- Proposed text changes

Grey represents the marked text to be removed

Green represents the marked text to be inserted

      - Derivation process for motion vector components and reference indices

Input to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xB, yB ) of the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

- luma motion vectors mvL0 and mvL1

- chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xP, yP ) specify the top-left sample location of the current luma prediction block relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

Let the variable currPic and ListX be the current picture and RefPicListX (with X being 0 or 1) of the current picture, respectively.

The function LongTermRefPic( picX, refIdx, LX ), with X being either 0 or 1, is defined as follows. If the picture with index refIdx from reference picture list LX of the picture picX was marked as "used for long term reference" at the time when picX was the current picture, LongTermRefPic( picX, refIdx, LX ) is equal to 1; otherwise LongTermRefPic( picX, refIdx, LX ) is equal to 0.

The variables vspModeFlag, ivpMvFlagL0 and ivpMvFlagL1 are set equal to 0.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as prefFlagL0 and prefFlagL1, the following applies.

- If PredMode[ xC ][ yC ] is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPbW, nPbH and the partition index partIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the flag vspModeFlag.  the following applies.

    - The derivation process for luma motion vectors for merge mode as specified in subclause 8.5.2.1.1 is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPbW, nPbH and the partition index partIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, the additional output parameters being the merge candidate lists mergeCandList, the availability flags availableFlagN of the neighbouring prediction units (with N being replaced by A0, A1, B0, B1 or B2).

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPbW, nPbH, the partition index partIdx, the merge candidate lists mergeCandList, the availability flags availableFlagN of the neighbouring prediction units (with N being replaced by A0, A1, B0, B1 or B2) as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the flag vspModeFlag.

- Otherwise, if PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with the luma location ( xC, yC ), luma location ( xP, yP ), variables nCS, nPbW and nPbH and the partition index partIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the flag vspModeFlag..the following applies.

    - The derivation process for luma motion vectors for merge mode as specified in subclause 8.5.2.1.1 is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPbW, nPbH and the partition index partIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, the additional output parameters being the merge candidate lists mergeCandList, the availability flags availableFlagN of the neighbouring prediction units (with N being replaced by A0, A1, B0, B1 or B2).

    - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 1, the derivation process for luma motion vectors for merge mode as specified in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with the luma location ( xC, yC ), the luma location ( xP, yP ), variables nCS, nPbW, nPbH, the partition index partIdx, the merge candidate lists mergeCandList, the availability flags availableFlagN of the neighbouring prediction units (with N being replaced by A0, A1, B0, B1 or B2) as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1, and the flag vspModeFlag.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX and in Pred_LX and in the syntax elements ref_idx_lX and MvdLX, the following applies.

    - The variables refIdxLX and predFlagLX are derived as follows.

    - If inter_pred_idc[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

          - refIdxLX = ref_idx_lX[ xP ][ yP ]		(Error: Reference source not found-1)
predFlagLX = 1		(Error: Reference source not found-2)

          - Otherwise, the variables refIdxLX and predFlagLX are specified by

          - refIdxLX = −1		(Error: Reference source not found-3)
predFlagLX = 0		(Error: Reference source not found-4)

    - The variable mvdLX is derived as follows.

        - mvdLX[ 0 ] = MvdLX[ xP ][ yP ][ 0 ]		(Error: Reference source not found-5)
mvdLX[ 1 ] = MvdLX[ xP ][ yP ][ 1 ]		(Error: Reference source not found-6)

    - When predFlagLX is equal to 1, the derivation process for luma motion vector prediction in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), variables nPbW and nPbH, refIdxLX, and the partition index partIdx as the inputs and the output being mvpLX.

    - When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

        - uLX[ 0 ] = (mvpLX[ 0 ] + mvdLX[ 0 ] + 216) % 216	(Error: Reference source not found-7)
mvLX[ 0 ] = ( uLX[ 0 ] >= 215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ]	(Error: Reference source not found-8)
uLX[ 1 ] = (mvpLX[ 1 ] + mvdLX[ 1 ] + 216) % 216	(Error: Reference source not found-9)
mvLX[ 1 ] = ( uLX[ 1 ] >= 215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]	(Error: Reference source not found-10)

NOTE 1 – The resulting values of mvLX[ 0 ] and mvLX[ 1 ] as specified above will always be in the range of −215 to 215 − 1.

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause 8.5.2.1.8 is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = xP.. ( xP + nPbW − 1 ), y = yP..( yP + nPbH− 1 ) (with X being either 0 or 1):

- IvpMvFlagLX[ x ][ y ] = ivpMvFlagLX		(Error: Reference source not found-11)
VspModeFlag[ x ][ y ] = vspModeFlag		(Error: Reference source not found-12)

[Ed.(GT): The complete derivation process for inter-view motion vectors candidates has the problem (in software as well as in this draft), that it is assumed that disparity vectors are related to the base view. However, in the draft and the software there are some loops looking for an arbitrary non-base view inter-view reference picture for inter-view motion vector prediction in the reference picture lists. This works for CTC, since other inter-view reference pictures apart from the base view are not allowed. However, to enable a more general approach, this should be discussed and possibly made more general in SW and text. (#5) ]

[Ed.(CY): To fix the above mentioned problem, the derived disparity vector should be associated with a reference view index.]

        - Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode[ xC ][ yC ] is equal to MODE_SKIP or PredMode[ xC ][ yC ] is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.,

- the merge candidate lists mergeCandList.

- the availability flags availableFlagN of the neighbouring prediction units are (with N being replaced by A0, A1, B0, B1 or B2).

Outputs of this process are

- the luma motion vectors mvL0 and mvL1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1,

- the flag vspModeFlag, specifying, whether the current PU is coded using view synthesis prediction,

The variables singleMCLFlag is derived as follows.

- If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

- Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPbW and nPbH are set equal to nCS.

NOTE 2  – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1, the disparity vector availability flags ivpMvFlagL0 and ivpMvFlagL1 and the flag vspModeFlag are derived as specified by the following ordered steps:

  - The derivation process for spatial merge candidates as specified in subclause G.H.8.5.2.1.2 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable singleMCLFlag, the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2The reference index for temporal merging candidate refIdxLX (with X being 0 or 1) is set equal to 0.

  - The derivation process for temporal luma motion vector prediction in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, refIdxLX, and mergeFlag being equal to 1 as the inputs and with the output being the availability flag availableFlagLXCol, the temporal motion vector mvLXCol, and the reference index refIdxLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

      - availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(Error: Reference source not found-13) 
predFlagLXCol = availableFlagLXCol		(Error: Reference source not found-14)

  - Depending on iv_mv_pred_flag[ nuh_layer_id ], the following applies.

  - If iv_mv_pred_flag[ nuh_layer_id ] is equal to 0, the flags availableFlagIvMC and availableFlagIvDC are set equal to 0.

  - Otherwise (iv_mv_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for the inter-view merge candidates as specified in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, as the inputs and the output is assigned to the availability flags availableFlagIvMC and availableFlagIvDC, the reference indices refIdxLXIvMC and refIdxLXIvDC, the prediction list utilization flags predFlagLXIvMC and predFlagLXIvDC, and the motion vectors mvLXIvMC and mvLXIvDC (with X being 0 or 1, respectively)..

  - Depending on view_synthesis_pred_flag[ nuh_layer_id ], the following applies.

  - If view_synthesis_pred_flag[ nuh_layer_id ] is equal to 0, the flag availableFlagVSP is set equal to 0.

  - Otherwise (view_synthesis_pred_flag[ nuh_layer_id ] is equal to 1), the derivation process for a view synthesis prediction merge candidate as specified in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with the luma locations ( xC, yC ) as input and the outputs are the availability flag availableFlagVSP, the reference indices refIdxL0VSP and refIdxL1VSP, the prediction list utilization flags predFlagL0VSP and predFlagL1VSP, and the motion vectors mvL0VSP and mvL1VSP.

  - Depending on DepthFlag, the following applies.

  - If DepthFlag is equal to 0, the variable availableFlagT is set equal to 0.

  - Otherwise ( DepthFlag is equal to 1), the derivation process for the texture merging candidate as specified in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH as the inputs and the outputs are the flag availableFlagT, the prediction utilization flags predFlagL0T and predFlagL1T, the reference indices refIdxL0T and refIdxL1T, and the motion vectors mvL0T and mvL1T.

  - The merge candidate lists mergeCandList and mergeCandIsVspFlag are constructed as specified by the following ordered steps:

  - The variable numMergeCand is set equal to 0.

  - The variable numMergeCandBase, numMergeCand3DV and  numA1B1B0 are set equal to 0.

  - When availableFlagA1 is equal to 1 and VspModeFlag[ xP − 1 ][ yP + nPbH − 1 ] is equal to 1, the entry mergeCandIsVspFlag[ numA1B1B0 ] is set equal to VspModeFlag[ xP − 1 ][ yP + nPbH − 1 ] and the variable numA1B1B0 is increased by 1.

  - When the following condition is true,

  - availableFlagT = = 0 && availableFlagIvMC  = = 0,

        - or one or more of the following conditions are true, with N being replaced by T and IvMC:

        - availableFlagN = = 1 && predFlagLXN ! = predFlagLXA1, (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && mvLXN ! = mvLXA1 (with X being replaced by 0 and 1),

        - availableFlagN = = 1 && refIdxLXN ! = refIdxLXA1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to A1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP − 1 ][ yP + nPbH − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB1 is equal to 1 and VspModeFlag[ xP + nPbW − 1 ][ yP − 1 ] is equal to 1, the entry mergeCandIsVspFlag[ numA1B1B0 ] is set equal to VspModeFlag[ xP + nPbW − 1 ][ yP − 1 ] and the variable numA1B1B0 is increased by 1.

  - When the following condition is true ,

  - availableFlagT = = 0 && availableFlagIvMC = = 0,

or one or more of the following conditions is true, with N being replaced by T and IvMC:

- availableFlagN = = 1 && predFlagLXN ! = predFlagLXB1, (with X being replaced by 0 and 1),

- availableFlagN = = 1 && mvLXN ! = mvLXB1 (with X being replaced by 0 and 1),

- availableFlagN = = 1 && refIdxLXN ! = refIdxLXB1 (with X being replaced by 0 and 1),

the entry mergeCandList[ numMergeCand ] is set equal to B1, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP + nPbW − 1 ][ yP − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagB0 is equal to 1 and VspModeFlag[ xP + nPbW ][ yP − 1 ] is equal to 1, the entry mergeCandIsVspFlag[ numA1B1B0 ] is set equal to VspModeFlag[ xP + nPbW ][ yP − 1 ] and the variable numA1B1B0 is increased by 1. the entry mergeCandList[ numMergeCand ] is set equal to B0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP + nPbW ][ yP − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1 and VspModeFlag[ xP − 1 ][ yP + nPbH ] is equal to 1, the entry mergeCandList[ numMergeCandBase + numA1B1B0 ] is set equal to A0, the entry mergeCandIsVspFlag[ numMergeCandBase + numA1B1B0 ] is set equal to VspModeFlag[ xP − 1 ][ yP + nPbH ] and the variable numMergeCandBase is increased by 1..

  - When availableFlagB2 is equal to 1 and VspModeFlag[ xP − 1 ][ yP − 1 ] is equal to 1, the entry mergeCandList[ numMergeCandBase + numA1B1B0 ] is set equal to B2, the entry mergeCandIsVspFlag[ numMergeCandBase + numA1B1B0 ] is set equal to VspModeFlag[ xP − 1 ][ yP − 1 ] and the variable numMergeCandBase is increased by 1.

  - The variable numMergeCandBase is set equal to 5.

  - When availableFlagT is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to T, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagT is equal to 1, the following applies:

  - The variable idxPrune is set equal to numMergeCandBase − 1. The variable bPrune is set to FALSE and the following steps are repeated until bPrune is equal to TRUE or idxPrune is greater than or equal to 0.

  - The following assignments are made with N being the candidate at position idxPrune in the merging candidate list mergeCandList ( N = mergeCandList[idxPrune] ) and X being replaced by 0 or 1:

- If mvLXN[ 0 ] is equal to the mvLXT[0] and mvLXN[ 1 ] is equal to the mvLXT[1], bPrune is set equal to TRUE.

- Otherwise, idxPrune is set to idxPrune.

- If bPrune is TRUE,  the following applies:

- k ranges from idxPrune + 1 to 1, inclusive, and mergeCandList[ k ] is set equal to mergeCandList[ k – 1 ], the variable numMergeCandBase is decreased by 1.

- If idxPrune < numA1B1B0, the variable numA1B1B0 is set to numA1B1B0 −  1.

- Otherwise, k ranges from numMergeCandBase to 1, inclusive, and mergeCandList[ k ] is set equal to mergeCandList[ k – 1 ].

- The entry mergeCandList[ numMergeCand ] is set equal to T, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0, the variable numMergeCand3DV is increased by 1 and the variable numMergeCand is increased by 1.

  - When availableFlagIvMC is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to IvMC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When availableFlagIvMC is equal to 1, the following applies:

  - The variable idxPrune is set equal to numMergeCandBase − 1. The variable bPrune is set to FALSE and the following steps are repeated until bPrune is equal to TRUE or idxPrune is greater than or equal to 0.

  - The following assignments are made with N being the candidate at position idxPrune in the merging candidate list mergeCandList ( N = mergeCandList[idxPrune] ) and X being replaced by 0 or 1:

- If mvLXN[ 0 ] is equal to the mvLXIvMC [0] and mvLXIvMC [ 1 ] is equal to the mvLXT[1], bPrune is set equal to TRUE.

- Otherwise, idxPrune is set to idxPrune.

- If bPrune is TRUE, the following applies:

- k ranges from idxPrune + 1 to 1, inclusive, and mergeCandList[ k ] is set equal to mergeCandList[ k – 1 ], the variable numMergeCandBase is decreased by 1.

- If idxPrune < numA1B1B0, the variable numA1B1B0 is set to numA1B1B0 −  1.

- Otherwise, k ranges from numMergeCandBase to 1, inclusive, and mergeCandList[ k ] is set equal to mergeCandList[ k – 1 ].

- the entry mergeCandList[ numMergeCand ] is set equal to IvMC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0, the variable numMergeCand3DV is increased by 1  and the variable numMergeCand is increased by 1.

  - The variable numMergeCand is set equal to numMergeCand + numA1B1B0.

  - When availableFlagIvDC is equal to 1, and one or more of the following conditions is true,

  - availableFlagA1 = = 0,

  - predFlagLXA1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

  - mvLXA1 ! = mvLXIvDC (with X being replaced by 0 and 1),

  - refIdxLXA1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

and one or more of the following conditions is true,

- availableFlagB1 = = 0,

- predFlagLXB1 ! = predFlagLXIvDC, (with X being replaced by 0 and 1),

- mvLXB1 ! = mvLXIvDC (with X being replaced by 0 and 1),

- refIdxLXB1 ! = refIdxLXIvDC (with X being replaced by 0 and 1),

k ranges from MaxNumMergeCand −  numMergeCand to numMergeCand + 1, inclusive, and mergeCandList[ k ] is set equal to mergeCandList[ k – 1 ].

the entry mergeCandList[ numMergeCand ] is set equal to IvDC, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0, the variable numMergeCand3DV is increased by 1 and the variable numMergeCand is increased by 1.

  - When availableFlagVSP is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to VSP, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal 1 and the variable numMergeCand is increased by 1.

  - When availableFlagA0 is equal to 1, the entry mergeCandList[ numMergeCand ] is set equal to A0, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP − 1 ][ yP + nPbH ] and the variable numMergeCand is increased by 1..

  - When availableFlagB2 is equal to 1 and numMergeCand is less than 4 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to B2, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to VspModeFlag[ xP − 1 ][ yP − 1 ] and the variable numMergeCand is increased by 1.

  - When availableFlagCol is equal to 1 and numMergeCand is less than 5 + iv_mv_pred_flag[ nuh_layer_id ] + DepthFlag, the entry mergeCandList[ numMergeCand ] is set equal to Col, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal to 0 and the variable numMergeCand is increased by 1.

  - When all of the following conditions is true,

  - numMergeCand is lesser then MaxNumMergeCand.

  - availableFlagVSP is equal to 1.

- k ranges from MaxNumMergeCand −  numMergeCand to numMergeCand + 1, inclusive, and mergeCandList[ k ] is set equal to mergeCandList[ k – 1 ].

- The entry mergeCandList[ numMergeCand ] is set equal to VSP, the entry mergeCandIsVspFlag[ numMergeCand ] is set equal 1, the variable numMergeCand3DV is increased by 1 and the variable numMergeCand is increased by 1.

  - The variable numMergeCand is set to (numMergeCandBase + numMergeCand3DV > MaxNumMergeCand ? MaxNumMergeCand : numMergeCandBase + numMergeCand3DV).

  - The derivation process for zero motion vector merging candidates specified in subclause 8.5.2.1.4 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive, and mergeCandIsVspFlag[ numOrigMergeCand + numCombMergeCand + m ] is set equal to 0.

  - The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

      - mvLX[ 0 ] = mvLXN[ 0 ]		(Error: Reference source not found-15)

      - mvLX[ 1 ] = mvLXN[ 1 ]		(Error: Reference source not found-16)

      - refIdxLX = refIdxLXN		(Error: Reference source not found-17)

      - predFlagLX = predFlagLXN		(Error: Reference source not found-18)

  - When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nPbW + nPbH ) is equal to 12, the following applies.

      - refIdxL1 = −1		(Error: Reference source not found-19)

      - predFlagL1 = 0		(Error: Reference source not found-20)

  - The variable vspModeFlag is set equal to mergeCandIsVspFlag[ merge_idx[ xP][ yP ] ]..

  - When vspModeFlag is equal to 1, the following applies:

  - If RefPicList0[ 0 ] is a long-term reference picture, refIdxL0 is set equal to 0.

  - Otherwise ( RefPicList0[ 0 ] is a short-term reference picture ), refIdxL0 is set equal to AltRefIdxL0.

  - The disparity availability flag ivpMvFlagLX is derived as follows (with X being replace by 0 or 1).

  - If all of the following conditions are true, ivpMvFlagLX is set equal to 1

- [Ed. (GT) There is some redundancy in draft and software since disparities equal for both lists.(#7) ]

- availableFlagIvMC = = 1

- merge_idx[ xP][ yP ] = = 0

- predFlagLXIvMC = = 1

[Ed. (GT): PredMode[ xC ][ yC ] = = MODE_SKIP might be added here instead of testing it in the disparity vector derivation process]

- Otherwise, ivpMvFlagLX is set equal to 0.

        - Derivation process for spatial merge candidates

The specifications in subclause 8.5.2.1.2 apply.

Inputs to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable singleMCLFlag,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP − 1,  yP + nPbH ), ( xP − 1,  yP + nPbH − 1 ), ( xP + nPbW,  yP − 1 ), ( xP + nPbW − 1,  yP − 1 ) or ( xP − 1,  yP − 1 ), the following applies.

–	When yP−1 is less than (( yC >> Log2CtbSizeY ) << Log2CtbSizeY), the following applies.

xB0 = ((xB0>>3)<<3) + ((xB0>>3)&1)*7		(8-21)
xB1 = ((xB1>>3)<<3) + ((xB1>>3)&1)*7		(8-22)
xB2 = ((xB2>>3)<<3) + ((xB2>>3)&1)*7		(8-23)

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xN, yN ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableN.

–	If one or more of the following conditions are true with X being replaced by 0 and 1, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN is set equal to −1 and predFlagLXN is set equal to 0.

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xN >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yN >> (log2_parallel_merge_level_minus2 + 2)).

- N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

- availableN is equal to FALSE

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and partIdx is equal to 1 and N is equal to B1

- singleMCLFlag is equal to 0 and PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and partIdx is equal to 1 and N is equal to A1

- N is equal to B1 and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to B0 and the prediction units covering luma location ( xB1, yB1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to A0 and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

- N is equal to B2 and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xN, yN ) have the same motion vectors and the same reference indices

–	Otherwise, availableFlagN is set equal to 1 and the following assignments are made.

mvLXN = MvLX[ xN ][ yN ]		(8-24)
refIdxLXN = RefIdxLX[ xN][ yN ]		(8-25)
predFlagLXN = PredFlagLX[ xN ][ yN ]		(8-26)

        - Derivation process for combined bi-predictive merging candidates

The specifications in subclause 8.5.2.1.3 apply.

Inputs of this process are

- a merging candidate list mergeCandList,

- a list mergeCandIsVspFlag,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0combCandk and refIdxL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

When numOrigMergeCand is greater than 1 and less than MaxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variable combStop is set to FALSE and the following steps are repeated until combStop is equal to TRUE.

  - The variables l0CandIdx and l1CandIdx are derived using combIdx as specified in Table 8-6.

  - The following assignments are made with l0Cand being the candidate at position l0CandIdx and l1Cand being the candidate at position l1CandIdx in the merging candidate list mergeCandList ( l0Cand = mergeCandList[ l0CandIdx ], l1Cand = mergeCandList[ l1CandIdx ] ).

  - When all of the following conditions are true,

  - mergeCandIsVspFlag[ l0CandIdx ] = = 0,

  - mergeCandIsVspFlag[ l1CandIdx ] = = 0,

  - predFlagL0l0Cand = = 1,

  - predFlagL1l1Cand = = 1,

  - PicOrderCnt( RefPicList0[ refIdxL0l0Cand ] ) != PicOrderCnt( RefPicList1[ refIdxL1l1Cand ] ) | | mvL0l0Cand != mvL1l1Cand,

the candidate combCandk with k equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = combCandk ) and the reference indices, the prediction list utilization flags and the motion vectors of combCandk are derived as follows and numMergeCand is incremented by 1.

      - refIdxL0combCandk = refIdxL0l0Cand		(Error: Reference source not found-27)

      - refIdxL1combCandk = refIdxL1l1Cand		(Error: Reference source not found-28)

      - predFlagL0combCandk = 1		(Error: Reference source not found-29)

      - predFlagL1combCandk = 1		(Error: Reference source not found-30)

      - mvL0combCandk[ 0 ] = mvL0l0Cand[ 0 ]		(Error: Reference source not found-31)

      - mvL0combCandk[ 1 ] = mvL0l0Cand[ 1 ]		(Error: Reference source not found-32)

      - mvL1combCandk[ 0 ] = mvL1l1Cand[ 0 ]		(Error: Reference source not found-33)

      - mvL1combCandk[ 1 ] = mvL1l1Cand[ 1 ]		(Error: Reference source not found-34)

      - numMergeCand = numMergeCand + 1		(Error: Reference source not found-35)

  - The variable combIdx is incremented by 1.

  - When combIdx is equal to ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numMergeCand is equal to MaxNumMergeCand, combStop is set to TRUE.

        - Derivation process for zero motion vector merging candidates

The specifications in subclause 8.5.2.1.4 apply.

        - Derivation process for luma motion vector prediction

The specifications in subclause 8.5.2.1.5 apply.

        - Derivation process for motion vector predictor candidates

The specifications in subclause 8.5.2.1.6 apply.

        - Derivation process for temporal luma motion vector prediction

The specifications in subclause 8.5.2.1.7 apply.

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a reference index refIdxLX (with X being 0 or 1),

- a flag mergeFlag specifying whether a merge candidate is derived.

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol,

- the reference index refIdxLX.

The function ViewId( picX ) is specified as follows:

- ViewId( picX ) = view_id of the picture picX		(Error: Reference source not found-36)

The function RefViewId( picX, refIdx, LX ) returns the view_id value of the reference picture with index refIdx from reference picture list LX of the picture picX and is specified as follows.

- RefViewId( picX, refIdx, LX ) = ViewId( RefPicListX[ refIdx ] of the picture picX)	(Error: Reference source not found-37)

- [Ed. (GT). The functions specified above might be specified in the scope view_id is parsed ].

The function RefPicOrderCnt( picX, refIdx, LX ) returns the picture order count PicOrderCntVal of the reference picture with index refIdx from reference picture list LX of the picture picX and is specified as follows.

- RefPicOrderCnt( picX, refIdx, LX ) = PicOrderCnt(RefPicListX[ refIdx ] of the picture picX)	(Error: Reference source not found-38)

Depending on the values of slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the picture that contains the collocated partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList1[ collocated_ref_idx ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), the variable colPic specifies the picture that contains the collocated partition as specified by RefPicList0[ collocated_ref_idx ].

Variable colPb and its position ( xPCol, yPCol ) are derived in the following ordered steps:

  - The variable colPb is derived as follows

      - yPRb = yP + nPbH		(Error: Reference source not found-39)

      - If ( yP >> Log2CtbSizeY ) is equal to ( yPRb >> Log2CtbSizeY ), the horizontal component of the right-bottom luma location of the current luma prediction block is defined by

        - xPRb = xP + nPbW		(Error: Reference source not found-40)

- and the variable colPb is set as the luma prediction block covering the modified location given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- Otherwise ( ( yP >> Log2CtbSizeY ) is not equal to ( yPRb >> Log2CtbSizeY ) ), colPb is marked as "unavailable".

  - When colPb is coded in an intra prediction mode or colPb is marked as "unavailable", the following applies.

  - Central luma location of the current prediction block is defined by

        - xPCtr = ( xP + ( nPbW >> 1 ) )		(Error: Reference source not found-41)

        - yPCtr = ( yP + ( nPbH >> 1 ) )		(Error: Reference source not found-42)

        - The variable colPb is set as the luma prediction block covering the modified location given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

  - ( xPCol, yPCol ) is set equal to the top-left sample of the colPb relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If one or more of the following conditions are true, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- colPb is coded in an intra prediction mode.

- colPb is marked as "unavailable".

- slice_temporal_mvp_enable_flag is equal to 0.

- Otherwise, the following ordered steps apply.

- The motion vector mvCol, the reference index refIdxCol, and the reference list identifier listCol are derived as follows.

- If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxL1[ xPCol ][ yPCol ], and L1, respectively. 
[Ed. (GT): Issue in base spec (JCTVC-J1003_d7: PredFlagL0,MvL1 and RefIdxL1 of which picture?]

- Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

- If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxL0[ xPCol ][ yPCol ], and L0, respectively.

- Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

- If PicOrderCnt( pic ) of every picture pic in every reference picture lists is less than or equal to PicOrderCntVal, mvCol, refIdxCol, and listCol are set equal to MvLX[ xPCol ][ yPCol ], RefIdxLX[ xPCol ][ yPCol ] and LX, respectively with X being the value of X this process is invoked for.

- Otherwise (PicOrderCnt( pic ) of at least one picture pic in at least one reference picture list is greater than PicOrderCntVal, mvCol, refIdxCol and listCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxLN[ xPCol ][ yPCol ] and LN, respectively with N being the value of collocated_from_l0_flag.

- The variables curIvFlag and colIvFlag, specifying whether inter-view prediction is utilized for the current and collocated PU are derived as:

        - curIvFlag = RefPicListX[ refIdxLX ] is a long term reference picture ? 1 : 0	(Error: Reference source not found-43)

        - colIvFlag = LongTermRefPic( colPic, refIdxCol, listCol ) 	(Error: Reference source not found-44)

        - When curIvFlag is not equal to colIvFlag, mergeFlag is equal to 1 and AltRefIdxLX is not equal to −1, the variables refIdxCol and colIvFlag are modified as follows:

        - refIdxLX = AltRefIdxLX		(Error: Reference source not found-45)

- curIvFlag = RefPicListX[ refIdxLX ] is a long term reference picture ? 1 : 0	(Error: Reference source not found-46)

- The motion vector mvLXCol is modified as follows.

- If curIvFlag is not equal to colIvFlag, availableFlagLXCol is set equal to 0.

- Otherwise, ( curIvFlag is equal to colIvFlag), availableFlagLXCol is set equal to 1 and the following applies.

- The variables distCur and distCol specifying a POC or ViewId difference are derived as follows.

- If curIvFlag is equal to 0 or ( ( ViewIdx != 0 ) && iv_mv_scaling_flag ) is equal to 0, the following applies.

              - distCur = PicOrderCntVal − PicOrderCnt( RefPicListX[ refIdxLX ] )	(Error: Reference source not found-47)

              - distCol = PicOrderCnt( colPic ) − RefPicOrderCnt( colPic, refIdxCol, listCol )	(Error: Reference source not found-48)

              - Otherwise, ( curIvFlag is equal to 1 and ( ( ViewIdx != 0 ) && iv_mv_scaling_flag ) is equal to 1), the following applies.

              - distCur = view_id[ nuh_layer_id ] − ViewId( RefPicListX[ refIdxLX ] )	(Error: Reference source not found-49)

              - distCol = ViewId( colPic ) − ViewId( colPic, refIdxCol, listCol )	(Error: Reference source not found-50)

              - The motion vector mvLXCol is derived as follows:

              - If distCur is equal to distCol, mvLXCol is set equal to mvCol.

              - Otherwise, ( distCur is not equal to distCol ),

              - mvLXCol is derived as scaled version of the motion vector mvCol as specified in the following:

                - td = Clip3( −128, 127, distCol )	(Error: Reference source not found-51)

                - tb = Clip3( −128, 127, distCur )	(Error: Reference source not found-52)

                - tx = ( 16384 + ( Abs( td ) >>1 ) ) / td	(Error: Reference source not found-53)

                - distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(Error: Reference source not found-54)

                - mvLXCol =  Clip3( −32768, 32767, Sign2( distScaleFactor * mvCol ) *
( (Abs( distScaleFactor * mvCol ) + 127 ) >> 8 ) )	(Error: Reference source not found-55)

        - Derivation process for chroma motion vectors

The specifications in subclause 8.5.2.1.8 apply.

        - Derivation process for inter-view merge candidates

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags availableFlagIvMC and availableFlagIvDC specifying whether the inter-view merge candidates are available,

- the reference indices refIdxLXIvMC and refIdxLXIvDC,

- the prediction list utilization flags predFlagLXIvMC and predFlagLXIvDC,

- the motion vectors mvLXIvMC and mvLXIvDC,

The temporal inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

  - The derivation process for a temporal inter-view motion vector candidate as specified in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the prediction list indication X , the view order index RefViewIdx[ xP ][ yP ], the disparity vector MvRefinedDisp[ xP ][ yP ], the flag mergeFlag being equal to 1, and the reference index refIdxLX being equal to −1, and as the inputs and the outputs are the flag availableFlagLXIvMC, the motion vector mvLXIvMC and the reference index refIdxLXIvMC.

  - The availability flag availableFlagIvMC, and the prediction utilization flags predFlagL0IvMC and predFlagL1IvMC are derived by

      - availableFlagIvMC = availableFlagL0IvMC | | availableFlagL1IvMC	(Error: Reference source not found-56)
predFlagL0IvMC = availableFlagL0IvMC		(Error: Reference source not found-57)
predFlagL1IvMC = availableFlagL1IvMC		(Error: Reference source not found-58)

The disparity inter-view motion vector merging candidate is derived by the following ordered steps.

  - For the prediction list indication X being 0 and 1 the following applies.

  - The derivation process for the disparity inter-view motion vector candidate as specified in subclause 㐀*%54%㘁%9%࠷8%58%㨀%9%࠻㰀⨾䀀䔀H䠀*䡋䩏䩑⩓吀ࡘ夀(࡜崀䩞ࡵ需￈需￈ý需烈Ǻ좙ഀȀ騀㫈좛V鰀㛈좝鸀(梟梠⢡ꈀh଀䠀⸀⸀Ā⸀Ȁ⸀̀⸀Ѐ＀_xffff_ヿ꠿『ĀĀ̀ༀ斄ᄁ鮄廾斄态鮄濾(.㽠ຨ1

ဃ萏ˊ萑ﺛ葞ˊ葠ﺛ⡯ȀĀ⸀退꠿㈎ĀĀ̀ༀ⾄ᄄ鮄廾⾄怄鮄濾(.㿀ຨ3

ဃ萏֔萑ﺛ葞֔葠ﺛ⡯Ȁ̀⸀덀㐎ĀĀ̀ༀ濾ᄆ鮄廾濾怆鮄濾(.䀰ຳ5

ဃ萏࡞萑ﺛ葞࡞葠ﺛ⡯ȀԀ⸀退덀㘎ĀĀ̀ༀ쎄ᄉ鮄廾쎄怉鮄濾(.䃀ຳ7

ဃ萏ନ萑ﺛ葞ନ葠ﺛ⡯Ȁ܀⸀덀㠎ĀĀ̀ༀ趄ᄌ鮄廾趄怌鮄濾(._xffff__xffff_䄠ຳ0

ဃ萏ť萑ﺛ葞ť葠ﺛ⡯Ȁ⸀倀덁ㄎĀĀ̀ༀ쪄ᄂ鮄廾쪄怂鮄濾(
.䆀ຳ2

ဃ萏Я萑ﺛ葞Я葠ﺛ⡯ȀȀ⸀뀀덁㌎ĀĀ̀ༀ钄ᄅ鮄廾钄怅鮄濾(.䈐ຳ4

ဃ萏۹萑ﺛ葞۹葠ﺛ⡯ȀЀ⸀䀀덂㔎ĀĀ̀ༀ庄ᄈ鮄廾庄怈鮄濾(.䉰ຳ6

ဃ萏ৃ萑ﺛ葞ৃ葠ﺛ⡯Ȁ؀⸀덃㜎ĀĀ̀ༀ⢄ᄋ鮄廾⢄怋鮄濾(.䌰ຳ8

ဃ萏಍萑ﺛ葞಍葠ﺛ⡯Ȁࠀ⸀＀_xffff__xffff__xffff__xffff_⋿܀圀圀㠀一甀洀㌀ࠀ圀圀㠀一甀洀㈀㄀ࠀ圀圀㠀一甀洀㄀㄀ࠀ圀圀㠀一甀洀㄀㌀＀⋿ሀ刀胾̚ऀԄऀĄऀ̄ऀԄऀĄऀ̄ऀԄऀሄ먀ϛ܀Ԅ܀Ą܀̄܀Ԅ܀Ą܀̄܀Ԅ܀ሄ ᪋㚰옐䂽 is invoked with the luma location ( xP, yP ), the variables nPSW and nPSH, the view order index RefViewIdx[ xP ][ yP ], the disparity vector MvRefinedDisp[ xP ][ yP ], the prediction list indication X, the flag mergeFlag being equal to 1, and the reference index refIdxLX being equal to −1, as the inputs and the outputs are the flag availableFlagLXIvDC, the motion vector mvLXIvDC, and the reference index refIdxLXIvDC.

  - The availability flag availableFlagIvDC, and the prediction utilization flags predFlagL0IvDC and predFlagL1IvDC are derived by

      - availableFlagIvDC = availableFlagL0IvDC | | availableFlagL1IvDC 	(Error: Reference source not found-59)
predFlagL0IvDC = availableFlagL0IvDC		(Error: Reference source not found-60)
predFlagL1IvDC = availableFlagL1IvDC		(Error: Reference source not found-61)

        - Derivation process for a temporal inter-view motion vector candidate

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a prediction list indication X,

- a reference view index refViewIdx.

- a disparity vector mvDisp,

- a flag mergeFlag specifying whether a merge candidate is derived,

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX.

Outputs of this process are:

- a flag availableFlagLXInterView specifying whether the temporal inter-view motion vector candidate is available,

- a temporal inter-view motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1),

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX,

The flag availableFlagLXInterView is set equal to 0, both components of mvLXInterView are set equal to 0.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

The reference layer luma location ( xRef, yRef ) is derived by

- xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 )  >>  1 ) + ( ( mvDisp[ 0 ] + 2 )  >>  2 ) )	(Error: Reference source not found-62)
yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 )  >>  1 ) + ( ( mvDisp[ 1 ] + 2 )  >>  2 ) )	(Error: Reference source not found-63)

Let refCU be the coding unit that covers the luma location ( xRef, yRef ) in the view component with ViewIdx equal to refViewIdx.

[Ed. (GT:) In software refCU is the coding unit that covers the luma location ( xRef, yRef ) in the view component with smallest ViewIdx and that is included in one of the reference lists of the current view component.(#6)]

[Ed. (CY): What is implemented in the software has no impact on the final results in common test condition, so it is preferred to have the current text and software changes may be desirable. ]

When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply, for Y in the range of X to (1 – X), inclusive:

  - The variable refPredFlagLY is set equal to the prediction utilization flag predFlagLY of the prediction unit refPU.

  - The variable refRefIdxLY, is set equal to the reference index refIdxLY of  the prediction unit refPU.

  - The variable refMvLY is set equal to the motion vector mvLY of the prediction unit refPU.

  - The variable refRefPicListLY, is set equal to the reference picture list RefPicListLY of the prediction unit refCU.

  - When refPredFlagLY is equal to 1, the following applies for each i from ( mergeFlag ? 0 : refIdxLX ) to ( mergeFlag ? num_ref_idx_lX_active_minus1 : refIdxLX), inclusive:

  - When availableFlagLXInterView is equal to 0, and the picture order count of the picture refRefPicListLY[ refRefIdxLY ] is equal to the picture order count of the picture RefPicListLX[ i ], the flag availableFlagLXInterView is set equal to 1 and the following applies.

  - The motion vector mvLXInterView is derived by:

        - mvLXInterView[ 0 ] = refMvLY[ 0 ]		(Error: Reference source not found-64)
mvLXInterView[ 1 ] = refMvLY[ 1 ]		(Error: Reference source not found-65)

        - When mergeFlag is equal to 1, the reference index refIdxLX is derived by:

        - refIdxLX = i		(Error: Reference source not found-66)

        - Derivation process for a disparity inter-view motion vector candidate

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a prediction list indication X,

- a  reference view index refViewIdx,

- a disparity vector mvDisp,

- a flag mergeFlag specifying whether a merge candidate is derived,

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX.

Outputs of this process are:

- a flag availableFlagLXInterView specifying whether the disparity inter-view motion vector candidate is available,

- a disparity inter-view motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1),

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX.

[GT (Ed.) This derivation process might be changed to use refViewIdx to restrict the reference view to be the base view or the view mvDisp is related to. ]

The flag availableFlagLXInterView is set equal to 0, both components of mvLXInterView are set equal to 0.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

For each i from ( mergeFlag ? 0 : refIdxLX ) to ( mergeFlag ? num_ref_idx_lX_active_minus1 : refIdxLX), inclusive, the following applies:

- When PicOrderCntVal of the picture RefPicListX[ i ] is equal to the PicOrderCntVal or the current picture and availableFlagLXInterView is equal to 0, availableFlagLXInterView is set equal to 1 and the following applies:

- The motion vector mvLXInterView is derived by

        - mvLXInterView[ 0 ] = mvDisp[ 0 ]		(Error: Reference source not found-67)
mvLXInterView[ 1 ] = 0		(Error: Reference source not found-68)

        - When mergeFlag is equal to 1, the reference index refIdxLX is derived by

        - refIdxLX = i		(Error: Reference source not found-69)

        - Derivation process for a view synthesis prediction merge candidate

Inputs to this process are:

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

Outputs of this process are

- the availability flag availableFlagVSP whether the VSP merge candidate is available,

- the reference indices refIdxL0VSP and refIdxL1VSP ,

- the prediction list utilization flags predFlagL0VSP and predFlagL1VSP,

- the motion vectors mvL0VSP and mvL1VSP.

- The variable availableFlagVSP is set equal to 1, the variables predFlagL0VSP and predFlagL1VSP are set equal to 0, the variables refIdxL0VSP and refIdxL1VSP are set equal to −1 and the variable refViewAvailableFlag is set equal to 0.

- For X in the range of 0 to 1, inclusive, the following applies:

- For i in the range of 0 to NumRefPicsLX – 1, inclusive, the following applies:

- When refViewAvailableFlag is equal to 0 and ViewIdx( RefPicListX[ i ] ) is equal to RefViewIdx[ xC ][ yC ], the following applies:

        - refViewAvailableFlag = 1		(Error: Reference source not found-70)
predFlagLXVSP = 1		(Error: Reference source not found-71)
mvLXVSP = MvDisp[ xC ][ yC ]		(Error: Reference source not found-72)
refIdxLXVSP = i		(Error: Reference source not found-73)
Y = 1 – X		(Error: Reference source not found-74)

When the current slice is a B slice and refViewAvailableFlag is equal to 1, refViewAvailableFlag is set equal to 0 and the following applies:

- For i in the range of 0 to NumRefPicsLY – 1, inclusive, the following applies.

- When refViewAvailableFlag is equal to 0 and ViewIdx( RefPicListY[ i ] ) is not equal to RefViewIdx[ xC ][ yC ] and ViewIdx( RefPicListY[ i ] ) is not equal to ViewIdx, the following applies:

        - refViewAvailableFlag = 1		(Error: Reference source not found-75)
predFlagLYVSP = 1		(Error: Reference source not found-76)
mvLYVSP = MvDisp[ xC ][ yC ]		(Error: Reference source not found-77)
refIdxLYVSP = i		(Error: Reference source not found-78)

[ Ed (GT): Mismatch to software. There the following applies:

predFlagL0 is set equal to 1, predFlagL1 is set equal to 0, refIdxLX is set equal to ( PicOrderCnt( RefPicListY[ i ] ) = = PicOrderCnt ) ? 0 : AltRefIdxLX for both lists.

This matches only for CTC with draft. ]

        - Derivation process for the texture merging candidate

This process is not invoked when DepthFlag is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

Outputs of this process are:

- a flag availableFlagT specifying whether the texture merging candidate is available,

- the prediction utilization flags predFlagL0T and predFlagL1T,

- the reference indices refIdxL0T and refIdxL1T (when availableFlagT is equal to 1),

- the motion vectors mvL0T and mvL1T (when  availableFlagT is equal to 1).

The variable availableFlagT is set equal to 0. The variables predFlagL0T and predFlagL1T are set equal to 0. The variables refIdxL0T and refIdxL1T are set equal to −1. Both components of the motion vectors mvL0T and mvL1T are set equal to 0.

The texture luma location ( xRef, yRef ) is derived by:

- xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 ) )	(Error: Reference source not found-79)

- yRef = Clip3( 0, PicHeightInSamplesL – 1, yP + ( ( nPSH – 1 ) >> 1 ) )	(Error: Reference source not found-80)

- [ Ed. (GT): Is clipping necessary? ]

Let textPic be the picture with PicOrderCntVal and ViewIdx equal to PicOrderCnt and ViewIdx of the current picture and DepthFlag being equal to 0 and let textPU the PU at covering the position ( xRef, yRef ) in textPic.

For X in the range of 0 to 1, inclusive, the following applies:

[Ed. (GT): It should be verified that the picture with refIdxLX is also a reference picture for depth. (#10)]

  - The variable textPredFlagLX is set equal to PredFlagLX of textPic. The variable textRefIdxLX is set equal to RefIdxLX of textPic. The variable textMvLX is set equal to the MvLX of textPic.

  - When X is equal to 0 or the current slice is a B slice, the following applies:

  - When textPredFlagLX[ xRef ][ yRef ] is equal to 1, the following applies:

      - mvLXT[ 0 ] = ( textMvLX[ xRef ][ yRef ][ 0 ] + 2 ) >> 2	(Error: Reference source not found-81)

      - mvLXT[ 1 ] = ( textMvLX[ xRef ][ yRef ][ 1 ] + 2 ) >> 2	(Error: Reference source not found-82)

      - refIdxLX = textRefIdxLX[ xRef ][ yRef ]		(Error: Reference source not found-83)

      - predFlagLXT = 1		(Error: Reference source not found-84)

      - availableFlagT = 1		(Error: Reference source not found-85)