| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0213 |
| --- | --- |

| Title: | 3D-CE3.h related: Simple merge candidate list construction for 3DV |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gun Bang, *Young Su Heo, *Kyung Yong Kim, *Gwang Hoon Park, Gwang Soon Lee, Namho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-6791 {gbang, gslee, namho} @etri.re.kr +82-31-201-3680 {* medist, * kimky, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution proposes a simplified method for constructing MCL (Merge Candidate List) for dependent view or depth. Proposed method is a simple and low-complexity way to implement the MCL construction module for the 3D-HEVC by re-using the existing MCL construction module of the HEVC. Simulation results show that the proposed algorithm only brings marginally-small loss on the coding performance while providing the MCL construction process for 3D-HEVC much simpler and also decreasing hardware implementation complexity significantly.

# Introduction

In the coding system of current 3D-HEVC, the independent base-view for the texture is encoded using the base encoder, e.g., original HEVC module. And the depth map and the dependent-view for the texture are encoded using the 3DV enhancement encoder, e.g., 3D-HEVC module, as shown in Fig. 1.

Fig. 1. Coding system of the current HTM (3D-HEVC)

The 3D-HEVC module has been integrated with many tools that can offer additional benefits for high coding-efficiency on the 3D videos, without considering H/W implementation comparability and constraints on the existing HEVC module. Therefore, on the other hand, the 3D-HEVC module has been more complex than the original HEVC module. It significantly increases the computational complexity and H/W implementation complexity. This design may be un-suitable in terms of H/W implementation, since the chip for 3D-HEVC module should be re-designed and re-implemented.

For example, there exist two kinds of merging candidate list (MCL) construction process, as shown in Fig. 2. One is the MCL construction process for the independent (base) texture view. The other is the MCL construction process for the dependent views and depth map. The MCL construction process for the dependent views and depth map is more complexly-combined than the MCL construction process for the independent texture view.

In the MCL construction process for the current 3D-HEVC, new merging candidates with high complexity for 3D-videos have been added in the MCL process of HEVC. And there exists the dependencies between merging candidate, e.g., combined-bi merging candidates using VSP information of the neighboring blocks.

Fig. 2. Current MCL construction process for the current 3D-HEVC

Fig. 3. Current MCL construction process in 3D-HEVC

Figure 3 shows current MCL construction process in the 3D-HEVC.

For the dependent views, the inserting processes for extra candidates (green-squared) are added and also the traditional processes (yellow-squared) in HEVC are changed. Therefore, redundant implementation should be carried out for traditional candidates in 3D-HEVC. This design may be un-suitable for implementation of the added and combined MCL process for 3D-HEVC in terms of H/W implementation, since the chip for 3D-HEVC module should be re-designed and re-implemented.

# Proposed Method

We propose a low-complexity way to implement the MCL construction process for the 3D-HEVC by re-using existing MCL construction module in the HEVC, as shown in Fig. 4. This design is suitable for implementing the new MCL processes, since the existing MCL construction module in the HEVC can be re-used.

Fig. 4 Proposed MCL construction process

For dependent views, the MCL construction process in HEVC (shown in Fig. 5) is used without any changes. Then as shown in Fig. 6, the additional MCL construction processes are carried out with the MCL1 and AvailableFlagN as inputs.

Fig. 5. MCL construction process in HEVC

Fig. 6. Proposed inserting process for extra candidates for dependent view

Following is a description of the inserting process for extra candidates for dependent views.

- Input/Output: The inputs are MCL1 which is the output from MCL construction process in HEVC, AvailableFlagN (with N being replaced by A0, A1, B0, B1 or B2) [1]. The output MCL2 is a complete MCL.

- N VSP: Set VSP flag of N in the MCL by checking VSP mode flag of N and AvailableFlagN (with N being replaced by A0, A1, B0, B1 or B2.).

- T: Shift right all valid candidates and insert the Texture candidate to the first space of the MCL.

- IvMC: Shift right all valid candidates and insert the Temporal Inter-view candidate to the first space of the MCL.

- IvDC: Shift right ranges from the A0 to the last valid candidate and insert the Disparity Inter-view candidate to after the B0 candidate.

- VSP: Shift right and ranges from the A0 to the last valid candidate and insert the VSP candidate to after the IVDC candidate.

- Zero: If the MCL is not full, fill empty spaces with default zero mv.

# Simulation Results

Common test conditions specified in JCT3V-D1100 [2] is used for the evaluation. TABLE 1 shows the simulation results of the proposed method. For the MCL construction process in HEVC, the process in HM 10.0rc1 was used. Therefore, the simulation results of the ‘video 0’ are slightly different from the anchor.

TABLE 1. Simulation results of the proposed method

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.03% | 0.62% | 0.13% | 0.21% | 0.19% | 0.14% |  | 99.9% | 100.0% |
| Kendo | 0.03% | 1.00% | 0.37% | 0.36% | 0.40% | 0.47% |  | 102.1% | 100.0% |
| Newspaper_CC | 0.04% | 0.32% | 0.08% | 0.14% | 0.10% | -0.12% |  | 96.9% | 100.0% |
| GT_Fly | -0.08% | 0.47% | 0.03% | 0.07% | 0.12% | 0.14% |  | 100.5% | 100.0% |
| Poznan_Hall2 | -0.04% | 0.40% | 0.27% | 0.21% | 0.20% | 0.30% |  | 98.2% | 100.0% |
| Poznan_Street | 0.01% | 0.27% | 0.32% | 0.16% | 0.16% | 0.16% |  | 97.1% | 100.0% |
| Undo_Dancer | 0.00% | 0.34% | 0.18% | 0.13% | 0.15% | 0.31% |  | 95.4% | 100.0% |
| 1024x768 | 0.03% | 0.65% | 0.19% | 0.24% | 0.23% | 0.16% |  | 99.7% | 100.0% |
| 1920x1088 | -0.03% | 0.37% | 0.20% | 0.14% | 0.16% | 0.23% |  | 97.8% | 100.0% |
| average | 0.00% | 0.49% | 0.20% | 0.18% | 0.19% | 0.20% |  | 98.6% | 100.0% |

# Conclusion

In this contribution, the simplified structure for constructing MCL for dependent view and depth is proposed. Simulation results show that the proposed algorithm only brings marginally-small loss on the coding performance while providing the MCL construction process for 3D-HEVC much simpler and also decreasing hardware implementation complexity significantly.

# Reference

[1]	G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Test Model 4”, 4th JCT3V Meeting.

[2] D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments”, 4th JCT3V Meeting.

# Patent rights declaration(s)

ETRI and Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).