| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0214 |
| --- | --- |

| Title: | 3D-CE3.h related: CTU-aligned DV for a temporal inter-view motion vector candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyung Yong Kim, Young Su Heo, Min Seong Lee, Yoon Jin Lee, Gwang Hoon Park Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-31-201-3680 {kimky, medist, yomega00, rainmake, ghpark} @khu.ac.kr |
| Source: | Kyung Hee University |  |  |

_____________________________

# Abstract

The disparity vector (DV) is used for deriving candidate of motion parameters for a current block in a dependent view. The vertical component of the derived DV mostly closes to zero, since the current 3DV test sequences are rectified. However, the DV with very large vertical component may come up, since there is no restriction for the vertical motion search range for the inter-view prediction. It increases the required memory-access bandwidth on the decoder side. Therefore, in this contribution, the CTU-aligned DV for a temporal inter-view motion vector candidate is proposed to reduce the memory-access bandwidth. Simulation results show that the performance of the proposed method is slightly better than that of the current HTM while the memory bandwidth to access the co-located motion data is far reduced.

# Problem Statement

In the process of the neighboring-blocks disparity vector (NBDV) of the current 3D-HEVC [1], the disparity vector (DV) can be derived using the spatial and temporal neighboring blocks. DV information is then used for deriving candidate of motion parameters for a current block in a dependent view.

The vertical component of the derived DV mostly closes to zero, since the current 3DV test sequences are rectified. However, when inter_view_mv_vert_constraint_flag is equal to 0 (in the CTC), there is no restriction for the vertical motion search range in the inter-view prediction. So, the worst case, such that the DV with very large vertical component may come up, can be happened. When this kind of worst case happens, as shown in Fig. 1, the required memory-access bandwidth on the decoder side is significantly increased, since the size (or area) of motion-information to keep in the memory, for deriving candidates of motion parameters for a current block in a dependent view, may be as large as an image frame (picture).

Fig. 1. The area of motion-information memory needed for deriving candidate motion parameters for a current block in a dependent view in current HTM (when inter_view_mv_vert_constraint_flag is equal to 0)

If inter_view_mv_vert_constraint_flag is equal to 1 (in the non-CTC), the vertical motion search range for the inter-view prediction is 56. In this case, the area of motion-information memory needed for deriving candidate motion parameters for a current block in a dependent view is reduced to the 3 lines of CTUs (3X CTU size), as shown in Fig. 2.

Fig. 2. Area of the motion-information memory needed for deriving candidate motion parameters for a current block in a dependent view in current HTM (when inter_view_mv_vert_constraint_flag is equal to 1)

Figure 3 shows more-detailed illustration example of co-located fetch-area of motion data needed for deriving candidate of motion parameters.

(a)                                         (b)

Fig. 3. An illustration example of co-located fetch-area of motion data needed for deriving candidate of motion parameters for a current block in a dependent view.

As shown in Fig. 3, in general, the vertical component of the derived DV of the block mostly closes to zero, since the current 3DV test sequences are rectified.

However, for example, the vertical components of the derived DVs of both block ‘D’ and block ‘O’ in Fig. 3(b) may have very large vertical values (blocks of D' and O' in Fig. 3(a)), since there are no restrictions for the vertical motion search ranges in the inter-view prediction. In Fig. 3, the value ‘N’ of the ‘NX CTU size’ can vary depending on the motion search range.

We found that the actual coding cases such that the outcomes of the vertical components of the derived DVs are greater than 300 (e.g. Undo_dancer sequence), when inter_view_mv_vert_constraint_flag is equal to 0. The NBDV process without vertical restrictions may produces those wrongly-derived DVs that have very large vertical values. It also significantly increases the required memory-access bandwidth on the decoder side.

# Algorithm Description of Proposed CTU-aligned (NX) DV

To reduce the memory-access bandwidth for deriving candidate of motion parameters for a current block in a dependent view, we propose the CTU-aligned DV for a temporal inter-view motion vector candidate.

(a)                                        (b)

Fig. 4. Proposed method 1: CTU-aligned (1X) DV

Figure 4 shows the illustration of proposed method using CTU-aligned (1X) DV. In the proposed method, if the top-left positions of blocks are located in the outside of the 1X CTU column, they are replaced by the vertically-closest ones of the current LCU column. In the example shown in Fig. 4, the top-left position of block A’, D’, F’, H’, K’, and O’ are replaced with the top-left position of block A’’, D’’, F’’, H’’, K’’, and O’’. In this way, the motion data required for deriving candidate motion parameter using derived DV won’t go beyond the size of the 1X CTU column.

The proposed method can vary depending on whether the rectification of the video sequence is carried out or not. The CTU-aligned (3X) DV may also be used for non-rectified sequences as shown in Fig. 5. In this case, the motion data required for deriving candidate motion parameter using derived DV won’t go beyond the size of the 3X CTU column.

(a)                                        (b)

Fig. 5. Proposed method 2: CTU-aligned (3X) DV

The modified derivation process using the CTU-aligned DV (1X) in the text of 3D-HEVC is as followings.

- H.8.5.2.1.10 Derivation process for a temporal inter-view motion vector candidate … The location ( xCtb, yCtb ) are set equal to the top-left luma sample of the current coding tree block relative to the top-left luma sample of the current picture. The reference layer luma location ( xRef, yRef ) is derived by - xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 ) + ( ( mvDisp[ 0 ] + 2 ) >> 2 ) ) (Error: Reference source not found-1) - yRef = Clip3(0, PicHeightInSamplesL – 1,yCtb, yCtb + CtbSizeY - 1, yP + ( ( nPSH – 1 ) >> 1 ) + ( ( mvDisp[ 1 ] + 2 ) >> 2 ) ) (Error: Reference source not found-2) Let refCU be the coding unit that covers the luma location ( xRef, yRef ) in the view component with ViewIdx equal to refViewIdx. …

The modified derivation process using the CTU-aligned (3X) DV in the text of 3D-HEVC can be as followings.

- H.8.5.2.1.10 Derivation process for a temporal inter-view motion vector candidate … The location ( xCtb, yCtb ) are set equal to the top-left luma sample of the current coding tree block relative to the top-left luma sample of the current picture. The variable minYBnd and maxYBnd is derived as follows: - minYBnd = max( 0, yCtb – CtbSizeY ) - maxYBnd = min( yCtb + (CtbSizeY << 1) - 1, PicHeightInSamplesL – 1 ) The reference layer luma location ( xRef, yRef ) is derived by - xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 ) >> 1 ) + ( ( mvDisp[ 0 ] + 2 ) >> 2 ) ) (Error: Reference source not found-3) - yRef = Clip3(0, PicHeightInSamplesL – 1, minYBnd, maxYBnd, yP + ( ( nPSH – 1 ) >> 1 ) + ( ( mvDisp[ 1 ] + 2 ) >> 2 ) ) (Error: Reference source not found-4) Let refCU be the coding unit that covers the luma location ( xRef, yRef ) in the view component with ViewIdx equal to refViewIdx. …

# Test Conditions

The proposed method was implemented into 3D-HTM v7.0r1 [2]. Common test conditions specified in JCT3V-D1100 [3] is used for the evaluation.

We used the below encoder configurations for all simulations.

DisparitySearchRangeRestriction : 0 # 0: No disparity restriction 1: Use disparity restriction

# Simulation Results

The following TABLE 1 shows the simulation results of the proposed methods using the CTU-aligned (1X) DV for the 3DV sequences.

TABLE 1. Proposed method 1 using the CTU-aligned (1X) DV vs. HTM7.0r1 (CTC, 3DV sequences)

As shown in TABLE 1, the proposed method using the CTU-aligned (1X) DV achieves an average of 0.05% BD-rate gains for dependent views (both video 1 and video 2), 0.02% BD-rate gain for video PSNR vs. video bitrate, and also 0.01% BD-rate gains for both video PSNR vs. total bitrate and synth PSNR vs. total bitrate, while maintaining far-reduced memory-access bandwidth (from full image size to 1X CTU size).

The following TABLE 2 shows the simulation results of the proposed methods using the CTU-aligned (3X) DV for the 3DV sequences.

TABLE 2. Proposed method 2 using the CTU-aligned (3X) DV vs. HTM7.0r1 (CTC, 3DV sequences)

As shown in TABLE 2, the proposed method using the CTU-aligned (3X) DV achieves the same performances in comparison with the anchor, while maintaining moderately-reduced memory-access bandwidth (from full image size to 3X CTU size).

For the non-rectified MVC test sequences [4], the impacts of the proposed methods are also investigated as shown in both TABLE 3 and TABLE 4.

TABLE 3. Proposed method using the CTU-aligned (1X) DV vs. HTM7.0r1 (CTC, MVC sequences)

As shown in TABLE 3, proposed method using the CTU-aligned (1X) DV obtains an average of 0.9% coding loss for video PSNR vs. total bitrates, while maintaining far-reduced memory-access bandwidth (from full image size to 1X CTU size)

Following TABLE 4 shows the results of the proposed methods using the CTU-aligned (3X) DV for the non-rectified MVC sequences.

TABLE 4. Proposed method using the CTU-aligned (3X) DV vs. HTM7.0r1 (CTC, MVC sequences)

As shown in TABLE 4, proposed method obtains an average of 0.07% coding loss while maintaining moderlately-reduced memory bandwidth (from full image size to 3X CTU size) to access the co-located motion data.

# Conclusion

In this contribution, the CTU-aligned (1X or 3X) DV for a temporal inter-view motion vector candidate is introduced to reduce the memory-access bandwidth. Simulation results show that the coding performance of the proposed method (1X case) is slightly better than that of the current HTM, while maintaining far-reduced memory-access bandwidth. Therefore, we recommend the proposed method to be adopted both in the 3DV-HTM and the 3D-HEVC Test Model.

# Reference

- G. Tech, K. Wegner, Y. Chen and S. Yea, “3D-HEVC Test Model 4,” The 4th JCT-3V Meeting, JCT3V-D1005, Incheon, KR, April 2013.

- 3D-HTM version 7.0r1: https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-7.0r1/.

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” The 4th JCT-3V Meeting, JCT3V-D1100, Incheon, KR, April 2013.

- O. Nakagami and T. Suzuki, “AHG13: On disparity vector constraints,” The 3rd JCT-3V Meeting, JCT3V-C0078, Geneva, CH, Jan 2013.

# Patent rights declaration(s)

Kyung Hee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).