# Proposed text changes

Insert the cyan marked text

Remove the green marked text

        - Derivation process for a temporal inter-view motion vector candidate

This process is not invoked when iv_mv_pred_flag[ nuh_layer_id ] is equal to 0.

Inputs to this process are:

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables nPSW and nPSH specifying the width and the height, respectively, of the current prediction unit,

- a prediction list indication X,

- a reference view index refViewIdx.

- a disparity vector mvDisp,

- a flag mergeFlag specifying whether a merge candidate is derived,

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX.

Outputs of this process are:

- a flag availableFlagLXInterView specifying whether the temporal inter-view motion vector candidate is available,

- a temporal inter-view motion vector candidate mvLXInterView (if availableFlagLXInterView is equal to 1),

- a reference index refIdxLX specifying a reference picture in the reference picture list RefPicListLX,

The flag availableFlagLXInterView is set equal to 0, both components of mvLXInterView are set equal to 0.

The location (xCtb, yCtb) are set equal to the top-left luma sample of the current coding tree block relative to the top-left luma sample of the current picture.

When X is equal to 1 and the current slice is not a B slice the whole decoding process specified in this subclause terminates.

The variable minYBnd and maxYBnd is derived as follows:

minYBnd = max( 0, yCtb – CtbSizeY )

maxYBnd = min( yCtb + (CtbSizeY <<  1) - 1, PicHeightInSamplesL – 1 )

The reference layer luma location ( xRef, yRef ) is derived by

- xRef = Clip3( 0, PicWidthInSamplesL – 1, xP + ( ( nPSW – 1 )  >>  1 ) + ( ( mvDisp[ 0 ] + 2 )  >>  2 ) )	(Error: Reference source not found-1)
yRef = Clip3( 0, PicHeightInSamplesL – 1minYBnd, maxYBnd, yP + ( ( nPSH – 1 )  >>  1 ) + ( ( mvDisp[ 1 ] + 2 )  >>  2 ) )	(Error: Reference source not found-2)

Let refCU be the coding unit that covers the luma location ( xRef, yRef ) in the view component with ViewIdx equal to refViewIdx.

[Ed. (GT:) In software refCU is the coding unit that covers the luma location ( xRef, yRef ) in the view component with smallest ViewIdx and that is included in one of the reference lists of the current view component.(#6)]

[Ed. (CY): What is implemented in the software has no impact on the final results in common test condition, so it is preferred to have the current text and software changes may be desirable. ]

When the variable PredMode for the coding unit refCU is equal to MODE_SKIP or MODE_INTER, the following ordered steps apply, for Y in the range of X to (1 – X), inclusive:

  - The variable refPredFlagLY is set equal to the prediction utilization flag predFlagLY of the prediction unit refPU.

  - The variable refRefIdxLY, is set equal to the reference index refIdxLY of  the prediction unit refPU.

  - The variable refMvLY is set equal to the motion vector mvLY of the prediction unit refPU.

  - The variable refRefPicListLY, is set equal to the reference picture list RefPicListLY of the prediction unit refCU.

  - When refPredFlagLY is equal to 1, the following applies for each i from ( mergeFlag ? 0 : refIdxLX ) to ( mergeFlag ? num_ref_idx_lX_active_minus1 : refIdxLX), inclusive:

  - When availableFlagLXInterView is equal to 0, and the picture order count of the picture refRefPicListLY[ refRefIdxLY ] is equal to the picture order count of the picture RefPicListLX[ i ], the flag availableFlagLXInterView is set equal to 1 and the following applies.

  - The motion vector mvLXInterView is derived by:

        - mvLXInterView[ 0 ] = refMvLY[ 0 ]		(Error: Reference source not found-3)
mvLXInterView[ 1 ] = refMvLY[ 1 ]		(Error: Reference source not found-4)

        - When mergeFlag is equal to 1, the reference index refIdxLX is derived by:

        - refIdxLX = i		(Error: Reference source not found-5)