| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0222 |
| --- | --- |

| Title: | CE6.h: CE results on removal of division operation for SDC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng， Yongbing Lin, Xu Chen, Jianhua Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution reports CE results for division free solution for SDC. Both test cases at common test condition and All intra condition for SDC are verified. Experimental results show that the proposed solution doesn't have the impact on the coding performance.

# Introduction

At current SDC designs, the DC value is the average of the sample values inside an intra block. The calculation is performed by the division between the summation of sample values and the number of samples which are inside a block for SDC. The division are shown as follows:

  - For p in the range of 0 to 1, inclusive,  the variable dcPred[ p ] is derived as specified in the following:

sumPred = 0
		numPred = 0. 
		for( x = 0; x < ( nT >> log2SubSample ); x++ ) { 
			xS  = x << log2SubSample  
			for ( y = 0; y < ( nT >> log2SubSample ) ;y++ ) { 
				yS = y << log2SubSample .
				if ( p = = wedgePattern[ xS ][ yS ] ) {
					sumPred += predSamples[ xS ][ yS ]
					numPred += 1
				}
			}
		dcPred[ p ] = ( numPred > 0 ) ? ( sumPred / numPred ) : 0

Because DMM might divide a square block into two non-rectangular partitions, the number of samples inside a block or inside the prediction samples might not be represented as 2N. Therefore the division operation exists in the calculation process of DC value for SDC, and can’t be converted to the right shift and addition operations.

Compared to other logic unit, divider is implemented with much complex circuit at cost of larger area and longer delay. In hardware design, the division operation can be achieved by the look-up table (LUT) as LUT can store all of results of the division operation. However, since the numerator and denominator vary in the same time at the DC calculation process, it will significantly improve the cost of LUT. The below table summarizes the range of the numerator and denominator, and the size of LUT in DMM and SDC.

Table 1: Cost of look-up table

|  | Numerator (summary of samples value) | Denominator (number of samples) | LUT size |
| --- | --- | --- | --- |
| SDC | 218 | 210 | 228 |

It can be easily concluded that the divider in current SDC design is expensive in hardware. This contribution provides two solutions that can significant reduce the division operation.

# Removal of division operation

Considering that the numerator and denominator in SDC change randomly, the size of LUT is the multiplication of the range of numerator and denominator. To reduce the LUT size, we propose to separate the calculation of the reciprocal of denominator and the calculation of DC value.

At the first step, the reciprocal of denominator is calculated as follow:

Int iX = (C + (numOfPix >> 1)) / numOfPix;

numOfPix is the number of samples (denominator), and C is a constant value. In order to improve the accuracy of iX, the constant value is set to the value which is larger than the maximum of the denominator (numOfPix). Thus, the value of “c” is set to 216 for SDC.

Secondly, the DC value is calculated by the multiplication of numerator and iX that has been calculated before.

DC =  Clip( (sumOfPix*iX + shiftBit>>1) >> shiftBit );

sumOfpix is the summary of the sample values (numerator), and shiftBit is set as 16 for SDC.

At the above solution, the division operation exists in the first step. Considering that the numerator of the first step is depended on the value of denominator, the LUT size for the storage of division results is reduced from from 228 to 210 in SDC. The cost of LUT is reduced significantly.

# Experimental results

The proposed method is implemented on HTM7.0. The coding efficiency is evaluated in random access (CTC) and All-Intra configurations. The table 2 and table 3 show the results

Table 2: Results for CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | -0.03% | -0.04% | -0.03% | 99.9% | 100.6% | 99.7% |
| Kendo | 0.0% | 0.1% | -0.1% | 0.00% | 0.04% | 0.01% | 99.6% | 110.3% | 102.9% |
| Newspaper_CC | 0.0% | 0.1% | 0.0% | 0.02% | 0.01% | 0.07% | 99.8% | 103.0% | 104.2% |
| GT_Fly | 0.0% | 0.1% | -0.1% | 0.00% | 0.00% | -0.06% | 100.0% | 100.7% | 101.1% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.4% | 0.06% | 0.04% | 0.00% | 99.8% | 102.2% | 102.6% |
| Poznan_Street | 0.0% | -0.1% | 0.0% | -0.03% | -0.01% | 0.00% | 99.8% | 100.6% | 99.3% |
| Undo_Dancer | 0.0% | -0.1% | 0.1% | -0.01% | -0.03% | -0.03% | 100.3% | 97.2% | 98.2% |
| 1024x768 | 0.0% | 0.1% | 0.0% | 0.00% | 0.01% | 0.02% | 99.8% | 104.7% | 102.3% |
| 1920x1088 | 0.0% | 0.0% | 0.1% | 0.00% | 0.00% | -0.02% | 100.0% | 100.2% | 100.3% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.01% | 99.9% | 102.1% | 101.1% |

Table 3: Results for all - intra

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | 0.01% | 100.0% | 100.2% | 98.5% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.06% | 100.0% | 101.3% | 99.2% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.02% | 99.9% | 102.4% | 98.7% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | 0.03% | 0.04% | 101.6% | 99.8% | 99.5% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | -0.07% | 98.0% | 100.5% | 101.1% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | 0.02% | 0.07% | 100.7% | 90.6% | 101.3% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 100.5% | 101.7% | 100.6% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.02% | 100.0% | 101.3% | 98.8% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.01% | 100.2% | 98.1% | 100.6% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | 0.00% | 100.1% | 99.5% | 99.9% |

# Changes to the description of Test Model

  - For p in the range of 0 to 1, inclusive,  the variable dcPred[ p ] is derived as specified in the following:

sumPred = 0
		numPred = 0. 
		for( x = 0; x < ( nT >> log2SubSample ); x++ ) { 
			xS  = x << log2SubSample  
			for ( y = 0; y < ( nT >> log2SubSample ) ;y++ ) { 
				yS = y << log2SubSample .
				if ( p = = wedgePattern[ xS ][ yS ] ) {
					sumPred += predSamples[ xS ][ yS ]
					numPred += 1
				}
			}
		if (numPred = 0) 
       {
           dcPred[p] = 0

}
       else 
       {
           iX = (0x10000 + numPred>>1) / numPred
           dcPred[p] = Clip (0, 255, (iX x sumPred + 0x8000) >> 16)
       }

# Conclusion

This contribution provides two division removal solutions for SDC mode. Compare to the anchor, the proposed solutions doesn't have impact on all test cases. Considering the tradeoff between complexity reduction and the impact on the coding performance, it is recommended to incorporate the proposed solution into next version of 3D-HEVC Test Model and HTM8.

# Patent rights declaration(s)

Hisilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).