| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0225 |
| --- | --- |

| Title: | CE5.h related: Simplification on DMM1 pattern generation mechanism |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiran Li Xiaozhen Zheng， Yun He | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Tsinghua University, Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution provides a simplified DMM1 pattern generation mechanism, which targets at the simplification of pattern generation, reduce the number of patterns and reduce encoding time. Experimental results show that the proposed method doesn’t have impact on coding performance and can achieve 10% encoding time reduction for all-intra case, and achieve 0.1% gain for CTC case.

# Introduction

DMM1 uses explicit signalling method to signal the position of start and end points of the DMM partition line. Basically, all possible combinations of start and end points located at different image block edge are used at the pattern generation, whose process is illustrated at Fig. 1 [1]. The accuracy of the start and end points are depended on the size of intra block. For 32 x 32 and 16 x 16 block, the accuracy is restricted to 2 samples. For 8 x 8 and 4 x 4 block, full-sample and half-sample accuracy are used, respectively. The redundant patterns that are identical to the already generated pattern are checked and removed during the pattern initialization process. The comprehensive search for the DMM patterns and redundant pattern checking complicate the pattern generation process.

Fig. Illustration of DMM1 patterns

According to the current DMM1 pattern generation mechanism, a considerable huge number of patterns are generated at each block-size. Table 1 summary the total number of the patterns for each block-size. Since DMM patterns contain the binary information to indicate the relationship between the location of a sample and its belonging partitions, they are stored and used at the encoding and decoding process. The cost for the storage of patterns in current mechanism is somewhat large.

Table 1: Number of patterns and signalling bits for different block-size

|  | 4 x 4 | 8 x 8 | 16 x 16 | 32 x32 |
| --- | --- | --- | --- | --- |
| Num. of patterns | 86 | 782 | 1394 | 1503 |
| Length of signalling bits | 7 | 10 | 11 | 11 |

Regarding the signalling method, DMM1 uses the fixed-length bits to signal the pattern index whose length is depended on the block-size and summarized in Table 1. According to the signalling method, the number of patterns doesn’t reach the maximum allowed number for the signalling bits at each block-size. The current signalling causes some kind of waste.

Base on the drawbacks of current DMM1 pattern generation mechanism, this contribution proposes a hardware-friendly DMM1 pattern generation method that can simplify the pattern generation process, reduce both pattern number and signalling bits.

# Proposed pattern generation method

## Basic principle

Different to the current DMM1 pattern generation design that is based on the start and end points, the proposed method explores the patterns’ geometry relationship and provides a simple generation method. In general, a square block can be partitioned into two types of partitions: rectangle partition and non-rectangle partition. Therefore, in our design, a syntax partition_type is used to indicate those two partition types.

For rectangle partition case, the partitions can be further classified to horizontal rectangle partition, and vertical rectangle partition categories. For non-rectangle case, the partitions can be categorised by the partition line’s slope. Five partition line’s slopes: 1:1, 2:1, 1:2, 4:1 and 1:4 are used for the categorization that is signalled by the syntax scalar. For each category, the parameter offset is used to signal the position of the partition line. At non-rectangle cases, an additional parameter ori is used to indicate the orientation of partition line.

By using the above parameters, DMM patterns can be represented and reconstructed. Meanwhile, total amount of DMM pattern is significantly reduced.

## Syntax design

The partition_type is used to indicate three partition type used in DMM1: non-rectangle partition, horizontal partition and vertical partition. The signalling is show in Table 2.

Table 2: Binarization for partition_type

| partition_type | Bin string |
| --- | --- |
| non-rectangle partition | 1 |
| horizontal partition | 00 |
| vertical partition | 01 |

For horizontal partition and vertical partition, the parameter offset is used to indicate the distance between the intersection of partition line and the image block boundary, and the base point. Fig. 2a and Fig. 2b shows the illustration of horizontal partition and vertical partition. Define the size of an image block as 4n x 4n, where n = 1, 2, 4, 8. In Fig. 2a (horizontal partition) and Fig. 2b (vertical partition), the base point is set as (2n-1, 0) and (0, 2n-1) respectively, which are the central point of the left image boundary and up image boundary. The accuracy of offset in in rectangle partitions is full-sample.

a                                      b

Fig. 2: Illustration of horizontal partition and vertical partition

For the non-rectangle partition, the scalar is used as the representation of a partition line’s slope, which can be calculated by the start and end points’ coordinate mathematically. Define the size of an image block as 4n x 4n, where n = 1, 2, 4, 8. Let the start and end points represented as (y1, x1) and (y2, x2). The value of the scalar is calculated as follow:

scalar=y1−y2x1−x2

By using the scalar, the shape of the partition can be represented. In our design, the scalar 1:1, 2:1, 1:2, 4:1 and 1:4 are used in the non-rectangle partitions. Fig. 3a and Fig. 3b show the examples of 2:1 and 1:2 scalar respectively.

Considering a non-rectangle partition can have two partitioning orientations: left-down to right up partitioning and left-up to right down partitioning, which is show in Fig 3.a and Fig. 3b respectively. The syntax ori is used to indicate the two partitioning orientation.

Lastly, the parameter offset is used to indicate the distance between the partition line and the central partition line. The central partition line has the same scalar as the partition line, and divides the image block into two partitions with the equal size. To simplify the determination of partition line location, the base point for different scalars is defined that is used to locate the partition line through the value of offset. The base point is the intersection between the central partition line and the image block’s up boundary. For the cases of 1:2 and 1:4 scalar, the base point locates at the extension of the image’s up boundary that is illustrated at Fig 3.b.

The accuracy of offset is depended on the value of scalar, which is listed as follows:

- 1:1 scalar: full-sample

- 2:1 and 1:2 scalars: full-sample for –n < offset < n, two-samples for other offset values

- 4:1 and 1:4 scalars: full-sample for –1.5n < offset < 1.5n, two-samples for other offset values

offset is signalled by the fixed length code. Base on the definition of offset accuracy, the length for offset of 1:1 scalar is log2(4n), and for other cases are always log2(2n). This design can guarantee that the amount of offset value can reach the maximum allowed number for the fixed length code.

a

b

Fig. 3: Illustration of 2:1 (a) and 1:2 (b) partitioning. The blue line is the partition line, and the dash line is the central partition line that divides the image block into two partitions with equal size.

Through the usage of the parameters partition_type, scalar, offset and ori, a specified partition pattern can be determined. Table 3 summaries the relationship between the partition categories and the value of those parameters, and their binarization scheme.

Table 3: The binarization scheme for partition_type, scalar, offset and ori

|  |  | partition_type | scalar | offset | ori |
| --- | --- | --- | --- | --- | --- |
| rectangle partition | Horizontal partition | 00 | - | FL, cMax = 2n - 1 | - |
|  | Vertical partition | 01 | - | FL, cMax = 2n – 1 | - |
| non-rectangle partition | 1:1 partition | 1 | 0 | FL, cMax = 4n - 1 | 0 or 1 |
|  | 2:1 partition | 1 | 100 | FL, cMax = 2n – 1 | 0 or 1 |
|  | 1:2 partition | 1 | 101 | FL, cMax = 2n – 1 | 0 or 1 |
|  | 4:1 partition | 1 | 110 | FL, cMax = 2n – 1 | 0 or 1 |
|  | 1:4 partition | 1 | 111 | FL, cMax = 2n - 1 | 0 or 1 |

## Generation of DMM1 pattern

The DMM1 pattern is generated based on the parameters scalar, offset and ori, and base point. The base point is set based on 16 x 16 block-size and is depended on the partition category listed in Table 4:

Table 4: The relationship between base Point value and different partition categories

|  | horizontal partition | vertical partition | 1:1 partition | 2:1 partition | 1:2 partition | 4:1 partition | 1:4 partition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| base Point | 8 | 8 | 0 | -4 | 8 | -6 | 24 |

The value of base Point for other block-sizes can be derived based on the scale through from the value for 16 x16 block-size.

Define the size of an image block as 4n x 4n, where n = 1, 2, 4, 8. Define basePointOffset for base point, and scalarFactor for scalar as follows:

basePointOffset [] = {0, -4, 8, -6, 24}

scalarFactor[] = {1, 2, 1/2, 4, 1/4}

The generation process for different partition categories is listed as follows:

Horizontal partition:

terminalPos = 8 * 4n  / 16

Partition 0: if y > terminalPos – offset

Partition1: others

Vertical partition:

terminalPos = 8 * 4n / 16

Partition 0: if x > terminalPos – offset

Partition1: others

Non-rectangle partition

if (!ori)

{

terminalPos = 4n - 1 + (4n x basePointOffset[scalar] / 16)

Partition 0: if terminalPos – n * offset - x > y/scalarFactor[scalar]

Partition 1: others

}

else

{

terminalPos = -(4n x basePointOffset[scalar] / 16)

Partition 0: if x - terminalPos – n*offset < y/scalarFactor[scalar]

Partition 1: others

}

n = 4 for scalar is equal to 4, n = 2 for scalar is equal to 2, n = 1 for other cases

## Benefits of the proposed pattern generation method

The total amount of the DMM1 patterns generated by the proposed method is significantly reduced. Moreover the overhead of the signalling bits for DMM1 pattern are also reduced. Table 5 compares the difference between the current DMM1 pattern generation method and the proposed method.

Table 5: The comparison between the current DMM1 and proposed DMM1 pattern generation scheme

|  | 4 x 4 |  | 8 x 8 |  |  | 16 x 16 |  | 32 x32 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | current | proposed | current | proposed |  | current | proposed | current | proposed |
| Num. of patterns | 86 | 32 | 782 |  | 100 | 1394 | 212 | 1503 | 436 |
| Length of signalling bits | 7 | 4~7 | 10 |  | 5~8 | 11 | 6~9 | 11 | 7~10 |

Since the DMM1 pattern number is reduced, the cost for the pattern storage is also reduced greatly. Moreover, considering that the proposed pattern generation mechanism is based on the partitioning geometry, the generation scheme doesn’t need to check the all possible combinations of partition line’s start and end points, and the redundant patterns. Therefore, the generation mechanism is much simplified than the current method.

# Experimental results

The proposed method is implemented on HTM7.0. The coding efficiency is evaluated in random access (CTC) and All-Intra configurations. The table 1 and table 2 show the test results.

Table 1: Results for all - intra

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 90.0% | 95.3% | 95.7% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | 90.2% | 97.5% | 96.4% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 91.0% | 102.5% | 101.1% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 90.4% | 98.8% | 99.4% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.4% | 90.8% | 99.8% | 100.5% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 90.2% | 101.1% | 99.5% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.3% | 89.6% | 100.3% | 102.9% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 90.4% | 98.4% | 97.7% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 90.3% | 100.0% | 100.6% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 90.3% | 99.3% | 99.3% |

Table 2: Results for CTC

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 99.3% | 95.9% | 100.3% |
| Kendo | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 99.4% | 105.2% | 104.2% |
| Newspaper_CC | 0.0% | -0.2% | -0.1% | -0.1% | -0.2% | -0.3% | 98.9% | 96.7% | 101.3% |
| GT_Fly | 0.0% | 0.1% | -0.3% | 0.0% | -0.1% | -0.1% | 98.5% | 92.9% | 94.1% |
| Poznan_Hall2 | 0.0% | 0.2% | 0.3% | 0.1% | 0.0% | -0.1% | 99.8% | 96.3% | 98.9% |
| Poznan_Street | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 101.1% | 101.5% | 102.1% |
| Undo_Dancer | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 101.4% | 98.3% | 96.2% |
| 1024x768 | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.2% | 99.2% | 99.3% | 101.9% |
| 1920x1088 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 97.3% | 97.8% |
| average | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 99.8% | 98.1% | 99.6% |

# Conclusion

This contribution provides a simplified DMM1 pattern generation mechanism, which simplify the pattern generation process and reduce the pattern amount. The proposed method doesn't have impact on all test cases. The encoding time for all-intra case also has 10% reduction. Therefore, it is recommended to incorporate the proposed sub-sampling solution into next version of 3D-HEVC Test Model and HTM8.

# References

[1] G. Tech, K. Wegner, Y. Chen, S.Yea, 3D-HEVC Test Model 4, JCT3V-D1005, Incheon, Korea, April, 2013

# Patent rights declaration(s)

Tsinghua University, Hisilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).