| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0226 |
| --- | --- |

| Title: | Modification of Binaryzation for Depth Intra Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen, Xiaozhen Zheng， Yongbing Lin, Jianhua Zheng | Tel: Email: | +86-755-28425333 anderson.chen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

D0195 proposes a unified intra mode signalling design for all the new Intra modes added into 3D-HEVC, including Depth Modeling Mode (DMM) modes, the Chain Coding Mode (CCM) and Simplified Depth Coding (SDC) modes.

This contribution proposes a modification of binaryzation for depth intra mode, which is more suitable for signalling 4×4 and 8×8-32×32 depth intra mode. Solution1 achieves 0.04% or 0.05% BD-rate gains and 0.02% or 0.03% video gain compared to 3DV-HTM7.0r1 under CTC without complexity improvement. Solution2 achieves 0.01% BD-rate gains and 0.05% video lost compared to 3DV-HTM7.0r1 under CTC without complexity improvement.

# Introduction

In 3D-HEVC, 9 depth intra modes are applied for coding unit, which are list depth modeling table semantics as follow.

Table xx: Interpretation of depth_intra_mode

| depth_intra_mode | DepthIntraMode |
| --- | --- |
| 0 | SDCM_Planar |
| 1 | HEVC intra (HEVC_INTRA_PRED) |
| 2 | SDCM_DMM_1 |
| 3 | DMM_1 (MODE_DMM_WFULL) |
| 4 | DMM_4 (MODE_DMM_CPREDTEX) |
| 5 | DMM_3 (MODE_DMM_WPREDTEX) |
| 6 | SDCM_DC |
| 7 | DMM_2 (MODE_DMM_WPREDDIR) |
| 8 | CCM |

The current design has the following problem:

Compare with 4×4 or 64×64 coding unit, 8×8 needs more bits to signal all intra modes. In depth modeling table semantics, depth intra mode 0, 1, 2 modes applied more than other modes. 2 bits for signaling depth intra mode 0 and 3 bits for signaling depth intra mode 1 and 2 is not suitable enough.

For 4×4 coding unit, 2 bits for signaling depth intra mode 3 is not suitable enough since depth intra mode 5 or 8 are few applied in coding units. 1 bit is enough for signaling depth intra mode 3 when no depth intra mode 5, 8 are applied in 64×64.

# Proposed method

The proposal is summarized as follows:

To modify the syntax table of 4×4 and 8×8-32×32, fewer bits are used for signalling depth intra mode 0-3 which brings greater impact than other modes behind.

Solution1:

Add sig_intramode_det0, which for signaling weather mode 5 or 8 applied in detecting unit(64×64). sig_intramode_det0 only exist once in detecting unit(64×64) that with 4×4 inside and the flag determine the signaling depth intra mode way of all 4×4s inside detecting unit.

Solution2:

Add sig_intramode_det1, which for signaling weather mode 3-8 applied in detecting unit(64×64). sig_intramode_det1 only exist once in detecting unit(64×64) that with 8×8-32×32 inside and the flag determine the signaling depth intra mode way of all 8×8-32×32s inside detecting unit.

Other syntax and semantics are the same as D0195.

# Experimental Results

The proposed method is implemented in the HTM7.0r1. And it was simulated under both common test condition configuration and all-intra configuration. Results are summarized as Solution1 Table 1-3 and Solution2 Table 1-2.

Solution1

Table 1: Coding gain of proposed method compared to anchor (CTC)

(anchor is HTM7.0r1) (modify on HTM7.01)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.05% | 0.03% | -0.01% | 0.00% | 0.00% |
| Kendo | 0.00% | 0.06% | -0.20% | -0.02% | 0.02% | -0.05% |
| Newspaper_CC | 0.00% | 0.10% | -0.13% | -0.01% | -0.01% | -0.31% |
| GT_Fly | 0.00% | 0.73% | 0.59% | 0.09% | 0.08% | 0.04% |
| Poznan_Hall2 | 0.00% | 0.10% | 0.12% | 0.02% | -0.01% | 0.04% |
| Poznan_Street | 0.00% | 0.14% | 0.10% | 0.02% | 0.03% | 0.03% |
| Undo_Dancer | 0.00% | -0.84% | -0.98% | -0.31% | -0.31% | -0.11% |
| 1024x768 | 0.00% | 0.04% | -0.10% | -0.01% | 0.00% | -0.12% |
| 1920x1088 | 0.00% | 0.03% | -0.04% | -0.04% | -0.05% | 0.00% |
| average | 0.00% | 0.04% | -0.07% | -0.03% | -0.03% | -0.05% |

Table 2: Coding gain of proposed method compared to anchor (CTC)

(anchor is HTM7.0r1) (modify on HTM7.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.02% | 0.02% | 0.00% | 0.00% | 0.00% |
| Kendo | 0.00% | 0.06% | -0.07% | 0.00% | 0.03% | 0.01% |
| Newspaper_CC | 0.00% | 0.08% | -0.03% | 0.01% | 0.03% | -0.22% |
| GT_Fly | 0.00% | 0.21% | 0.02% | 0.02% | 0.02% | 0.01% |
| Poznan_Hall2 | 0.00% | 0.10% | -0.36% | -0.07% | -0.04% | -0.06% |
| Poznan_Street | 0.00% | -0.28% | -0.05% | -0.05% | -0.04% | -0.06% |
| Undo_Dancer | 0.00% | -0.10% | -0.08% | -0.03% | -0.04% | 0.03% |
| 1024x768 | 0.00% | 0.04% | -0.03% | 0.00% | 0.02% | -0.07% |
| 1920x1088 | 0.00% | -0.02% | -0.12% | -0.03% | -0.03% | -0.02% |
| average | 0.00% | 0.01% | -0.08% | -0.02% | 0.00% | -0.04% |

Table 3: Coding gain of proposed method compared to anchor (All Intra)

(anchor is HTM7.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.04% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.02% | 0.05% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 0.02% |
| Poznan_Hall2 | Not complete |  |  | Not complete |  |  |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.02% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | 0.03% |
| 1920x1088 | Not complete |  |  | Not complete |  |  |
| average | Not complete |  |  | Not complete |  |  |

Solution2

Table 1: Coding gain of proposed method compared to anchor (CTC)

(anchor is HTM7.0r1) (modify on HTM7. 0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | -0.17% | -0.08% | -0.05% | 0.02% | -0.01% |
| Kendo | 0.00% | 0.05% | -0.02% | 0.01% | 0.10% | 0.06% |
| Newspaper_CC | 0.00% | -0.01% | -0.05% | -0.01% | 0.09% | -0.14% |
| GT_Fly | 0.00% | 0.26% | 0.06% | 0.03% | 0.07% | 0.04% |
| Poznan_Hall2 | 0.00% | 0.21% | -0.05% | 0.01% | 0.05% | -0.04% |
| Poznan_Street | 0.00% | -0.03% | 0.05% | -0.01% | 0.06% | -0.01% |
| Undo_Dancer | 0.00% | -0.17% | -0.25% | -0.05% | -0.02% | 0.02% |
| 1024x768 | 0.00% | -0.04% | -0.05% | -0.02% | 0.07% | -0.03% |
| 1920x1088 | 0.00% | 0.07% | -0.05% | 0.00% | 0.04% | 0.00% |
| average | 0.00% | 0.02% | -0.05% | -0.01% | 0.05% | -0.01% |

Table 2: Coding gain of proposed method compared to anchor (All Intra)

(anchor is HTM7.0r1)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.00% | 0.00% | 0.00% | 0.00% | 0.08% | -0.01% |
| Kendo | 0.00% | 0.00% | 0.00% | 0.00% | 0.10% | 0.01% |
| Newspaper_CC | 0.00% | 0.00% | 0.00% | 0.00% | 0.11% | 0.01% |
| GT_Fly | 0.00% | 0.00% | 0.00% | 0.00% | 0.09% | 0.00% |
| Poznan_Hall2 | Not complete |  |  | Not complete |  |  |
| Poznan_Street | 0.00% | 0.00% | 0.00% | 0.00% | 0.06% | 0.00% |
| Undo_Dancer | 0.00% | 0.00% | 0.00% | 0.00% | 0.05% | -0.02% |
| 1024x768 | 0.00% | 0.00% | 0.00% | 0.00% | 0.10% | 0.00% |
| 1920x1088 | Not complete |  |  | Not complete |  |  |
| average | Not complete |  |  | Not complete |  |  |

# References

- Qin Yu, Li Zhang, Ying Chen, Siwei Ma, Marta Karczewicz “Unification of new intra modes in 3D-HEVC,” JCT3V-D0195, Incheon, KR, Apr. 2013.

- Dmytro Rusanovskyy, Karsten Müller, Anthony Vetro, “Common Test Conditions of 3DV Core Experiments,” JCT3V-C1100, Geneva, CH, Jan. 2013.

# Decoding process

- Binaryzation process for depth_intra_mode

Input to this process is a request for a binarization for the syntax element depth_intra_mode, a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, and a variable cLog2CbSize specifying the current luma coding block size.

Output of this process is the binarization of the syntax element.

The binarization for part_mode is given by xx depending on PartMode[ xC ][ yC ] and cLog2CbSize.

Solution1

Table H-xx –4×4 and 8×8-32×32 Binaryzation for depth_intra_mode

(with depth intra mode 3 or 5 in 64×64)

| depth_intra_mode | DepthIntraMode | Bin String |  |  |
| --- | --- | --- | --- | --- |
|  |  | (cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N) ¦¦ (cLog2CbSize > 3 && cLog2CbSize < 6) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | Sig_intramode_det0 |
| 0 | SDCM_Planar | 0 0 | - | 1 |
| 1 | HEVC intra (HEVC_INTRA_PRED) | 0 1 | 0 |  |
| 2 | SDCM_DMM_1 | 1 0 | - |  |
| 3 | DMM_1 (MODE_DMM_WFULL) | 1 1 0 0 | 1 0 |  |
| 4 | DMM_4 (MODE_DMM_CPREDTEX) | 1 1 0 1 | - |  |
| 5 | DMM_3 (MODE_DMM_WPREDTEX) | 1 1 1 0 | 1 1 0 |  |
| 6 | SDCM_DC | 1 1 1 1 0 | - |  |
| 7 | DMM_2 (MODE_DMM_WPREDDIR) | 1 1 1 1 1 0 | - |  |
| 8 | CCM | 1 1 1 1 1 1 | 1 1 1 |  |

Table H-xx –4×4 and 8×8-32×32 Binarization for depth_intra_mode

(w/o depth intra mode 5,8 in 64×64)

| depth_intra_mode | DepthIntraMode | Bin String |  |  |
| --- | --- | --- | --- | --- |
|  |  | (cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N) ¦¦ (cLog2CbSize > 3 && cLog2CbSize < 6) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | Sig_intramode_det0 |
| 0 | SDCM_Planar | 0 0 | - | 0 |
| 1 | HEVC intra (HEVC_INTRA_PRED) | 0 1 | 0 |  |
| 2 | SDCM_DMM_1 | 1 0 | - |  |
| 3 | DMM_1 (MODE_DMM_WFULL) | 1 1 0 0 | 1 |  |
| 4 | DMM_4 (MODE_DMM_CPREDTEX) | 1 1 0 1 | - |  |
| 5 | DMM_3 (MODE_DMM_WPREDTEX) | 1 1 1 0 | - |  |
| 6 | SDCM_DC | 1 1 1 1 0 | - |  |
| 7 | DMM_2 (MODE_DMM_WPREDDIR) | 1 1 1 1 1 0 | - |  |
| 8 | CCM | 1 1 1 1 1 1 | - |  |

Solution2

Table H-xx –8×8-32×32 Binaryzation for depth_intra_mode

(with depth intra mode 3-8 in 64×64)

| depth_intra_mode | DepthIntraMode | Bin String |  |  |
| --- | --- | --- | --- | --- |
|  |  | (cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N) ¦¦ (cLog2CbSize > 3 && cLog2CbSize < 6) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | Sig_intramode_det1 |
| 0 | SDCM_Planar | 0 0 | - | 1 |
| 1 | HEVC intra (HEVC_INTRA_PRED) | 0 1 | 0 |  |
| 2 | SDCM_DMM_1 | 1 0 | - |  |
| 3 | DMM_1 (MODE_DMM_WFULL) | 1 1 0 0 | 1 0 |  |
| 4 | DMM_4 (MODE_DMM_CPREDTEX) | 1 1 0 1 | - |  |
| 5 | DMM_3 (MODE_DMM_WPREDTEX) | 1 1 1 0 | 1 1 0 |  |
| 6 | SDCM_DC | 1 1 1 1 0 | - |  |
| 7 | DMM_2 (MODE_DMM_WPREDDIR) | 1 1 1 1 1 0 | - |  |
| 8 | CCM | 1 1 1 1 1 1 | 1 1 1 |  |

Table H-xx –8×8-32×32 Binarization for depth_intra_mode

(w/o depth intra mode 3-8 in 64×64)

| depth_intra_mode | DepthIntraMode | Bin String |  |  |
| --- | --- | --- | --- | --- |
|  |  | (cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_2Nx2N) ¦¦ (cLog2CbSize > 3 && cLog2CbSize < 6) | cLog2CbSize = = 3 && PartMode[ xC ][ yC ] = = PART_NxN | Sig_intramode_det1 |
| 0 | SDCM_Planar | 0 | - | 0 |
| 1 | HEVC intra (HEVC_INTRA_PRED) | 1 0 | 0 |  |
| 2 | SDCM_DMM_1 | 1 1 | - |  |
| 3 | DMM_1 (MODE_DMM_WFULL) | - | 1 0 |  |
| 4 | DMM_4 (MODE_DMM_CPREDTEX) | - | - |  |
| 5 | DMM_3 (MODE_DMM_WPREDTEX) | - | 1 1 0 |  |
| 6 | SDCM_DC | - | - |  |
| 7 | DMM_2 (MODE_DMM_WPREDDIR) | - | - |  |
| 8 | CCM | - | 1 1 1 |  |

# Patent rights declaration(s)

Hisilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).