| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0230 |
| --- | --- |

| Title: | AHG13: MFC Conformance Testing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Peng Yin, Taoran Lu, Hariharan Ganapathy, Tao Chen, Walt Husak 432 Lakeside Dr. Sunnyvale CA 94085 USA Dong Tian MERL 201 Broadway Cambridge, MA 02139 | Tel: Email: | +1 408 330 3251 {pyin,wjh}@dolby.com tian@merl.com |
| Source: | Dolby Laboratories Inc. and MERL |  |  |

_____________________________

# Abstract

This document provides information about MFC conformance Testing. It has two attachments. The first attachment is the draft text of MFC conformance testing. The second attachment is a set of candidate bitstreams for MFC conformance testing.

# MFC conformance test bitstreams

The candidate conformance bitstreams are listed in Table 1, which include various combinations of syntax elements introduced by MFC. The software and encoder configuration is based on the MFC reference software in [1]. Each zip file package contains a bitstream (.264), a short description (.txt), an md5sum (md5sum.txt) of the bitstream, the decoded BL and EL yuv (normative), and the fully reconstructed left and right view yuv (non-normative). One 1080 24p sequence is used for progressive case and one 1080 50i sequence is used for interlaced case. For each sequence, 17 frames are encoded with one I frame only. The GOP size is set to 8 with hierarchical B coding structure.

Table 1 Bitstreams for the MFC High profile

| Categories | Bitstream | Frame Rate (Frame/Sec) | mfc_format_idc | default_grid_ position_flag | rpu_filter_ enabled_flag | rpu_field_ processing_flag |
| --- | --- | --- | --- | --- | --- | --- |
| RPU Operational Modes | MFCRFT-1 | 24p | 0 | 1 | 1 | 0 |
|  | MFCRFT-2 | 24p | 0 | 1 | 0 | 0 |
| RPU Interlaced Coding Tools | MFCFLD-1 | 50i | 0 | 1 | 1 | 1 |
|  | MFCFLD-2 | 50i | 0 | 1 | 1 | 1 |
| MFC Format | MFCMFM-1 | 24p | 1 | 1 | 1 | 0 |
| Grid Position | MFCGRD-1 | 24p | 0 | 0 | 1 | 0 |

# References

- P. Yin, T. Lu, H. Ganapathy, T. Chen, and W. Husak, “AHG13: MFC Reference Software Update”, JCT3V-E0231, JCT3V 5th Meeting, Vienna, AT, July 2013.