Information technology – Coding of audio-visual objects – Part 4: Conformance test, AMENDMENT X: Conformance test for MFC extension of AVC

In 10.6.5.7, add the following text at the end of the subclause:

A decoder that conforms to the MFC High profile at a specific level shall be capable of decoding all bitstreams in which all active MVC sequence parameter sets have any of the following:

–	profile_idc equal to 128

–	profile_idc equal to 134

–	profile_idc equal to 100 or 77

–	profile_idc equal to 88 and constraint_set1_flag equal to 1

–	profile_idc equal to 66 and constraint_set1_flag equal to 1

and in which level_idc or the combination of level_idc and constraint_set3_flag for all active MVC sequence parameter sets represent a level less than or equal to the specific level. In addition to the bitstreams defined in Table-AMDxx-1, a decoder that conforms to the MFC High profile shall be capable of decoding the Main profile bitstreams specified in Table AMD6-1, and the High profile bitstreams specified in Table AMD 9-2.

After 10.6.6.xx. x, add the following text:

### 10.6.6. xx Test bitstreams – MFC High Profile

#### 10.6.6. xx.1 Test bitstream #MFCRFT-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of two views with RPU Filter mode, using SbS base layer and default grid position.

Purpose: Check that the decoder can properly decode two view components with RPU filter mode.

#### 10.6.6. xx.1 Test bitstream #MFCRFT-2

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 0. rpu_field_processing_flag is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of two views with RPU DC mode, using SbS base layer and default grid position.

Purpose: Check that the decoder can properly decode two view components with RPU DC mode.

#### 10.6.6. xx.1 Test bitstream #MFCFLD-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. field_pic_flag is equal to 1. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of two views with RPU field processing, using SbS base layer, default grid position, RPU filter mode, and each view component is coded as a field picture.

Purpose: Check that the decoder can properly decode two view components with RPU field processing for field pictures.

#### 10.6.6. xx.1 Test bitstream #MFCFLD-2

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mb_adaptive_frame_field_flag  is equal to 1. field_pic_flag is equal to 0. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 1. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of two views with RPU field processing, using SbS base layer, default grid position, RPU filter mode, and each view component is coded as an mbaff frame picture.

Purpose: Check that the decoder can properly decode two view components with RPU field processing for mbaff frame pictures.

#### 10.6.6. xx.1 Test bitstream #MFCMFM-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mfc_format_idc is equal to 1. default_grid_position_flag is equal to 1. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of two views with TaB base layer MFC format configuration, using default grid position and RPU filter mode.

Purpose: Check that the decoder can properly decode two view components with TaB MFC format configuration.

#### 10.6.6. xx.1 Test bitstream #MFCGRD-1

Specification: All slices are coded as I, P or B slices. Only the first picture is coded as an IDR access unit. Each view component contains only one slice. num_views_minus1 is equal to 1. mfc_format_idc is equal to 0. default_grid_position_flag is equal to 0. rpu_filter_enabled_flag is equal to 1. rpu_field_processing_flag is equal to 0. All NAL units are encapsulated into the byte stream format specified in Annex B in ITU-T Rec. H.264 | ISO/IEC 14496-10.

Functional stage: Decoding of two views with non-default grid position configuration, using SbS base layer and RPU filter mode and

Purpose: Check that the decoder can properly decode two view components with non-default grid positions.

Add the following after Table XX:

Table XXX – Bitstreams for the MFC High profile

| Categories | Bitstream | Donated by | File Name | MFC High | Level | Frame Rate (Frame/Sec) |
| --- | --- | --- | --- | --- | --- | --- |
| RPU Operational Modes | MFCRFT-1 | Dolby |  | X | 4.1 | 24p |
|  | MFCRFT-2 | Dolby |  | X | 4.1 | 24p |
| RPU Interlaced Coding Tools | MFCFLD-1 | Dolby |  | X | 4.1 | 50i |
|  | MFCFLD-2 | Dolby |  | X | 4.1 | 50i |
| MFC Format | MFCMFM-1 | Dolby |  | X | 4.1 | 24p |
| Grid Position | MFCGRD-1 | Dolby |  | X | 4.1 | 24p |

[Ed.Note: The above table will be revised during bitstream exchange and bitstream generation.]