| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0234 |
| --- | --- |

| Title: | 3D-CE6.h related: Cross check on applying depth look-up table to Intra modes of depth map (JCT3V-E0157) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao Zhongguancun Bldg 27 Zhongguancun Road Beijing, 100080 (China) | Tel: Email: | 86-10-5776-0696 xinzhao@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the cross check results of JCT3V-E0157, “CE 6.h related: Applying Depth Look-up Table to Intra Modes of Depth Map”, proposed by LG Electronics. Crosschecker confirmed that the coding results obtained in this cross-verification match with those provided by the proponents.

# Introduction

In JCT3V-E0157 [1], for Intra depth CU which is not coded by SDC modes, an alternative residual generation method is proposed instead of directly calculating residual as the difference of original and prediction pixel value.

To generate residual value for each position within a residual block, it is proposed to firstly convert both original and prediction pixel values of this position to index values by Depth Look-up Table (DLT), and then the difference between these two index values is derived as the residual value of this position. Whether the proposed method is applied is signalled by a proposed one-bit flag for each Intra depth CU which is not coded by SDC modes.

# Crosscheck Results

As a cross-checker, we received the source code from LG Electronics. The source code is compiled and tested on Linux 64-bit OS with below macro settings,

#define LGE_APPLY_DLT_INTRA_DEPTH_CODING  1

#if LGE_APPLY_DLT_INTRA_DEPTH_CODING

#define LGE_APPLY_DLT_INTRA_ENC_SIMP      0

#define LGE_APPLY_DLT_INTRA_CABAC_SIMP    1

#endif

Simulations are performed under both common test condition (CTC) and All-Intra configuration as specified in JCT3V-D1105 [2], and detailed results are listed in below Table 1 and Table 2, respectively.

Table 1: Simulation results of JCT3V-E0157 under CTC

Table 2: Simulation results of JCT3V-E0157 under All-Intra configuration

Moreover, the proposed method is tested on top of the method proposed in JCT3V-E0156, which is applied for inter-coded depth CU. The source code is compiled and tested on Linux 64-bit OS with below additional macro settings,

#define LGE_INTER_SDC                     1

#if LGE_INTER_SDC

#define LGE_INTER_SDC_NON0_ONLY           1

#define LGE_INTER_SDC_PRECISION           1

#define LGE_INTER_SDC_MRSAD               1

#endif

Simulations are performed under common test condition (CTC) as specified in JCT3V-D1100 [3], and detailed results are listed in below Table 3.

Table 3: Simulation results of JCT3V-E0157 + JCT3V-E0156 under CTC

# Conclusions

The results of this crosscheck match with those provided in JCT3V-E0157. The cross-checker confirmed that the source code is consistent with the proposal and specification changes. There is some deviation on the runtime measurements for encoder and decoder, which is probably caused by different test environments.

# References

- H. Liu, J. Jia, "CE 6.h related: Applying Depth Look-up Table to Intra Modes of Depth Map", JCT3V-E0157, Vienna, AU, July 2013.

- P. Merkle, "Description of Core Experiment 5 (CE5) on Depth Bi-partition Modes", JCT3V-D1105, Incheon, KR, April 2013.

- D. Rusanovskyy, K. Mueller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-D1100, Incheon, KR, April 2013.