| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0316_r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0239_r2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Initial inter-layer reference picture list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V and JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Usman Hakeem Färögatan 6 16480 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com usman.hakeem@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution addresses handling of inter-layer reference pictures in the initial reference picture list construction in MV-HEVC, 3D-HEVC, and SHVC. Two reference picture sets are created, which are inserted at predefined positions making it possible to minimize use of RPLM commands. Two options have been proposed. In option 1, the reference picture sets are inserted at predefined positions. In option 2, a position for one of the two defined reference picture sets is signaled in the slice header.

# Introduction

In order to save bits for signaling of reference picture list modification commands, it is important that the initial reference picture list construction provides the order of reference pictures similar to the desired reference picture list. It is also desired to have a procedure that is generic and can be applied for both 3D and scalable extensions. However, the desirable order of pictures may differ depending on the configuration extension (scalable or 3D). For example, in case of 3D, the correlation will be closer from the views, which are spatially closer to the current view (see Figure 1). In case of scalable extension, the prediction is usually done from the spatially closer layer.

Figure 1 In case of 3D, pictures with view_id closer to the current view have better correlation with it

In order to have a flexible initialization procedure, which are applicable for both 3D and scalable extensions, the following is proposed:

- Inter-layer reference pictures are split into two parts:

  - MV-HEVC: part0 contains  pictures with viewId to the left from the current view, part1 can contain pictures to the right from the current view

  - SHVC: part0 contains picture(s) from lower layers, part1 is empty

- These two parts are inserted at different locations in initial RefPicList0 and RefPicList1 (similar to the current short-term positive and negative pictures)

- Division of pictures into two set and sorting inside each set is done in the encoder and signaled to the decoder

There are two proposed alternatives regarding positions of these two sets in initial RPL:

- Positions are predetermined (Option 1)

- Positions for part0 in list 0 and for part 1 in list 1 are signaled (Option2)

A general illustration of this approach (Option 1) is shown in Figure 2, while Figure 3 shows how this approach looks on the Common Test Conditions.

Figure 2 Example of initial reference picture list construction with the proposed approach (Option 1)

Figure 3 Example of initial reference picture list construction with the proposed approach for MV-HEVC common test conditions. RefPicSetIL1 is empty on MV-HEVC Common Test Conditions (PIP configuration).

# Proposed methods

## Option 1

Step 1:  Two values for the number of pictures num_inter_layer_ref_pics_0 (set0) and num_inter_layer_ref_pics_1 (set1) are signaled in the slice header, which split values in the inter_layer_pred_layer_idc[ i ] into two parts.

Step 2: Inter-layer reference pictures corresponding to these two sets are inserted to the RefPicListTemp0 and RefPicListTemp1 in the following order

RefPicListTemp0:

RefPicSetStCurrBefore, RefPicSetInterLayer0, RefPicSetStCurrAfter, RefPicSetLtCurr, RefPicSetInterLayer1.

RefPicListTemp1 as in the following:

RefPicSetStCurrAfter,  RefPicSetInterLayer1, RefPicSetStCurrBefore, RefPicSetLtCurr, RefPicSetInterLayer0.

### Proposed syntax and semantics modifications

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_0 | u(v) |
| if (NumDirectRefLayers[ nuh_layer_id ] > num_inter_layer_ref_pics_0) |  |
| num_inter_layer_ref_pics_1 | u(v) |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| … |  |

- G.7.4.7.1 General slice segment header semantics

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictures that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0  | |  !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

num_inter_layer_ref_pics_0 specifies the number of pictures NumInterLayerRefPics0 in the InterLayerRefPicSet0. The length of the num_inter_layer_ref_pics_0 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] + 1) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] , inclusive. When not specified,  the value of num_inter_layer_ref_pics_0  is inferred equal to NumDirectRefLayers[ nuh_layer_id ].

num_inter_layer_ref_pics_1 specifies the number of pictures in the InterLayerRefPicSet1. The length of the num_inter_layer_ref_pics_1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] − num_inter_layer_ref_pics_0 + 1) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to ( NumDirectRefLayers[ nuh_layer_id ] − num_inter_layer_ref_pics_0, inclusive. When not specified, the value of num_inter_layer_ref_pics_1 is inferred equal to 0.

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0  | |  !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_0 + num_inter_layer_ref_pics_1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

- G.8.3.4	Decoding process for reference picture lists construction

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	if( InterRefEnabledInRPLFlag ) {
		for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0; i < NumInterLayerRefPics0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ]  
		for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(Error: Reference source not found-1)
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	}	else{
		for( i = 0; i < NumInterLayerRefPics0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ]  
	}

for( i = NumInterLayerRefPics0; i < NumActiveRefLayerPics; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ]  
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++)			(Error: Reference source not found-2)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	if( InterRefEnabledInRPLFlag ) {
		for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = NumInterLayerRefPics0; i< NumActiveRefLayerPics; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
		for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(Error: Reference source not found-3)
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	}
	else{
		for( i = NumInterLayerRefPics0; i< NumActiveRefLayerPics; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
	}
	for( i = 0; i< NumInterLayerRefPics0; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(Error: Reference source not found-4)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

## Option 2

Step 1 (same as in Option1):  Two values for the number of pictures num_inter_layer_ref_pics_0 (set0) and num_inter_layer_ref_pics_1 (set1) are signaled in the slice header, which split values in the inter_layer_pred_layer_idc[ i ] into two parts.

Step 1 is exactly the same as in the Option 1.

Step 2: Positions for RefPicSetInterLayer0 in RefPicListTemp0 and for RefPicSetInterLayer1 in RefPicListTemp1 are signaled. If the positions are not signaled (e.g. position 0 is signaled) then the positions take the default positions defined as in Option 1.

Step 3: Inter-layer reference pictures corresponding to these two sets are inserted to the RefPicListTemp0 and RefPicListTemp1 in the following order

RefPicListTemp0:

RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetLtCurr, RefPicSetInterLayer1.

Then RefPicSetInterLayer0 is inserted at a specified position.

RefPicListTemp1:

RefPicSetStCurrAfter,  RefPicSetStCurrBefore, RefPicSetLtCurr, RefPicSetInterLayer0.

Then RefPicSetInterLayer1 is inserted at a specified position.

### Proposed syntax and semantics modifications

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_0 | u(v) |
| if (NumDirectRefLayers[ nuh_layer_id ] > num_inter_layer_ref_pics_0) |  |
| num_inter_layer_ref_pics_1 | u(v) |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| … |  |
| if( slice_type != I_SLICE && nuh_layer_id > 0 && num_inter_layer_ref_pics_0) |  |
| inter_layer_ref_pics_0_position_L0_plus1 | ue(v) |
| if( slice_type = = B_SLICE && nuh_layer_id > 0 && num_inter_layer_ref_pics_1) |  |
| inter_layer_ref_pics_1_position_L1_plus1 | ue(v) |

- G.7.4.7.1 General slice segment header semantics

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictures that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0  | |  !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

num_inter_layer_ref_pics_0 specifies the number of pictures NumInterLayerRefPics0 in the InterLayerRefPicSet0. The length of the num_inter_layer_ref_pics_0 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] + 1) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] , inclusive. When not specified,  the value of num_inter_layer_ref_pics_0 is inferred equal to NumDirectRefLayers[ nuh_layer_id ].

num_inter_layer_ref_pics_1 specifies the number of pictures NumInterLayerRefPics1 in the InterLayerRefPicSet1. The length of the num_inter_layer_ref_pics_1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] − num_inter_layer_ref_pics_0 + 1) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to ( NumDirectRefLayers[ nuh_layer_id ] − num_inter_layer_ref_pics_0), inclusive. When not specified, the value of num_inter_layer_ref_pics_1 is inferred equal to 0.

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0  | |  !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_0 + num_inter_layer_ref_pics_1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

…

inter_layer_ref_pics_0_position_L0_plus1 specifies the starting position of the pictures in the inter-layer reference picture set0 in reference picture list 0 after reference picture list initialization. The range of values of inter_layer_ref_pics_0_position_L0_plus1 is 0 to Min( num_ref_idx_l0_active_minus1 + 1, NumPocStCurrBefore + NumPocStCurrAfter + NumRefPicSetLtCurr + num_inter_layer_ref_pics_1 –  1), inclusive. For non-zero values, inter_layer_ref_pics_0_position_L0_plus1 –  1 denotes the starting position of inter-layer reference pictures from inter_layer_ref_pics_0 in the initial reference picture list.

The variable ILRefSet0StartL0 is derived as follows:

if( ! inter_layer_ref_pics_0_position_L0_plus1)
	ILRefSet0StartL0 = NumPocStCurrBefore 
else
	ILRefSet0StartL0 = inter_layer_ref_pics_0_position_L0_plus1  –  1 +  (inter_layer_ref_pics_0_position_L0_plus1 –  1  < NumPocStCurrBefore ? 0 : 1)

inter_layer_ref_pics_1_position_L1_plus1 specifies the starting position of the pictures in the inter-layer reference picture set1 in reference picture list 1 after reference picture list initialization. The range of values of inter_layer_ref_pics_1_position_L1_plus1 is 0 to Min( num_ref_idx_l1_active_minus1 + 1, NumPocStCurrBefore + NumPocStCurrAfter + NumRefPicSetLtCurr + num_inter_layer_ref_pics_1 –  1), inclusive. For non-zero values, inter_layer_ref_pics_1_position_L1_plus1 –  1 denotes the starting position of inter-layer reference pictures from inter_layer_ref_pics_1 in the initial reference picture list.

The variable ILRefSet1StartL1 is derived as follows:

if( ! inter_layer_ref_pics_1_position_L1_plus1)
	ILRefSet1StartL1 = NumPocStCurrAfter 
else
	ILRefSet0StartL0 = inter_layer_ref_pics_1_position_L1_plus1  –  1 +  (inter_layer_ref_pics_1_position_L1_plus1 –  1  < NumPocStCurrAfter ? 0 : 1)

- G.8.3.4	Decoding process for reference picture lists construction

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 – NumInterLayerRefPics0) {
	if( InterRefEnabledInRPLFlag ) {
		for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(Error: Reference source not found-5)
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	}

for( i = NumInterLayerRefPics0; i < NumActiveRefLayerPics; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ]  
}

for( cIdx = NumRpsCurrTempList0 – 1; cIdx >= ILRefSet0StartL0 + NumInterLayerRefPics0; cIdx - - )
	RefPicListTemp0[ cIdx ] = RefPicListTemp0[ cIdx – NumInterLayerRefPics0]

for( i = 0; cIdx = ILRefSet0StartL0; i < NumInterLayerRefPics0; cIdx+ +, i+ + )
	RefPicListTemp0[ cIdx ] = RefPicSetInterLayer[ i ]

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++)			(Error: Reference source not found-6)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 – NumInterLayerRefPics1 ) {
	if( InterRefEnabledInRPLFlag ) {
		for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(Error: Reference source not found-7)
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	}
	for( i = 0; i < NumInterLayerRefPics0; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

for( cIdx = NumRpsCurrTempList1 – 1; cIdx >= ILRefSet1StartL1 + NumInterLayerRefPics1; cIdx - - )
	RefPicListTemp1[ cIdx ] = RefPicListTemp1[ cIdx – NumInterLayerRefPics1]

for( i = NumInterLayerRefPics0; cIdx = ILRefSet1StartL1; i < NumActiveRefLayerPics; cIdx+ +, i+ + )
	RefPicListTemp1[ cIdx ] = RefPicSetInterLayer[ i ]

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(Error: Reference source not found-8)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

# Results

The numerical results according to the document JCT3V-D0314 [2] are provided in the following.

## Bit count in slice header

The following results for counting the bits in the slice header have been provided. The simulations have been done according to the conditions described in the JCT3V-D0314. The software has been implemented based on HTM-DEV1.0 and SHM2.0. The HTM-DEV1.0 software has been used in the experiments since it contains the syntax for inter_layer_pred_layer_idc[ i ] that is used in the proposal.

Tables 1 – 5 provide the total RPL related bits in slice header.

The following syntax elements have been included in the bit count (some elements are present in the anchor or a certain proposal only, which is specified in brackets):

num_ref_idx_active_override_flag

num_ref_idx_l0_active_minus1

num_ref_idx_l1_active_minus1

num_inter_layer_ref_pics_minus1				(Anchor only)

num_inter_layer_ref_pics_0					(Option 1 and 2)

num_inter_layer_ref_pics_1					(Option 1 and 2)

inter_layer_pred_layer_idc[ i ]

inter_layer_ref_pics_0_position_L0_plus1			(Option 2 only)

inter_layer_ref_pics_1_position_L1_plus1			(Option 2 only)

ref_pic_list_modification_flag_l0

list_entry_l0[ i ]

ref_pic_list_modification_flag_l1

list_entry_l1[ i ]

Table 1 HTM-DEV1.0 (MV-HEVC), PIP configuration, Undo Dancer (QP 25)

|  | Anchor PIP | PIP Op1 | PIP Opt1 (%) | PIP Op2 | PIP Op2 (%) |
| --- | --- | --- | --- | --- | --- |
| Layer 0 | 363 | 363 | 100.0% | 363 | 100.0% |
| Layer 1 | 2858 | 1889 | 66.1% | 1529 | 53.5% |
| Layer 2 | 2858 | 1889 | 66.1% | 1529 | 53.5% |
| Total | 6079 | 4141 | 68.1% | 3421 | 56.3% |

Table 2 HTM1.0-DEV (MV-HEVC), IBP configuration, Undo Dancer (QP 25),

|  | Anchor IBP | IBP Op1 | IBP Op1 (%) | IBP Op2 | IBP Op2 (%) |
| --- | --- | --- | --- | --- | --- |
| Layer 0 | 363 | 363 | 100.0% | 363 | 100.0% |
| Layer 1 | 2858 | 1889 | 66.1% | 1529 | 53.5% |
| Layer 2 | 5785 | 4468 | 77.2% | 3282 | 56.7% |
| Total | 9006 | 6720 | 74.6% | 5174 | 57.5% |

Table 3 SHM2.0, RA configuration, Kimono 2x QP26

|  | Anchor RA | RA Op1 | RA Op1 (%) | RA Op2 | RA Op2 (%) |
| --- | --- | --- | --- | --- | --- |
| Layer 0 | 350 | 350 | 100.0% | 350 | 100.0% |
| Layer 1 | 5442 | 4677 | 85.9% | 4354 | 80.0% |
| Total | 5792 | 5027 | 86.8% | 4704 | 81.2% |

Table 4 SHM2.0, LD-B configuration, Kimono 2x QP26

|  | Anchor ld-main | ld-main Op1 | ld-main Op1 (%) | ld-main Op2 | ld-main Op2 (%) |
| --- | --- | --- | --- | --- | --- |
| Layer 0 | 253 | 253 | 100.0% | 253 | 100.0% |
| Layer 1 | 3091 | 3091 | 100.0% | 3331 | 107.8% |
|  | 3344 | 3344 | 100.0% | 3577 | 107.2% |

Table 5 SHM2.0, LD-P, Kimono 2x QP26

|  | ld-p Anchor | ld-p Op1 | ld-p Op1(%) | ld-p Op2 | ld-p Op2 (%) |
| --- | --- | --- | --- | --- | --- |
| Layer 0 | 246 | 246 | 100.0% | 246 | 100.0% |
| Layer 1 | 1666 | 1666 | 100.0% | 1906 | 114.4% |
|  | 1912 | 1912 | 100.0% | 2152 | 112.6% |

# Results for proposal without additional syntax

As the result of the BoG discussion, it was stated that the proposal without signaling the additional syntax elements is more desirable such as JCT3V-D0220 from the Incheon meeting, which is similar to Option 1 of the current proposal but does not include additional syntax. The bit count results were requested for a proposal without additional syntax elements and its comparison with the JCTVC-N0082 proposal. The results for RPLM bit count based on HTM6.2 are provided in Table 6. The proposal was implemented on top of the JCTVC-N0082 software. The software is attached to the contribution as well as the proposed changes to the specification text. The results for HTM6.2 PIP configuration and SHM2.0 are not provided since the proposal behaves identically to JCTVC-N0082 on those configurations.

Table 6 Comparison of the proposal without syntax with HTM6.2 and JCTVC-N0082

| Scalability | Coding structure | HTM6.2 [bit] | N0082 [bit] | N0082 [%] | Proposal [bit] | Proposal vs HTM6.2 [%] | Proposal vs N0082 [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MV/3D-HEVC | IBP (3t) | 7 175 | 6 206 | 86.5% | 4 370 | 60.9% | 70.4% |
| MV/3D-HEVC | IBP (3t+3d) | 14 350 | 12 412 | 86.5% | 8 740 | 60.9% | 70.4% |

Table 7 shows layer-wise comparison of the proposal versus JCTVC-N0082 on IBP (3t+3d) configuration of HTM6.2.

Table 7 Comparison of the proposal without syntax to JCTVC-N0082 (total RPL bits (patch), and RPLM bits)

| Layer | RPL patch bits N0082 | RPL patch bits proposal | RPLM bits N0082 | RPLM bits proposal |
| --- | --- | --- | --- | --- |
| layer 0 | 1743 | 1743 | 0 | 0 |
| layer 1 | 1743 | 1743 | 0 | 0 |
| layer 2 | 3209 | 3209 | 1496 | 1496 |
| layer 3 | 3209 | 3209 | 1496 | 1496 |
| layer 4 | 6453 | 4617 | 4710 | 2874 |
| layer 5 | 6453 | 4617 | 4710 | 2874 |
| Total: | 22810 | 19138 | 12412 | 8740 |

As it was mentioned, the specification text changes are attached to the contribution (see JCT3V-E0239_JCTVC-N0316_Spec_text_r1.doc)

The fill simulations on the JCT-VC common test conditions have also been performed. The proposal has been compared with HTM6.2 and with JCTVC-N0082. The result are provided in Table 8  and  Table 9.

One can see that the proposal improves relative to JCTVC-N0082 on video 2 and HTM6.2 on video2 and video 1.

Table 8 Comparison of proposal with HTM6.2 anchor (IBP configuration)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | -0.3% | -0.1% | -0.1% | -0.1% | 82.8% | 92.7% | 76.8% |
| Kendo | 0.0% | -0.1% | -0.3% | -0.1% | -0.1% | -0.1% | 81.8% | 86.7% | 88.3% |
| Newspaper_CC | 0.0% | -0.1% | -0.3% | -0.1% | -0.1% | -0.1% | 86.1% | 99.2% | 88.4% |
| GT_Fly | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 91.9% | 115.1% | 106.9% |
| Poznan_Hall2 | 0.0% | -0.1% | -0.3% | -0.1% | -0.2% | -0.2% | 84.1% | 96.2% | 90.6% |
| Poznan_Street | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | 87.5% | 100.0% | 105.9% |
| Undo_Dancer | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 90.5% | 90.9% | 94.2% |
| 1024x768 | 0.0% | -0.1% | -0.3% | -0.1% | -0.1% | -0.1% | 83.5% | 92.8% | 84.5% |
| 1920x1088 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 88.5% | 100.5% | 99.4% |
| average | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 86.4% | 97.2% | 93.0% |

Table 9 Comparison of proposal with JCTVC-N0082 as anchor (IBP configuration)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | 87.4% | 83.4% | 74.8% |
| Kendo | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | 78.9% | 90.3% | 86.1% |
| Newspaper_CC | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | 86.9% | 103.7% | 88.3% |
| GT_Fly | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 86.3% | 112.0% | 106.0% |
| Poznan_Hall2 | 0.0% | 0.0% | -0.3% | -0.1% | -0.1% | -0.1% | 83.1% | 95.6% | 90.5% |
| Poznan_Street | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 93.4% | 105.4% | 107.6% |
| Undo_Dancer | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 90.0% | 98.4% | 107.5% |
| 1024x768 | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | -0.1% | 84.4% | 92.5% | 83.0% |
| 1920x1088 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 88.2% | 102.8% | 102.9% |
| average | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 86.6% | 98.4% | 94.4% |

# Conclusions

The proposed changes to the initial picture list construction result provide reference picture list construction that is suitable for both MV-HEVC and SHVC scalable extension. The procedure is generic for the reference picture list extensions and allows high flexibility in reference picture list construction The bit count savings are reported.  It is proposed to adopt the described modifications to the reference picture list constructions to the next version of MV-HEVC, SHVC, and 3D-HEVC text drafts and the next version of HTM and SHM reference software.

It is suggested to adopt the proposal without additional syntax elements to the next version of MV-HEVC and 3D-HEVC test drafts and the reference software.

# References

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCTV3V-D1100, Incheon, KR, Apr. 2013

- A. Norkin, "Report of BoG on guidelines for evaluation of reference picture list modification proposals",  JCT3V-D0314, Incheon, Korea, Apr. 2013

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).