| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0240 |
| --- | --- |

| Title: | On 3D reference displays information SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 16480 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes changes to the 3D reference displays SEI message. The proposed changes are related to the persistence scope of the SEI message and the note addressing the use of the SEI message.

# Introduction

The 3D reference SEI message has been adopted at the Incheon meeting and included into the MV-HEVC specification [1]. There are still open issues related to the persistence scope of the SEI message and the informative note describing the use of the SEI. This document proposes changes to the SEI text that address the two aforementioned issues.

# Proposed changes to the text

This section includes the proposed changes to the MV-HEVC draft text. The changes are marked with editing tracking.

G.14.2.1 3D reference display information SEI message semantics.

A reference displays information SEI message contains information about the reference display width(s) and reference viewing distance(s) as well as information about the corresponding  reference stereo-pair(s), i.e. the pair(s) of views to be displayed for the viewer's left and right eyes on the reference display at reference viewing distance. This information enables a view renderer to generate a proper stereo-pair for the target screen width and the viewing distance. The reference display width and viewing distance values are signalled in units of centimetres. The reference pair of views specified in this SEI message can be used to extract or infer parameters related to the distance between the camera centers in the reference stereo-pair, which can be used for generation of views for the target display. For multi-view displays, the reference stereo-pair corresponds to a pair of views that can be simultaneously observed by the viewer's left and right eyes.

When present, this SEI message shall be associated with IRAP access units. This SEI message can also be associated with a non-IRAP access unit, if all access units that follow this access unit in the decoding order, also follow it in the display order. The 3D reference display information message should be applied for the access unit, it is associated with and the access units which follow this access unit in both the display and decoding order until the next IRAP access unit or the next access unit containing a 3D reference displays information SEI message.

NOTE 1  – The reference displays information SEI message specifies display parameters for which the 3D sequence was optimized and the corresponding reference parameters. Each reference display (i.e. a reference display width and possibly a corresponding viewing distance) is associated with one reference pair of views by signalling their ViewId. The difference between the values of ViewId is referred to as the baseline distance (i.e. the distance between the centers of the cameras used to obtain the video sequence).

The following equations can be used for determining the baseline and horizontal shift for the receiver's display when the ratio between the receiver's viewing distance and the reference viewing distance is the same as the ratio between the receiver screen width and the reference screen width:

baseline = refBaseline[ i ] * ( refDisplayWidth [ i ]÷ displayWidth )

shift = refShift[ i ] * ( refDisplayWidth [ i ]÷ displayWidth )

where refBaseline[ i ] is equal to right_view_id[ i ] - left_view_id[ i ] signalled in this SEI message. Other parameters related to the view generation may be obtained determined by using a similar equation.

parameter = refParameter[ i ] * ( refDisplayWidth[ i ] ÷ displayWidth[ i ] ),

where refParameter [i] is a parameter related to view generation that corresponds to the reference pair of views signalled by left_view_id[ i ] and right_view_id[ i ]. In the above equations, the width of the visible part of the display used for showing the video sequence should be understood under "display width". The same equations can also be used for determining the pair of views, horizontal shift or/and other view synthesis parameters when the viewing distance is not scaled proportionally to the screen width compared to the reference display parameters. In this case, the effect of applying the above equations would be to keep the perceived depth in the same proportion to the viewing distance as in the reference setup.

When the view synthesis related parameters that correspond to the reference stereo-pair change from one access unit to another, they should be scaled with the same scaling factor as the parameters in the access unit that the SEI message is associated with. Therefore, the above formulas should also be applied to obtain the parameters for a following access unit, where the refParameter is the parameter related to the reference stereo-pair associated the following access unit.  The horizontal shift for the receiver's display should also be modified by scaling it with the same factor as that used to scale the other view synthesis parameters).

# Conclusions

It is proposed to adopt the changes to the SEI message text and integrate them in the next version MV-HEVC draft text.

# References

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, "MV-HEVC Draft Text 4", JCT3V-D1004, Incheon, KR, Apr. 2013.

- A. Norkin, "3D reference displays information SEI message", JCT3V-D0218, Incheon, KR, Apr. 2013.

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).