| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0245 |
| --- | --- |

| Title: | 3D-AVC: Report on coding performance of the depth coding tools for the 3D-AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *Gun Bang, **Kyung Yong Kim, **Jung Hyun Kim, **Young Su Heo, **Gwang Hoon Park, *Gwang Soon Lee, *Namho Hur *ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea **Media Lab, Kyung Hee University, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea Ying Chen, Xin Zhao, Li Zhang, Marta Karczewicz, 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +82-42-860-5069 {*gbang, *gslee, *namho} @etri.re.kr +82-31-201-3680 {**kimky, **heyheyhey, **medist, **ghpark} @khu.ac.kr +1-858-845-6589 cheny@qti.qualcomm.com +86-10-5776-0696 xinzhao@qti.qualcomm.com +1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | ETRI, Kyung Hee University and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, all depth coding tools have been tested under the CTC and other configurations. To evaluate the depth coding tools such as DISP, PSIP and IVMP, we prepare additionally full resolution depth case and the texture first coding order configuration. In addition, DISP uses just only AVC deblocking without any changing process and VSO impact are also considered in this contribution.

In the result of DISP with AVC deblocking and VSP off case, the efficient coding gain and the complexity reduction could be investigated under CTC configuration and the texture first coding order with MB-level NBDV, as proposed in [3]. The result of this contribution shows that the DISP following AVC deblocking and IVMP for depth coding can improve coding gain and the depth coding tools are very useful especially for the full resolution depth coding.

# Test Conditions and Simulation Results

Both the common test conditions [2] and texture first coding order condition are used in simulations. For texture first coding order, the coding order is configured to be “T0D0T1D1T2D2”. Test sets are shown in the following Error: Reference source not found1.

Table 1. Test sets

|  | Anchor | Depth coding tools | VSO | Depth resolution | Coding order |
| --- | --- | --- | --- | --- | --- |
| Test 1 | 3DV-ATM8.0 | DISP (with AVC deblocking) and IVMP | On | Half | CTC |
| Test 2 | 3DV-ATM8.0 | DISP (with AVC deblocking) and IVMP | On | Full | CTC |
| Test 3 | 3DV-ATM8.0 | DISP (with AVC deblocking) and IVMP | Off | Half | CTC |
| Test 4 | 3DV-ATM8.0 | DISP (with AVC deblocking) and IVMP | Off | Full | CTC |
| Test 5 | MVC+D | NBDV and DISP (with AVC deblocking) and IVMP | On | Half | Non-CTC (T0D0T1D1T2D2) |
| Test 6 | MVC+D | NBDV and DISP (with AVC deblocking) and IVMP | On | Full | Non-CTC (T0D0T1D1T2D2) |
| Test 7 | MVC+D | NBDV and DISP (with AVC deblocking) and IVMP | Off | Half | Non-CTC (T0D0T1D1T2D2) |
| Test 8 | MVC+D | NBDV and DISP (with AVC deblocking) and IVMP | Off | Full | Non-CTC (T0D0T1D1T2D2) |

In VSO off case, the slightly higher gains are investigated in the summary results of Table 2. And 1.48% gain and 5.63% gain are shown in half resolution depth (Test 3) and full resolution depth (Test 4), respectively. When compared to MVC+D, the whole package including depth coding tools could provide 18%-21% coding gain for synthesized views when texture first coding is enabled.

Table 2. Summary of the results

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesized PSNR) |  | Complexity estimate (ratio to anchor) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR | dPSNR | dBR | dPSNR | dBR | dPSNR | dBR | dPSNR | Enc. time | Dec. time |
| Test.1 | 0.26 | -0.01 | -2.25 | 0.16 | -1.41 | 0.06 | -1.14 | 0.04 | 99% | 103% |
| Test.2 | 0.20 | -0.01 | -5.34 | 0.30 | -5.14 | 0.20 | -4.93 | 0.19 | 98% | 104% |
| Test.3 | 0.15 | -0.01 | -7.74 | 0.43 | -1.51 | 0.06 | -1.36 | 0.05 | 100% | 100% |
| Test.4 | 0.10 | 0.00 | -12.20 | 0.69 | -5.57 | 0.22 | -5.50 | 0.21 | 100% | 103% |
| Test.5 | -20.86 | 0.93 | -11.13 | 0.83 | -21.51 | 0.96 | -19.22 | 0.79 | 113 | 112 |
| Test.6 | -21.10 | 0.94 | -10.97 | 0.76 | -23.18 | 1.03 | -20.90 | 0.86 | 111 | 113 |
| Test.7 | -21.08 | 0.94 | -15.64 | 1.03 | -21.40 | 0.94 | -18.90 | 0.77 | 111 | 113 |
| Test.8 | -21.26 | 0.95 | -17.77 | 1.19 | -23.20 | 1.02 | -20.95 | 0.86 | 112 | 111 |

# Conclusions

We recommend JCT-3V to reconsider the efficient 3D-AVC depth-map coding tools, such as DISP (without PSIP) with AVC deblocking and IVMP that can improve the coding efficiency with very marginal complexity burden, to be included into the 3D-AVC standard.

# References

- ISO/IEC JTC1/SC29/WG11, “Text of ISO/IEC 14496-10:2012/DAM3 AVC compatible video-plus-depth extension,” N13555, Incheon, KR, April 2013.

- D. Rusanovskyy, K. Müller and A. Vetro, “Common Test Conditions of 3DV Core Experiments,” The 4th JCT-3V Meeting, JCT3V-D1100, Incheon, KR, April 2013.

- X. Zhao, Y. Chen, L. Zhang, J. Kang, Y.-K. Wang, R. Joshi, M. Karczewicz, “CE7: MB-level NBDV for 3D-AVC,” The 5th JCT-3V Meeting, JCT3V-E0136, Vienna, AU, July 27–Aug. 2, 2013.

# Patent rights declaration(s)

ETRI and KyungHee University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).