| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0246 |
| --- | --- |

| Title: | 3D-CE1.h related: Improvement of View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

3D-HEVC supports view synthesis prediction (VSP), where prediction image is synthesized by sub-PU based disparity compensation. In this contribution, it is proposed to synthesize prediction images by utilizing both inter-view and temporal correlations. Experiments reportedly show the average bitrate reduction of 0.06% and 0.01% for coded and synthesis views, respectively. The maximum bitrate reduction of 0.78% on a dependent view is achieved for the GT_Fly sequence.

# Introduction

The current test model of 3D-HEVC support view synthesis prediction (VSP) [1]. VSP is a concept where the synthesized picture is utilized as a prediction picture. In the current design, a picture is synthesized by utilizing inter-view correlations. However, it is also possible to synthesize a picture by utilizing temporal correlations.

In this contribution, in order to improve VSP, it is proposed to synthesize a prediction picture by utilizing both spatial and temporal correlations.

# Proposal

The proposed method generates prediction signals by averaging motion compensated prediction (MCP) and disparity compensated prediction (DCP), which is already performed for every sub-PU (4x4) in the current VSP design. For motion compensated prediction, it is necessary to derive motion parameters. In order to avoid encoding additional information, motion parameters are also derived from reference views.

Figure 1 shows the concept of the proposed view synthesis prediction by using both DCP and MCP. Both compensations are performed every sub-PU, 4x4 in the current design. There is no change on the signalling method and stored data on VSP coded PUs.

Figure 1: Concept of the proposed view synthesis prediction

# Experimental results

The proposed methods are built on the top of the reference software 3DV-HTM v7.0r1. And the experiments are conducted according to the common test condition [2].

TABLE 1 shows the simulation results by the proposed VSP improvement. It can be seen that the proposed method can achieve some improvements except for Undo_Dancer sequence.

TABLE 1: HTM v7.0r1 vs. proposed adaptive partitioning for 2nd DCP in VSP

# Conclusion

This contribution proposes an improvement of view synthesis for VSP. The experimental results show the average bitrate reduction of 0.06% and 0.01% for coded and synthesis views, respectively. Although the current improvement is minor, but this is the preliminary work and it might be possible to improve by utilizing temporal correlations in VSP. It is suggested to include this concept in the next round of CE.

# Patent rights declaration

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “3D-HEVC Test Model 4”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-D1005, June 2013, Incheon, Korea.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-D1100, April 2013, Incheon, Korea.