| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AU, 29 July – 2 August 2013 | Document: JCT3V-E0247 |
| --- | --- |

| Title: | 3D-HEVC: Crosscheck on Proposal JCT3V-E0124 by Qualcomm /Peking University |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Dong Tian (MERL) 201 Broadway, 8th floor Cambridge, MA 02139 | Tel: Email: | +1-617-621-7556 tian@merl.com |
| Source: | Mitsubishi Electric Research Labs (MERL) |  |  |

_____________________________

# Abstract

This document reports the outcome of cross-verification of JCT3V-E0124, "Further improvements on advanced residual prediction", performed by Mitsubishi Electric Research Labs (MERL). The coding/synthesis results obtained for this cross-verification match those provided by the proponents. Crosscheckers confirmed that the software and the text provided by the proponent are consistent.

# Introduction

In contribution JCT3V-E0124 [1], several improvements are proposed to enhance the advanced residual prediction (ARP). Upon request, MERL performed the crosschecking on four test cases presented in the proposal.

# Performance of proposed method

Source code of the proposal was provided and the crosscheckers confirm that the software is consistent with the proposal. The proposal is consisted of four aspects.

1) Inter-view residual prediction. Extend the ARP to the inter-view coded picture such that the inter-view residual is predictively coded using a previous inter-view coded picture. The modification corresponds to a compiling switch QC_ARP_INTERVIEW.

2) Disparity refinement. The disparity vector is always derived from NBDV process in ARP. With the improvement, if reference CU in the temporal reference picture is coded in inter-view mode, use the disparity vector of the reference CU as a finer disparity vector. This modification corresponds to a compiling switch QC_ARP_DVREFINE.

3) Harmonization with IC. With the modification, ic_flag is not signaled when ARP weighting factor is not equal to zero. This modification corresponds to a compiling switch QC_ARP_IC_HARMONIZE.

4) One additional reference PU to be checked. In the first two modifications of this proposal, only the reference CU (CR) that covers the center sample of the reference block is used to return a temporal MV or spatial DV. With the fourth modification, one additional reference PU (BR) that covers the bottom right sample will be checked if the CR does not have MV or DV information. This modification corresponds to a compiling switch QC_ARP_CT_BR.

The four test cases that are cross checked will be described below, respectively.

Test case 1 (Full proposal vs. Anchor, CTC)

With this test, the HTM 7.0r1 is used to create the anchor bitstreams under CTC. For the proponent software, all the modifications are turned ON using compiling switches. The results obtained by proponents and crosscheckers are shown in Table 1.a and 1.b; which are matched.

Test case 2 (Proposal w/ No. 1 modification vs. Anchor, CTC)

With this test, the HTM 7.0r1 is used to create the anchor bitstreams under CTC. For the proponent software, only the modification for interview residual prediction is enabled (QC_ARP_INTERVIEW). The results obtained by proponents and crosscheckers are shown in Table 2.a and 2.b; which are matched.

Test case 3 (Proposal w/ No. 2 modification vs. Anchor, CTC)

With this test, the HTM 7.0r1 is used to create the anchor bitstreams under CTC. For the proponent software, only the modification for refined disparity is enabled (QC_ARP_DVREFINE). The results obtained by proponents and crosscheckers are shown in Table 3.a and 3.b; which are matched.

Test case 4 (Full proposal except No. 4 modification vs. Anchor, CTC)

With this test, the HTM 7.0r1 is used to create the anchor bitstreams under CTC. For the proponent software, all the modifications are turned ON using compiling switches, except the switch for the fourth modification (QC_ARP_CT_BR). The results obtained by proponents and crosscheckers are shown in Table 4.a and 4.b; which are matched.

Table 1.a Proponent’s results (Full proposal vs. Anchor, CTC)

Table 1.b. Crosschecker’s results (Full proposal vs. Anchor, CTC)

Table 2.a Proponent’s results (Proposal w/ No. 1 modification vs. Anchor, CTC)

Table 2.b. Crosschecker’s results (Proposal w/ No. 1 modification vs. Anchor, CTC)

Table 3.a Proponent’s results (Proposal w/ No. 2 modification vs. Anchor, CTC)

Table 3.b. Crosschecker’s results (Proposal w/ No. 2 modification vs. Anchor, CTC)

Table 4.a Proponent’s results (Proposal w/o No. 4 modification vs. Anchor, CTC)

Table 4.b. Crosschecker’s results (Proposal w/o No. 4 modification vs. Anchor, CTC)

# References

- L. Zhang, Y. Chen, X. Zhao, M. Karczewicz (Qualcomm), Y. Cai, S. Ma (PKU), "Further improvements on advanced residual prediction", JCT3V-E0124, Vienna, AU, July 2013

- D. Rusanovskyy, K. Mueller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-D1100, Incheon, KR, January 2013