| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0251 |
| --- | --- |

| Title: | Cross-check of On low-delay checking process for MV-HEVC (JCT3V-E0064) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

- This contribution reports a cross-check of collocated motion vector derivation modification for MV-HEVC. Experiment results perfectly match with those provided by the proponent. The software was checked and verified that the modified low-delay checking can be carried out at picture level and the proposal follows the description.

- It is reported that the scheme shows 0.7 % and 0.7 % gain in 2 view and 3 view configuration,  respectively.

# Introduction

In the current spec, the colPic is derived from either collocated_ref_layer_idx or collocated_from_l0_flag and collocated_ref_idx. But LX component of the colPic can be decided by low-delay checking which corresponds to the following condition.

If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice

In JCT3V-E0064, in addition to the above condition, the same POC (inter-layer prediction) check is added to select more appropriate LX, which is benefitial to improve coding efficiency.

      - If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice, or if DiffPicOrderCnt( colPic, currPic ) is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xColPb ][ yColPb ], refIdxLXCol[ xColPb ][ yColPb ] and LX, respectively.

The modified low-delay checking can be carried out at picture level because it only depends on slice level parameter which is constant in a picture.

# Experimental results

Table 1. Test results of 3 view configuration (anchor: HTMdev1.0)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -5.4% | -3.7% | -2.4% | -2.4% | #VALUE! | 102.3% | 103.7% | #NUM! |
| Kendo | 0.0% | -3.5% | -2.4% | -1.5% | -1.5% | #VALUE! | 102.1% | 103.6% | #NUM! |
| Newspaper_CC | 0.0% | -0.4% | 1.7% | 0.3% | 0.3% | #VALUE! | 101.2% | 101.3% | #NUM! |
| GT_Fly | 0.0% | -0.8% | -0.3% | -0.8% | -0.8% | #VALUE! | 96.6% | 112.4% | #NUM! |
| Poznan_Hall2 | 0.0% | -3.2% | 0.3% | -0.6% | -0.6% | #VALUE! | 101.8% | 102.5% | #NUM! |
| Poznan_Street | 0.0% | -0.1% | 0.8% | 0.0% | 0.0% | #VALUE! | 100.7% | 104.5% | #NUM! |
| Undo_Dancer | 0.0% | -1.1% | 0.1% | -0.2% | -0.2% | #VALUE! | 101.9% | 104.8% | #NUM! |
| 1024x768 | 0.0% | -3.1% | -1.5% | -1.2% | -1.2% | #VALUE! | 101.9% | 102.9% | #NUM! |
| 1920x1088 | 0.0% | -1.3% | 0.2% | -0.4% | -0.4% | #VALUE! | 100.3% | 106.1% | #NUM! |
| average | 0.0% | -2.1% | -0.5% | -0.7% | -0.7% | #VALUE! | 100.9% | 104.7% | #NUM! |

Table 2. Test results of 2 view configuration (anchor: HTMdev1.0)

|  | video 0 | video 1 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -5.4% | -1.9% | -1.9% | #VALUE! |
| Kendo | 0.0% | -3.5% | -1.2% | -1.2% | #VALUE! |
| Newspaper_CC | 0.0% | -0.4% | -0.1% | -0.1% | #VALUE! |
| GT_Fly | 0.0% | -0.8% | -0.7% | -0.7% | #VALUE! |
| Poznan_Hall2 | 0.0% | -3.2% | -0.9% | -0.9% | #VALUE! |
| Poznan_Street | 0.0% | -0.1% | -0.1% | -0.1% | #VALUE! |
| Undo_Dancer | 0.0% | -1.1% | -0.2% | -0.2% | #VALUE! |
| 1024x768 | 0.0% | -3.1% | -1.1% | -1.1% | #VALUE! |
| 1920x1088 | 0.0% | -1.3% | -0.5% | -0.5% | #VALUE! |
| average | 0.0% | -2.1% | -0.7% | -0.7% | #VALUE! |

# References

[1] X. Xiu, Y. Ye, Y. He, Y. He, Y. Lin, X. Zheng, X. Chen, J. Zheng, “On low-delay checking process for MV-HEVC”, JCT3V-E0064, JCT3V 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).