| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0260 |
| --- | --- |

| Title: | 3D-AVC: Cross-check of the report on coding performance of the depth coding tools for the 3D-AVC (JCT3V-E0245) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shinya Shimizu*, Shiori Sugimoto (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | *+81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck on the coding performance of the depth coding tools for 3D-AVC in JCT3V-E0245. In this document, only texture-first coding order cases were crosschecked. The verification task was done successfully. No mismatch is identified. The simulation result reportedly shows the average bitrate reduction of 19.22% and 20.90% for synthesis views with around 15% decoding runtime increase relative to MVC+D when coding quarter and full resolution depth maps, respectively.

# Introduction

The contribution JCT3V-E0245 reported coding performance of the depth coding tools for 3D-AVC. In this crosscheck report, the simulations are conducted with the same software used in [1].

# Crosscheck Results

The simlations are conducted on the texture first coding order, i.e. T0D0T1D1T2D2. Both VSO on and off configurations are evaluated. The other configurations are follow by the common test condition [2]. TABLE 1 summarizes the test sets crosschekd in this report. The provided source code was compiled on 64-bit Windows 7 environment by using Microsoft Visual Studio 2010 Professional Edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7. Regarding the decoding time, we use the average decoding times of three decoding trials.

TABLE 1: Test sets crosschecked in this report

|  | Anchor | Depth coding tools | VSO | Depth map resolution | Coding order |
| --- | --- | --- | --- | --- | --- |
| Test 5 | MVC+D | NBDV and DISP (with AVC deblocking) and IVMP | On | Half in both dimensions | Texture first (T0D0T1D1T2D2) |
| Test 6 | MVC+D | NBDV and DISP (with AVC deblocking) and IVMP | On | Full | Texture first (T0D0T1D1T2D2) |
| Test 7 | MVC+D | NBDV and DISP (with AVC deblocking) and IVMP | Off | Half in both dimensions | Texture first (T0D0T1D1T2D2) |
| Test 8 | MVC+D | NBDV and DISP (with AVC deblocking) and IVMP | Off | Full | Texture first (T0D0T1D1T2D2) |

No mismatch is identified between encoder and decoder. The coding results matched with those provided by the proponents. The obtained results are reported in Error: Reference source not found. Detailed results are available in the attached EXCEL files.

TABLE 2: Results of crosscheck simulation

- Test 5

- Test 6

- Test 7

- Test 8

# Conclusions

The results obtained during this crosscheck match those provided by the proponents. Experimental result reportedly shows the average bitrate reduction of 19.22% and 20.90% for synthesis views with around 15% decoding runtime increase relative to MVC+D when coding quarter and full resolution depth maps, respectively. Source codes were not reviewed due to the lack of time.

# References

- G. Bang, G.S. Lee, N.H. Hur, K.Y. Kim, J.H. Kim, Y.S.Heo, G.H. Park, Y. Chen, X. Zhao, L. Zhang, and M. Karczewicz, “3D-AVC: Report on coding performance of the depth coding tools for the 3D-AVC”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-E0245, July 2013, Vienna, Austria.

- JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, “Common Test Conditions of 3DV Core Experiments”, JCT-3V of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JCT3V-D1100, April 2013, Incheon, Korea.