| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 |  |  | Document: JCTVC-E0261 |
| --- | --- | --- | --- |
| Title: | CE3-related: Cross check of Depth-based Block Partitioning (JCT3V-E0118) |  |  |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sebastiaan Van Leuven, Glenn Van Wallendael, Robin Bailleul, Jan De Cock, Rik Van de Walle Gaston Crommenlaan 8 bus 201 9050 Ghent-Ledeberg, Belgium | Tel: Email: | +32 93314957 sebastiaan.vanleuven@ugent.be |
| Source: | Ghent University – iMinds |  |  |

_____________________________

# Abstract

This contribution provides a cross verification of a depth-based block partitioning (DBBP) as proposed in JCT3V-E0118 [1], as a CE3.h related technique. The DBBP has been evaluated both against the CTC [2] and a depth-first configuration. HTM6.1 has been used as the anchor and starting point for the software modifications.

The RD results show a perfect match with the tests conducted by the proponents. The source code has been verified.

# Encoding and decoding

Cluster computer with 54 computing nodes (IBM iDataPlex dx360M2), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5520 (Intel Nehalem microarchitecture, 2.27 GHz, 8 MB L3 cache per quad-core chip), thus 8 cores / node

- memory: 12 GB RAM (DDR3 PC3-8500S 1067 MHz)

- storage (local): one 156 GB 3.5" 15k RPM SAS disk

The encoder executables were generated with g++ 4.1.2.

# Simulation Results

The simulations were performed according the common test conditions, a depth first configuration, and the CTC configuration with a modified VSO string.

## Random Access Coding Configuration (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | -0,3% | -1,4% | -0,3% | 2,0% | 1,2% |
| Kendo | 0,0% | -0,9% | -1,3% | -0,4% | 2,8% | 2,1% |
| Newspaper_CC | 0,0% | -0,5% | -1,0% | -0,3% | 2,2% | 0,8% |
| GT_Fly | 0,0% | -1,1% | -1,5% | -0,3% | 1,8% | 1,6% |
| Poznan_Hall2 | 0,0% | -0,2% | -1,1% | -0,3% | 3,3% | 2,8% |
| Poznan_Street | 0,0% | -1,1% | -0,8% | -0,3% | 1,6% | 1,7% |
| Undo_Dancer | 0,0% | -3,5% | -5,1% | -1,2% | 0,7% | 0,6% |
| 1024x768 | 0,0% | -0,6% | -1,2% | -0,3% | 2,3% | 1,4% |
| 1920x1088 | 0,0% | -1,5% | -2,1% | -0,5% | 1,8% | 1,7% |
| average | 0,0% | -1,1% | -1,8% | -0,4% | 2,0% | 1,5% |

## Depth first configuration

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | -2,4% | -2,4% | -1,1% | -1,0% | -0,8% |
| Kendo | 0,0% | -5,0% | -5,7% | -2,3% | -2,0% | -1,7% |
| Newspaper_CC | 0,0% | -1,6% | -2,1% | -0,8% | -0,7% | -0,6% |
| GT_Fly | 0,0% | -0,8% | -1,1% | -0,2% | -0,2% | -0,2% |
| Poznan_Hall2 | 0,0% | -0,6% | -2,0% | -0,7% | -0,6% | -0,3% |
| Poznan_Street | 0,0% | -1,2% | -1,5% | -0,4% | -0,4% | -0,4% |
| Undo_Dancer | 0,0% | -3,1% | -4,0% | -0,9% | -0,9% | -0,8% |
| 1024x768 | 0,0% | -3,0% | -3,4% | -1,4% | -1,2% | -1,0% |
| 1920x1088 | 0,0% | -1,4% | -2,1% | -0,6% | -0,5% | -0,4% |
| average | 0,0% | -2,1% | -2,7% | -0,9% | -0,8% | -0,7% |

## CTC anchor with modified VSO configuration

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate |
| --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0,0% | -0,3% | -1,4% | -0,3% | 1,6% | 0,9% |
| Kendo | 0,0% | -0,9% | -1,3% | -0,4% | 2,1% | 1,6% |
| Newspaper_CC | 0,0% | -0,5% | -1,0% | -0,3% | 1,7% | 0,6% |
| GT_Fly | 0,0% | -1,1% | -1,5% | -0,3% | 1,0% | 0,8% |
| Poznan_Hall2 | 0,0% | -0,2% | -1,1% | -0,3% | 2,3% | 2,0% |
| Poznan_Street | 0,0% | -1,1% | -0,8% | -0,3% | 1,0% | 1,0% |
| Undo_Dancer | 0,0% | -3,5% | -5,1% | -1,2% | 0,2% | 0,0% |
| 1024x768 | 0,0% | -0,6% | -1,2% | -0,3% | 1,8% | 1,0% |
| 1920x1088 | 0,0% | -1,5% | -2,1% | -0,5% | 1,1% | 1,0% |
| average | 0,0% | -1,1% | -1,8% | -0,4% | 1,4% | 1,0% |

# References

x

| [1] | Fabian Jäger, "CE3-related: Depth-based Block Partitioning," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG, Vienna, Austria, Doc. JCT3V-E0118, 2013. |
| --- | --- |