| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0265 |
| --- | --- |

| Title: | CE5.h related: Crosscheck on simplified DMM prediction method (JCT3V-E0179) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | X. Zheng | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Hisilicon Technologies |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of proposal E0179. The source code was implemented based on HTM-7.0r1. We compiled, inspected, and ran the code of these cases with CTC and all-intra configurations. The cross-check has been completed successfully and the results match those provided by proponents of JCT3V- E0179.

# Test description and crosscheck results

This document reports the cross-check results of E0179. The encoder and decoder reconstructed YUV files generated from the above settings are matched. The experimental results of the rate-distortion are illustrated in Error: Reference source not found to Table 2, which show results of CTC and all-intra configurations of the proposed solution in E0181. These results match the BD-rate results provided by proponents of JCT3V- E0179.

Table 1: HTM7.0r1 anchor vs. proposed method in E0179 (CTC)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.1% | 0.0% | -0.01% | 0.00% | -0.02% | 98.6% | 98.4% | 98.5% |
| Kendo | 0.0% | 0.1% | -0.1% | -0.01% | 0.00% | -0.01% | 98.2% | 105.3% | 102.0% |
| Newspaper_CC | 0.0% | 0.1% | -0.1% | 0.01% | 0.01% | -0.33% | 98.5% | 105.9% | 104.6% |
| GT_Fly | 0.0% | -0.1% | -0.1% | -0.03% | -0.03% | -0.08% | 99.0% | 101.0% | 101.1% |
| Poznan_Hall2 | 0.0% | 0.1% | -0.4% | -0.09% | -0.08% | -0.11% | 98.5% | 99.5% | 98.9% |
| Poznan_Street | 0.0% | -0.1% | 0.0% | 0.00% | -0.01% | -0.09% | 98.8% | 96.7% | 97.4% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | -0.01% | -0.03% | -0.01% | 100.5% | 97.4% | 98.0% |
| 1024x768 | 0.0% | 0.0% | -0.1% | -0.01% | 0.00% | -0.12% | 98.4% | 103.2% | 101.7% |
| 1920x1088 | 0.0% | 0.0% | -0.1% | -0.03% | -0.04% | -0.07% | 99.2% | 98.7% | 98.8% |
| average | 0.0% | 0.0% | -0.1% | -0.02% | -0.02% | -0.09% | 98.9% | 100.6% | 100.1% |

Table 2: HTM7.0r1 anchor vs. proposed method in E0179 (all-intra)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.03% | 103.0% | 96.6% | 96.4% |
| Kendo | 0.0% | 0.0% | 0.0% | 0.00% | 0.01% | -0.03% | 103.2% | 98.2% | 94.7% |
| Newspaper_CC | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.10% | 102.1% | 97.2% | 95.4% |
| GT_Fly | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.12% | 101.9% | 96.5% | 95.6% |
| Poznan_Hall2 | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.08% | 102.8% | 97.1% | 95.9% |
| Poznan_Street | 0.0% | 0.0% | 0.0% | 0.00% | -0.03% | -0.12% | 101.9% | 84.3% | 97.5% |
| Undo_Dancer | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.03% | 102.8% | 91.1% | 96.3% |
| 1024x768 | 0.0% | 0.0% | 0.0% | 0.00% | 0.00% | -0.05% | 102.7% | 97.3% | 95.5% |
| 1920x1088 | 0.0% | 0.0% | 0.0% | 0.00% | -0.02% | -0.09% | 102.3% | 92.3% | 96.3% |
| average | 0.0% | 0.0% | 0.0% | 0.00% | -0.01% | -0.07% | 102.5% | 94.4% | 96.0% |