| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AU, 29 July – 2 August 2013 | Document: JCT3V-E0271 |
| --- | --- |

| Title: | 3D-CE3.h related: Crosscheck on Advanced temporal residual prediction (JCT3V-E0185) |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Li Zhang 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the outcome of cross-verification of JCT3V-E0185, "3D-CE3.h related: Advanced temporal residual prediction", performed by Qualcomm. The coding/synthesis results obtained for this cross-verification match those provided by the proponents.

# Introduction

In contribution JCT3V-E0185 [1], it is proposed to apply the advanced residual prediction (ARP) to disparity compensated prediction, i.e., the reference picture is from a different view component.

In the proposed method, when the reference picture of current prediction unit (PU) in RefPicListX is an inter-view reference picture, the motion information of the prediction unit  (denoted by Base) covering the top-left of center position of the corresponding reference block may be utilized to locate additional two reference blocks in a different access unit. In one case that Base is inter-coded, the motion information of Base in RefPicListX is firstly checked, followed by RefPicList(1-X). If there is one reference picture in RefPicListX (with X being 0 or 1) of current PU that has the same POC value as that in RefPicListX of Base, the motion information is utilized. If such a reference picture couldn’t be found or Base is intra coded, zero motion vector and the first available temporal reference picture in RefPicListX of current PU are used to locate two reference blocks in a different access unit. The additional two blocks are then utilized to generate the residual predictor and multiplied by a weighting factor as in current ARP.

Upon request, Qualcomm performed the crosschecking on the test case presented in the proposal.

# Performance of proposed method

Simulation results generated by the cross-checker are provided in Table 1 using the common test conditions [2] of 3D-HTMv7.0r1.

Table 1: Coding gain of JCT3V-E0185 vs 3D-HTMv7.0r1 (CTC)

|  | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate |  | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | -0.85% | -0.95% | -0.39% | -0.32% | -0.26% |  | 101% | 109% | 93% |
| Kendo | -1.37% | -1.85% | -0.63% | -0.54% | -0.50% |  | 95% | 103% | 88% |
| Newspaper_CC | -0.89% | -0.76% | -0.36% | -0.27% | -0.03% |  | 104% | 103% | 96% |
| GT_Fly | -0.04% | -0.43% | -0.07% | -0.07% | -0.05% |  | 97% | 97% | 87% |
| Poznan_Hall2 | -0.70% | -0.37% | -0.23% | -0.19% | -0.13% |  | 102% | 100% | 92% |
| Poznan_Street | -1.05% | -0.77% | -0.35% | -0.33% | -0.22% |  | 104% | 102% | 95% |
| Undo_Dancer | -0.59% | -0.60% | -0.17% | -0.18% | -0.13% |  | 102% | 97% | 93% |
| 1024x768 | -1.04% | -1.19% | -0.46% | -0.37% | -0.26% |  | 100% | 105% | 92% |
| 1920x1088 | -0.59% | -0.54% | -0.21% | -0.19% | -0.13% |  | 101% | 99% | 92% |
| average | -0.78% | -0.82% | -0.31% | -0.27% | -0.19% |  | 101% | 101% | 92% |

# References

- J. An, K. Zhang, J.-L. Lin, S. Lei, " 3D-CE3.h related: Advanced temporal residual prediction", JCT3V-E0185, Vienna, AU, July 2013.

- D. Rusanovskyy, K. Mueller, A. Vetro, "Common Test Conditions of 3DV Core Experiments", JCT3V-D1100, Incheon, KR, April 2013.