| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0288 |
| --- | --- |

| Title: | Cross-check on JCT3V-Inter-view MAD prediction for rate control of 3D multi-view video coding (JCT3V-E0227) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Cross check |  |  |
| Author(s) or Contact(s): | Yoon Jin Lee, Min Seong Lee, Young Su Heo, Gwang Hoon Park Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-31-201-3680 {rainmake, yomega00, medist, ghpark} @khu.ac.kr |
| Source: | Kyung Hee University |  |  |

_____________________________

# Abstract

This document presents the cross-check results of JCT3V-E0227 [1] which is about JCT3V-Inter-view MAD prediction for rate control of 3D multi-view video coding.

# Experimental Condition

KHU received the package of crosscheck data to be presented by JCT3V-E0228 [1] from KWU and the result cross-checked. Then we found no problem in compiling, building, encoding and decoding.

The test configurations are summarized in Table 1.

Table 1. Test configurations

| Test number | Rate Conrol Algorithm |
| --- | --- |
| 1 | Rate control using R-lambda only |
| 2 | Rate control using R-lambda with proposed method |
| 3 | Rate control based on URQ model only |
| 4 | Rate control based on URQ model with proposed method |

# Results

Percentage error in the actual bitrate and BD-rate of each experimental result are summarized in Table 2 and Table 3

Table 2 shows the results of percentage error in the actual bitrate.

- R-lambda model-based rate control vs. R-lambda model-based rate control with the proposed algorithms)

- URQ model-based rate control vs. URQ model-based rate control with the proposed algorithms

Table 3 shows the results of BD-rate of the proposed algorithms.

- R-lambda model-based rate control vs. R-lambda model-based rate control with the proposed algorithms)

- URQ model-based rate control vs. URQ model-based rate control with the proposed algorithms

Table 2. Percentage error in the actual bitrate (%)

- R-lambda model-based rate control vs. R-lambda model-based rate control with the proposed algorithms

| Sequence | R-lambda model-based rate control |  |  |  | R-lambda model-based rate control + the proposed methods |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | average view0 | average view1 | average view2 | total | average view0 | average view1 | average view2 | total |
| Balloons | 0.79 | 1.10 | 1.05 | 0.89 | 0.79 | 1.60 | 1.59 | 1.08 |
| Kendo | 0.25 | 0.89 | 0.82 | 0.47 | 0.25 | 0.96 | 0.91 | 0.50 |
| Newspaper | 1.00 | 1.04 | 0.99 | 1.01 | 1.00 | 1.37 | 1.27 | 1.12 |
| GTFly | 0.48 | 0.90 | 0.96 | 0.50 | 0.48 | 3.80 | 4.26 | 1.20 |
| Poznanhall2 | 0.45 | 1.31 | 1.01 | 0.49 | 0.45 | 1.34 | 1.52 | 0.50 |
| Poznanstreet | 2.73 | 1.58 | 1.58 | 2.40 | 2.73 | 3.63 | 4.02 | 3.00 |
| Dancer | 0.58 | 0.44 | 0.45 | 0.53 | 0.58 | 0.81 | 0.82 | 0.59 |
| Average | 0.90 | 1.04 | 0.98 | 0.90 | 0.90 | 1.93 | 2.06 | 1.14 |

- URQ model-based rate control vs. URQ model-based rate control with the proposed algorithms

| Sequence | URQ model-based rate control |  |  |  | URQ model-based rate control + the proposed methods |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | average view0 | average view1 | average view2 | total | average view0 | average view1 | average view2 | total |
| Balloons | 0.79 | 0.54 | 0.68 | 0.71 | 0.79 | 0.54 | 0.53 | 0.67 |
| Kendo | 0.67 | 0.51 | 0.49 | 0.56 | 0.67 | 0.51 | 0.49 | 0.56 |
| Newspaper | 1.68 | 2.14 | 1.74 | 1.77 | 1.68 | 2.22 | 2.05 | 1.84 |
| GTFly | 0.50 | 0.95 | 6.25 | 0.50 | 0.50 | 0.48 | 6.66 | 0.48 |
| Poznanhall2 | 0.76 | 2.45 | 2.76 | 0.67 | 0.76 | 1.39 | 2.73 | 0.78 |
| Poznanstreet | 0.46 | 1.43 | 4.95 | 0.73 | 0.46 | 1.13 | 3.32 | 0.64 |
| Dancer | 0.54 | 0.55 | 0.73 | 0.45 | 0.54 | 0.45 | 0.82 | 0.44 |
| Average | 0.77 | 1.22 | 2.51 | 0.77 | 0.77 | 0.96 | 2.37 | 0.77 |

Table 3. BD-rate of the proposed algorithms

- R-lambda model-based rate control vs. R-lambda model-based rate control with the proposed algorithms

| Sequence | BD-rate of R-lambda only vs. R-lambda + the proposed |  |
| --- | --- | --- |
|  | average view1 | average view2 |
| Balloons | -4.12 | -3.78 |
| Kendo | -2.75 | -2.71 |
| Newspaper | -6.79 | -8.10 |
| GTFly | 4.36 | 4.77 |
| Poznanhall2 | -5.50 | -6.49 |
| Poznanstreet | -2.44 | -2.69 |
| Dancer | -1.47 | -1.38 |
| Average | -2.67 | -2.91 |

- URQ model-based rate control vs. URQ model-based rate control with the proposed algorithms

| Sequence | Bdrate of URQ only vs. URQ + the proposed |  |
| --- | --- | --- |
|  | average view1 | average view2 |
| Balloons | 0.30 | -0.31 |
| Kendo | 1.70 | 1.35 |
| Newspaper | 1.12 | 3.19 |
| GTFly | -0.80 | -0.34 |
| Poznanhall2 | -0.83 | -0.13 |
| Poznanstreet | 4.03 | -3.60 |
| Dancer | -0.18 | 0.35 |
| Average | 0.76 | 0.07 |

# Conclusion

The results of JCT3V-E0227 were confirmed. Experimental results perfectly match with the one provided by the proponent.

# References

- W.Lim, H.Jo, J.Yoo, D.Sim, “JCT3V –Inter-view MAD prediction for rate control of 3D multi-view video coding”, The 5th JCT-3V meeting, JCT3V-E0227, July. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, The 4th JCT-3V meeting, JCT3V-D1100, Inchon, KR, April. 2013.