| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0289 |
| --- | --- |

| Title: | 3D-CE7: Cross-check Report on MB-level NBDV for 3D-AVC (JCT3V-E0136) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Cross check |  |  |
| Author(s) or Contact(s): | Gun Bang, *Yoon Jin Lee, *Jung Hyun Kim, *Gwang Hoon Park, Gwang Soon Lee, Namho Hur ETRI, 218 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea *Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 446-701, Korea | Tel: Email: | +82-42-860-5069 {gbang, gslee, namho} @etri.re.kr +82-31-201-3680 {*rainmake, *heyheyhey, *ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This document presents the cross-check results of JCT3V-E0136 [1] which is about NBDV Process to gain coding efficiency in texture-first cording in 3DV-AVC.

# Experimental Condition

ETRI and KHU received the package of crosscheck data to be presented by JCT3V-E0136 [1] from Qualcomm and the result has been cross-checked. Then we found no problem in compiling, building, encoding and decoding.

The test configurations are summarized in Table 1.

Table 1. Test configurations

| Test | Anchor | Tested |
| --- | --- | --- |
| Test 1. A | MVC+D | CTC |
| Test 2. A | MVC+D | QC_NBDV(BVSP on) |
| Test 3. A | MVC+D | QC_NBDV(BVSP off) |
| Test 4. A | CTC | QC_NBDV(BVSP on) |
| Test 5. A | MVC+D | ATM Texture First |
| Test 1. B | CTC | MVC+D |
| Test 2. B | QC_NBDV(BVSP on) | MVC+D |
| Test 3. B | QC_NBDV(BVSP off) | MVC+D |
| Test 4. B | QC_NBDV(BVSP on) | CTC |
| Test 5. B | ATM Texture First | MVC+D |

# Results

R-D performance of each experimental result are summarized in Table2 to Table11

Table 2 shows the results of Test 1. A (MVC+D vs CTC)

Table 3 shows the results of Test 2. A (MVC+D vs QC_NBDV(BVSP on))

Table 4 shows the results of Test 3. A (MVC+D vs QC_NBDV(BVSP off))

Table 5 shows the results of Test 4. A (CTC vs QC_NBDV(BVSP on))

Table 6 shows the results of Test 5. A (MVC+D vs ATM Texture First)

Table 7 shows the results of Test 1. B (anchor vs MVC+D)

Table 8 shows the results of Test 2. B (QC_NBDV(BVSP on) vs MVC+D)

Table 9 shows the results of Test 3. B (QC_NBDV(BVSP off) vs MVC+D)

Table 10 shows the results of Test 4. B (QC_NBDV(BVSP on) vs CTC)

Table 11 shows the results of Test 5. B (ATM Texture First vs MVC+D)

Table 2. Test 1. A

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -30.90 | 1.15 | -1.66 | 0.10 | -28.46 | 1.06 | -25.82 | 0.95 |
| S02 | -12.55 | 0.38 | -1.19 | 0.05 | -11.64 | 0.36 | -9.95 | 0.31 |
| S03 | -18.07 | 0.68 | -0.69 | 0.06 | -17.00 | 0.66 | -14.29 | 0.49 |
| S04 | -23.55 | 0.97 | -26.36 | 1.87 | -21.62 | 0.89 | -21.27 | 0.83 |
| S05 | -23.17 | 1.26 | -0.82 | 0.04 | -19.68 | 1.00 | -18.46 | 0.85 |
| S06 | -27.37 | 1.61 | -1.35 | 0.06 | -24.89 | 1.41 | -22.38 | 1.15 |
| S08 | -15.05 | 0.69 | 11.32 | -0.48 | -18.89 | 0.88 | -12.42 | 0.47 |
| Average | -21.52 | 0.96 | -2.96 | 0.24 | -20.31 | 0.89 | -17.80 | 0.72 |

Table 3. Test 2. A

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -30.59 | 1.14 | -1.27 | 0.07 | -28.26 | 1.05 | -25.76 | 0.95 |
| S02 | -12.35 | 0.38 | -2.12 | 0.09 | -11.69 | 0.36 | -9.92 | 0.31 |
| S03 | -17.70 | 0.67 | 2.78 | -0.22 | -16.48 | 0.63 | -14.00 | 0.48 |
| S04 | -23.25 | 0.96 | -26.47 | 1.87 | -21.31 | 0.88 | -21.02 | 0.82 |
| S05 | -22.03 | 1.20 | -1.58 | 0.08 | -18.78 | 0.95 | -17.75 | 0.81 |
| S06 | -26.62 | 1.57 | -2.86 | 0.13 | -24.45 | 1.38 | -21.85 | 1.12 |
| S08 | -14.48 | 0.67 | 10.85 | -0.46 | -18.58 | 0.87 | -12.12 | 0.46 |
| Average | -21.00 | 0.94 | -2.95 | 0.22 | -19.94 | 0.88 | -17.49 | 0.71 |

Table 4. Test 3. A

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -27.36 | 1.03 | -1.43 | 0.07 | -25.35 | 0.95 | -23.15 | 0.87 |
| S02 | -8.76 | 0.27 | -2.04 | 0.08 | -8.41 | 0.26 | -7.10 | 0.22 |
| S03 | -15.29 | 0.58 | 2.45 | -0.19 | -14.26 | 0.55 | -11.86 | 0.41 |
| S04 | -17.21 | 0.69 | -26.56 | 1.88 | -15.67 | 0.63 | -16.04 | 0.61 |
| S05 | -20.27 | 1.12 | -1.64 | 0.08 | -17.67 | 0.91 | -15.90 | 0.74 |
| S06 | -25.23 | 1.49 | -2.69 | 0.12 | -23.28 | 1.32 | -20.54 | 1.06 |
| S08 | -14.04 | 0.65 | 10.71 | -0.46 | -18.27 | 0.86 | -11.64 | 0.44 |
| Average | -18.31 | 0.83 | -3.03 | 0.23 | -17.56 | 0.78 | -15.18 | 0.62 |

Table 5. Test 4. A

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 0.43 | -0.01 | 0.40 | -0.03 | 0.27 | -0.01 | 0.08 | 0.00 |
| S02 | 0.22 | -0.01 | -0.94 | 0.04 | -0.05 | 0.00 | 0.03 | 0.00 |
| S03 | 0.44 | -0.01 | 3.49 | -0.28 | 0.63 | -0.02 | 0.30 | -0.01 |
| S04 | 0.39 | -0.01 | -0.07 | 0.00 | 0.39 | -0.01 | 0.31 | -0.01 |
| S05 | 1.53 | -0.07 | -0.76 | 0.04 | 1.15 | -0.05 | 0.93 | -0.04 |
| S06 | 1.03 | -0.05 | -1.53 | 0.07 | 0.59 | -0.03 | 0.70 | -0.03 |
| S08 | 0.67 | -0.03 | -0.86 | 0.04 | 0.37 | -0.02 | 0.35 | -0.01 |
| Average | 0.67 | -0.03 | -0.04 | -0.02 | 0.48 | -0.02 | 0.39 | -0.02 |

Table 6. Test 5. A

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -0.50 | 0.02 | -1.75 | 0.09 | -0.62 | 0.03 | -1.12 | 0.05 |
| S02 | -1.28 | 0.04 | -2.00 | 0.08 | -1.51 | 0.05 | -1.16 | 0.04 |
| S03 | -0.10 | 0.00 | 2.08 | -0.17 | 0.05 | 0.00 | -0.51 | 0.02 |
| S04 | 0.11 | 0.00 | -26.41 | 1.87 | 0.64 | -0.03 | -2.22 | 0.09 |
| S05 | -1.70 | 0.10 | -1.57 | 0.08 | -1.60 | 0.08 | -1.51 | 0.07 |
| S06 | -2.58 | 0.14 | -2.91 | 0.13 | -2.68 | 0.14 | -2.27 | 0.11 |
| S08 | -1.97 | 0.09 | 10.93 | -0.47 | -7.99 | 0.37 | -2.03 | 0.08 |
| Average | -1.15 | 0.06 | -3.09 | 0.23 | -1.96 | 0.09 | -1.55 | 0.06 |

Table 7. Test 1. B

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 44.71 | -1.15 | 1.68 | -0.10 | 39.79 | -1.06 | 34.82 | -0.95 |
| S02 | 14.35 | -0.38 | 1.20 | -0.05 | 13.18 | -0.36 | 11.05 | -0.31 |
| S03 | 22.05 | -0.68 | 0.70 | -0.06 | 20.49 | -0.66 | 16.67 | -0.49 |
| S04 | 30.81 | -0.97 | 35.80 | -1.87 | 27.58 | -0.89 | 27.01 | -0.83 |
| S05 | 30.15 | -1.26 | 0.83 | -0.04 | 24.50 | -1.00 | 22.64 | -0.85 |
| S06 | 37.68 | -1.61 | 1.37 | -0.06 | 33.14 | -1.41 | 28.84 | -1.15 |
| S08 | 17.72 | -0.69 | -10.17 | 0.48 | 23.29 | -0.88 | 14.18 | -0.47 |
| Average | 28.21 | -0.96 | 4.49 | -0.24 | 25.99 | -0.89 | 22.17 | -0.72 |

Table 8. Test 2. B

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 44.07 | -1.14 | 1.28 | -0.07 | 39.39 | -1.05 | 34.69 | -0.95 |
| S02 | 14.09 | -0.38 | 2.16 | -0.09 | 13.24 | -0.36 | 11.02 | -0.31 |
| S03 | 21.51 | -0.67 | -2.70 | 0.22 | 19.73 | -0.63 | 16.27 | -0.48 |
| S04 | 30.29 | -0.96 | 36.00 | -1.87 | 27.08 | -0.88 | 26.62 | -0.82 |
| S05 | 28.25 | -1.20 | 1.60 | -0.08 | 23.13 | -0.95 | 21.58 | -0.81 |
| S06 | 36.27 | -1.57 | 2.95 | -0.13 | 32.36 | -1.38 | 27.96 | -1.12 |
| S08 | 16.93 | -0.67 | -9.79 | 0.46 | 22.83 | -0.87 | 13.79 | -0.46 |
| Average | 27.34 | -0.94 | 4.50 | -0.22 | 25.39 | -0.88 | 21.71 | -0.71 |

Table 9. Test 3. B

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 37.66 | -1.03 | 1.45 | -0.07 | 33.96 | -0.95 | 30.12 | -0.87 |
| S02 | 9.60 | -0.27 | 2.08 | -0.08 | 9.18 | -0.26 | 7.64 | -0.22 |
| S03 | 18.04 | -0.58 | -2.39 | 0.19 | 16.63 | -0.55 | 13.46 | -0.41 |
| S04 | 20.78 | -0.69 | 36.16 | -1.88 | 18.58 | -0.63 | 19.10 | -0.61 |
| S05 | 25.42 | -1.12 | 1.66 | -0.08 | 21.46 | -0.91 | 18.91 | -0.74 |
| S06 | 33.74 | -1.49 | 2.77 | -0.12 | 30.35 | -1.32 | 25.85 | -1.06 |
| S08 | 16.33 | -0.65 | -9.67 | 0.46 | 22.36 | -0.86 | 13.17 | -0.44 |
| Average | 23.08 | -0.83 | 4.58 | -0.23 | 21.79 | -0.78 | 18.32 | -0.62 |

Table 10. Test 4. B

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | -0.43 | 0.01 | -0.40 | 0.03 | -0.27 | 0.01 | -0.08 | 0.00 |
| S02 | -0.22 | 0.01 | 0.95 | -0.04 | 0.05 | 0.00 | -0.03 | 0.00 |
| S03 | -0.44 | 0.01 | -3.38 | 0.28 | -0.63 | 0.02 | -0.30 | 0.01 |
| S04 | -0.39 | 0.01 | 0.07 | 0.00 | -0.39 | 0.01 | -0.31 | 0.01 |
| S05 | -1.50 | 0.07 | 0.77 | -0.04 | -1.14 | 0.05 | -0.92 | 0.04 |
| S06 | -1.02 | 0.05 | 1.56 | -0.07 | -0.58 | 0.03 | -0.70 | 0.03 |
| S08 | -0.66 | 0.03 | 0.87 | -0.04 | -0.37 | 0.02 | -0.35 | 0.01 |
| Average | -0.67 | 0.03 | 0.06 | 0.02 | -0.48 | 0.02 | -0.38 | 0.02 |

Table 11. Test 5. B

|  | Texture Coding |  | Depth Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 0.50 | -0.02 | 1.78 | -0.09 | 0.62 | -0.03 | 1.14 | -0.05 |
| S02 | 1.30 | -0.04 | 2.04 | -0.08 | 1.53 | -0.05 | 1.18 | -0.04 |
| S03 | 0.10 | 0.00 | -2.04 | 0.17 | -0.05 | 0.00 | 0.51 | -0.02 |
| S04 | -0.11 | 0.00 | 35.88 | -1.87 | -0.64 | 0.03 | 2.27 | -0.09 |
| S05 | 1.73 | -0.10 | 1.60 | -0.08 | 1.62 | -0.08 | 1.54 | -0.07 |
| S06 | 2.65 | -0.14 | 3.00 | -0.13 | 2.75 | -0.14 | 2.33 | -0.11 |
| S08 | 2.01 | -0.09 | -9.85 | 0.47 | 8.69 | -0.37 | 2.07 | -0.08 |
| Average | 1.17 | -0.06 | 4.63 | -0.23 | 2.07 | -0.09 | 1.58 | -0.06 |

It was confirmed that these results perfectly match with the one provided by the proponent.

# Conclusion

The results of JCT3V-E0136 were confirmed. Experimental results perfectly match with the one provided by the proponent.

# References

- Xin Zhao, Ying Chen, Li Zhang , Jewon Kang, Ye-Kui Wang, Rajan Joshi, Marta Karczewicz
, “3D-CE7: MB-level NBDV for 3DV-AVC”, The 5th JCT-3V meeting, JCT3V-E0136, July. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, "Common Test Conditions of 3DV Core Experiments," ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-C1100, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013.