| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0296 |
| --- | --- |

| Title: | 3D-CE3.h related: Cross-check result on the CTU-aligned DV for temporal inter-view motion vector candidate from KHU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Cross check |  |  |
| Author(s) or Contact(s): | Woong Lim, Jonghoon Yoo, Donggyu Sim 512 Bima-building, Kwangwoon Univ. Wolgye-dong, Seoul, Korea. | Tel: Email: | +82-10-2744-6098 limwoong@kw.ac.kr mmyjh@kw.ac.kr dgsim@kw.ac.kr |
| Source: | Kwangwoon University (KWU) |  |  |

_____________________________

# Abstract

This document presents the cross-check results of JCT3V-E0214 [1] which is about the CTU-aligned DV for a temporal inter-view motion vector candidate to reduce the memory-access bandwidth.

# Experimental Condition

KWU received the package of crosscheck data to be presented by JCT3V-E0214 [1] from KHU and the result cross-checked. Then we found no problem in compiling, building, encoding and decoding.

The test configurations are summarized in Table 1.

Table 1. Test configurations

| Test number | CTU-align range |
| --- | --- |
| Method 1 | 1 x CTU aligned |
| Method 2 | 3 x CTU aligned |

# Crosscheck Results

R-D performances of each experimental result are summarized from Table 2 to Table 5. Table 2 shows the results of method 1 for 3DV sequences (anchor vs. 1xCTU aligned)

Table 2. Experimental results of the method 1 using the CTU-aligned (1X) DV vs. HTM7.0r1 (CTC, 3DV sequences)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | -0.06% | -0.02% | -0.01% | -0.01% | -0.01% | N/A | N/A | N/A |
| Kendo | 0.0% | 0.01% | -0.08% | -0.01% | -0.01% | -0.01% | N/A | N/A | N/A |
| Newspaper_CC | 0.0% | -0.02% | 0.00% | 0.00% | 0.01% | -0.01% | N/A | N/A | N/A |
| GT_Fly | 0.0% | 0.02% | 0.02% | 0.00% | 0.00% | 0.00% | N/A | N/A | N/A |
| Poznan_Hall2 | 0.0% | -0.16% | -0.10% | -0.06% | -0.06% | 0.00% | N/A | N/A | N/A |
| Poznan_Street | 0.0% | -0.01% | 0.00% | -0.01% | 0.00% | 0.00% | N/A | N/A | N/A |
| Undo_Dancer | 0.0% | -0.11% | -0.15% | -0.03% | -0.03% | -0.04% | N/A | N/A | N/A |
| 1024x768 | 0.0% | -0.02% | -0.03% | -0.01% | 0.00% | -0.01% | N/A | N/A | N/A |
| 1920x1088 | 0.0% | -0.06% | -0.06% | -0.02% | -0.02% | -0.01% | N/A | N/A | N/A |
| average | 0.0% | -0.05% | -0.05% | -0.02% | -0.01% | -0.01% | N/A | N/A | N/A |

Table 3 shows the results of method 2 for 3DV sequences (anchor vs. 3xCTU aligned)

Table 3. Experimental results of the method 2 using the CTU-aligned (3X) DV vs. HTM7.0r1 (CTC, 3DV sequences)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | synth PSNR / total bitrate | enc time | dec time | ren time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloons | 0.0% | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | N/A | N/A | N/A |
| Kendo | 0.0% | -0.01% | 0.01% | 0.00% | 0.01% | 0.01% | N/A | N/A | N/A |
| Newspaper_CC | 0.0% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | N/A | N/A | N/A |
| GT_Fly | 0.0% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | N/A | N/A | N/A |
| Poznan_Hall2 | 0.0% | 0.03% | -0.03% | 0.00% | 0.02% | 0.04% | N/A | N/A | N/A |
| Poznan_Street | 0.0% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | N/A | N/A | N/A |
| Undo_Dancer | 0.0% | -0.02% | -0.02% | -0.01% | 0.00% | 0.02% | N/A | N/A | N/A |
| 1024x768 | 0.0% | 0.00% | 0.01% | 0.00% | 0.00% | 0.00% | N/A | N/A | N/A |
| 1920x1088 | 0.0% | 0.00% | -0.01% | 0.00% | 0.00% | 0.01% | N/A | N/A | N/A |
| average | 0.0% | 0.00% | 0.00% | 0.00% | 0.00% | 0.01% | N/A | N/A | N/A |

For MVC test sequences, we also crosschecked the method of JCT3V-E0214 using 1xCTU and 3xCTU conditions. Experimental results to evaluate RD-performances are summarized in both Table 4 and Table 5. Table 4 shows the experimental results of method 1 for non-rectified MVC sequences (anchor vs. 1xCTU aligned).

Table 4. Experimental results of the method 1 using the CTU-aligned (1X) DV vs. HTM7.0r1 (CTC, MVC sequences)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AkkoKayo_48_26_70 | 0.0% | 3.4% | 5.2% | 2.2% | 2.2% | N/A | N/A |
| AkkoKayo_48_46_50 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | N/A | N/A |
| BreakDancers_4_1_7 | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | N/A | N/A |
| Flamenco_0_1_3 | 0.0% | 2.5% | 2.6% | 1.4% | 1.4% | N/A | N/A |
| Flamenco_2_1_3 | 0.0% | 2.1% | 0.8% | 0.6% | 0.6% | N/A | N/A |
| average | 0.0% | 1.6% | 1.7% | 0.9% | 0.9% | N/A | N/A |

Table 5 shows the experimental results of method 2 for non-rectified MVC sequences (anchor vs. 3xCTU aligned).

Table 5. Experimental results of the method 2 using the CTU-aligned (3X) DV vs. HTM7.0r1 (CTC, MVC sequences)

|  | video 0 | video 1 | video 2 | video PSNR / video bitrate | video PSNR / total bitrate | enc time | dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AkkoKayo_48_26_70 | 0.00% | 0.42% | 0.83% | 0.33% | 0.33% | N/A | N/A |
| AkkoKayo_48_46_50 | 0.00% | 0.02% | -0.03% | 0.00% | 0.00% | N/A | N/A |
| BreakDancers_4_1_7 | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% | N/A | N/A |
| Flamenco_0_1_3 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | N/A | N/A |
| Flamenco_2_1_3 | 0.00% | 0.08% | -0.07% | 0.00% | 0.00% | N/A | N/A |
| average | 0.00% | 0.10% | 0.15% | 0.07% | 0.07% | N/A | N/A |

It was confirmed that these results for coding performance perfectly match with the one provided by the proponent.

# Conclusion

The results of JCT3V-E0214 were confirmed. Experimental results perfectly match with the one provided by the proponent.

# References

- K.Y. Kim, Y.S. Heo, M.S. Lee, Y.J. Lee, G.H. Park, “3D-CE3.h related: CTU-aligned DV for temporal inter-view motion vector candidate”, The 5th JCT-3V meeting, JCT3V-E0214, July. 2013.

- D. Rusanovskyy, K. Müller, A. Vetro, “Common Test Conditions of 3DV Core Experiments”, The 4th JCT-3V meeting, JCT3V-D1100, Inchon, KR, April. 2013.