| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0374v7 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0306v7 |
| --- | --- |

| Title: | BoG report on SHVC/MV-HEVC HLS topics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | BoG Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill@vidyo.com |
| Source: | BoG Chair |  |  |

_____________________________

# Abstract

The BoG met 27 July to review the following contributions:

- 6.4.4.5 Reference picture list construction (4)

  - JCTVC-N0082, JCTVC-N0095, JCTVC-N0216, JCTVC-N0316

- 6.4.3.3 Efficient parameter set parameters signalling (3)

  - JCTVC-N0162, JCTVC-N0200, JCTVC-N0212

- 6.6.2 Layer presence and dependency change SEI messages (2)

  - JCTVC-N0173, JCTVC-N0174

The BoG recommends the following:

- Adopt JCTVC-N0082, initialization process of reference picture lists for HEVC extensions

- Adopt JCTVC-N0371, scaling list prediction in SPS and PPS (harmonization of JCTVC-N0162 and JCTVC-N0200 variant 3)

- Layer dependency change SEI message be removed from specification.  If the SEI message does remain, to adopt JCTVC-N0174 (with some editorial improvements).

The BoG plans to meet again to revisit:

- Initialization of reference picture lists: Revision to JCT3V-D0220 (in JCTVC-N0316), and harmonization of JCTVC-N0095 and JCTVC-N0216, with experimental results compared to JCTVC-N0082

The BoG also met 28 July to review the following contributions:

- 6.4.4.3  General inter-layer RPS signalling and derivation (7) (TBP)

  - N0059, N0081, N0118, N0154, N0195, N0217, N0131

- Portions of 6.4.3.1 General (4)

  - N0130, N0195, N0217

- 6.4.4.6 Management of resampled or filtered inter-layer reference pictures (2)

  - N0128, N0282, N0056

The BoG recommends the following:

- Adopt from JCTVC-N0057 second proposal, change decoding process to add condition that when SamplePredEnabledFlag equal to 1, don’t include picture in the motion pred ref list. [not needed with removal of alt collocated index]

- Adopt a condition on signaling inter_layer_pred_layer_idc[ i ], to avoid sending when NumDirectRefLayers equals NumActiveRefLayerPics, and instead infer values.  From JCTVC-N0081, JCTVC-N0195 proposal 1, JCTVC-N0154, and JCTVC-N0217.

- Adopt an Inter Layer Reference Picture (ILRP) presence flag in the VPS, conditioning the presence of ILRP syntax elements in the slice segment header, similar to JCTVC-N0195 proposal 2.

- Change order of sorting of inter-layer reference pictures to be in descending rather than ascending order, from JCTVC-N0131 Proposal 4.

  - It is noted that during the JCT-3V review of these BoG recommendations, there was objection to this recommendation.

  - [Decision in the joint session to not do this.]

- Add constraint when splitting_flag is used, that the sum of the lengths be less than or equal to 6, from JCTVC-N0195 5th proposal.

The BoG suggests the editors consider the following:

- A text bug fix identified in JCTVC-N0059, in which motion resampling is currently only invoked when alt collocated idc flag is equal to 1, while it should be invoked whenever inter-layer motion prediction is performed and the current and reference layer differ in resolution.

- Add an editorial note for SHVC encoders to avoid use of TMVP when the inter-layer reference picture is the only one in the list.

The BoG suggests to the Track that:

- JCTVC-N0128 Proposal 2 be reviewed in the track, as the topic was difficult to address in the BoG.

- The Track revisit the concept of sending IL RPS candidates in VPS with slice indication, as in JCTVC-N0118and JCTVC-N0081.

When the BoG meets again, to revisit JCTVC-N0217 Proposals 4.1.1 and 4.1.2.

The v4 version of this document make corrections to the exact language of the recommendations from the 30 July meeting.

The BoG also met again on 30 July to review the following contributions

### 6.4.4.5 Reference picture list construction (4)

  - Revisit of JCTVC-N0316

### 6.4.4.2  Sub-layer related inter-layer prediction signalling (4)

  - Revisits of JCTVC-N0120, N0109, N0196

- 6.4.5 Tiles and parallel processing (4)

  - JCTVC-N0158,  JCTVC-N0160,  JCTVC-N0199

The BoG recommends the following:

- Adopt JCTVC-N0316, inter-layer reference picture list construction initialization process, in which inter-layer reference pictures are classified into two lists (“before” and “after” views), inserted at different positions.

- Adopt a presence flag for signaling of max_tid_il_ref_pics_plus1[i], from JCTVC-N0120.

- Add constraint to not use tiles in a layer and wavefronts in a different layer, when one layer is a direct dependent of the other layer, from JCTVC-N0158.

  - [During joint session discussion, decision was made no to do this.]

- Move num_ilp_restricted_ref_layers and related offset delay syntax elements from SPS VUI to VPS VUI, and change to a num_ilp_restricted_ref_layers flag per direct dependent layer for each layer, from JCTVC-N0160.

- Modify calculation of xRef[ ] and yRev[ ] variables in semantics for offset delay syntax element semantics, from JCTVC-N0160

- Adopt JCTVC-N0199 proposal 2 variant 2 (also in JCTVC-N0160) to move tile boundaries alignment flag from the SPS VUI to the VPS VUI, and also signal the flag per direct dependent layer for each layer, from JCTVC-N0199.

The BoG met again on 31 July to cover the following topics:

### 6.6.1 Motion and prediction constrained SEI messages (5)

  - JCTVC-N0088, JCTVC-N0117, JCTVC-N0236

  - Revisits of JCTVC-N0069,  JCTVC-N0159

- 6.2.5 Sampling position

  - Revisit of JCTVC-N0334

The BoG recommends the following:

- Add inter-layer prediction tile constraint indication to the motion constrained tile information SEI message. Exact syntax to be provided in new contribution JCTVC-N0383, which harmonizes JCTVC-N0069, JCTVC-N0087, JCTVC-N0088,  and JCTVC-N0236.

The BoG met again on 1 August to review the following:

- 6.4.1 Generic HLS issues

  - JCTVC-N0355, JCTVC-N0356, JCTVC-N0357, JCTVC-N0135, JCTVC-N0065, JCTVC-N0244

- 6.4.2 Random access, layer switching structures and cross-layer alignment of pictures types

  - JCTVC-N0066, JCTVC-N0147

- 6.5.1 SHVC Generic HLS issues

  - JCTVC-N0108

- 6.6.1 Motion and prediction constrained SEI messages

  - JCTVC-N0383

The BoG recommends the following:

- Adopt JCTVC-N0244, to use a reserved slice header bit for a POC reset flag, plus signal POC LSB in enhancement layer IRAP pictures from JCTVC-N0065, to maintain POC alignment between layers when IRAP pictures are not aligned.

  - [During joint session discussion, decided to align with #3, and only require inclusion of the slice flag when the VPS alignment flag indicates non-aligned IRAPs possible.]

- Adopt JCTVC-N0066 version 2 layer-wise start up decoding process.

- Add a flag in VPS extension to indicate if all IRAP pictures are aligned in set of dependent layers, from JCTVC-N0147.

- Improve text clarity by adding explicit constraint that sample resampling may be done once per enhancement layer picture, and motion field resampling may be done once per enhancement layer picture, from JCTVC-N0108.

  - SHVC track may also wish to discuss if sample prediction and motion field prediction from different reference layers can be performed when coding an enhancement layer picture in the Scalable Main profile.

  - [During joint session discussion, decision to impose such a constraint.]

- Adopt JCTVC-N0383 inter-layer predication constraint SEI message, plus modification to motion constrained tile set SEI message to align syntax.

- v7 also includes results of review of the recommendations in the joint session of JCTVC and JCT3V.

# Meeting notes

## 6.4.4.5 Reference picture list construction (4)

JCTVC-N0082 MV-HEVC/SHVC HLS: On initialization process of reference picture lists for HEVC extensions [O. Nakagami, T. Suzuki (Sony)]

This contribution is a follow-up of JCTVC-M0081/JCT3V-D0058.

It is proposed that inter-layer reference picture (ILRP) is inserted between RefPicSetStCurrBefore and RefPicSetStCurrAfter in creating the temporal list. The motivation is to reduce reference picture list modification (RPLM) signalling for common coding structures.

The proposal is tested under the recommended condition described in JCTVC-M0461. It is reported that the RPLM bit for SHVC(RA), 3D(PIP) and 3D(IBP) are reduced to 51.1, 60.7 and 86.5 [%], respectively.

It is recommended to modify the list0 initialization process as proposed.

The proportion is kept since the proposal doesn’t affect the process for LD-B case.

Current:

RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr, RefPicSetIvCurr,

Proposed:

RefPicSetStCurrBefore, RefPicSetIvCurr, RefPicSetStCurrAfter and RefPicSetLtCurr.

No new syntax, just a different default order for list 0.  No complexity impact.

See notes under JCTVC-N0316.

JCTVC-N0095 MV-HEVC/SHVC HLS: Inter-layer reference pictures in reference picture list initialization [A. K. Ramasubramonian, Y. Chen, L. Zhang (Qualcomm)]

It is proposed that explicitly signaling the start position of inter-layer reference pictures in the reference picture list can be used to reduce the number of reference picture list modification commands, and hence the number of bits used in the slice header, in typical cases in MV-HEVC and SHVC.

Proposes to add two ue(v) new syntax elements in slice segment header, for list 0 and list 1, to indicate position in the list for inter-layer reference picture.

Decoding process modified to insert inter-layer ref pics in list, and relocate later reference pictures.

No experimental results for this meeting using the common test conditions defined in JCTVC-M0461, but previously proposed and had results last meeting in JCT3V-D0198.

See notes under JCTVC-N0316.

JCTVC-N0216 MV-HEVC/SHVC HLS: On Reference Picture List Modification [Y He, X. Xiu, Y Ye (InterDigital)]

It is proposed to signal the position of inter-layer reference picture set in reference picture list modification (RPLM) to save the bits. In current common test condition, reference picture list modification is invoked to allocate inter-layer reference picture (ILRP) to the end of the active reference lists when the number of active reference pictures is less than the total number of available reference pictures. To avoid signaling both temporal and ILRP reference picture one by one within current list modification framework, extra syntax elements in RPLM are proposed to signal the position of inter-layer reference picture set directly.  The average bit saving of RPLM is around 57% for SHVC RA cases, and 51% for MV/3D-HEVC cases.

Similar to N0095, but with difference in the syntax.  Four syntax elements, two flags (for list 0 and list 1), and 2 positions as u(v).

The position to locate all of the inter-layer references is signalled with individual reference picture granularity.  Decoding process modified to insert inter-layer ref pics in list, and relocate later reference pictures.

Experimental results were provided for SHVC and MV-HEVC.

See notes under JCTVC-N0316.

JCTVC-N0316 MV-HEVC/SHVC HLS: Initial inter-layer reference picture list construction [Andrey Norkin, Usman Hakeem (Ericsson)] [late]

This contribution addresses handling of inter-layer reference pictures in the initial reference picture list construction in MV-HEVC, 3D-HEVC, and SHVC. Two reference picture sets are created, which are inserted at predefined positions making it possible to minimize use of RPLM commands. Two options have been proposed. In option 1, the reference picture sets are inserted at predefined positions. In option 2, a position for one of the two defined reference picture sets is signaled in the slice header.

Option 1

Proposes to separate before and after inter-layer reference pictures.  For SHVC, only have before, while for MV-HEVC can have both before and after.

However, have more flexibility to separate the list of ref pics into lists 0 and 1 that are not necessarily exactly before and after.  Change the num_inter_layer_ref_pics syntax element into two syntax elements, for list 0 and list 1 (before and after).

May be possible to derive before and after lists without changing syntax element.  Was previously proposed in JCT3V-D0220.

For SHVC, list construction will be identical in behavior to N0082.  For MV-HEVC will differ: additional bits required for signaling, but potential savings from different position for after list.

Proposed:

RefPicListTemp0:

RefPicSetStCurrBefore, RefPicSetInterLayer0, RefPicSetStCurrAfter, RefPicSetLtCurr, RefPicSetInterLayer1.

RefPicListTemp1 as in the following:

RefPicSetStCurrAfter,  RefPicSetInterLayer1, RefPicSetStCurrBefore, RefPicSetLtCurr, RefPicSetInterLayer0.

Option 2

Similar to N0095, except signaling position of the two lists at a reference picture granularity, using ue(v) coding.

Option 1 outperforms Option 2 for SHVC Kimono LDB and LDP example because of additional cost to signal.  Option 2 outperforms Option 1 for all MV-HEVC cases.

BoG agree to N0082

Revisit later this meeting a D0220 solution (without sorting) once updated text and simulation results are available.  Can also revisit harmonized N0095 and N0216.   Comparison data vs N0082 should be provided.  Both RPL and absolute bitrate savings numbers should be provided.  Any software may be used, but both proposal and N0082 results should use the same software.

## 6.4.3.3 Efficient parameter set parameters signalling (3)

JCTVC-N0162 MV-HEVC/SHVC HLS: Inter-layer scaling list inheritance for HEVC extensions [Martin Pettersson, Thomas Rusert]

This proposal is a follow-up of the JCTVC-M0134 proposal. The contribution addresses a redundancy problem that is asserted to be present when two or more different SPSs or PPSs containing identical scaling list (quantization matrix) data are present for different layers in multi-layer HEVC extensions. If scaling list data is present in SPS or PPS it typically takes up a major part of the amount of bits in the particular SPS or PPS. The scaling list data would in the current version of the draft be duplicated, causing bit overhead for parameter set transmission and storage.

It is proposed for HEVC extensions to introduce inter-layer scaling list inheritance to reduce bit redundancy. This implies extending the definition of SPS and PPS in the extension specifications. In the proposed SPS/PPS inheritance scheme, scaling list data is inherited from a reference SPS/PPS, determined as the active SPS/PPS of the reference layer, which is signaled in the referencing SPS/PPS.

See notes under JCTVC-N0200.

JCTVC-N0200 On Scaling List Data Signaling [S. Deshpande (Sharp), S. Liu (MediaTek), S. Lei (MediaTek), K. Sato (Sony)]

This document proposes syntax and semantics for signaling scaling list data for SHVC layers with nuh_layer_id > 0. Three variant syntax alternatives are proposed. PPS and SPS activation method and bitstream constraint is proposed in relation to the proposed syntax and semantics.

Variant 1 has a parsing dependency of the SPS on the VPS

Should restrict such that can only predict from a dependent layer.

In what scenarios would scaling list inheritance be desirable when parameter set sharing couldn’t be used?  When coding tools differed in the layers, including LCU size, min CU size, min transform size, RQT depth, in SPS. Picture resolution in SPS currently disallows SPS sharing in spatial scalability layer.  Also, if SPS is different, PPS can’t be shared.

Preferred solution: N0162 syntax (same as N0200 variant 3), except changing the reference layer to be an absolute nuh_layer_id with u(6). Includes bitstream constraint that the reference layer must not itself infer from another layer.  Activation language needed from N0200 (to be checked by editors).  Include a constraint that the reference layer must be a dependent layer of the current layer (when the SPS/PPS activated).  Proponents to create new contribution N0371 which implements the description above.

BoG: Agree to recommend adoption of combined contribution JCTVC-N0371 as described in preferred solution above.

JCTVC-N0212 SHVC HLS: On Inter Layer Parameter Set [Y. He, Y. Ye, Y. He (InterDigital)]

This contribution document proposes a new parameter set, inter-layer parameter set (IPS), to address the redundant issue of SPS syntax elements across multiple layers. The proposed IPS aggregates groups of parameters shared by multiple layers, and each layer could refer to the IPS parameters. As a result, the number of bits used for SPS would be significantly reduced when multiple layers shares the identical parameter values. The IPS could further remove the parsing dependency when the parameters placed in different parameter sets could be included in the same IPS. In addition, the proposed new parameter set also avoids the problems caused by SPS to SPS, or VPS to SPS parameter set prediction.

Propose a new parameter set, called the inter_layer_parameter_set, similar to the VPS, but not used for capabilities exchange.  Would need a new NUT for the IPS.

Scaling list prediction savings only applies to SPS, not to PPS.

Similar to previously proposed header parameter set.

Would lose bits for 2 layer case, but small savings for 3 or more layers sending separate SPSs without sharing.  (Only RBSP bits were counted. )

It was suggested that even if IPS existed, the capability of signaling complete info in the SPS should be maintained.

Would IPS be optional?  No existing parameter sets are optional.

Further study.  What would the size of the IPS be, and could it fit inside the VPS?  Bitrates for the different parameter subsets, especially the totals with and without SPS scaling list parameters.

## 6.6.2 Layer presence and dependency change SEI messages (2)

JCTVC-N0173 MV-HEVC/SHVC HLS: On Layers Present SEI message [J. W. Kang (ETRI), J. Lee, H. Lee, J. S. Choi, T. C. Thang (UoA)]

Currently the layer present SEI message provides the information of non-present layers after some modification. The current solution just focuses on the layers identified by the nuh_layer_id parameter. In practice, a CVS could be modified in temporal dimension as well. This contribution proposes to describe the (temporal) sub-layers which are not present in a CVS.

Usefulness of the proposed SEI modifications questioned. No action.

JCTVC-N0174 MV-HEVC/SHVC HLS: On Layer Dependency Change SEI message [T. C. Thang (UoA), J. W. Kang, J. Lee, H. Lee, J. S. Choi (ETRI)]

Currently, in the layer dependency change SEI message, the information for all layers is specified. In fact, the layer dependency information of some (or even many) layers may be unchanged. Obviously, it is unnecessary to describe again the layer dependency information of such layers. This contribution proposes to add a flag to indicate whether the dependency change information is available for a given layer.

BoG recommends that the layer dependency change SEI message be removed. If the SEI message does remain, BoG recommends to adopt contribution, with some editorial improvements.

## 6.4.4.3  General inter-layer RPS signalling and derivation (7) (TBP)

N0059, N0081, N0118, N0154, N0195, N0217, N0131

JCTVC-N0059 MV-HEVC/SHVC HLS: On slice segment header extension [T. Ikai, T. Uchiumi (Sharp)]

This contribution proposes to move all slice segment header changes by SHVC and MV-HEVC to slice segmentation header extension. It is asserted that extension is more suitable for clear definition because the extension part is strictly restricted within one part. The relocation is also beneficial because these syntax elements can be effectively omitted and inferred.

The extension length field is effectively a presence flag.

Some experts expressed that there was no backwards compatibility issue with the current design, so no need to change.  No action.

Seems to have identified an editorial bug 6.8.1.4 where motion resampling is only invoked when alt_collocated_indication-flag is equal to 1.  The appropriate condition should be if the collocated picture is interlayer and there is a size change.  BoG agrees that bug fix be assigned to the editors.

JCTVC-N0057 MV-HEVC/SHVC HLS: On inter-layer picture selection in RPS and colPic [T. Ikai, T. Uchiumi (Sharp)]

First part reviewed in track.

Second part: Proposes that when SamplePredEnabledFlag equal to 1, don’t include those pictures in the motion pred ref list (for an individual slice, by adding condition in inter_layer_pred_layer_idc semantics). Could shorten the list.

BoG agrees to add condition, only if alt collocated idc remains in the design.

JCTVC-N0081 MV-HEVC/SHVC HLS: On inter-layer prediction related syntax [J. Xu, A. Tabatabai, O. Nakagami, T. Suzuki (Sony)]

This contribution suggests cleanups and improvement regarding the inter-layer prediction related syntax.

First, it is proposed to modify the signalling position of the syntax under P or B-slice condition in slice header to reduce redundancy. Also, it is proposed to add a condition to signal inter_layer_pred_layer_idc[ i ] for the same reason.

Second, inter_layer_ref_pic_set is proposed to arrange the syntax. The motivation is efficiently to signal the inter-layer reference information like short_term_ref_pic_set.

Finally, inter-layer sample only prediction and TMVP interaction aspect is pointed. It is proposed to check that inter_layer_sample_pred_only_flag is false before signaling collocated_ref related syntax.

In the revision1 document, an internal variable approach is alternatively added to the third aspect. InterLayerSamplePredOnlyFlag, which is derived from the number of inter reference pictures, is proposed to use for the condition to send TMVP syntax instead of checking inter_layer_sample_pred_only_flag.

For the first proposal, some experts expressed that this information should also be sent for I slices, since must be the same for all slices in the picture.

BoG agrees that if NumDirectRefLayers equals NumActiveRefLayerPics no need to send inter_layer_pred-layer_idc, and instead infer the value.  N0195 proposal 1,   N0154, and N0217 are similar. Editors to determine exact semantics for inference.

Second proposal introduces parsing dependency on VPS.  Introduces a inter layer reference picture list, similar to short term ref picture list in version 1, sent in the SPS.  Similar to N0118 which signals in the VPS.

Third proposal suggests use of inter_layer_sample_pred_only_flag to gate presence of alt collocated idc flag,

Suggestion to add an editorial NOTE to spec for SHVC encoders to use avoid using TMVP when the inter-layer ref pic is the only one in the list.  BoG suggests to delegate this to the editors.

JCTVC-N0118 MV-HEVC/SHVC HLS: On Inter layer Prediction Signaling [J. Chen, Y. Chen, Hendry, Y.-K. Wang, K. Rapaka (Qualcomm)]

This contribution proposes a mechanism to avoid slice header inter-layer prediction signaling in two aspects.

The first aspect is to cover two reportedly typical encoding configurations in MV-HEVC and SHVC. The first typical is that pictures of all direct reference layers specified by the VPS are included in the inter-layer RPS subset. The second configuration is that only the picture of the highest direct reference layer specified by the VPS is included in the inter-layer RPS subset. Furthermore, it is proposed to avoid signalling of the layer IDs of pictures to be included in the inter-layer RPS subset when the total number is equal to the number of direct reference layers specified by the VPS, as the information in this case is known.

The second aspect is to signal a set of candidate inter-layer prediction reference picture sets (ILP-RPS) in the VPS and a candidate can be indicated in a slice header to be used for derivation of the ILP-RPS for the picture.

Similar to JCTVC-N0081. but signaled in VPS, with optimizations for two typical cases for SHVC and MV-HEVC, signaled used flags not nuh_layer_id.  Putting the ILP-RPS in the SPS would reduce the opportunity for sharing of SPS by different layers.

JCTVC-N0154 MV-HEVC/SHVC HLS: On signalling of inter-layer RPS in slice segment header [J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

In slice segment header of the current SHVC as well as MV-HEVC draft text, there are syntax elements to signal direct reference layers, which are a subset of inter-layer RPS signalled in VPS extension, used by the current slice for inter-layer prediction.

This contribution asserts that it is not necessary to signal direct reference layers used by the current slice for inter-layer prediction in the slice segment header when those are identical to the inter-layer RPS for a picture having nuh_layer_id equal to layer_id of the current picture signalled in VPS extension. Two alternative syntax changes are proposed to solve the problem.

Proposal 1, see notes under JCTVC-N0081.

Proposal 2 provides an additional optimization, where a flag is signaled in the slice segment header to indicate if the list should directly match those available in the VPS.

JCTVC-N0195 / JCT3V-E0078 Comments On SHVC and MV-HEVC [S. Deshpande (Sharp)]

This document proposes comments on SHVC Draft Text 2 and MV-HEVC Draft Text 4.  The following is proposed:

- In slice segment header signalling if NumActiveRefLayerPics is equal to NumDirectRefLayers[nuh_layer_id] then the inter_layer_pred_idc[i] syntax elements are not signalled as they can be inferred.

- A gating flag is proposed for signaling syntax elements related to inter-layer prediction in slice segment header.

- A bitstream constraint related to values for syntax elements direct_dependency_flag[i][j] and max_one_active_ref_layer_flag is proposed.

In r1 revision of this document following proposals are added:

- A restriction on the alignment of IDR and BLA pictures within the same access unit is proposed to be relaxed.

- A bitstream constraint related to the values of syntax elements splitting_flag and dimension_id_len_minus1[i] is proposed.

Proposals 1 and 2 reviewed in this BoG.

For proposal 1, see notes under JCTVC-N0081.

Proposal 2.  Add presence flag in SPS for presence of inter-layer prediction syntax elements in slice header.  Provides optimization for the typical case.

JCTVC-N0217 MV-HEVC/SHVC HLS: On SHVC High Level Syntax [Y. He, X. Xiu, Y. Ye, Y. He (Interdigital)]

This contribution proposes several modifications to the current high level syntax in VPS and slice header to improve signaling flexibility and save bit overhead. These changes include (1) replace VPS syntax element max_one_active_ref_layer_flag with max_num_active_ref_layers_minus1 to improve signaling flexibility; (2) set condition on signaling inter_layer_pred_layer_idc to save slice header bits; (3) relocate sample and motion prediction syntax elements from slice header to PPS; (4) improved slice header signaling when inter_layer_sample_pred_only_flag is set to 1.

Proposal 2, see notes under JCTVC-N0081.

Proposal 3, move slice inter-layer signaling to the PPS from the slice segment header, for coding efficiency to avoid repeating in all slice headers.  Similar to JCTVC-N0131.

Proposal 4.1.1, to not include inter_layer_sample_pred_only_flag for IDR pics.  Revisit in BoG after discussion about inter_layer_sample_pred_only_flag.

Proposal 4.1.2, avoid sending alt_collocated_idc when only inter-layer prediction is allowed for SHVC. Revisit in BoG after discussion about alt_collocated_idc.

Proposal 1. Change max_one_active_ref_layer_flag to max_num_active_ref_layers_minus1.  No action.

JCTVC-N0131 MV-HEVC/SHVC HLS: On interlayer reference picture set [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

The improvements of the inter-layer reference picture set (IL-RPS) are proposed.

- Signaling num_max_active_ref_layer instead of max_one_active_ref_layer_flag.

- Moving the IL-RPS related syntax from slice header to PPS extension.

- Replacing inter_layer_pred_layer_idc with inter_layer_pred_layer_idc_delta

- Forcing RefLayerId[i] to be included in RefPicLayerId[ i ] in a descending order.

Proposal 1. Rename max_one_active_ref_layer_flag , Didn’t adjust semantics for num_inter_layer_ref_pics_minus1, which is needed.  Similar to N0217.

Proposal 2: move inter-layer predictor info to PPS from slice header

Proposal 3: Replace inter_layer_pred_layer_idc with inter_layer_pred_layer_idc_delta

Interaction with MV-HEVC coding of views in a different order than view position could lead to negative deltas.  No action.

Proposal 4: Change order of inter-layer reference pictures to be descending rather than ascending order

BoG agrees to change the default order of sorting of inter-layer predictors to be descending rather than ascending

JCTVC-N0129 MV-HEVC/SHVC HLS: On single layer for non-IRAP pictures [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

Other aspects covered in track.

Proposal 3, proposes to avoid sending inter-layer RPS when single_layer_for_non_irap_flag

No action.

General discussion

What is the best location(s) for inter-layer reference picture (ILRP) signaling – slice header, PPS, SPS, VPS?

Presence flag?  optimization for typical SHVC/MV-HEVC cases.

ILP-RPS candidates in parameter set, indication index in slice header?

Putting ILRP info in the VPS is only useful if there are multiple candidates in an RPS.

Total bitrate of ILRP signaling information is low as a percentage of total bitrate.

The solution that is simplest to implement and most efficient in the typical case is to send a presence flag in the SPS (or VPS) and conditionally sent ILRP info in the slice header only when the presence flag is set.

May need to update the inference rules in the case of different frame rates between layers.

BoG agrees to recommend to put a ILRP signaling enable flag in the VPS, conditioning the presence of the syntax elements in the slice header upon the presence flag, similar to JCTVC-N0195 proposal 2.

BoG recommends revisit by the larger group of the ILP RPS candidates in VPS concept, with slice indication.

## 6.4.3.1 General (4)

JCTVC-N0195 / JCT3V-E0078 Comments On SHVC and MV-HEVC [S. Deshpande (Sharp)]

Proposals 3 and 5 in this category

3rd proposal: A bitstream constraint related to values for syntax elements direct_dependency_flag[i][j] and max_one_active_ref_layer_flag is proposed.

No action.  No non-proponent support.

5th proposal: A bitstream constraint related to the values of syntax elements splitting_flag and dimension_id_len_minus1[i] is proposed.

Constrain that the sum of lengths be less than or equal to 6.

BoG agrees to this constraint, with editorial modifications to use “shall”.

## 6.4.4.6 Management of resampled or filtered inter-layer reference pictures (2)

N0128, N0282, plus addition of N0056 (recategorized from 6.4.6.2)

JCTVC-N0128 MV-HEVC/SHVC HLS: Reference picture marking and picture removal [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

Two proposals are presented in this contribution.

First, the marking and the picture removal of the re-sampled reference pictures and the decoded pictures used for up-scaling are proposed. The re-sampled reference pictures are marked as “not needed for output” by default, and marked as “unused for reference” immediately after used for inter-layer prediction. The decoded pictures with the original spatial resolution are marked as “unused for reference” immediately after the re-sampled reference pictures are generated, if those pictures are not used for inter-prediction by the following pictures.

Second, the picture removal process from a decoded picture buffer (DPB) is proposed. When the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB except the pictures belonging to the same access unit are emptied. The purpose of the second item is to avoid the removal of inter-layer reference pictures before inter-layer prediction.

1st proposal: It is asserted that this reduces memory requirements. However, there is no requirement currently that the upsampled reference picture be stored, as this is an implementation choice. No action.

2nd proposal:  DPB operation.  BoG recommends this proposal be reviewed in the track, not in the BoG.

JCTVC-N0282 MV-HEVC/SHVC HLS: On handling of filtered inter-layer reference [P. Lai, S. Liu, S. Lei (MediaTek)]

This contribution presents high-level syntax and decoding process for handling situations wherein alternative filtered inter-layer references are inserted into the RPS to be utilized as reference pictures in the reference picture lists. The presented changes allow both reference pictures with and without the alternative filter from the same layer to co-exist in RPS and reference picture lists, facilitating block-level switching between the two reference pictures from the same layer.

Proposal is dependent upon SCE3 inter-layer filtering.  Further study encouraged if inter-layer filtering is included in the design.

JCTVC-N0056 MV-HEVC/SHVC HLS: On inter-layer reference picture output marking [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

JCT3V-E0039 and JCTVC-N0056 contain the same proposal commonly applicable to HLS of MV-HEVC and SHVC. This contribution proposes to set picture output flag for inter-layer reference picture. The proposed changes are asserted to provide clearer operation of picture buffer management since inter-layer reference picture can be handled in the bumping process without need of distinguishing inter-layer reference picture and other types of reference pictures.

Some experts say that the resampled picture is not managed by the DPB, so the proposal not needed.  No action.

## 6.4.4.5 Reference picture list construction

JCTVC-N0316 MV-HEVC/SHVC HLS: Initial inter-layer reference picture list construction [Andrey Norkin, Usman Hakeem (Ericsson)] [late]

This contribution addresses handling of inter-layer reference pictures in the initial reference picture list construction in MV-HEVC, 3D-HEVC, and SHVC. Two reference picture sets are created, which are inserted at predefined positions making it possible to minimize use of RPLM commands. Two options have been proposed. In option 1, the reference picture sets are inserted at predefined positions. In option 2, a position for one of the two defined reference picture sets is signaled in the slice header.

No new syntax, creates new variables NumActiveRefPics0 and NumActiveRefPics1Decoding process changed once per picture.

Proposal saves approx 14.5 bits per slice, for view IBP case.  Saves 367.2 bits/sec for 1 slice/pic for test case.  At QP35, bitrate of UndoDancer is 1331 kbps, for 0.03% overall savings for QP=35,

BoG recommends to adopt.  Exact decoding process might require slight modification based upon review of contributions related to view_id.

### 6.4.4.2  Sub-layer related inter-layer prediction signalling (4)

Revisits of JCTVC-N0120, N0109, N0196

JCTVC-N0120 MV-HEVC/SHVC HLS: Signaling for Inter-layer prediction indication [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

Revisit. (Was originally reviewed in track.)

Add presence flag for signaling of max_tid_il_ref_pics_plus1[i], and inference when not present.

BoG recommends to adopt, with a minor editorial change to move location of inference.

JCTVC-N0109 MV-HEVC/SHVC HLS: Signalling for sub-layer dependency [V. Seregin, Y.-K. Wang (Qualcomm)]

Revisit. (Was originally reviewed in track.)

No new syntax elements added, but different condition for presence of ILRP syntax elements in slice header.  If max_tid_for_inter_layer is used, allows to save some bits for signaling ILRP parameters.  Question about the potential bitrate savings given the adoption of the presence flag in the VPS can avoid the need in the typical cases for slice level signaling of the ILRP parameters.

No action.

JCTVC-N0196 On Sub-layer Non-reference Pictures Indication for Inter-layer Prediction [S. Deshpande (Sharp)]

Revisit. (Was originally reviewed in track.)

Adds syntax in VPS, and changes decoding process – picture marking process, could potentially save memory to remove pictures earlier.

Some potential interaction with single loop decoding constraints. May be beneficial for pre-created base layer bitstream which didn’t use temporal scalability.

Further study.  Need better understanding of use cases, and of possible profile restrictions, especially for single loop decoding.

## 6.4.5 Tiles and parallel processing (4)

JCTVC-N0158 MV-HEVC/SHVC HLS: Bitstream restrictions on tiles and wavefronts across layers [K. Rapaka, Y.-K. Wang, A. K. Ramasubramonian, J. Chen (Qualcomm)]

This document proposes various bitstream restrictions on use of tiles and wavefront parallel processing (WPP) across layers.

For the first proposal, it was expressed that this constraint could reduce coding efficiency, especially for SNR scalability. This proposal asserts to provide some additional opportunities for synchronized parallel decoding between layers, when the tile alignment indication is not used.

For the second proposal, several proposed constraints regarding tiles and wavefront combinations.

BoG agrees to avoid using tiles in one layer and using wavefronts in another layer that is a direct dependent layer, or wavefronts in one layer and tiles in a direct dependent layer.  Could express the constraint that an enhancement layer can’t exercise a different tool than its direct dependent layers.

Also proposes deblocking across tiles shall be the same for all layers. May want to be the case only when tiles are aligned. Some concern expressed that QP may be very different at different layers.

For the third proposal, proposes alignment of CTB sizes across layers for SNR scalability. However, coding efficiency is best when low bitrates used large CTB sizes and high bitrates use small CTB sizes. May provide benefit to decrease latency when decoding aligned between layers.  Further study to provide estimates of coding efficiency costs.

For the forth proposal, interaction between level definition and tile constraints.  Suggestion to defer consideration until level definition has taken place.

JCTVC-N0160 MV-HEVC/SHVC HLS: On signaling of offset delay parameters and tile alignment [K. Rapaka, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

This document proposes various improvements related to signaling of cross-layer parallel processing offset delay parameters and tile alignment in MV-HEVC and SHVC. The discussions in this contribution are divided into four sections. The first section proposes signaling of offset delay parameters in the VPS instead of in the SPS. The second section proposes methods for signaling offset delay parameters when extended spatial scalability is in use. The third section proposes improvements in signaling of ctu_based_offset_enabled_flag. The last section proposes signaling of tile_boundaries_aligned_flag in the VPS instead of in the SPS.

In first proposal, num_ilp_restricted_ref_layers and offset delay syntax elements are currently in the SPS VUI, but this reduces opportunities for SPS sharing. Track already decided to create a VPS VUI section.  BoG agrees to send one num_ilp_restricted_ref_layers flag per layer, and move additional syntax that it signals to the VPS VUI section.

In second proposal, proposes modified variable calculations for offset delay parameters when extended spatial scalability is in use. BoG agrees to modify the variable calculation of xRef[ ] and yRef[ ] accordingly.

In the third proposal, proposes that the ctu based offset be the same for all layers. This limits flexibility and some experts preferred to not make that change. No action on this point.

In the fourth proposal, move tile_boundaries_aligned_flag from the SPS VUI to the VPS VUI. Similar proposal in JCTVC-N0199 proposal 2.  BoG agrees.  Also see notes under JCTVC-N0199.

Proponents to upload marked text version including text adoptions.

JCTVC-N0199 On Tile Alignment [S. Deshpande (Sharp), K. Misra (Sharp)]

This document proposes:

- Bitstream conformance constraints on various syntax elements when tile_boundaries_aligned_flag is equal to 1 are proposed.

- Alternatively syntax and semantics for picture parameter set (PPS) is proposed to be modified when nuh_layer_id > 0 and tile_boundaries_aligned_flag is equal to 1 to not signal certain tile related syntax elements whose value could be inferred.

- Signaling tile alignment information regarding layers in video parameter set (VPS).

In the r1 revision of this document additional bitstream constraints and equations for inference of syntax elements not signaled in PPS are added.

See notes in JCTVC-N0160.

Proposal2 variant 2 proposes more flexibility in the tile boundary align flag, to be associated with each direct dependent layer. Location of the syntax in the VPS extension is awkward, and should be moved to the VPS VUI.  BoG agrees to add the flag per direct dependent layer for each layer, but to move to the VPS VUI, with the exact location at editors’ discretion.

Proposal 1 proposes constraint language for tile_boundaries_aligned_flag related to bitstream conformance.  Imposing bitstream constraints on syntax elements based upon VUI values is not generally done in the text.  No action.

Proponent to upload marked text version including text adoptions.

## 6.6.1 Motion and prediction constrained SEI messages (5)

JCTVC-N0087 Inter-layer constrained tile sets SEI message [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI)]

This contribution proposes an SEI message that indicates inter-layer decoding independence between tiles. The syntax is aligned with the (temporal) motion constrained tile sets SEI message. In addition to region of interest based decoding, a decoder can use the information provided by the SEI message for load assignment, e.g. parallel decoding of tiles.

See notes under N0069.

JCTVC-N0159 MV-HEVC/SHVC HLS: Parallel Processing Indications for Tiles in HEVC Extensions [K. Rapaka, X. Li, J. Chen, W. Pu, Y.-K. Wang, M. Karczewicz (Qualcomm)]

Additional review in BoG.

Proposes a flag in VUI to indicate that PUs along tile borders are not allowed to use inter-layer prediction.

SEI message to indicate for each individual tile if it allows inter-layer prediction. Similar proposal in JCTVC-N0069.

See notes under N0069.

JCTVC-N0069 Motion and inter-layer prediction constrained SEI message [K. Ugur, M. M. Hannuksela (Nokia)]

Proposes modification to motion constrained tile set SEI message, which is in the RExt draft.

ROI indication.

Combining both motion and inter-layer prediction constraints in one SEI message is helpful for a decoder to more easily determine that it can do independent decoding.  BoG agrees to combine both functionalities into a single SEI message, with a presence flag per constraint type.

Other aspect of proposal: ROI for skipped tile sets.  When tiles are mapped into tile sets, the leftover region is the skipped tile set.

BoG agrees to add “skipped” tile set indicator functionality.  Ask proponents to combine following aspects into a design, in a new contribution, JCTVC-Nxxxx (K. Ugur):

    - single SEI message, with both MC and ILP constraint, able to signal either or both constraints in same message

    - shortcut flag to indicate one tile per tile set

    - when one tile per tile set shortcut flag not set, send mappings of tiles into tile sets

    - individually control whether MC and/or ILP constraint applies for each tile set

    - add shortcut (per MC and ILP) to indicate if constraint applies for all tile sets, for no tile sets, or individually signaled per tile set

    - “skipped” tile indicator

JCTVC-N0236 HLS: Extensions of motion-constrained tile sets SEI message for interactivity [C. Auyeung, J. Xu (Sony)]

It is asserted that in the motion-constrained tile sets in Range Extensions text specification: Draft 3 does not have the functionality to signal all tiles are independently decodable and to signal the regions of interest (ROI) that may have more than one tile per ROI. This contribution proposes an extension to the motion-constrained tile sets SEI message to provide this functionality. With this extension, it enables the functionality to re-define any independently decodable region of interest in a CVS without re-encoding the CVS at CTU level based on user interactivity. Consequently this proposal supports the interactivity needed for various applications such as interactive UHDTV application, dynamic high-quality zoom-in application, interactive on-demand, e-learning, smart surveillance, and etc.

BoG agrees to add shortcut (per MC and ILP) to indicate if constraint applies for all tile sets, for not tile sets, or individually signaled per tile set.  See notes under JCTVC-N0069.

JCTVC-N0088 Extension of (temporal) motion constrained tile sets SEI message [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI)]

See notes under N0069.

JCTVC-N0117 HLS: Extensions to Motion-constrained tile sets SEI message [S. Hattori, O. Nakagami, T. Suzuki (Sony)]

This contribution proposes an extension to a motion-constrained tile sets SEI message to indicate the level for a decoder to decode each defined motion-constrained tile set. The proposal provides flexibility for HEVC tile structure to be applied for various applications such as in interactive UHDTV application, dynamic high-quality zoom-in application and interactive on-demand e-learning etc.

Proposes to send a level_idc for each motion constrained tile set. It was generally agreed that such functionality would be useful.

Additional semantic text would be required to more fully describe how to apply a level calculation to a tile rather than a coded picture. Further study encouraged to better define what a level indicator means for a tile.

JCTVC-N0217 MV-HEVC/SHVC HLS: On SHVC High Level Syntax [Y. He, X. Xiu, Y. Ye, Y. He (Interdigital)]

During revisit in BoG, no action needed because of removal of related syntax elements.

## 6.2.5 Sampling position

JCTVC-N0334 Derivation of picture and slice level information for resampled interlayer reference picture [J. Chen, V. Seregin, X. Li, K. Rapaka, M. Karczewicz (Qualcomm)] [late]

This contribution provides the specification text of derivation process for picture and slice level information of resampled interlayer reference picture.

No real change to design, but change to specification text to support intended design.  (The software already implements the intended design.)

Proposes a constraint to disallow MV prediction in enhancement layer when base layer utilizes different slice types.   It is asserted that without this restriction, it is complicated to describe the decoding process of the enhancement layer, and that the decoder complexity could be reduced if the restriction would be applied.

The common test conditions to do not exploit the flexibility of different slice types in a picture, so difficult to measure any coding efficiency loss associated with the proposed restriction.  Inter-layer MV prediction generally provides approx. 1% of gain, so that is an upper bound.

A problem has been identified with the current text.  The editors are suggested to study this issue.

## 6.4.1 Generic HLS issues

JCTVC-N0355 / JCT3V-E0092 3D/MV-HEVC HLS: Extending the supported number of layers [K. Suehring, G. Tech, R. Skupin, T. Schierl (FhG HHI)] [late]

Two approaches discussed, reserved nuh_layer_id value of 63 as an escape, vs. the LSB in the slice header approach.

Further study encouraged. Suggestion to add an AHG mandate (in HLS AHG) to study comparison of the proposed approach with reserving nuh_layer_id value of 63 to provide an extension, in terms of functionality supported, signalling efficiency when feature used.

JCTVC-N0356 / JCT3V-E0223 3D/MV-HEVC HLS: Dependency signaling for extending the supported number of layers [G. Tech, K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (HHI)] [late]

In this contribution an alternative method for dependency signalling for "Extending the supported number of layers" as presented in JCT3V-E0092 is proposed. It is suggested to signal layer dependencies for the layer id extension mechanism hierarchically. The existing direct_dependency_flag syntax element is used to indicate dependencies between layer subsets, whereby a subset consists of the layers associated with the same particular nuh_layer_id value. Only for dependent subsets, dependencies for the layers within the subsets are signalled. This sparse representation of dependencies allows bit savings in the VPS. Moreover the hierarchical approach enables network elements to determine dependencies based on nuh_layer_id and direct_dependency_flag values, without considering the extension mechanism in the slice header.

Asserted benefit that an existing middle box could still extract clusters of layer using the existing nuh_layer_id values and existing direct dependency flags.

Also includes a mode where nuh_layer_id value of 63 is reserved for an extension.

Suggestion to include this proposed approach in the AHG mandate.

JCTVC-N0357 / JCT3V-E0224 3D/MV-HEVC HLS: Flexible layer clustering for extending the supported number of layers [G. Tech, K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (HHI)] [late]

Includes a variable size subset of nuh_layer_id values which can be associated with multiple layers.

Suggestion to include this proposed approach in the AHG mandate.

JCTVC-N0135 MV-HEVC/SHVC HLS: Extended maximum number of layers [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)] [late]

No need to review again.

Suggestion to include the proposed use case of large number of layers in the AHG mandate.

JCTVC-N0267 / JCT3V-E0087 MV-HEVC/SHVC HLS: On changing of the highest layer ID across AUs and multi-mode bitstream extraction [Y.-K. Wang, Y. Chen (Qualcomm)]

Proposed to disallow use of single_layer_for_non_irap_flag for SNR scalability.  It is understood that it would be stupid of an encoder to use it, but the benefit of disallowing it was questioned.

Propose to add two new sub-bitstream extraction modes, to make use of the max_tid_il_ref_pics_plus1.  Question if the resulting sub-bitstream is conformant?  Perhaps not for the second mode.

This could introduce a problem with AU boundary detection.  Further study encouraged.

JCTVC-N0065 / JCT3V-E0051 MV-HEVC/SHVC HLS: On IDR picture constraints [M. M. Hannuksela (Nokia)]

Was originally presented in the track. Reviewed again in BoG to consider POC alignment issues.

Document has been revised several times. Revision 2 proposes delta POC alignment rather than POC alignment, similar to what exists in SVC.  Revision 3 was presented, to be uploaded.

A variable value is maintained to save the POC difference between the layers, which is reset whenever any layer has an IDR.  Use POC plus this variable in the bumping process.

One aspect of the proposal is to send POC LSB in the enhancement layer IDR pictures.

Question about the impact of the proposal when layer upswitching is performed.

JCTVC-N0244 / JCT3V-E0075 MV-HEVC/SHVC HLS: Cross-layer POC alignment [Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

Proposes adding a bit to every slice header, in the extra slice header bits reserved section, even including the base layer using a reserved slice header bit.

Causes problem with pre-encoded base layer bitstream, with enhancement layer bitstream added later. Question about error resiliency.  Error resiliency issue could potentially be addressed with an SEI message.

Suggestion to only require the extra slice bit when a flag indicates that IDRs need not be aligned.

Question about impact on SEI message persistence based on POC.

Suggested to have flag in the base layer only to indicate an MSB reset in the enhancement layer.

Should be a constraint that can only set the flag with TL0 pictures.

This contribution could be combined with signaling POC LSB for IRAP pictures in enhancement layers, as proposed in JCTVC-N0066.

Have technically reviewed N0244 and have identified only the following potential technical concerns:

    - Error resiliency when a picture with the reset flag set is lost

    - The persistence scope of some SEI messages depends upon POC, so there may be some interaction, which should be studied, especially for post-processing units that handle the SEI messages

Another aspect of the contribution which should be considered is the requirement for the base layer to use one of the reserved slice header bits.

BoG recommends to adopt JCTC-N0244, with addition of POC LSB in IRAP pictures in enhancement layers.

## 6.4.2 Random access, layer switching structures and cross-layer alignment of pictures types

JCTVC-N0066 / JCT3V-E052 MV-HEVC/SHVC HLS: Layer-wise startup of the decoding process [M. M. Hannuksela (Nokia)]

BoG recommends to adopt JCTVC-N0066 version 2 layer-wise start up decoding process.

Suggestion to add a flag to indicate if all IRAPs are aligned between layers.  See notes under JCTVC-N0147.

JCTVC-N0147 / JCT3V-E0085 MV-HEVC/SHVC HLS: On restriction and indication of cross-layer IRAP picture distribution [J. Chen, Y.-K. Wang, K. Rapaka, A.-K. Ramasubramonian, Hendry (Qualcomm)]

Proposes  vps_cross_layer_irap_align_idc in the VPS to indicate 3 sets of constraints:

    - value 0: all IRAPs are aligned in one dependency chain of layers

This would enable a profile to constrain the values of the indicator.

BoG recommends to adopt a single flag to indicate value 0.

## 6.5.1 SHVC Generic HLS issues

JCTVC-N0108 SHVC HLS: Avoid resampling process for pictures used only for inter-layer motion prediction [V. Seregin, Y.-K. Wang, Y. Chen (Qualcomm)]

In current spec constrains only one resampling process, which is unclear.  Proposes placing separate constraints that sample resampling may be done once per enhan layer picture, and motion field resampling may be done once per enhan layer picture.

BoG recommends to change text constraint as described above.  Low level tool discussion suggested about question if sample prediction and motion prediction may be from different reference layers, by coding tool experts.

Should correct text to properly name decoding process sections, (Resampling process of  picture motion field ).  (Editorial only.)

## 6.6.1 Motion and prediction constrained SEI messages

JCTVC-N0383, Indication of inter-layer prediction constraints [K. Ugur, M. Hannuksela (Nokia), K. Suehring, R. Skupin, Y. Sanchez (FhG HHI), K. Rapaka, J. Chen,Y.-K Wang (Qualcomm) C. Auyeung,  S. Hattori (Sony)]

Suggestion to put into two separate SEI messages instead of combining into one.

BoG recommends to adopt inter-layer constrained tile set SEI message.  BoG also recommends to modify the syntax of the motion constrained tile set SEI message to align with the inter-layer constrained tile sets SEI message.