| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0374 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0306 |
| --- | --- |

| Title: | BoG report on SHVC/MV-HEVC HLS topics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | BoG Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill@vidyo.com |
| Source: | BoG Chair |  |  |

_____________________________

# Abstract

The BoG met 27 July to review the following contributions:

- 6.4.4.5 Reference picture list construction (4)

  - JCTVC-N0082, JCTVC-N0095, JCTVC-0216, JCTVC-0316

- 6.4.3.3 Efficient parameter set parameters signalling (3)

  - JCTVC-N0162, JCTVC-N0200, JCTVC-N0212

- 6.6.2 Layer presence and dependency change SEI messages (2)

  - JCTVC-N0173, JCTVC-N0174

The BoG recommends the following:

- Adopt JCTVC-N0082, initialization process of reference picture lists for HEVC extensions

- Adopt JCTVC-N0371, scaling list prediction in SPS and PPS (harmonization of JCTVC-N0162 and JCTVC-N0200 variant 3)

- Layer dependency change SEI message be removed from specification.  If the SEI message does remain, to adopt JCTVC-N0174 (with some editorial improvements).

The BoG plans to meet again to revisit:

- Initialization of reference picture lists: Revision to JCT3V-D0220, and harmonization of JCTVC-N0095 and JCTVC-N0216, with experimental results compared to JCTVC-N0082

# Meeting notes

## 6.4.4.5 Reference picture list construction (4)

JCTVC-N0082 MV-HEVC/SHVC HLS: On initialization process of reference picture lists for HEVC extensions [O. Nakagami, T. Suzuki (Sony)]

This contribution is a follow-up of JCTVC-M0081/JCT3V-D0058.

It is proposed that inter-layer reference picture (ILRP) is inserted between RefPicSetStCurrBefore and RefPicSetStCurrAfter in creating the temporal list. The motivation is to reduce reference picture list modification (RPLM) signalling for common coding structures.

The proposal is tested under the recommended condition described in JCTVC-M0461. It is reported that the RPLM bit for SHVC(RA), 3D(PIP) and 3D(IBP) are reduced to 51.1, 60.7 and 86.5 [%], respectively.

It is recommended to modify the list0 initialization process as proposed.

The proportion is kept since the proposal doesn’t affect the process for LD-B case.

Current:

RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr, RefPicSetIvCurr,

Proposed:

RefPicSetStCurrBefore, RefPicSetIvCurr, RefPicSetStCurrAfter and RefPicSetLtCurr.

No new syntax, just a different default order for list 0.  No complexity impact.

See notes under JCTVC-N0316.

JCTVC-N0095 MV-HEVC/SHVC HLS: Inter-layer reference pictures in reference picture list initialization [A. K. Ramasubramonian, Y. Chen, L. Zhang (Qualcomm)]

It is proposed that explicitly signaling the start position of inter-layer reference pictures in the reference picture list can be used to reduce the number of reference picture list modification commands, and hence the number of bits used in the slice header, in typical cases in MV-HEVC and SHVC.

Proposes to add two ue(v) new syntax elements in slice segment header, for list 0 and list 1, to indicate position in the list for inter-layer reference picture.

Decoding process modified to insert inter-layer ref pics in list, and relocate later reference pictures.

No experimental results for this meeting using the common test conditions defined in JCTVC-M0461, but previously proposed and had results last meeting in JCT3V-D0198.

See notes under JCTVC-N0316.

JCTVC-N0216 MV-HEVC/SHVC HLS: On Reference Picture List Modification [Y He, X. Xiu, Y Ye (InterDigital)]

It is proposed to signal the position of inter-layer reference picture set in reference picture list modification (RPLM) to save the bits. In current common test condition, reference picture list modification is invoked to allocate inter-layer reference picture (ILRP) to the end of the active reference lists when the number of active reference pictures is less than the total number of available reference pictures. To avoid signaling both temporal and ILRP reference picture one by one within current list modification framework, extra syntax elements in RPLM are proposed to signal the position of inter-layer reference picture set directly.  The average bit saving of RPLM is around 57% for SHVC RA cases, and 51% for MV/3D-HEVC cases.

Similar to N0095, but with difference in the syntax.  Four syntax elements, two flags (for list 0 and list 1), and 2 positions as u(v).

The position to locate all of the inter-layer references is signalled with individual reference picture granularity.  Decoding process modified to insert inter-layer ref pics in list, and relocate later reference pictures.

Experimental results were provided for SHVC and MV-HEVC.

See notes under JCTVC-N0316.

JCTVC-N0316 MV-HEVC/SHVC HLS: Initial inter-layer reference picture list construction [Andrey Norkin, Usman Hakeem (Ericsson)] [late]

This contribution addresses handling of inter-layer reference pictures in the initial reference picture list construction in MV-HEVC, 3D-HEVC, and SHVC. Two reference picture sets are created, which are inserted at predefined positions making it possible to minimize use of RPLM commands. Two options have been proposed. In option 1, the reference picture sets are inserted at predefined positions. In option 2, a position for one of the two defined reference picture sets is signaled in the slice header.

Option 1

Proposes to separate before and after inter-layer reference pictures.  For SHVC, only have before, while for MV-HEVC can have both before and after.

However, have more flexibility to separate the list of ref pics into lists 0 and 1 that are not necessarily exactly before and after.  Change the num_inter_layer_ref_pics syntax element into two syntax elements, for list 0 and list 1 (before and after).

May be possible to derive before and after lists without changing syntax element.  Was previously proposed in JCT3V-D0220.

For SHVC, list construction will be identical in behavior to N0082.  For MV-HEVC will differ: additional bits required for signaling, but potential savings from different position for after list.

Proposed:

RefPicListTemp0:

RefPicSetStCurrBefore, RefPicSetInterLayer0, RefPicSetStCurrAfter, RefPicSetLtCurr, RefPicSetInterLayer1.

RefPicListTemp1 as in the following:

RefPicSetStCurrAfter,  RefPicSetInterLayer1, RefPicSetStCurrBefore, RefPicSetLtCurr, RefPicSetInterLayer0.

Option 2

Similar to N0095, except signaling position of the two lists at a reference picture granularity, using ue(v) coding.

Option 1 outperforms Option 2 for SHVC Kimono LDB and LDP example because of additional cost to signal.  Option 2 outperforms Option 1 for all MV-HEVC cases.

BoG agree to N0082

Revisit later this meeting a D0220 solution (without sorting) once updated text and simulation results are available.  Can also revisit harmonized N0095 and N0216.   Comparison data vs N0082 should be provided.  Both RPL and absolute bitrate savings numbers should be provided.  Any software may be used, but both proposal and N0082 results should use the same software.

## 6.4.3.3 Efficient parameter set parameters signalling (3)

JCTVC-N0162 MV-HEVC/SHVC HLS: Inter-layer scaling list inheritance for HEVC extensions [Martin Pettersson, Thomas Rusert]

This proposal is a follow-up of the JCTVC-M0134 proposal. The contribution addresses a redundancy problem that is asserted to be present when two or more different SPSs or PPSs containing identical scaling list (quantization matrix) data are present for different layers in multi-layer HEVC extensions. If scaling list data is present in SPS or PPS it typically takes up a major part of the amount of bits in the particular SPS or PPS. The scaling list data would in the current version of the draft be duplicated, causing bit overhead for parameter set transmission and storage.

It is proposed for HEVC extensions to introduce inter-layer scaling list inheritance to reduce bit redundancy. This implies extending the definition of SPS and PPS in the extension specifications. In the proposed SPS/PPS inheritance scheme, scaling list data is inherited from a reference SPS/PPS, determined as the active SPS/PPS of the reference layer, which is signaled in the referencing SPS/PPS.

See notes under JCTVC-N0200.

JCTVC-N0200 On Scaling List Data Signaling [S. Deshpande (Sharp), S. Liu (MediaTek), S. Lei (MediaTek), K. Sato (Sony)]

This document proposes syntax and semantics for signaling scaling list data for SHVC layers with nuh_layer_id > 0. Three variant syntax alternatives are proposed. PPS and SPS activation method and bitstream constraint is proposed in relation to the proposed syntax and semantics.

Variant 1 has a parsing dependency of the SPS on the VPS

Should restrict such that can only predict from a dependent layer.

In what scenarios would scaling list inheritance be desirable when parameter set sharing couldn’t be used?  When coding tools differed in the layers, including LCU size, min CU size, min transform size, RQT depth, in SPS. Picture resolution in SPS currently disallows SPS sharing in spatial scalability layer.  Also, if SPS is different, PPS can’t be shared.

Preferred solution: N0162 syntax (same as N0200 variant 3), except changing the reference layer to be an absolute nuh_layer_id with u(6). Includes bitstream constraint that the reference layer must not itself infer from another layer.  Activation language needed from N0200 (to be checked by editors).  Include a constraint that the reference layer must be a dependent layer of the current layer (when the SPS/PPS activated).  Proponents to create new contribution N0371 which implements the description above.

BoG: Agree to recommend adoption of combined contribution JCTVC-N0371 as described in preferred solution above.

JCTVC-N0212 SHVC HLS: On Inter Layer Parameter Set [Y. He, Y. Ye, Y. He (InterDigital)]

This contribution document proposes a new parameter set, inter-layer parameter set (IPS), to address the redundant issue of SPS syntax elements across multiple layers. The proposed IPS aggregates groups of parameters shared by multiple layers, and each layer could refer to the IPS parameters. As a result, the number of bits used for SPS would be significantly reduced when multiple layers shares the identical parameter values. The IPS could further remove the parsing dependency when the parameters placed in different parameter sets could be included in the same IPS. In addition, the proposed new parameter set also avoids the problems caused by SPS to SPS, or VPS to SPS parameter set prediction.

Propose a new parameter set, called the inter_layer_parameter_set, similar to the VPS, but not used for capabilities exchange.  Would need a new NUT for the IPS.

Scaling list prediction savings only applies to SPS, not to PPS.

Similar to previously proposed header parameter set.

Would lose bits for 2 layer case, but small savings for 3 or more layers sending separate SPSs without sharing.  (Only RBSP bits were counted. )

It was suggested that even if IPS existed, the capability of signaling complete info in the SPS should be maintained.

Would IPS be optional?  No existing parameter sets are optional.

Further study.  What would the size of the IPS be, and could it fit inside the VPS?  Bitrates for the different parameter subsets, especially the totals with and without SPS scaling list parameters.

## 6.6.2 Layer presence and dependency change SEI messages (2)

JCTVC-N0173 MV-HEVC/SHVC HLS: On Layers Present SEI message [J. W. Kang (ETRI), J. Lee, H. Lee, J. S. Choi, T. C. Thang (UoA)]

Currently the layer present SEI message provides the information of non-present layers after some modification. The current solution just focuses on the layers identified by the nuh_layer_id parameter. In practice, a CVS could be modified in temporal dimension as well. This contribution proposes to describe the (temporal) sub-layers which are not present in a CVS.

Usefulness of the proposed SEI modifications questioned. No action.

JCTVC-N0174 MV-HEVC/SHVC HLS: On Layer Dependency Change SEI message [T. C. Thang (UoA), J. W. Kang, J. Lee, H. Lee, J. S. Choi (ETRI)]

Currently, in the layer dependency change SEI message, the information for all layers is specified. In fact, the layer dependency information of some (or even many) layers may be unchanged. Obviously, it is unnecessary to describe again the layer dependency information of such layers. This contribution proposes to add a flag to indicate whether the dependency change information is available for a given layer.

BoG recommends that the layer dependency change SEI message be removed. If the SEI message does remain, BoG recommends to adopt contribution, with some editorial improvements.