| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0310 |
| --- | --- |

| Title: | BoG report on draft text for level definitions in MV-HEVC stereo profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Anthony Vetro, Mitsubishi Electric Miska Hannuksela, Nokia Ying Chen, Qualcomm | Email: | avetro@merl.com miska.hannuksela@nokia.com cheny@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

The BoG reviewed level limits for MV-HEVC, and recommends to resolve the open questions on DPB/HRD and to specify the level constraints highlighted below accordingly for inclusion into the draft text.

- [Ed: Given that the level limits are only being defined for stereo and DPB/HRD may be specified on a per layer basis, the level limits for MV-HEVC may only require minimal changes.]

  - Tiers and levels

    - General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag equal to 0 is considered to be a lower tier than the tier with general_tier_flag equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this annex:

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0 (i.e. the 0-th access unit).

- Let picture n be the coded picture or the corresponding decoded picture of access unit n.

- Let the variable CpbBrVclFactor be equal to 1000.

- Let the variable CpbBrNalFactor be equal to 1100.

The value of mvScaleFactor is set equal to 2. [Ed: this may not be necessary]

The value of NumViews is set equal to 2 (for the stereo profile). [Ed: this may not be necessary]

[Ed: PicSizeInSamplesY corresponds to the spatial resolution of a picture; it is assumed that the picture size in each view is the same.]

Bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

- PicSizeInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A-1.

- The value of pic_width_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of pic_height_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 shall be less than or equal to MaxDpbSize, which is derived as follows:

if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf, 16 )
else if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, 16 )	(A-1)
else if( PicSizeInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxDpbSize = maxDpbPicBuf

where MaxLumaPs is specified in Table A-1 and maxDpbPicBuf is equal to 6.

[Ed: The above needs to be considered depending on the outcome of the DPB spec. If DPB is specified per layer, then no change is needed, it just needs to be clarified that these constraints apply per layer. But if the DPB is for all views, then certain parameters (e.g., MaxLumaPs and maxDpbBuf) should be doubled.]

- For level 5 and higher levels, the value of CtbSizeY shall be equal to 32 or 64.

- The value of NumPocTotalCurr shall be less than or equal to 8.

- The value of num_tile_columns_minus1 shall be less than MaxTileCols and num_tile_rows_minus1 shall be less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A-1.

- For the VCL HRD parameters, CpbSize[ i ] shall be less than or equal to CpbBrVclFactor * MaxCPB for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxCPB is specified in Table A-1 in units of CpbBrVclFactor bits.

- For the NAL HRD parameters, CpbSize[ i ] shall be less than or equal to CpbBrNalFactor * MaxCPB for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxCPB is specified in Table A-1 in units of CpbBrNalFactor bits.

Table A-1 specifies the limits for each level of each tier.

A tier and level to which the bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc as follows:

–	general_tier_flag equal to 0 indicates conformance to the Main tier, and general_tier_flag equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A-1. general_tier_flag shall be equal to 0 for levels below level 4 (corresponding to the entries in Table A-1 marked with "-").

–	general_level_idc shall be set equal to a value of 30 times the level number specified in Table A-1.

Table A-1 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (1000 bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |

Informative subclause Error: Reference source not found shows the effect of these limits on picture rates for several example picture formats.

    - Profile-specific level limits for the Main and Main 10 profiles

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

Bitstreams conforming to the Main or Main 10 profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

- The nominal removal time of access unit n (with n greater than 0) from the CPB, as specified in subclause Error: Reference source not found, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( PicSizeInSamplesY  MaxLumaSr, fR ) for the value of PicSizeInSamplesY of picture n − 1, where MaxLumaSr is the value specified in Table A-2 that applies to picture n − 1.

- The difference between consecutive output times of pictures of the same layer from the DPB, as specified in subclause Error: Reference source not found, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( PicSizeInSamplesY  MaxLumaSr, fR ) for the value of PicSizeInSamplesY of picture n, where MaxLumaSr is the value specified in Table A-2 for picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

- The removal time of access unit 0 shall satisfy the constraint that the number of slice segments in picture 0 is less than or equal to Min( MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSliceSegmentsPerPicture * PicSizeInSamplesY / MaxLumaPs, MaxSliceSegmentsPerPicture ), for the value of PicSizeInSamplesY of picture 0, where MaxSliceSegmentsPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Table A-1 and Table A-2, respectively, that apply to picture 0.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slice segments in picture n is less than or equal to Min( MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ), MaxSliceSegmentsPerPicture ), where MaxSliceSegmentsPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Table A-1 and Table A-2, respectively, that apply to picture n.

- For the VCL HRD parameters, BitRate[ i ] shall be less than or equal to CpbBrVclFactor * MaxBR for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxBR is specified in Table A-2 in units of CpbBrVclFactor bits/s.

- For the NAL HRD parameters, BitRate[ i ] shall be less than or equal to CpbBrNalFactor * MaxBR for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxBR is specified in Table A-2 in units of CpbBrNalFactor bits/s.

- The sum of the NumBytesInNalUnit variables for access unit 0 shall be less than or equal to 1.5 * ( Max( PicSizeInSamplesY, fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr for the value of PicSizeInSamplesY of picture 0, where MaxLumaSr and MinCr are the values specified in Table A-2 that apply to picture 0.

- The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) shall be less than or equal to 1.5 * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AyCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr and MinCr are the values specified in Table A-2 that apply to picture n.

- The removal time of access unit 0 shall satisfy the constraint that the number of tiles in picture 0 is less than or equal to Min( MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * PicSizeInSamplesY / MaxLumaPs, MaxTileCols * MaxTileRows ), for the value of PicSizeInSamplesY of picture 0, where MaxTileCols and MaxTileRows are the values specified in Table A-1 that apply to picture 0.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in picture n is less than or equal to Min( MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ), MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Table A-1 that apply to picture n.

Table A-2 – Tier and level limits for the Main and Main 10 profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (1000 bits/s) |  | Min Compression Ratio MinCr |
| --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |
| 1 | 552 960 | 128 | - | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 |