| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0383 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0312 |
| --- | --- |

| Title: | Indication of inter-layer and motion constrained prediction constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Ugur, M. Hannuksela (Nokia), K. Suehring, R. Skupin, Y. Sanchez (FhG HHI), K. Rapaka, J. Chen,Y.-K Wang (Qualcomm) C. Auyeung, S. Hattori (Sony) | Email: | karsten.suehring@hhi.fraunhofer.de |
| Source: | BoG on HLS |  |  |

_____________________________

# Abstract

The BoG on HLS reviewed the documents related to inter-layer constrained tiles, and the related SEI messages. The BoG decided to include some of the functionalities common in some proposals and authors were requested to write the SEI message combining the proposals. Related contributions are JCTVC-N0069, JCTVC-N0087, JCTVC-N0088, JCTVC-N0159 and JCTVC-N0236

# Inter-layer constrained tile sets SEI message

The following syntax and semantics are proposed for a new Inter-layer constrained tile sets SEI message:

| inter_layer_constrained_tile_sets ( payloadSize ) { | Descriptor |
| --- | --- |
| il_all_tiles_exact_sample_value_match_flag | u(1) |
| il_each_tile_one_tile_set_flag | u(1) |
| if (!each_tile_one_tile_set_flag) { |  |
| il_num_sets_in_message_minus1 | ue(v) |
| if(il_num_sets_in_message_minus1) |  |
| skipped_tile_set_present_flag | u(1) |
| numSignificantSets = il_num_sets_in_message_minus1– skipped_tile_set_present_flag+1 |  |
| for( i = 0; i <= numSignificantSets; i++) { |  |
| ilcts_id[ i ] | ue(v) |
| il_num_tile_rects_in_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_tile_rects_in_set_minus1[ i ]; j++) { |  |
| il_top_left_tile_index[ i ][ j ] | ue(v) |
| il_bottom_right_tile_index[ i ][ j ] | ue(v) |
| } |  |
| ilc_idc[ i ] | u(2) |
| if ( ! il_all_tiles_exact_sample_value_match_flag ) |  |
| il_exact_sample_value_match_flag[ i ] | u(1) |
| } |  |
| } else |  |
| all_tiles_ilc_idc | u(2) |
| } |  |

- Inter-layer constrained tile sets SEI message semantics

The scope of the inter-layer constrained tile sets SEI message is the complete CVS. When an inter-layer tile sets SEI message is present in any access unit of a CVS, it shall be present for the first access unit of the CVS in decoding order and may also be present for other access units of the CVS.

The inter-layer constrained tile sets SEI message shall not be present for a layer when tiles_enabled_flag is equal to 0 for any PPS that is active in the layer.

The inter-layer constrained tile sets SEI message shall not be present for a layer unless every PPS that is active for the layer has tile_boundaries_aligned_flag equal to one or fulfills the conditions that would be indicated by tile_boundaries_aligned_flag being equal to one.

The presence of the inter-layer tile sets SEI message indicates that the inter-layer inter prediction process is constrained such that no sample value outside each identified tile set, and no sample value at a fractional sample position that is derived using one or more sample values outside the identified tile set, is used for inter prediction of any sample within the identified tile set.

NOTE 1 – When loop filtering is applied across tile boundaries, inter prediction of any samples within a inter-layer constrained tile set that refers to samples within 4 samples from a motion-constrained tile set boundary that is not also a picture boundary may result in propagation of mismatch error. An encoder can avoid such potential error propagation by avoiding the use of motion vectors that cause such references.

When more than one inter-layer constrained tile sets SEI message is present within the access units of a CVS, they shall contain identical content.

The number of inter-layer constrained tile sets SEI messages in each access unit shall not exceed 5.

il_all_tiles_exact_sample_value_match_flag equal to 0 indicates that, within the coded video sequence, when the coding tree blocks that are outside an identified tile are not decoded and the boundaries of the identified tile is treated as picture boundaries for purposes of the decoding process, the value of each sample in the identified tile may not be exactly the same as the value of the same sample when all the coding tree blocks of the picture are decoded. il_all_tiles_exact_sample_value_match_flag equal to 1 indicates that, within the coded video sequence, when the coding tree blocks that do not belong to an identified tile are not decoded and the boundaries of the identified tile is treated as picture boundaries for purposes of the decoding process, the value of each sample in the tile would be exactly the same as the value of the sample that would be obtained when all the coding tree blocks of all pictures in the coded video sequence are decoded.

il_each_tile_one_tile_set_flag equal to 1 indicates that each tile forms an own inter-layer constrained tile set.  If il_each_tile_one_tile_set_flag is equal to zero, tile sets are signalled explicitly.

il_num_sets_in_message_minus1 plus 1 specifies the number of inter-layer tile sets identified in the SEI message. The value of il_num_sets_in_message_minus1 shall be in the range of 0 to 255, inclusive.

skipped_tile_set_present_flag equal to 1 specifies that the last tile set consists of those tiles that are not included in any earlier tile sets in the same message and all the prediction blocks that are inside the tile set having nuh_layer_id equal to mctsNuhLayerId are inter-layer predicted from inter-layer reference pictures with nuh_layer_id equal to RefLayerId[ mctsNuhLayerId ][ NumDirectRefLayers[ mctsNuhLayerId ] – 1 ] and no residual_coding syntax structure is present in any transform unit of the identified tile set. skipped_tile_set_present_flag equal to 0 indicates that the last tile set may or may not consist of those tiles that are not included in any earlier tile sets in the same message and all the prediction blocks that are inside the identified tile set having nuh_layer_id equal to mctsNuhLayerId may or may not be inter-layer predicted and residual_coding syntax structure may or may not be present in any transform unit of the identified tile set.

ilcts_id[ i ] contains an identifying number that may be used to identify the purpose of the i-th identified tile set (for example, to identify an area to be extracted from the coded video sequence for a particular purpose). The value of pan_scan_rect_id shall be in the range of 0 to 232 − 2, inclusive.

Values of ilcts_id[ i ] from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of ilcts_id[ i ] from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of ilcts_id[ i ] in the range of 256 to 511 or in the range of 231 to 232 − 2 shall ignore (remove from the bitstream and discard) it.

il_num_tile_rects_in_set_minus1[ i ] plus 1 specifies the number of rectangular regions of tiles in the i-th identified inter-layer constrained tile set. The value of il_num_tile_rects_in_set_minus1[ i ] shall be in the range of 0 to (num_tile_columns_minus1 + 1) * (num_tile_rows_minus1 + 1) − 1, inclusive.

il_top_left_tile_index[ i ][ j ] and il_bottom_right_tile_index[ i ][ j ] identify the tile position of the top-left tile and the tile position of the bottom-right tile in a rectangular region of the i-th identified inter-layer constrained tile set, respectively, in tile raster scan order.

il_exact_sample_value_match_flag[ i ] equal to 0 indicates that, within the coded video sequence, when the coding tree blocks that are outside of the i-th identified inter-layer constrained tile set are not decoded and the boundaries of the inter-layer constrained tile set are treated as picture boundaries for purposes of the decoding process, the value of each sample in the identified tile set may not be exactly the same as the value of the same sample when all the coding tree blocks of the picture are decoded. il_exact_sample_value_match_flag[ i ] equal to 1 indicates that, within the coded video sequence, when the coding tree blocks that do not belong to the inter-layer constrained tile set are not decoded and the boundaries of the inter-layer constrained tile set are treated as picture boundaries for purposes of the decoding process, the value of each sample in the inter-layer constrained tile set would be exactly the same as the value of the sample that would be obtained when all the coding tree blocks of all pictures in the coded video sequence are decoded.

NOTE 1 – It should be feasible to use exact_sample_value_match_flag equal to 1 when using certain combinations of loop_filter_across_tiles_enabled_flag, pps_loop_filter_across_slices_enabled_flag, pps_deblocking_filter_disabled_flag, slice_loop_filter_across_slices_enabled_flag, slice_deblocking_filter_disabled_flag, sample_adaptive_offset_enabled_flag, slice_sao_luma_flag, and slice_sao_chroma_flag.

ilc_idc[ i ] equal to 0 specifies that the inter-layer prediction process may or may not be constrained for the identified tile set having nuh_layer_id equal to mctsNuhLayerId. ilc_idc[ i ] equal to 1 specifies that no sample value outside each identified tile set and no sample value at a fractional sample position that is derived using one or more sample values outside the identified tile set is used for inter-layer prediction of any sample within the identified tile set with nuh_layer_id equal to mctsNuhLayerId. ilc_idc[ i ][ j ] equal to 2 specifies that no inter-layer prediction takes place for the indicated tile set with nuh_layer_id equal to mctsNuhLayerId. ilc_idc[ i ] equal to 3 is reserved.

all_tiles_ilc_idc equal to 0 specifies that for each tile the inter-layer prediction process may or may not be constrained for the tile having nuh_layer_id equal to mctsNuhLayerId. all_tiles_ilc_idc equal to 1 specifies that no sample value outside each tile and no sample value at a fractional sample position that is derived using one or more sample values outside each tile set is used for inter-layer prediction of any sample within the identified tile set with nuh_layer_id equal to mctsNuhLayerId. all_tiles_ilc_id] equal to 2 specifies that no inter-layer prediction takes place for the indicated tile set with nuh_layer_id equal to mctsNuhLayerId. The value of all_tiles_ilc_idc equal to 3 is reserved.

# Motion constrained tile sets SEI message

The motion constrained tile sets SEI message is proposed to be changed in the following way.

| temporal_motion _constrained_tile_sets ( payloadSize ) { | Descriptor |
| --- | --- |
| mc_all_tiles_exact_sample_value_match_flag | u(1) |
| each_tile_one_tile_set_flag | u(1) |
| if ( !each_tile_one_tile_set_flag ) { |  |
| num_sets_in_message_minus1 | ue(v) |
| for( i = 0; i <= num_sets_in_message_minus1; i++) { |  |
| mcts_id[ i ] | ue(v) |
| num_tile_rects_in_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_tile_rects_in_set_minus1[ i ]; j++) { |  |
| top_left_tile_index[ i ][ j ] | ue(v) |
| bottom_right_tile_index[ i ][ j ] | ue(v) |
| } |  |
| if ( ! mc_all_tiles_exact_sample_value_match_flag ) |  |
| exact_sample_value_match_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

mc_all_tiles_exact_sample_value_match_flag equal to 0 indicates that, within the coded video sequence, when the coding tree blocks that are outside any identified tile are not decoded and the boundaries of the identified tile is treated as picture boundaries for purposes of the decoding process, the value of each sample in the tile may not be exactly the same as the value of the same sample when all the coding tree blocks of the picture are decoded. all_tile_exact_sample_value_match_flag equal to 1 indicates that, within the coded video sequence, when the coding tree blocks that do not belong to any identified tile are not decoded and the boundaries of the identified tile is treated as picture boundaries for purposes of the decoding process, the value of each sample in the tile would be exactly the same as the value of the sample that would be obtained when all the coding tree blocks of all pictures in the coded video sequence are decoded.

each_tile_one_tile_set_flag equal to 1 indicates that each tile forms an own motion-constrained tile set.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. and Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).