| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E1008 |
| --- | --- |

| Title: | MFC Test Model 2 |  |  |
| --- | --- | --- | --- |
| Status: | Draft Output Document of JCT-3V |  |  |
| Purpose: | Test Model Description |  |  |
| Author(s) or Contact(s): | Peng Yin, Walt Husak 432 Lakeside Dr. Sunnyvale CA 94085 USA | Tel: Email: | +1 408 330 3251 {pyin,wjh}@dolby.com |
| Source: | Dolby Laboratories Inc. |  |  |

_____________________________

# Abstract

This document serves as a source of general tutorial information on MFC Reference Test Model for MFC specification.

CONTENTS

Page

Abstract	1

List of figures	3

1	Introduction	4

2	Description of MFC Test Model	4

2.1	General overview of MFC	4

2.2	MFC EL Input Signal Generation	5

2.3	MFC Codec Description	6

2.4	MFC Enhanced Resolution Reconstruction	7

2.5	Encoder control	7

2.6	Software Implementation Description and Usage	7

3	Reference	8

- List of figures

Figure 2-1: Spectrum visualization of white noise in FC system	4

Figure 2-2: Spectrum visualization of white noise in MFC system	5

Figure 2-3: Enhancement layer signal generation	5

Figure 2-4 – MFC RPU (filter mode)	6

Figure 2-5: Vertical filtering in interlaced RPU	6

Figure 2-6: MFC Codec	6

Figure 2-7: MFC Enhanced Resolution Reconstruction	7

# Introduction

The MFC test model is based on a proposal [1] in response to the CfP of the Multi-resolution Frame-Compatible (MFC) extension of AVC and is further simplified and improved [2][3][4]. The specification of MFC is in [5]. The solution is an MVC based framework, in which the enhancement layer carries high frequencies missing in the orthogonal direction of the frame compatible base layer. In order to generate a good prediction reference for the enhancement layer which has the orthogonal frame packing format of the base layer, a reference process unit (RPU) is introduced for frame packing format conversion. The RPU is operated in two modes. It can generate either a low-pass filtered carrier picture or a flat gray picture as the inter-view reference. The first mode provides higher coding efficiency while the second mode has lower complexity.

# Description of MFC Test Model

## General overview of MFC

MFC is a scalable extension of the Frame Compatible (FC) AVC base layer to full resolution with an addition of an enhancement layer. The base layer (BL) carries FC signal, thus legacy decoders can still decode a pair of half-resolution views and perform the proper up-sampling on the BL pictures. The enhancement layer (EL) carries the orthogonal high frequencies missing in the base layer. The base layer and enhancement layer use orthogonal FC formats. Figure 2-1(a) shows a “flat” frequency spectrum (of a white noise image) at full resolution, Figure 2-1 (b) shows the spectrum of the upsampled frame compatible white noise image with FC-SbS packing. Figure 2-1 (c) is similar to (b) but using FC-TaB packing. Due to the use of low pass filtering at one direction to create the frame compatible signal, FC-SbS preserves vertical frequencies but loses horizontal high frequencies. FC-TaB, on the contrary, preserves horizontal frequencies but loses vertical high frequencies.

MFC brings back full resolution by sending the missing high frequencies in an FC upsampled image through an enhancement layer. When the base layer is SbS and the enhancement layer is TaB, MFC preserves vertical high frequencies in the base layer and horizontal high frequencies in the enhancement layer, respectively. When the base layer is TaB and the enhancement is SbS, MFC preserves horizontal high frequencies in the base layer and vertical high frequencies in the enhancement layer, respectively. Figure 2-2(c) shows the spectrum of white noise using MFC assuming BL is FC-SbS.

(a)	FR                           (b) FC-SbS                        (c) FC-TaB

Figure 2-1: Spectrum visualization of white noise in FC system

(a) FR                               (b) FC-SbS                              (c) MFC

Figure 2-2: Spectrum visualization of white noise in MFC system

## MFC EL Input Signal Generation

The input generation of EL signal is not normative. The EL signal is the differential residue from the original full resolution image and the upsampled base layer FC image. One recommended way is to use the uncompressed upsampled base layer FC signal instead of the reconstructed signal, so as to keep low overhead of coding the enhancement layer signal. In this way, the coding artifacts in the base layer are not included in the EL. For the coding of the residual picture, a low frequency carrier signal is generated from BL signal using the RPU and is added to the residue picture (as shown in Figure 2-3). This allows us to have maximum freedom to decouple the design of muxing filters in input and RPU filters in codec [4].

Figure 2-3: Enhancement layer signal generation

The RPU has two operational modes: filter mode and DC mode. In the DC mode, the carrier image pels are set to a constant value, 128, which avoids the filtering process and thus has the minimum complexity. In the filter mode, RPU includes a 1D downsampling filter and 1D upsampling filter for the format conversion from the base layer FC format to the enhancement layer FC format. Because the high frequencies in a carrier image are not used for reconstruction and the purpose to have such a signal is to increase coding efficiency for the enhancement layer signal, the downsampling filter is designed to have a very low cutoff frequency. To maintain the integrity of the residual image, the range of the carrier signal is reduced by half. The right shift operation is integrated into the downsampling filter process in the RPU. The filters are also designed to have a low complexity. Figure 2-4 illustrates how the RPU filter mode works for a FC-SbS base layer signal. The RPU will first perform vertical downsampling on the base layer picture, and then it will perform horizontal upsampling to generate a TaB prediction picture. As for the RPU processing for interlaced sequences, vertical filtering is done for each field independently. The top field and bottom field of a frame are separated first, and then vertical downsampling/upsampling is performed for each field separately. The top and bottom field are then merged (line interleaved) back to create the frame. Figure 2-5 shows the vertical filtering process for interlaced sequences in the RPU.

Figure 2-4 – MFC RPU (filter mode)

Figure 2-5: Vertical filtering in interlaced RPU

The generation of the enhancement layer signal is described as follows, assuming the base layer is in FC-SbS format:

- Horizontally upsample BL signal to full size;

- Create the full size residue signal by subtracting the original upsampled BL signal from the FR original source signal;

- Mux full size residue signal from Step 2) into TaB format after vertical downsampling;

- If RPU filter mode is on, generate a TaB carrier signal through the RPU using the original BL SbS signal by converting SbS signal to TaB; otherwise, generate a constant value image;

- Add the TaB residue signal from Step 3) and the TaB carrier signal from Step 4) to form an EL signal.

## MFC Codec Description

MFC codec is built upon MVC. Figure 2-6 shows a simplified block diagram of MFC coding process. RPU is used to process decoded BL picture for inter-view prediction. The inter-view reference is incorporated into the reference lists used for EL picture coding. There is no DPB modification required.

Figure 2-6: MFC Codec

## MFC Enhanced Resolution Reconstruction

After decoding, MFC needs a reconstruction process (the 3D-Demux process in Figure 2-6) to combine the orthogonal frequencies carried by the base layer and the enhancement layer to form the full resolution output image. The reconstruction is not normative. One recommended method is shown in Figure 2-7. Assuming the base layer signal is in SbS format and the enhancement layer signal is in TaB format, respectively. The reconstruction involves the following steps:

1)	Horizontally upsample the base layer FC-SbS signal to full size;

2)	Created the residue image by subtracting the RPU processed base layer signal in TaB format from the enhancement layer signal to get the lost orthogonal high frequency information;

3)	Vertically upsample the residual signal from Step 2) to full size;

4)	Add the upsampled base layer signal from Step 1) and the upsampled residue signal from Step 3).

Figure 2-7: MFC Enhanced Resolution Reconstruction

## Encoder control

MFC is implemented using MVC framework in the AVC reference software JM18.3. All mode decision and motion estimation operations are performed as in the original JM18.3 software using the well-known Lagrangian technique with the cost measure Distortion + λ⋅Rate. λ is the Lagrangian multiplier that is derived based on the used quantization parameter. No specific encoder optimization is added in the simulations presented hereby [1][2][3][4]. The base layer configuration follows the FC anchor case defined in MFC CfP [6]. The enhancement layer uses the same setting as MVC in JM18.3.

## Software Implementation Description and Usage

MFC is implemented based on JM18.3 reference software with use of C source code. The functions related to the MFC algorithm are implemented as static libraries in a SDK and built with a clean interface in JM 18.3 software. The SDK is split into encoder_layer, rpu_layer, decoder_layer and utilities_layer to facilitate integration of MFC to any MVC based encoder or decoder with minimal effort. The SDK has standalone test routines which make it easier to track the flow of MFC code. The core RPU functions are referenced using function pointers during initialization. This means that the RPU functions optimized for different platforms can be easily added and enabled in the MFC SDK.

MFC software and the software manual can be found in [7]. A simple usage of the software is described as follows. For detailed usage of the software, please refer to the software manual.

Example usage of encoder: lencod.exe –d encoder_mfc.cfg

Example usage of decoder:

- If only decode the base layer and enhancement layer bitstream: ldecod.exe -d decoder.cfg -p ExportViews=0

- If fully reconstruct left and right view: ldecod.exe -d decoder.cfg -p ExportViews=1

# Reference

- P. Yin, T. Lu, T. Chen, H. Ganapathy, G. Lakshminarayanan and W. Husak, “Description of MFC Video Coding Technology Proposal by Dolby”, m26661,102th MPEG Meeting, Shanghai, China, October, 2012

- P. Yin, T. Lu, T. Chen, and W. Husak, “Revision of m26661”, m27249,102th MPEG Meeting, Shanghai, China, October, 2012

- P. Yin, H. Ganapathy, T. Lu, T. Chen, and W. Husak, “Unification of Upsampling Filters in MFC”, JCT3V-C0037, JCT3V 3rd Meeting: Geneva, CH, Jan. 2013

- T. Lu, H. Ganapathy, P. Yin, T. Chen, and W. Husak, “Study of Carrier Signal in MFC”, JCT3V-D0097, JCT3V 4th Meeting: Incheon, KR, Apr. 2013

- P. Yin, Y-K. Wang, and G. J. Sullivan, “Draft 3 of Multi-resolution Frame Compatible Stereo Coding (MFC)”, JCT3V-D1007(N13558), JCT3V 3rd Meeting: Incheon, Korea, Apr. 2013

- ISO/IEC JTC 1/SC 29/WG 11, “Call for Proposals on MFC”, MPEG doc. w12961, July 2012, Stockholm, Sweden

- P. Yin and D. Tian, “Draft 1 of MFC Reference Software”, JCT3V-E1013, JCT3V 5th Meeting: Vienna, AT, July 2013