| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E1011 |
| --- | --- |

| Title: | 3D Video Subjective Quality Assessment Test Plan |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Testing |  |  |
| Author(s) or Contact(s): | Anthony Vetro, Mitsubishi Electric Touradj Ebrahimi, EPFL Vittorio Baroncini, FUB | Email: | avetro@merl.com touradj.ebrahimi@epfl.ch vittorio@fub.it |
| Source: | JCT-3V |  |  |

_____________________________

# Abstract

This document describes the subjective quality assessment test plan for AVC-based coding of multiview video and depth including test conditions, evaluation methodology and timeline.

# Introduction

The primary usage scenario for 3D video is to support 3D video applications, where 3D depth perception of a visual scene is provided by a 3D display system. There are many types of 3D display systems including classic stereo systems that require special-purpose glasses to more sophisticated multiview auto-stereoscopic displays that do not require glasses.

A new generation of 3D Video Coding technology that goes beyond the capabilities of existing standards to enable both advanced stereoscopic display processing and improved support for auto-stereoscopic multi-view displays has been the primary subject of work by the JCT-3V group. A new data format and associated compression technology to enable the high-quality reconstruction of synthesized views for 3D displays have been developed for both AVC and HEVC-based coding frameworks. As part of this work, two amendments of the AVC standard have been developed as outlined below.

- MVC compatible extension including depth (MVC+D): The main target of this extension is to enable 3D enhancements while maintaining MVC stereo compatibility.  The method invokes an independent second stream for the representation of depth maps that are treated as monochrome video data, as well as high-level syntax signaling of the necessary information to express the interpretation of the depth data and its association with the video data. There are no changes to the macroblock-level syntax, semantics and decoding processes of AVC/MVC.  The specification of this extension (ISO/IEC 14496-10:2012/Amd.2) has reached FDAM status in January 2013.

- AVC compatible video-plus-depth extension (3D-AVC): Stereo and multi-view video systems are challenging and require a higher data rate, even more when depth data are included in the representation. Therefore, a second extension has been developed that aims for higher compression efficiency by jointly compressing texture and depth data. With this approach, compatibility with MVC is not necessarily retained, however applications exist which only require monoscopic AVC compatibility. The specification of this extension (ISO/IEC 14496-10:2012/Amd.3) has reached DAM status in April 2013.

The above codec configurations will be subject to 3D video quality assessment testing, with the purpose of obtaining evaluation data that could be used to develop objective quality metrics that better correlate with subjective scores. In the following sections, the timeline for this test, the test conditions and evaluation procedure are described.

# Timeline

2013/08/30:		Finalize 3D-ATM v9 software

2013/09/13:		Prepare test bitstreams and viewing material with cross-verification

2013/09/20:		Transfer viewing material and test data to test site

2013/12/20:		Conduct subjective viewing

2014/01/10:		Report results at 7th JCT-3V meeting in San Jose

# Test Conditions

## Input Test Sequences

The multiview test sequences with associated depth data, and corresponding input views to be used for experiments are specified in the table below.

| Seq. ID | Test Sequence | 2-view input | 3-view input |
| --- | --- | --- | --- |
| S01 | Poznan_Hall2 | 7-6 | 7-6-5 |
| S02 | Poznan_Street | 5-4 | 5-4-3 |
| S03 | Undo_Dancer | 1-5 | 1-5-9 |
| S04 | GT_Fly | 9-5 | 9-5-1 |

## Encoder Configuration

Two AVC-based codec configurations are considered for the subjective test:

- MVC+D: anchor codec for multiview and depth without block-level changes to decoding process

- 3D-AVC: enhanced compression of multiview and depth with modifications to block-level decoding process for dependent texture views

The encoder configuration settings for both codec are consistent with the CTC as given in JCT3V-E1100, which are also outlined below:

- Inter-view coding structure

  - 3 view case: center-left-right (in coding order)

  - 2 view case: right-left (in coding order)

  - P-I-P inter-view prediction

- Temporal prediction structure: GOP 8, intra every 24 frames (random access at ~1sec)

- Full resolution texture coding

- Anchor software: 3DV-ATM v9.0 (http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/)

- Encoder configurations are provided as part of the “Starter-kit” (./configs )

The QP settings of the base view for both texture and depth, as well as the depth resolution for each sequence, are summarized in the below table:

| Seq. ID | Test Sequence | QP Settings | Depth Resolution |
| --- | --- | --- | --- |
| S01 | Poznan_Hall2 | 26, 31, 36, 41 | Quarter |
| S02 | Poznan_Street | 26, 31, 36, 41 | Quarter |
| S03 | Undo_Dancer | 31, 38, 41, 46 | Full |
| S04 | GT_Fly | 26, 31, 36, 41 | Quarter |

## Output Views

Based on the reconstructed multiview video and associated depth data, which have been encoded according to the test conditions specified in this section, two sets of stereo pairs shall be generated as given in the below table. The first stereo pair consists of 2 synthesized views, while the second stereo pair consists of one original and one synthesized view.

| Seq. ID | Test Sequence | Input Views | Output Stereo #1 (2 synthesized views) | Output Stereo #2 (1 original, 1 synthesized) |
| --- | --- | --- | --- | --- |
| S01 | Poznan_Hall2 | 7-6-5 | (6.25-5.75) | (6-5.5) |
| S02 | Poznan_Street | 5-4-3 | (4.25-3.75) | (4-3.5) |
| S03 | Undo_Dancer | 1-5-9 | (4-6) | (5-7) |
| S04 | GT_Fly | 9-5-1 | (6-4) | (7-5) |

The view synthesis algorithm to be used for generating synthesized views is the “VSRS-1D-Fast” software (https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware).

# Evaluation Procedure

The different codec configurations will be evaluated through formal subjective testing on stereoscopic displays. The evaluation will be performed based on the output stereo views as specified in the table of section 3.4.

## Evaluation Methodology

The Double Stimulus Impairment Scale (DSIS) test method will be used with 11 quality levels, where 10 indicates the highest quality and 0 indicates the lowest quality. The tests will be carried out with naïve viewers. The Variant II of the DSIS method will be adopted; timing of the Basic Test Cell (BTC) is shown in figure here below.

Example of DSIS Variant II Basic Test Cell

The messages showing the letters A and A* will indicate to the viewers that the uncompressed original is going to be shown, while the messages showing the letters B and B* will indicate to the viewers that the video to be evaluated is going to be shown. All the messages showing the letters will stand on the screen for 1 second. The message showing the sentence “Vote N” will stand for 6 seconds. The test subjects will be allowed to express their vote during that time period only. The quality votes will be expressed by the test subjects using paper scoring sheets (see example below).

|  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

Example of DSIS with 11 quality levels Scoring Sheet

The scoring sheets will foresee a section (made of a column with 11 boxes) dedicated to the evaluation of each BTC; each column is numbered to allow an easy synchronization with the message present on the screen and the column to be used.

The whole test will be organized in test sessions. The design of the test sessions will be done in a way the first part of each session is made of a “stabilization” phase; the stabilization phase will be made of 4 BTC selected from those in that session, taking care of choosing one with high quality one with low quality and two of mid quality. This will allow subjects to create their quality impression in a more clear way before the actual test begins. The existence of the stabilization phase is not known to the test subjects.

Considering the particular care that a 3D visual test requires, the test sessions will be of a length never exceeding 20 minutes. The length and the design of the test sessions will be set by the Test Coordinator, taking into account the number of tests to be conducted.

## Raw Data Processing

Mean Opinion Scores (MOS) Standard Deviation (SD) and Confidence Interval (CI) with 95% precision will be computed on the raw data. Overlapping of CI values, centered on the MOS values, will indicate that, in a rather acceptable way, no Statistical Significant Difference exists between two test points. Tables with the MOS and CI values for each test points will be provided together with graphs. Alternative analysis methods such as T-Student test may also be considered.

## Test Subjects selection, screening and training

The tests will be conducted using naïve subjects only; the subjects will be selected among students aged 18 to 30 years old; all test subjects will be pre-screened for visual acuity, color blindness and stereo vision, according to what foreseen by the current literature.

The screening of the test subjects will consist of two steps: a first visual screening (as described earlier) and a post screening. The post screening will be done after the training phase is completed.

Training phase will be done of a short test session using test material and coding conditions similar to those of the actual test and will be made of just 5 or 6 BTC; The goal of the “training” session is to allow subjects to learn about the task they have to do and the timing of the voting. During the training session the test administrators are present and help subjects in their behavior; as training sessions is done, questions and doubts from subjects will be answered.

The “screening” 3D test session will be conducted, made of test material different form that of the actual test, but same length and similar content and impairments of an actual test session.

After the above steps are completed, the approved subjects will run the actual test sessions during the same day.

## Laboratory setup and equipment

The test areas in the test laboratories will be done of wide rooms with no reflecting ceiling walls and floor, and insulated by any external audible and visual pollution.

The dimension of the test area must allow for the following:

- Deep enough to seat subjects at 4H for a 52’ monitor (around four meters)

- Have additional depth to put the monitor at least at 1 meter form the wall behind the monitor

- Wide enough to seat 5 subjects in comfortable way

The wall behind the monitor has to be illuminated with a uniform light source, not directly hitting the viewers; the light level must not exceed the 5% of the monitor peak luminance (when functioning as stereo monitor); the light source has to be at 6500 K° and driven with an high frequency switching device (in any case not at power frequency; e.g. 50 or 60 HZ). The ambient illumination has to be provided only by the light illuminating the wall behind the monitor.

It is planned to use a Hyundai 46” stereo display (interlaced stereo IF) with passive glasses for the subjective evaluation (Model: S465D). The subjects will be seated in front of the monitor as noted below.

- Subjects per monitor: 2

- Viewing distance: 4H

- Angle of vision: < 60°

# Appendix A: Test Sequences

## S01: Poznan_Hall2

## S02: Poznan_Street

Poznan University of Technology

ftp://multimedia.edu.pl/3DV/
username: 3DV / password: ftvftv

directory: CFP

Copyright: Individuals and organizations extracting sequences from this archive agree that the sequences and all intellectual property rights therein remain the property of the respective owners listed below. These materials may only be used for the purpose of developing, testing and promulgating technology standards and for academic usage. Acknowledgement and reference to the following source document: Marek Domañski, Tomasz Grajek, Krzysztof Klimaszewski, Maciej Kurc, Olgierd Stankiewicz, Jakub Stankowski, Krzysztof Wegner "Poznañ Multiview Video Test Sequences and Camera Parameters", ISO/IEC JTC1/SC29/WG11 MPEG 2009/M17050, Xian, China, October 2009, are required in all documents that report any usage of the materials. The respective owners make no warranties with respect to the materials and expressly disclaim any warranties regarding their fitness for any purpose.

Owners: Poznan University of Technology, Poznañ, Poland.

## S03: Undo_Dancer

## S04: GT_Fly

Nokia

ftp://mpeg3dv.research.nokia.com

username: mpegmember / password: S9"12#sHD)3

Copyright: The supplied data and content (“Supplied Data”) is provided free of charge and made available for use by the Licensee, who shall be a member of the MPEG standardization committee or a respondent to a the Call for Proposals for the MPEG standard for 3D video coding that is to be issued March 2011, under the following conditions:

The Licensee agrees that the Supplied Data and all intellectual property rights therein remain the property of Nokia Corporation and its licensors (owners). The Supplied Data may only be used for the purpose of responding to the Call for Proposals that is to be issued March 2011, developing, testing and promulgating the MPEG technology standard for 3D video coding resulting from the Call for Proposals that is to be issued March 2011. The respective owners make no warranties with respect to the Supplied Data and expressly disclaim any warranties regarding its fitness for any purpose.

As a way of promulgating the MPEG technology standard for 3D video coding, the Licensee may present parts or modifications of the Supplied Data at academic conferences and publications.

The Licensee agrees not to provide the data to any third parties without permission from the owners and that the data shall not be sold, let for hire, or by way of trade, offered or exposed for sale or hire. This restriction shall apply to the original material or to any reproduction of it in whole or in part and to any modifications of the Supplied Data.

The texture views of the Supplied Data are copyright © UNDO. (See http://www.undo.fi)

Owners: Nokia, Finland.

In the context of this 3D testing, an “uncompressed original” refers to video clips in which the synthesis result is generated without compression.