| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E1103 |
| --- | --- |

| Title: | Description of Core Experiment 3 (CE3) on Inter-view/motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang Sehoon Yea | Tel: Email: Tel: Email: | +886-3-5670766 ext. 25314 yulin.chang@mediatek.com +82-10-3968-0602 sehoon.yea@lge.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document defines Core Experiment 3 on motion/mode parameter prediction to be performed for the 6th JCT-3V meeting.

# Introduction

The goal of this Core Experiment (CE) is to investigate the methods for motion/mode parameter prediction proposed at the 6th JCT-3V meeting. Tools under test will be evaluated according to their impact on compression efficiency and implementation complexity

# Participants

## CE3

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Li Zhang Vijayaraghavan Thirumalai | lizhang@qti.qualcomm.com vthiruma@qti.qualcomm.com | P/C |
| INRIA | Laurent Guillo Christine Guillemot | Laurent.Guillo@inria.fr Christine.Guillemot@inria.fr | C |
| LG | Taesup Kim Sunmi Yoo JungHak Nam Sehoon Yea | taesup.kim@lge.com sunmi.yoo@lge.com junghak.nam@lge.com sehoon.yea@lge.com | P/C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Kai Zhang Jicheng An Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com kai.zhang@mediatek.com jicheng.an@mediatek.com yuwen.huang@mediatek.com | P/C |
| Sharp | Tadashi Uchiumi Tomohiro Ikai | uchiumi.tadashi@sharp.co.jp ikai.tomohiro@sharp.co.jp | C |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | C |
| Orange Labs | J. Jung | joelb.jung@orange.com | C |
| MERL | Dong Tian | tian@merl.com | C |
| Hisilicon | Yongbing Lin | yblin@huawei.com | C |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | C |
| Zhejiang University | Yichen Zhang | felixzyc@gmail.com | - |
| ETRI | Gun Bang | gbang@etri.re.kr | P |
| KHU | Young Su Heo | medist@khu.ac.kr | P |
| RWTH Aachen | Fabian Jäger | jaeger@ient.rwth-aachen.de | P |
| Huawei | Jacek Konieczny | jacek.konieczny@huawei.com | P |

# Tools under Test

## CE3

There are totally five topics to be investigated in CE3. They are merge candidate list construction, AMVP candidate list construction, interview motion prediction, depth prediction, and depth based block partitioning. Total eight contributions are included.

### Merge candidate list construction

JCT3V-E0213 3D-CE3.h related: Simple merge candidate list construction for 3DV [G. Bang (ETRI), Y.S. Heo, K.Y. Kim, G.H. Park (KHU), G.S. Lee, N.H. Hur (ETRI)]

This contribution proposes a simplified method for constructing MCL (Merge Candidate List) for dependent view or depth. The proposed method is a simple and low-complexity way to implement the MCL construction module for the 3D-HEVC by re-using the existing MCL construction module of the HEVC. One potential benefit is utilizing existing processes in HEVC.

 To further investigate into the simplification of the proposal on utilizing existing merge candidate list construction process in HEVC

### AMVP candidate list construction

JCT3V-E0189 CE3.h related: AMVP candidate list construction for DCP blocks [S. Yoo, T. Kim, J. Nam, S. Yea (LGE)]

At the last meeting, a list construction process for AMVP candidates was changed identically as one for simulcast HEVC, which means the AMVP process does not consider any inter-view prediction benefit. However, the AMVP process can be improved when the reference picture is an inter-view reference picture. Therefore, the AMVP list construction process for DCP blocks is proposed. For the proposed method, the refined disparity of neighboring blocks, which has been used for merge process, is inserted into the AMVP candidate list. As a result, it achieved approximately 0.1% of coding gain from texture video.

 To further investigate ways to improve the AMVP candidate list on migrated HTM

### Interview motion prediction

JCT3V-E0184 3D-CE3.h related: Sub-PU level inter-view motion prediction [J. An, K. Zhang, J.-L. Lin, S. Lei (MediaTek)]

This contribution proposes a sub-PU level inter-view motion prediction method used for the temporal inter-view merge candidate. The original inter-view motion prediction is extended by the Sub-PU partitioning concept.

 To confirm the coding performance of the Sub-PU interview motion prediction on migrated HTM

### Depth prediction

JCT3V-E0133 Inter-view motion vector prediction for depth coding [V. Thirumalai, L. Zhang, Y. Chen, M. Karczewicz (Qualcomm)]

This proposal extends the inter-view motion prediction scheme currently used for texture coding to depth coding. In order to improve the accuracy of the additional merge candidates for the dependent depth views two aspects are added. In the first aspect, a disparity vector is derived from the neighboring reconstructed depth pixels and more merge candidates are derived for a depth block by utilizing the motion information from the reference depth views. In the second aspect, the center pixel, with which a reference block is identified, has been shifted by (1, 1) to provide more efficient merge candidates.

 To further investigate ways to improve the inter-view motion vector prediction for depth coding

JCT3V-E0174 3D-CE3.h related: A disparity derived depth coding method in 3D-HEVC [K. Zhang (MediaTek), J. An, J.-L. Lin, Y.-L. Chang, S. Lei (MediaTek)]

This contribution presents a disparity derived depth (DDD) coding method in 3D-HEVC. When the collocated texture block of the current depth block is predicted by disparity-compensated prediction, a DDD candidate will be inserted into the merging candidate list right after the texture candidate. In a DDD candidate, prediction samples are derived from the disparity vector in the collocated texture block.

 To further investigate ways of using disparity vectors to improve depth coding

JCT3V-E0183 3D-CE3.h related: An additional texture merging candidate in 3D-HEVC [K. Zhang, J. An, J.-L. Lin, Y.-L. Chang, S. Lei (MediaTek)]

In 3D-HEVC depth map coding, a current texture merging candidate inherits motion parameters from the center position of the collocated texture prediction unit (PU). This contribution proposes an additional texture merging candidate, which inherits motion parameters from the right bottom position of the collocated texture PU.

 To evaluate the texture merging candidate methods provided in JCT3V-E0183 and JCT3V-E0229 in terms of complexity and coding performance.

JCT3V-E0229 Enhanced motion parameter inheritance for depth coding in 3D-HEVC [L. He, L. Zhang, Y. Chen, M. Karczewicz (Qualcomm)]

In current 3D-HEVC, motion parameter inheritance (MPI) is enabled for depth views where a texture merging candidate is derived from the co-located texture block and inserted to the merge candidate list. To further improve the coding performance of MPI, in this proposal, additional two MPI candidates may be generated. One of them is used to replace the current MPI candidate when the current MPI candidate is unavailable and the other one is added after pruning to improve the accuracy of the motion vector prediction.

 To evaluate the texture merging candidate methods provided in JCT3V-E0183 and JCT3V-E0229 in terms of complexity and coding performance.

### Depth based block partitioning

JCT3V-E0118 CE3-related: Depth-based Block Partitioning [F. Jäger (RWTH Aachen University), J. Konieczny, G. Cordara (Huawei Technologies)]

In the proposed method, the dependent views' depth information is coded before the texture component to be able to utilize this coded depth data. Based on a binary segmentation mask computed from the reconstructed depth map, an arbitrarily shaped block partitioning for the collocated texture block is derived. Each of the two partitions (resembling foreground and background) is motion compensated and afterwards merged based on the depth-based segmentation mask.
Simulation results show the benefit of the proposed tools with 0.8% BD-rate savings on the video component (w.r.t. total bitrate) under flexible coding order.

- Further investigate the depth based block partitioning with both flexible coding order and virtual depth.

- The experiments with flexible coding order will be carried out on HTM 8.1.

# Mandates

## CE3

The goals of this core experiment are as follows:

- To further investigate into the simplification of the proposal on utilizing existing merge candidate list construction process in HEVC. [JCT3V-E0213]

- To further investigate ways to improve the AMVP candidate list on migrated HTM. [JCT3V-E0189]

- To confirm the coding performance of the Sub-PU interview motion candidate on migrated HTM. [JCT3V-E0184]

- To further investigate ways to improve the inter-view motion vector prediction for depth coding. [JCT3V-E0133]

- To further investigate ways of using disparity vectors to improve depth coding. [JCT3V-E0174]

- To evaluate the texture merging candidate methods provided in JCT3V-E0183 and JCT3V-E0229 in terms of complexity and coding performance.

- To further investigate the depth based block partitioning with both flexible coding order and virtual depth. [JCT3V-E0118]

# Software, Configuration and Evaluation

## Software

### CE3

Experiments in CE3 will use the 3D-HTM 8.0 software that is recommended in JCT3V-E1100 except the experiments with flexible coding order. The experiments with flexible coding order will use the HTM 8.1 software. Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the test sequences, configuration and conditions that are recommended in JCT3V-E1100. Moreover, proponents and cross checkers are required to provide simulation results for the Random access configuration as specified in JCT3V-E1100.

## Evaluation of CE Results

- Measure impact on bitrate/PSNR. PSNR shall be calculated for the decoded texture views, relative to original texture views and for the synthesized views relative to uncompressed synthesized views. Use 4-point BD-PSNR and BD-Rate according to common conditions. The anchors will be generated according to common test conditions.

- Measure impact on encoding and decoding runtimes for all test cases relative to the unmodified test model software.

- Report comments of subjective visual quality according to common conditions.

# Timelines

## CE3

2013/08/09		Finalize CE description

2013/08/31		Release HTM 8.0

2013/09/07		Release HTM 8.1

2013/10/11		Make source code, simulation results, and draft text available for ALL CE3 participants, not only for the cross-checks.

2013/10/18		Register documents for the 6th JCT-3V meeting

2013/10/18		Upload simulation and cross check results to the JCT-3V document repository

2013/10/25-11/01	Start the 6th JCT-3V meeting

# References

- D. Rusanovksyy, K. Müller, and A. Vetro, "Common Test Conditions for 3D Video Extensions Development," Joint Collaborative Team on 3D Video Coding Extension Development (JCT-3V) of ITU-T VCEG and ISO/IEC MPEG, JCT3V-E1100, Vienna, Jul., 2013.