| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 02 Aug. 2013 | Document: JCT3V-E1105 |
| --- | --- |

| Title: | Description of Core Experiment 5 (CE5) on Depth Intra Modes |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Fabian Jäger Institut für Nachrichtentechnik RWTH Aachen University | Tel: Email: | +49 (0) 241 80 27678 jaeger@ient.rwth-aachen.de |
| Source: | JCT-3V Core Experiment Coordinator |  |  |

_____________________________

# Abstract

This document describes Core Experiment 5 on depth intra modes (DIM) to be performed for the 6th JCT-3V meeting in Geneva, Switzerland.

# Introduction

The goal of this Core Experiment (CE) is to investigate methods for simplified depth coding and depth bi-partition modes (DMM and RBC) proposed to the 5th JCT-3V meeting. This CE covers proposals to 3D-HEVC, which are targeting depth coding. The tested methods either reduce complexity of existing coding tools or introduce new tools that are less complex than the current anchor methods. The corresponding test model is described in JCT3V-E1005.

The objectives of tools under test in CE5 are simplifications and improvements of existing depth coding methods.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| LG Electronics | H. Liu, J. Nam, J. Heo Jie Jia | hongbin.liu@lge.com, junghak.nam@lge.com, jin78.heo@lge.com jie.jia@lge.com | X | X |
| Samsung | J. Y. Lee, M. W. Park | jinyoung79.lee@samsung.com, m.w.park@samsung.com | X | X |
| Tsinghua U. | W. Li Y. He | liwr11@mails.tsinghua.edu.cn , hey@tsinghua.edu.cn | X | X |
| Hisilicon | X. Zheng | zhengxiaozhen@huawei.com | X | X |
| HHI | P. Merkle | Philipp.merkle@hhi.fraunhofer.de |  | X |
| RWTH Aachen University | F. Jäger | jaeger@ient.rwth-aachen.de |  | X |
| Sharp | T. Ikai | ikai.tomohiro@sharp.co.jp |  | X |
| Qualcomm | X. Zhao, L. Zhang, Y. Chen | xinzhao@qti.qualcomm.com , lizhang@qti.qualcomm.com cheny@qti.qualcomm.com |  | X |
| MediaTek | Y.-L. Chang, J.-L. Lin, Y.-W. Chen, K. Zhang, Y.-W. Huang | yulin.chang@mediatek.com, jl.lin@mediatek.com, yiwen.chen@mediatek.com, kai.zhang@mediatek.com, yuwen.huang@mediatek.com |  |  |
| Zhejiang University | Peng Lu | yiqiu8932@gmail.com |  |  |
| HiSilicon | X. Zheng J. Zheng | zhengxiaozhen@huawei.com , zhengjianhua@huawei.com |  | X |

(P=proponent, C=cross checker)

# Tools Under Test

## JCT3V-E0147: Simplified depth coding based on most probable mode

The proposed SDC mode employs the most probable modes to improve the coding performance, instead of the fixed modes such as Planar, DC, and DMM1 modes. When the most probable modes are decided, if the depth modeling modes such as DMM and Chain modes are used, Planar mode is instead used. Also, the proposed method removes the residual signalling parts, so the DC calculation parts are not needed and the syntaxes and contexts related to the SDC residual are simplified.

## JCT3V-E0157: Applying Depth Look-up Table to Intra Modes of Depth Map

Inspired by DLT used in SDC [1], this contribution proposes an alternative residual generation method for all intra modes excluding SDC. Instead of calculating residual as the difference of original pixel value and prediction pixel value directly, both original pixel value and prediction pixel value are firstly mapped to depth index through DLT, and then residual is generated as the difference of these two depth indexes. Let org[x], pred[x] and resi[x] be the original value, intra predicted value and residual at position x respectively, and let DLT[a] be the index of depth value a. In proposed method, resi[x] is generated as:

resi[x] = DLT[org[x]] – DLT[pred[x]]

Rate distortion optimized selection between proposed method and method in 3D-HEVC is performed at encoder and one flag is encoded for each intra coded (excluding SDC) depth CU to indicate which method is used.

## JCT3V-E0225: Simplification of DMM pattern generation and signaling

A modification of the DMM pattern generation mechanism is proposed in this contribution. Compared to the current pattern generation and signaling of DMM segmentations, the proposed method reduces the number of patterns for each block size by a description and construction mechanism based on partition type, offset, scalar and orientation flag. The signaling of DMM1 segmentation information is modified accordingly. At the decoder the segmentation pattern can be directly reconstructed from the signaled parameters without requiring the lookup list. Furthermore, the reduced number of patterns leads to a reduction in encoder complexity. Direction-dependent sublists for DMM3 are also reduced due to the reduction of DMM1 pattern number.

# Core Experiment Conditions

## Software

Core Experiment CE5 will use the 3D-HTM 8.0 software that is described in the common test conditions (CTC), JCT3V-E1100. 
Proponents are requested to provide software that can be compiled under Windows and Linux platforms.

## Test Sequences, Bit Rates and Coding Conditions

The CE will use test sequences, configuration and conditions that are described in the common test conditions (CTC), JCT3V-E1100. Moreover, proponents and cross checkers are required to provide simulation results for the following two configurations:

- Random access configuration as specified in JCT3V-E1100.

- All-intra configuration (configuration files to be provided by CE coordinator).

## Evaluation of CE Results

The performance measurements are evaluated by switching on and off individual tools to identify their relative performance. The following measurements are to be used in this core experiment. A corresponding Excel sheet for reporting the simulation results will be provided by the CE coordinator.

Coding Performance Measurements: PSNR values shall be computed for the decoded texture views relative to original texture views and for the synthesized views relative to the synthesized views based on uncompressed texture and depth. The 4-point BD-rate according to common test conditions is used to report the results for random access as well as all-intra configuration.

Complexity measurements: For the complexity measurement, the reference software and the reference software with the proposed method implemented will be executed on the same machine with the same configuration and the computational time will be measured. A time ratio will then be calculated between the reference software and the reference software with the proposed method implemented.

## Timeline

2013/08/09		Final description of CE available 
2013/10/11		Make source code, simulations results and draft text available to all participants
2013/10/18		Document registration and upload deadline
2013/10/24		CE summary report available