| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0004 |
| --- | --- |

| Title: | AHG4 report: 3D-AVC Software Integration |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | AhG report |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy (LGE), J.Y. Lee (Samsung), J.-L. Lin (MediaTek), O.Stankiewicz (Poznan) | Email: | dmytro.rusanovskyy@ieee.org jinyoung79.lee@samsung.com jl.lin@mediatek.com tian@merl.com ostank@multimedia.edu.pl |
| Source: | AhG4 |  |  |

_____________________________

# Abstract

This document reports on activities performed by JCT-3V AHG4 on 3D-AVC Software Integration between 5th and 6th JCT-3V meetings. Activities focused on the integration and cross-check of tools adopted at the 5th meeting, resolving known 3DV-ATM software bugs and delivering MPEG reference software for Multiview Depth High profile (JCT3V-E1010).

# Introduction

At the 5th JCT-3V meeting, the following mandates have been specified for AHG4:

- Coordinate development of the 3DV-ATM software and its distribution to JCT-3V members

- Produce documentation of software usage for distribution with the software

- Prepare and deliver a subset of the ATM software for standalone implementation of the Multiview Depth High profile (JCT3V-E1010, expected to be available and cross-checked by Aug. 9).

- Prepare and deliver 3DV-ATM v9.0 software version and the reference configuration encodings according to JCT3V-E1100 based on common conditions (expected within 3 weeks after the meeting).

- If necessary, prepare and deliver 3DV-ATM v9.1 software version for further improvements and bug fixes.

- Perform analysis and reconfirmation checks of the behavior of technical changes adopted into the draft design, and report the results of such analysis.

- Suggest configuration files for additional testing of tools.

- Coordinate with 3D-AVC Draft and Test Model editing (AHG2) to identify any mismatches between software and text.

- Set up a bug tracking system.

# Report

With respect to the mandates 1-2, JCT-3V AhG4 coordinated development of the 3D-AVC software and distributions to the JCT-3V community. The 3D-AVC Test Model description document JCT3V-E1003 was updated with new tools description and examples of software usage.

## Mandate 3

Reference MPEG software for Multiview Depth High profile (JCT3V-E1010/ WG 11 N 13747) was prepared, cross-checked and made available by the specified timeline. The software package formed a base for MVC+D conformance bitrstream production and work of AhG9. AhG4 would like to thank ITRI, MERL and Poznan University for their active participation in software improvement and cross-check of the reference MVC+D software. Reference software package is available for download from:

http://phenix.int-evry.fr/jct3v/doc_end_user/current_document.php?id=1364

## Mandates 4 and 5

Following this, AhG4 coordinated integration of tools and changes adopted at the last JCT-3V meeting with planned release of 3DV-ATM v9.0 on 1.Sept.2013. Unfortunately, software integration process was delayed by approximately 1 month and 3DV-ATM v9.0 software was made available on 30.Sept.2013. This situation was happen due to availability of resources of one of the proponents. The delay seemed to have a limited impact on the technical development of 3D-AVC due its late stage of standardization, however it lead to delay in releasing the JCT3V-E1003/WG11 N 13753  documents.

3DV-ATM v9.0 software and encoding configuration produced according to the JCT3V-E1100 are available

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/3DV-ATMv9.0/

Simulation results produced for under the Common Test Conditions and anchor templates are available at the following location:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/tags/3DV-ATMv9.0/results/

The software 3D-ATM v9.1 was not released, since all know updates and bug fixes have been incorporated to the version 9.0.

## Mandates 6 and 7

Analysis and reconfirmation checks of the behavior of technical changes adopted into the draft design were performed. Full set of simulation results produced at the integration stage is available for information purposes from the following location:

http://mpeg3dv.research.nokia.com/svn/mpeg3dv/branches/JCT-3V_5_post_integration/results/

Simulation results produced with 3DV-ATM v8.0 has been compared against the previous version 7.0, summary of the results is given in Table 1.

Table 1. Comparative results of 3DV-ATM v9.0 vs. 8.0

|  | Texture Coding |  | Total (Coded PSNR) |  | Total (Synthesed PSNR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB | dBR, % | dPSNR,dB |
| S01 | 0.06 | 0.00 | 0.04 | 0.00 | -0.42 | 0.02 |
| S02 | 0.00 | 0.00 | 0.05 | 0.00 | -0.04 | 0.00 |
| S03 | -0.06 | 0.00 | 0.67 | -0.02 | 0.04 | 0.00 |
| S04 | 0.06 | 0.00 | 0.42 | -0.01 | 0.31 | -0.01 |
| S05 | -0.05 | 0.00 | 0.03 | 0.00 | -0.06 | 0.00 |
| S06 | 0.01 | 0.00 | 0.09 | 0.00 | -0.11 | 0.00 |
| S08 | 0.01 | 0.00 | 0.22 | -0.01 | -0.28 | 0.01 |
| Average | 0.00 | 0.00 | 0.22 | -0.01 | -0.08 | 0.00 |

## Mandate 7-8

Adoption of the JCT3V-E0166 proposal resulted in significant complications to the encoder configuration. Proponent of the E0166 (Nokia) requested an action point to further improve software and introduce certain automatization of newly introduced encoder parameters.

AhG4 communicated with AhG2 on identifying mismatched between SW and texts of the 3D-AVC Draft and Test Model editing (AHG2). Up to this moment, there are no mismatches known.

## Mandates 9

A bug-tracking system for 3D-AVC related development has been released and is available from following location: https://hevc.hhi.fraunhofer.de/trac/3d-avc/

The bug-tracking covers the following components:

- 3D-ATM software for tickets relating to the 3D-ATM software and MVC+D, 3D-AVC implementation.

- 3D-AVC text for tickets relating to the 3D-AVC specification text

- MVC+D text for tickets relating to the MVC+D specification text

- 3D-AVC test model for tickets relating to the 3D-AVC test model text

AhG4 would like to thank Heinrich Hertz Institute for hosting this tracking system. And we would like to personally thank Karsten Suehring and Gerhard Tech for their efforts and assistance on this development.

## Others

At this moment, AhG4 aware of 2 bugs/inconsistencies in 3DV-ATM software which to be resolved at the next integration round:

1. Incomplete support of Un-paired MVD in MVC+D implementation (problem of "single depth component").

2. Consistence of the package for Windows platform. Currently provided project file for MS Visual Studio 9.0 cannot be compiled.

AhG4 is planning to form the next integration round to resolve these issues.