| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0007 |
| --- | --- |

| Title: | JCT-3V AHG report: 3D High level syntax (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. M. Hannuksela (Chair), J. Boyce, Y. Chen, A. Norkin, Y.-K. Wang (vice chairs) | Email: | miska.hannuksela@nokia.com jill@vidyo.com cheny@qti.qualcomm.com andrey.norkin@ericsson.com yekuiw@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This report summarizes the activities of the ad-hoc group of 3D high level syntax (AHG7).

# Mandates

- Study high level syntax design extensions of AVC for 3DV functionalities.

- Study the relations of AVC multiview/3D extensions and HEVC multiview/3D extensions on high-level syntax concepts, e.g., related to SEI messages.

- Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC.

- Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs.

- Study SEI messages and VUI syntax designs needed for HEVC extensions.

- Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs.

- Study the usage of auxiliary pictures (alpha channel) for the purpose of depth map coding in MV-HEVC, and the extension of HL syntax that would be necessary in this context.

- Study restrictions that need to be imposed w.r.t. spatial resolution of the views in MV-HEVC HL syntax

- Study methods for extending the number of layers in MV-HEVC and 3D-HEVC, as proposed in documents JCT3V-E0092, JCT3V-E0223 and JCT3V-E0224.

- Compare the method of allowing multiple layers for all nuh_layer_id values as proposed in JCT3V-E0092 with the approach of reserving nuh_layer_id value of 63 to provide an extension of the number of layers, in terms of functionality supported, signalling efficiency.

- Coordinate efforts with JCT-VC AHG 9 (HLS) and AHG 20 (HRD)

# Activities

The auxiliary picture layer mechanism proposed in JCTVC-N0063/JCT3V-E0049 was studied and a follow-up contribution JCTVC-O0041/JCT3V-F0031 refining a few details was submitted on 19th September and announced in the email reflector.

There were no other emails related to the mandates of this ad-hoc group over the reflector between the 5th JCT-3V meeting and the 6th JCT-3V meeting.

The ad-hoc group has met collocated with JCT-VC on 23-25 October, 2013. The JCT-VC meeting notes document the recommendations reached.

# Recommendations

AHG7 recommends reviewing all related input contributions.