| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0021 |
| --- | --- |

| Title: | CE1: Summary report on View Synthesis Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shinya Shimizu (NTT) 1-1, Hikari-no-Oka, Yokosuka, Kanagawa, Japan | Tel: Email: | +81-46-859-2703 shimizu.shinya@lab.ntt.co.jp |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This document is the summary report of Core Experiment 1 (CE1) on view synthesis prediction. This Core Experiment investigates various aspects of view synthesis prediction. Experiments are related to 3D-HTM. There are 1 CE1 proposal and 11 CE1 related proposals.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| NTT | S. Shimizu | shimizu.shinya@lab.ntt.co.jp | P/C |
| Mitsubishi Electric | D. Tian | tian@merl.com |  |
| MediaTek | Y.-L. Chang Y.-W. Chen Y.-W. Huang | yulin.chang@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com | P/C |
| Qualcomm | X. Zhao J. Kang L. Zhang | xinzhao@qti.qualcomm.com jewonk@qti.qualcomm.com lizhang@qti.qualcomm.com | P |
| Nokia |  |  |  |
| Zhejiang University | Y. Zhang | felixzyc@gmail.com | P |
| LG Electronics | S. Yea T. Kim | sehoon.yea@lge.com taesup.kim@lge.com | P/C |
| Samsung | J. Y. Lee M. W. Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | P/C |
| ETRI | G. Bang | gbang@etri.re.kr |  |
| KyungHee University | K. Y. Kim | kimky@khu.ac.kr |  |
| Sharp | T. Ikai | ikai.tomohiro@sharp.co.jp | P |
| KDDI | K. Kawamura | kei@kddilabs.jp |  |
| GIST | Y. Song Y.-S. Ho | ysong@gist.ac.kr hoyo@gist.ac.kr |  |
| HHI | H. Brust | heribert.brust@hhi.fraunhofer.de |  |
| NCKU | C.-F. Chen | n28991146@mail.ncku.edu.tw |  |
| ITRI | Erh-Chung Ke | JohnKe@itri.org.tw | P |

(P = proponent, C = cross checker)

# Tools under Investigation

The tools to be investigated in this Core Experiment use the decoded texture and depth view components of the already coded views to compute a prediction signal for the texture view components of the coding target views. This process is done by means of disparity compensation, where the disparity values are derived from the decoded depth information.

There is no experiment in the context of 3D-AVC.

The following two experiments were set up for 3D-HEVC:

- Disparity vector for fetching depth map

  - JCT3V-F0111

  - JCT3V-F0141 (CE-related)

  - JCT3V-F0173 (CE-related)

- Depth consistency check

  - No

There are many CE-related contributions, which are roughly categorized into the following three categories:

- VSP for AMP

  - JCT3V-F0102

  - JCT3V-F0130

  - JCT3V-F0140

  - JCT3V-F0175

- Sub-PU partitioning

  - JCT3V-F109

  - JCT3V-F120

- Others

  - JCT3V-F0162

  - JCT3V-F0176

  - JCT3V-F0177

## List of contributions

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| CE contributions |  |  |  |
| MediaTek | JCT3V-F0111 | 3D-CE1: Simplified view synthesis prediction | Proposal |
| LG | JCT3V-F0203 | CE1 : Crosscheck for Simplified View Synthesis Prediction (JCT3V-F0111) | Crosscheck |
| CE-related contributions |  |  |  |
| Sharp | JCT3V-F0102 | CE1-related: VSP partitioning for AMP | Proposal |
| MediaTek | JCT3V-F0219 | 3D-CE1 related: Crosscheck of VSP partitioning for AMP by Sharp (JCT3V-F0102) | Crosscheck |
| MediaTek | JCT3V-F0109 | 3D-CE1 related: A simplified block partitioning method for view synthesis prediction | Proposal |
| Samsung | JCT3V-F0197 | 3D-CE1 related: Cross check of A simplified block partitioning method for view synthesis prediction (JCT3V-F0109) | Crosscheck |
| LG | JCT3V-F0120 | CE1 related : Simplified VSP Block Partitioning Process | Proposal |
| NTT | JCT3V-F0180 | CE1-related: Crosscheck on Simplified VSP Block Partitioning Process (JCT3V-F0120) | Crosscheck |
| Qualcomm | JCT3V-F0130 | CE1 related: BVSP for asymmetric motion partitioning | Proposal |
| NTT | JCT3V-F0181 | CE1-related: Crosscheck on BVSP for asymmetric motion partitioning (JCT3V-F0130) | Crosscheck |
| Samsung | JCT3V-F0140 | 3D-CE1 related: Problem fix on View Synthesis Prediction | Proposal |
| NTT | JCT3V-F0182 | CE1-related: Crosscheck for Problem fix on View Synthesis Prediction (JCT3V-F0140) | Crosscheck |
| Samsung | JCT3V-F0141 | 3D-CE1 related: Improvement of View Synthesis Prediction | Proposal |
| NTT | JCT3V-F0183 | CE1-related: Crosscheck on Improvement of View Synthesis Prediction (JCT3V-F0141) | Crosscheck |
| ITRI | JCT3V-F0162 | 3D-CE1.h related: Vertical Filtering for View Synthesis Prediction | Proposal |
| MediaTek | JCT3V-F0192 | CE1-related: Crosscheck on Vertical Filtering for View Synthesis Prediction (JCT3V-F0162) | Crosscheck |
| Zhejiang University | JCT3V-F0173 | CE1.h related: View Synthesis Prediction with DV difference | Proposal |
| NTT | JCT3V-F0175 | CE1-related: Sub-PU partitioning for VSP with AMP | Proposal |
| Samsung | JCT3V-F0196 | 3D-CE1 related: Cross check of Sub-PU partitionaing for VSP with AMP (JCT3V-F0175) | Crosscheck |
| NTT | JCT3V-F0176 | CE1-related: Motion parameters stored at VSP-coded blocks | Proposal |
| MediaTek | JCT3V-F0194 | CE1-related: Crosscheck on motion parameters stored at VSP-coded blocks (JCT3V-F0176) | Crosscheck |
| NTT | JCT3V-F0177 | CE1-related: View synthesis prediction via motion field synthesis | Proposal |

## Summary of results

### Test (a): [CE] Disparity vector for fetching depth map

In this category, several kind of disparity vectors are tested as the vector used on referring depth map, mainly, in the case that VSP mode is inherited from spatial neighbors.

| Doc. | Disparity vector for default VSP | Disparity vector for inherited VSP | VSP inheritance cross the CTU row boundary | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | Data storage for NBDV and VSP flag | # of memory access (worst) | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HTM 8.0 | NBDV of the current CU | NBDV of the spatial neighbor | Allowed |  |  | 9360 bits for FHD | 100% |  |
| F0111 | NBDV of the current CU | NBDV of the current CU | Allowed | 0.07% | 0.05% | 9360 bits for FHD | 50% | F0203 |
|  | NBDV of the current CU | NBDV of the spatial neighbor | Disallowed | 0.04% | 0.01% | 0 bit | 100% |  |
|  | NBDV of the current CU | NBDV of the current CU | Disallowed | 0.05% | 0.02% | 0 bit | 50% |  |
| F0141 | DoNBDV of the current CU | NBDV of the spatial neighbor | Allowed | -0.09% | -0.12% | 9360 bits for FHD | 100% | F0183 |
| F0173 | NBDV of the current CU | NBDV of the spatial neighbor + delta DV (coded) | Allowed | 0.01% | 0.00% | 9360 bits for FHD | 100% | No |

### Test (b): [CE] Depth consistency check

No result is available.

### Test (c): [CE-related] VSP for AMP

The current WD contains an error on the adaptive block partitioning on VSP when AMP is used for the CU which size is 16x16. Several solutions are proposed.

| Doc. | 16x4 | 16x12 | 4x16 | 12x16 | Other AMPs | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | enc time | dec time | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HTM 8.0 | 8x4 or 4x4 | 8x4, 4x8, or 4x4 | 4x8 or 4x4 | 8x4, 4x8, or 4x4 | 8x4 or 4x8 |  |  |  |  |  |
| F0102 | 8x4 | 8x4 | 4x8 | 4x8 | 8x4 or 4x8 | 0.00% | -0.04% | 100.2% | 100.4% | F0219 |
| F0130 | 8x4 | 8x4 or 4x8 | 4x8 | 8x4 or 4x8 | 8x4 or 4x8 | 0.00% | -0.02% | 101.1% | 104.3% | F0181 |
| F0140 | 16x4 | 16x12 | 4x16 | 12x16 | 8x4 or 4x8 | 0.00% | 0.00% | 100.1% | 100.1% | F0182 |
| F0175 | disable | disable | disable | disable | disable | -0.01% | -0.01% | 99.8% | 99.5% | F0196 |
|  | 8x4 | 8x4 or 4x8 | 4x8 | 8x4 or 4x8 | 8x4 or 4x8 | 0.00% | -0.01% | 99.9% | 99.2% |  |

Note: The difference between F0130 and F0175 is unclear.

| (A) HTM 8.0 |  |
| --- | --- |
| (B) F0102 | (C) F0130 |
| (D) F0140 | (E) F0175 |

Figure 1: Sub-block partitions (the adaptive partitioning decision is performed for the colored blocks)

### Test (d): [CE-related] Sub-PU partitioning

In the current WD, sub-PU partition is decided for each 8x8 in PU. Two contribution propose to decide either 4x8 or 8x4 once per one PU. It is noted that the methods proposed in F0109 and F0120 are identical.

| Doc. | Sub-PU size | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | enc time | dec time | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- |
| HTM 8.0 | 4x8 or 8x4 decided for each 8x8 block |  |  |  |  |  |
| F0109 | 4x8 or 8x4 decided for each PU | 0.00% | -0.03% | 99.6% | 97.0% | F0197 |
| F0120 | 4x8 or 8x4 decided for each PU | 0.00% | -0.03% | 100.0% | 101.3% | F0180 |

### Test (e): [CE-related] others

#### JCT3V-F0162: Vertical Filtering for View Synthesis Prediction

In the current VSP design, the vertical component on sub-PU based disparity compensation prediction (DCP) is always set to zero. This contribution proposes to utilize the vertical part of disparity vector used on fetching depth map for sub-PU based DCP; the vertical component on sub-PU based DCP is set to the same value with the vertical component of disparity vector used on fetching depth map when the magnitude of the vertical component of the disparity vector is less than 1 pixel.

| Doc. | Vertical component of disparity vector | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | enc time | dec time | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- |
| HTM 8.0 | zero |  |  |  |  |  |
| F0162 | Same with disparity vector used on fetching depth map | -0.06% | -0.05% | 99.7% | 99.0% | F0192 |

Necessary to be clarified: The vertical component of NBDV is proposed to be utilized in the contribution while that of the disparity vectors on fetching depth map is utilized in the proposed WD.

#### JCT3V-F0176: Motion parameters stored at VSP-coded blocks

In the current VSP design, the disparity parameters utilized to fetch depth map are stored in the motion storage for the VSP coded blocks. This contribution proposes to store the disparity parameters used for each sub-PU on generating prediction signals.

| Doc. | Stored motion parameters | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | enc time | dec time | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- |
| HTM 8.0 | NBDV of either the current CU or the neighboring CU |  |  |  |  |  |
| F0176 | Disparity parameters derived from virtual depth map (different for each sub-PU) | -0.07% | -0.07% | 99.8% | 99.3% | F0194 |

#### JCT3V-F0177: View synthesis prediction via motion field synthesis

The current VSP directly synthesizes prediction signals using disparity information derived from depth map. This contribution proposes to first synthesize motion vector field using depth map and then generate prediction signals based on the synthesized motion vector field.

| Doc. | Sub-PU block size (prediction mode) | BD-Rate (Video PSNR) | BD-Rate (Synth. PSNR) | enc time | dec time | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- |
| HTM 8.0 | 8x4 or 4x8 (DCP) |  |  |  |  |  |
| F0177 | 4x4 (MCP) | -0.63% | -0.38% | 99.6% | 125.3% | No |
|  | 8x8 (MCP) |  |  |  |  |  |

Note: There are some commonality with JCT3V-F0110 (CE3 proposal) where single DoNBDV is utilized on the derivation of motion parameter for all sub-PU.

# Recommendations

- To review the relevant input contributions

- ...