| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0022 |
| --- | --- |

| Title: | CE2: Summary report on Disparity Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | Ying Chen 5775 Morehouse Drive San Diego, CA 92121 USA Sehoon Yea | Tel: Email: | +1-858-845-6589 cheny@qti.qualcomm.com +82-10-3968-0602 sehoon.yea@lge.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of the Core Experiment 2 (CE2) on Disparity Vector Derivation. During the 5th JCT-3V meeting, three contributions have been proposed to simplify current design of NBDV process or improve the coding efficienty and they were considered for further study in a Core Experiment. The goal of this CE is to evaluate them in the context of the 3D-HTM 8.0.

11 companies were involved in the CE with 3 CE proposals and 6 CE-related proposals.

# Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| ETRI | Gun Bang | gbang@etri.re.kr | C |
| KHU | Kyung Yong Kim | kimky@khu.ac.kr | C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com yuwen.huang@mediatek.com | P |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | P |
| LG | Sehoon Yea | sehoon.yea@lge.com | P/C |
| Qualcomm | Jewon Kang Li Zhang | jewonk@qti.qualcomm.com lizhang@qualcomm.com | P/C |
| INRIA | Laurent Guillo | Laurent.guillo@inria.fr |  |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp |  |
| MERL | Dong Tian | tian@merl.com |  |
| Zhejiang University | Yichen Zhang | felixzyc@gmail.com | C |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp | C |

# Tools under investigation

## Results on a simplified DV derivation method [JCT3V-F0106]

In this proposal, it is proposed to inherit the NBDV results of an available neighboring block (left and above in order, or just left), when no DV can be found from any temporal or spatial neighboring block. With the proposed method, the checking on whether a neighboring block is a DV-MCP block is removed.

This proposal was crosschecked by Hisilicon and detail information can be referred to JCT3V-F0221.

## Derived disparity vector for 3D-HEVC [JCT3V-F0124]

In this proposal, a single derived disparity vector, namely Derived Disparity Vector (DDV), is initially set to zero, at the beginning of each coding tree block (CTB) row and updated to be the derived disparity vector of that CU. With the proposed method, the implicitly stored Disparity Vector (IDV) is replaced by DDV.

This proposal was crosschecked by NTT and detail information can be referred to JCT3V-F0185.

## Results on 2-Step Disparity Vector Derivation [JCT3V-F0142]

In this proposal, a 2-step DV derivation method is proposed. The 2-step DV derivation consists of two steps. The first DV is derived from the depth block reached by a DV derived using a default depth value (e.g., 128), then the final DV is derived from the corresponding depth block pointed by the first DV. The proposed method is applied to inter-view motion prediction and BVSP, excluding ARP.

Two sets of tests are provided:

- Test #1: use the final DV as the input to BVSP and the output of DoNBDV.

- Test #2: use the first DV as the input to BVSP and the final DV as the output to DoNBDV.

This proposal was crosschecked by NTT and detail information can be referred to JCT3V-F0186.

# Experiments and results

Table 1: IDV simplifications

| Test Conditions | Proposals | Simulation Results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Video 1 | Video 2 | Video PSNR / Video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate |
| CTC | JCT3V-F0106 (left and above) | 0.05% | -0.06% | 0.00% | 0.00% | -0.02% |
|  | JCT3V-F0106 (left) | 0.05% | -0.08% | -0.01% | -0.01% | -0.04% |
|  | JCT3V-F0124 | 0.26% | 0.07% | 0.06% | 0.05% | 0.03% |
| BVSP off | JCT3V-F0106 (left and above) | -0.14% | -0.24% | -0.07% | -0.06% | -0.06% |
|  | JCT3V-F0106 (left) | -0.11% | -0.18% | -0.06% | -0.05% | -0.04% |
|  | JCT3V-F0124 | -0.01% | -0.03% | -0.02% | -0.01% | -0.01% |

Table 2: Two step DoNBDV

| Test Conditions | Proposals | Simulation Results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Video 1 | Video 2 | Video PSNR / Video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate |
|  | JCT3V-F0142 Test #1 | -0.27% | -0.67% | -0.17% | -0.18% | -0.21% |
|  | JCT3V-F0142 Test #2 | -0.12% | -0.55% | -0.18% | -0.18% | -0.19% |
|  | JCT3V-F0142 Test #1/ #2 | n/a | n/a | n/a | n/a | n/a |

Table 3: Simplifcation of neighboring blocks

| Test conditions | Test cases | Simulation Results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Video 1 | Video 2 | Video PSNR / Video bitrate | Video PSNR / total bitrate | Synth PSNR / total bitrate |
| CTC | Test 3.a | 0.03% | -0.05% | 0.00% | 0.00% | -0.03% |
|  | Test 3.b | 0.04% | -0.01% | 0.01% | 0.01% | 0.00% |
|  | Test 3.c | 0.05% | 0.04% | 0.02% | 0.05% | 0.04% |
| BVSP off | Test 3.a | -0.04% | 0.02% | 0.00% | 0.00% | -0.01% |
|  | Test 3.b | -0.05% | 0.01% | -0.01% | 0.00% | 0.01% |
|  | Test 3.c | 0.26% | 0.22% | 0.09% | 0.08% | 0.04% |

Table 4: Definitions of tests in CE2 [1]

| Test 3.a | 5 spatial DMV candidates | Check the performance drop/gain of using only 2 DMV candidates |
| --- | --- | --- |
| Test 3.b | 5 spatial IDV candidates | Check the performance drop/gain of using only 2 IDV candidates |
| Test 3.c | 4 (2 x 2 pics.) temporal DMV candidates | Check the performance drop/gain of using only 2 temporal candidates (1 per pic) |

# Related proposals

## Group 1: default DV

### Simplification on NBDV derivation in 3D-HEVC (JCT3V-F0116)

In this proposal, it is proposed to remove the above neighboring block in the derviation process of NBDV. It is also proposed to replace the default zero DV with the DV converted from the middle depth value (128). It is reported that there is 0.0% coding loss for CTC.

This proposal was crosschecked by Sharp and detail information can be referred to JCT3V-F0211.

### Default Disparity Vector Improvement (JCT3V-F0143)

In this proposal, it is proposed to remove the DV-MCP process from the NBDV derivation process, and to change the default disparity vector value from (0, 0) to the disparity vector derived from depth value 128 and to use the default disparity vector for the residual prediction in order to make up for DV-MCP coding performance. It is reported that there is 0.0% coding gain for CTC and BVSP and DoNBDV off.

This proposal was crosschecked by MediaTek and detail information can be referred to JCT3V-F0218

## Group 2: PU-level NBDV

### PU-based Disparity Vector Derivation in 3D-HEVC (JCT3V-F0202)

In this proposal, it is proposed to reverse NBDV from CU level to PU level. It is repoted that there is 0.1% coding gain.

This proposal was crosschecked by Sharp and detail information can be referred to JCT3V-F0215.

## Group 2: DV improvements

### Derivation of motion-aware temporal disparity vector (JCT3V-F0163)

In this proposal, it is proposed to replace co-located block with block pointed by a temoral MV in NBDV derivation process, and the temporal MV is derived from left and above neighboring block. It is reported that there is -0.1% coding gain for both texture and synthesis view.

This proposal was crosschecked by Samsung and detail information can be referred to JCT3V-F0200.

### Modification of disparity vector derivation based on depth-oriented disparity (JCT3V-F0164)

In this proposal, it is proposed to update disparity vector (DV) derived in NBDV namely DVNBDV, and DV derived in DoNBDV namely DVDoNBDV. It is proposed to chose DV (among all DV candidaites involved in NBDV derivation process) with minimum difference from DVDoNBDV as the updated DV namely DVNBDV_NEW and then use DVNBDV_NEW to update DVDoNBDV. It is reported that there is -0.1% coding gain for synthesis view.

This proposal was crosschecked by Samsung and detail information can be referred to JCT3V-F0227.

## Others

### Problem fix of the DV derivation in 3D-HEVC(JCT3V-F0115)

In this proposal, it is proposed to use a default zero vector pointing to the inter-view reference picture with the smallest view index as default output for NBDV when no DV could be derived from the neighbor blocks. It is reported that the default zero vector with zero view index in NBDV of 3D-HEVC may point to non-existing referernce picture and proposed method can solve this problem. This bug-fix has already been included in 3D-HTM v8.0.

### Compression of Disparity Vectors based on DV-MCP (JCT3V-F0165)

Withdrawn.

# Reference

- “Description of Core Experiment 2 (CE2) on Disparity Vector Derivation,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-E1102, 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013.