| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0023 |
| --- | --- |

| Title: | 3D-CE3: Summary report on inter-view/motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang Sehoon Yea | Tel: Email: Tel: Email: | +886-3-5670766 ext. 25314 yulin.chang@mediatek.com +82-10-3968-0602 sehoon.yea@lge.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

A summary of proposals and cross-check results of CE3 is reported.

# Introduction

A summary of proposals and cross-check results of CE3 is reported.

## Mandates

# Mandates

## CE3

The goals of this core experiment are as follows:

- To further investigate into the simplification of the proposal on utilizing existing merge candidate list construction process in HEVC. [JCT3V-E0213]

- To further investigate ways to improve the AMVP candidate list on migrated HTM. [JCT3V-E0189]

- To confirm the coding performance of the Sub-PU interview motion candidate on migrated HTM. [JCT3V-E0184]

- To further investigate ways to improve the inter-view motion vector prediction for depth coding. [JCT3V-E0133]

- To further investigate ways of using disparity vectors to improve depth coding. [JCT3V-E0174]

- To evaluate the texture merging candidate methods provided in JCT3V-E0183 and JCT3V-E0229 in terms of complexity and coding performance.

- To further investigate the depth based block partitioning with both flexible coding order and virtual depth. [JCT3V-E0118]

## Participants

Participants with proposals at this meeting in CE3 are as follows:

## CE3

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Li Zhang Vijayaraghavan Thirumalai | lizhang@qti.qualcomm.com vthiruma@qti.qualcomm.com | P/C |
| INRIA | Laurent Guillo Christine Guillemot | Laurent.Guillo@inria.fr Christine.Guillemot@inria.fr | C |
| LG | Taesup Kim Sunmi Yoo JungHak Nam Sehoon Yea | taesup.kim@lge.com sunmi.yoo@lge.com junghak.nam@lge.com sehoon.yea@lge.com | P/C |
| MediaTek | Jian-Liang Lin Yi-Wen Chen Kai Zhang Jicheng An Yu-Wen Huang | jl.lin@mediatek.com yiwen.chen@mediatek.com kai.zhang@mediatek.com jicheng.an@mediatek.com yuwen.huang@mediatek.com | P/C |
| Sharp | Tadashi Uchiumi Tomohiro Ikai | uchiumi.tadashi@sharp.co.jp ikai.tomohiro@sharp.co.jp | C |
| Samsung | Jin Young Lee Min Woo Park | jinyoung79.lee@samsung.com m.w.park@samsung.com | C |
| Orange Labs | J. Jung | joelb.jung@orange.com | C |
| MERL | Dong Tian | tian@merl.com | C |
| Hisilicon | Yongbing Lin | yblin@huawei.com | C |
| NTT | Shinya Shimizu | shimizu.shinya@lab.ntt.co.jp | C |
| Zhejiang University | Yichen Zhang | felixzyc@gmail.com | - |
| ETRI | Gun Bang | gbang@etri.re.kr | P |
| KHU | Young Su Heo | medist@khu.ac.kr | P |
| RWTH Aachen | Fabian Jäger | jaeger@ient.rwth-aachen.de | P |
| Huawei | Jacek Konieczny | jacek.konieczny@huawei.com | P |

# List of input documents

## CE3

A total of 28 input contributions were gathered in this category, among which were 16 proposals. Among the 16 proposals, six of them are CE contributions, and ten of them are non-CE contributions. The input contributions can be roughly listed as following table. Please note that the CE contributions are in blue blocks.

| Doc. No. | Contributions | Proponent(s) | Cross-checker(s) |
| --- | --- | --- | --- |
| JCT3V-F0093 | 3D-CE3.h: Results on simple merge candidate list construction for 3DV | ETRI & KHU | F0245 (LG) |
| JCT3V-F0104 | CE3-related: Removal of redundancy on VSP, ARP and IC | Sharp & LG | F0198 (Samsung) |
| JCT3V-F0107 | 3D-CE3: Results on additional merging candidates for depth coding | MediaTek | F0205 (Samsung) |
| JCT3V-F0110 | 3D-CE3: Sub-PU level inter-view motion prediction | MediaTek | F0237 (Qualcomm) |
| JCT3V-F0114 | 3D-CE3 related: A bug-fix for the disparity-based Skip and Direct modes in 3D-AVC | MediaTek |  |
| JCT3V-F0121 | CE3 : AMVP candidate list construction for DCP blocks | LGE | F0187 (NTT) |
| JCT3V-F0125 | CE3: Inter-view motion vector prediction for depth coding | Qualcomm | F0188 (RWTH Aachen) |
| JCT3V-F0127 | CE3 related: Simplifications to sub-PU level inter-view motion prediction | Qualcomm | F0229 (MediaTek) |
| JCT3V-F0128 | CE3 related: Sub-PU based MPI | Qualcomm | F0230 (MediaTek) |
| JCT3V-F0129 | CE3 related: combined bi-predictive merging candidates for 3D-HEVC | Qualcomm | F0234 (ETRI&KHU) |
| JCT3V-F0135 | CE3 related: HEVC compatible de-blocking for sub-PUs | Qualcomm | F0242 (Ericsson) |
| JCT3V-F0137 | CE3: Depth-based Block Partitioning | RWTH & Huawei | F0233 (ETRI&KHU) |
| JCT3V-F0144 | 3D-CE3 related: Additional Depth-based DV Candidate | Samsung | F0212 (Sharp) |
| JCT3V-F0150 | 3D-CE3 related: MPI candidate in depth merge mode list construction | Samsung | F0240 (MediaTek) |
| JCT3V-F0174 | 3D-CE3.h related: On Split Flag Prediction | SYSU | F0228 (MediaTek) |
| JCT3V-F0195 | CE3-related: Interview motion vector prediction by DoNBDV in depth coding | MediaTek | F0252 (NTT) |

# Summary of proposals & results

## CE3 results

The following table describes the best coding gain in each CE contribution.

| Doc. No. | Contributions | Video /total BR | Synthesized /total BR |
| --- | --- | --- | --- |
| Simplification of existing merge candidate list |  |  |  |
| JCT3V-F0093 | 3D-CE3.h: Results on simple merge candidate list construction for 3DV | 0.02% | 0.02% |
| AMVP candidate list |  |  |  |
| JCT3V-F0121 | CE3 : AMVP candidate list construction for DCP blocks | -0.09% | -0.08% |
| Sub-PU interview motion candidate |  |  |  |
| JCT3V-F0110 | 3D-CE3: Sub-PU level inter-view motion prediction | -1.44% | -1.27% |
| Interview motion vector prediction for depth coding |  |  |  |
| JCT3V-F0125 | CE3: Inter-view motion vector prediction for depth coding | -0.07% | -0.29% |
| Using disparity vectors to improve depth coding |  |  |  |
| Texture merging candidate |  |  |  |
| JCT3V-F0107 | 3D-CE3: Results on additional merging candidates for depth coding | -0.02% | -0.27% |
| Depth-based block partitioning |  |  |  |
| JCT3V-F0137 | CE3: Depth-based Block Partitioning | -0.18% | -0.12% |

### Simplification of existing merge candidate list

JCT3V-F0093 is the CE3 results for merge list construction.

- JCT3V-F0093 [ETRI and KHU] 3D-CE3.h: Results on simple merge candidate list construction for 3DV (Cross-checked in JCT3V-F0245 [LG])

This contribution mainly proposes three aspects:

- Implement the MCL construction process for the 3D-HEVC by re-using existing MCL construction module in the HEVC.

- Simplifies the pruning process for inter-view candidate, disparity candidate compared to the previous contribution (JCT3V-E0213)

- Use the same combined index table for combined bi-predictive candidates as the one in HEVC.

One CE-related contribution JCT3V-F0129 also proposes a similar idea to use the same combined index table for combined bi-predictive candidates as the one in HEVC.

#### Coding results of the CE contribution F0093

| F0093 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.8% | 101.2% |

### AMVP candidate list

- JCT3V-F0121 [LGE] CE3 : AMVP candidate list construction for DCP blocks (Cross-checked in JCT3V-F0187[NTT])

This contribution proposes to fill the empty candidate with the refined disparity of neighboring blocks (DoNBDV) instead of the zero vector into the AMVP candidate list. It also proposes to fill the list with DoNBDV and zero vector after checking whether the number of mvp candidates are less than two.

#### Coding results of the CE contribution F0121

| F0121 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | -0.14% | -0.18% | -0.08% | -0.09% | -0.08% | 99.45% | 99.91% |

### Sub-PU interview motion candidate

- JCT3V-F0110 [MediaTek] 3D-CE3: Sub-PU level inter-view motion prediction (Cross-checked in JCT3V-F0237[Qualcomm])

In this contribution, it is proposed to use a sub-PU level inter-view motion prediction (SPIVMP) method as the temporal inter-view merge candidate, which means the temporal inter-view merge candidate is automatically retrieving the interview motion vector and do motion compensation in sub-PUs. The Sub-PU sizes equal to 4x4, 8x8, and 16x16 are tested.

#### Coding results of the CE contribution F0110

| F0110 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4x4 | -6.4% | -6.4% | -1.8% | -1.5% | -1.4% | 98.9% | 108.2% |
| 8x8 | -5.9% | -6.0% | -1.7% | -1.4% | -1.3% | 98.8% | 102.9% |
| 16x16 | -4.1% | -4.1% | -1.1% | -1.0% | -0.9% | 99.0% | 101.6% |

One CE-related contribution JCT3V-F0127 proposes some simplification methods based on this sub-PU level inter-view motion prediction.

### Interview motion vector prediction for depth coding

- JCT3V-F0125 [Qualcomm] CE3: Inter-view motion vector prediction for depth coding (Cross-checked in JCT3V-F0188[RWTH Aachen])

In this contribution, three aspects are proposed to enable the inter-view motion vector prediction for depth coding:

- Derive a disparity vector from neighbor reconstructed depth pixels

- Add more merge candidates by using the motion information from the reference depth views which are generated in the same way as in texture coding.

- Shift the center pixel which is used to identify the reference block by (1, 1) for coding efficiency improvement

#### Coding results of the CE contribution F0125

| F0125 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vs CTC | -0.2% | -0.2% | -0.1% | -0.1% | -0.3% | 96% | 101% |

One CE-related contribution JCT3V-F0195 also proposes to enable the interview motion vector prediction for depth coding by almost the same scheme in texture coding.

### Using disparity vectors to improve depth coding and texture merging candidate

- JCT3V-F0107 [MediaTek] 3D-CE3: Results on additional merging candidates for depth coding (Cross-checked in JCT3V-F0205[Samsung])

This contribution combines two CE contributions JCT3V-E0174 and JCT3V-E0183 together. There are two parts in this contribution:

- A disparity derived depth (DDD) candidate was proposed to derive a depth prediction value converted from the disparity vector of a collocated texture block. The derived depth value is used to represent all prediction samples of the current block when the proposed candidate is selected.

- An additional texture merging candidate was proposed to inherit the motion parameters from the below-right position of the collocated texture PU. It is also proposed to add an additional pruning to further improve the coding efficiency.

#### Coding results of the CE contribution F0107

| F0107 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. DDD | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 99.7% | 100.6% |
| 2. Additional cand. | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 99.5% | 99.4% |
| 1+2 w/o pruning | 0.1% | 0.0% | 0.0% | 0.0% | -0.2% | 100.4% | 101.6% |
| 1+2 w/ pruning | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | 99.7% | 101.4% |

### Depth-based block partitioning (DBBP)

- JCT3V-F0137 [RWTH & Huawei] CE3: Depth-based Block Partitioning (Cross-checked in JCT3V-F0233[ETRI and KHU])

In this contribution, it is proposed to use an arbitrarily shaped block partitioning for the collocated texture block which is derived based on a binary segmentation mask computed from the corresponding (virtual) depth map. (For flexible coding order configuration, the binary segmentation mask is computed from the collocated depth block.) Each of the two partitions (resembling foreground and background) is motion compensated and afterwards merged based on the depth-based segmentation mask.

Two results are shown in this contribution. Configuration A is the results of the proposed method. Configuration B uses a simple 0.5/0.5 average filter along the segmentation boundary of DBBP blocks.

#### Coding results of the CE contribution F0137

| F0137 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | -0.4% | -0.4% | -0.2% | -0.2% | -0.1% | 103.3% | 101.2% |
| B | -0.5% | -0.5% | -0.2% | -0.2% | -0.1% | 103.2% | 102.2% |

## CE3 related contributions

The CE3-related contributions are roughly classified into three categories according to their purpose: simplifcation, coding efficiency, fixes. The following table shows the classfication and their coding results. Two sub-PU related contributions JCT3V-F0127 and JCT3V-F0128 are all based on CE contribution JCT3V-F0110. The anchor of these two contributions is the 8x8 sub-PU results of JCT3V-F0110.

| Doc. No. | Contributions | Video /total BR | Synthesized /total BR |
| --- | --- | --- | --- |
| Simplification |  |  |  |
| JCT3V-F0127 | CE3 related: Simplifications to sub-PU level inter-view motion prediction: Test 1 | 0.09% | 0.04% |
|  | CE3 related: Simplifications to sub-PU level inter-view motion prediction: Test 2 | 0.02% | 0.00% |
| JCT3V-F0129 | CE3 related: combined bi-predictive merging candidates for 3D-HEVC | 0.02% | 0.01% |
| Coding Efficiency |  |  |  |
| JCT3V-F0104 | CE3-related: Removal of redundancy on VSP, ARP and IC: Test 1 | -0.09% | -0.14% |
|  | CE3-related: Removal of redundancy on VSP, ARP and IC: Test 2 | -0.10% | -0.16% |
| JCT3V-F0128 | CE3 related: Sub-PU based MPI | -0.24% | -0.27% |
| JCT3V-F0135 | CE3 related: HEVC compatible de-blocking for sub-PUs | -0.26% | -0.18% |
| JCT3V-F0144 | 3D-CE3 related: Additional Depth-based DV Candidate | -0.01% | -0.05% |
| JCT3V-F0174 | 3D-CE3.h related: On Split Flag Prediction | -0.28% | -0.15% |
| JCT3V-F0195 | CE3-related: Interview motion vector prediction by DoNBDV in depth coding | -0.08% | -0.29% |
| Fixes |  |  |  |
| JCT3V-F0114 | 3D-CE3 related: A bug-fix for the disparity-based Skip and Direct modes in 3D-AVC | n/a | n/a |
| JCT3V-F0150 | 3D-CE3 related: MPI candidate in depth merge mode list construction: Case 1 | 0.02% | -0.04% |
|  | 3D-CE3 related: MPI candidate in depth merge mode list construction: Case 2 | 0.00% | 0.00% |

### Simplication

- JCT3V-F0127 [Qualcomm] CE3 related: Simplifications to sub-PU level inter-view motion prediction (Cross-checked in JCT3V-F0229 [MediaTek])

This contribution proposes simplifications to the sub-PU level inter-view motion prediction with two parts:

- Remove the pruning process for two spatial merge candidates when compared to the inter-view predicted merge candidate.

- Use the original PU-level interview merge candidate for cases that a sub-PU does not reach available motion from the reference block.

Experimental results show that, average 0.0% and 0.0% BD-rate is observed on synthesized views with the first and second proposed part, respectively. The anchor of the coding results is the 8x8 Sub-PU results in CE contribution JCT3V-F0110.

#### Coding results of the CE-related contribution F0127

| F0127 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Part 1 | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 106% | 110% |
| Part 2 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 103% | 103% |

- JCT3V-F0129 [Qualcomm] CE3 related: combined bi-predictive merging candidates for 3D-HEVC (Cross-checked by JCT3V-F0234 [ETRI & KHU])

Similar to one aspect in CE contribution JCT3V-F0093, this contribution proposes to align the bi-predictive merging candidate derivation process in 3D-HEVC the existing HEVC design. Simulation results show almost no impact on coding performance by doing this alignment.

#### Coding results of the CE-related contribution F0129

| F0129 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 105% | 108% |

### Coding Efficiency

- JCT3V-F0104 [Sharp / LG] CE3-related: Removal of redundancy on VSP, ARP and IC (Cross-checked in JCT3V-F0198 [Samsung])

In current 3D-HEVC, VSP, ARP, and IC are mutually exclusive, so it is redundant to allow the combination of these coding tools in one coding unit. Two aspects are proposed:

- Insert the VSP merge candidate only on the condition that both ARP and IC are disabled in the CU.

- IC flag is not signalled when ARP is enabled.

The experimental result reportedly shows 0.1 %, 0.1 %, and 0.1 % gain in texture, video and synthesis respectively.

On top of above, Test 2 added the following aspect:

- VSP motion information (VspModeFlag) is only inherited from neighboring blocks on the condition that both ARP and IC are disabled in the CU.

#### Coding results of the CE-related contribution F0104

|  | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F0104 test1 | -0.4% | -0.5% | -0.1% | -0.1% | -0.1% | 98.1% | 100.4% |
| F0104 test2 | -0.5% | -0.5% | -0.1% | -0.1% | -0.2% | 97.5% | 102.1% |

- JCT3V-F0128 [Qualcomm] CE3 related: Sub-PU based MPI (Cross-checked in JCT3V-F0230 [MediaTek])

This contribution proposes to extend the sub-PU level inter-view motion prediction method to the Motion Parameter Inheritance (MPI) merge candidate in 3D-HEVC. For each depth PU, 8x8 size sub-PUs can grab different motion vectors from the co-located 8x8 blocks in texture to form a MPI candidate. Experimental results show that the proposed method achieves additional 0.3% BD-rate saving for synthesized views with sub-PU inter-view motion prediction enabled in texture. The anchor of the coding results is the 8x8 Sub-PU results in CE contribution JCT3V-F0110.

#### Coding results of the CE-related contribution F0128

| F0128 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| vs F0110 | 0.0% | 0.0% | 0.0% | -0.2% | -0.3% | 108% | 106% |

- JCT3V-F0135 [Qualcomm] CE3 related: HEVC compatible de-blocking for sub-PUs (Cross-checked in JCT3V-F0242 [Ericsson])

The sub-PU based motion may lead to blocking artifacts because sub-blocks within a PU are allowed to be predicted using different motion vectors while the boundary of such sub-blocks are typically not considered edges for de-blocking. This contribution proposes to allow de-blocking sub-PU boundary by introducing transform unit boundaries before the de-blocking process.

It reportedly shows that blocking artifact caused by sub-PU prediction is suppressed. It is also reported that -0.26% and -0.18% (and -0.20% and -0.07% coding gain for Shark sequence) average coding gain are achieved on texture and synthesized view respectively for the first 7 sequences.

#### Coding results of the CE-related contribution F0135

| F0135 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | -0.49% | -0.50% | -0.26% | -0.26% | -0.18% | 111.4% | 122.0% |

- JCT3V-F0144 [Samsung] 3D-CE3 related: Additional Depth-based DV Candidate (Cross-checked in JCT3V-F0212 [Sharp])

It is described that the shifted DV candidates provide no coding gain in the current 3D-HEVC. It is proposed to replace the current shifted DV candidates with a proposed depth-based DV candidate. While the current depth-based DV is derived by choosing the maximum depth value from the corresponding depth block, the proposed depth-based DV is derived by choosing the minimum depth value from the corresponding depth block.

The proposed method reportedly provides 0.1% bit-saving for synthesized view.

#### Coding results of the CE-related contribution F0144

| F0144 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 99.0% | 96.3% |

- JCT3V-F0174 [SYSU] 3D-CE3.h related: On Split Flag Prediction (Cross-checked in JCT3V-F0228 [MediaTek])

This contribution provided a strategy to reduce the split flag signalling for dependent texture views. Based on JCT3V-E0173, the proposal introduces a modified CU splitting termination strategy in both encoder and decoder for split_cu_flag prediction. The proposed method checks whether the five interview neighboring blocks are in merge mode and the splitting depth is equal to or larger than the maximum depth of the CU splitting depths of the five interview neighboring blocks to further decide the signaling of the split_cu_flag.

Experimental results show that the proposed method can effective to reduce split flag signalling overhead and achieves 0.36% , 0.28% and 0.15% averaging BD-rate coding gains for texture views, overall video and synthesized views under common test condition, respectively.

#### Coding results of the CE-related contribution F0174

| F0174 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | -0.36% | -0.36% | -0.28% | -0.28% | -0.15% | 109.1% | 92.1% |

- JCT3V-F0195 [MediaTek] CE3-related: Interview motion vector prediction by DoNBDV in depth coding (Cross-checked in JCT3V-F0252[NTT])

It is proposed to enable the interview motion vector prediction and depth refinement for depth coding in 3D-HEVC. The NBDV and DoNBDV for depth coding is enabled to provide disparity vector for interview motion vector prediction. The IDV is disabled for depth coding in order to reduce additional memory buffers. The same as CE contribution JCT3V-F0125, the center pixel which is used to identify the reference block is shifted by (1, 1) for coding efficiency improvement

The experimental results reportedly show that 0.4% and 0.3% BD-Rate gains are achieved for video 1 and video 2, 0.2% BD-Rate gains is achieved for coded and synthesized views.

#### Coding results of the CE-related contribution F0195

| F0195 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% | 100.7% | 94.8% |

### Fixes

- JCT3V-F0114 [MediaTek] 3D-CE3 related: A bug-fix for the disparity-based Skip and Direct modes in 3D-AVC

The disparity-based skip/direct mode directly inherits the motion information, i.e., reference index(es) and motion vector(s), of a corresponding block in the base view as the motion information of the current block. However, an inherited reference index may exceed the maximum reference index of the current block, which may cause the encoder and decoder to crash. Therefore, it is proposed to add one additional check to see if the inherited reference index is valid. If the reference index is valid, the reference index and MVs of the corresponding block are directly inherited. Otherwise, the inherited inter-view motion information is viewed as unavailable. This bug-fix does not affect the coding performance under common test conditions.

- JCT3V-F0150 [Samsung] 3D-CE3 related: MPI candidate in depth merge mode list construction (Cross-checked in JCT3V-F0240 [MediaTek])

In this contribution, a flag is defined to enable or disable MPI in VPS, so the MPI candidate is only added when the flag is equal to 1. The current 3D-HTM employs 5 candidates in the depth merge mode, but WD describes 6. The max number of the texture merge candidates depends on an inter-view motion vector flag in VPS extension (5 +iv_mv_pred_flag). This contribution provides two options to align the max number of merge candidate of 3D-HTM and WD:

- The same as the texture merge mode list construction, it is proposed to employs 5 or 6 merge candidates, based on the MPI flag in VPS.

- Always uses 5 candidates regardless of the MPI flag.

#### Coding results of the CE-related contribution F0150

| F0150 | Video 1 | Video 2 | Video/video BR | Video/total BR | Synthesized/total BR | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Option 1 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 102.4% | 101.0% |
| Option 2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100.2% | 100.0% |

# Closing Remarks

It is recommended the CE be continued and investigate proposed designs in further details.