| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0092r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0040r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On nuh_layer_id of SPS and PPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye, Xiaoyu Xiu, Yuwen He 9710 Scranton Road, #250 San Diego, CA 92121 USA Takeshi Tsukuba Tomoyuki Yamamoto Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com +81-43-299-8526 tsukuba.takeshi@sharp.co.jp |
| Source: | InterDigital Communications, Inc; SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes constraint on the value of nuh_layer_id of SPS/PPS NAL unit to ensure the availability of the parameter sets after sub-bitstream extraction.

Based on BoG discussion, the modified specification text of the constraint is presented in section 2.

# Constraint on nuh_layer_id of SPS/PPS

SHVC working draft v3 specifies the order and activation of SPS/PPS. The current constraint on the nuh_layer_id value of SPS is the following, and the same constraint is applied to PPS.

“The nuh_layer_id value the NAL unit containing the SPS RBSP that is activated for nuh_layer_id equal to nuhLayerIdA shall be less than or equal to nuhLayerIdA. The same SPS RBSP may be the active layer SPS for more than one nuh_layer_id value.”

At the 14th JCTVC meeting, it was also agreed that SPS/PPS IDs with different values of nuh_layer_id share the same “value space” such that different layers may share the same SPS/PPS.

The nuh_layer_id constraint on SPS/PPS does not mandate the value of nuh_layer_id of SPS/PPS to be equal to the nuh_layer_id of the layer it activates, or the nuh_layer_id of the dependent layers of the layer it activates. During the sub-bitstream extraction process, the layer being activated and its dependent layers may be included in the target layer identification, layerIdListTarget; while the SPS/PPS NAL units may get removed if the value of its nuh_layer_id is not among the values in the layerIdListTarget. If an essential SPS/PPS gets removed in bitstream extraction, then the layers being activated may not be decoded correctly due to missing SPS and/or PPS. Figure 1 shows such an example.

Here a further constraint on the nuh_layer_id value of SPS and PPS is proposed:

- The nuh_layer_id of value the NAL unit containing the SPS/PPS RBSP that is activated for nuh_layer_id equal to iNuhLId shall be equal to iNuhLId or the nuh_layer_id value of the dependent layer of the layer with nuh_layer_id value equal to iNuhLId.

Or an alternative option is to mandate all SPS/PPS nuh_layer_id to be equal to 0:

- The nuh_layer_id value the NAL unit containing the SPS/PPS RBSP shall be equal to 0

- Sub-bitstream extraction example

# Specification text for the constraint

Based on BoG discussion, the modified specification text of the proposed constraint on nuh_layer_id of SPS and PPS is as follows:

The nuh_layer_id value of the NAL unit containing the PPS that is activated for a layer layerA with nuh_layer_id equal to nuhLayerIdA shall be equal to 0, or nuhLayerIdA, or the nuh_layer_id of a direct or indirect reference layer of layerA.

The nuh_layer_id value of the NAL unit containing the SPS that is activated for a layer layerA with nuh_layer_id equal to nuhLayerIdA shall be equal to 0, or nuhLayerIdA, or the nuh_layer_id of a direct or indirect reference layer of layerA.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).