| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0110 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0043 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On picture output marking process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Tel: Email: | {yongjin9.cho, b.d.choi, m.w.park, jinyoung79, hc.wey, dionism}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes PicOutputFlag marking process based on a signalled output layer set in the vps extension.

# Introduction

In the current working draft of scalable high efficiency video coding, it is not clearly stated how to mark PicOutputFlag given target output layers. This contribution specifies the process of marking the flag.

# Proposal

Addition to the current draft

The list of output layer flags, TargetOutputLayerFlag, which specifies the target output layer(s) from TargetDecLayerIdList, is specified as follows:

–	If TargetOutputLayerSetIdx, which identifies the target output layer set index and takes the values from vps_number_layer_sets_minus1+1 to numOutputLayerSets – 1, inclusive , is specified by external means not specified in this Specification,

- TargetOutputLayerFlag[i] is set equal to output_layer_flag[TargetOutputLayerSetIdx][i]  for all values of i from 0 to NumLayerInIdList[output_layer_set_idx_minus1[TargetOutputLayerSetIdx]+1]-1.

Otherwise (if TargetOutputLayerSetIdx is not set by external means not specified in this Specification),

–	If the value of default_one_target_output_layer_flag is 0, TargetOutputLayerFlag[i] is set equal to 1 for all values of i that are equal to entries of TargetDecLayerIdList, and is set equal to 0 for all other values of i (that are not equal to entries of TargetDecLayerIdList).

–	Otherwise, TargetOutputLayerFlag[i] is set equal 1 for the value of i that is equal to the highest nuh_layer_id value in TargetDecLayerIdList, and is set equal to 0 for the other values of i (that is not equal to the highest nuh_layer_id value in TargetDecLayerIdList).

F.8.1.2 	Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0

PicOutputFlag is set as follows:

–	If TargetOutputLayerFlag[i] is equal to 0 for the value of i, where nuh_layer_id is equal to TargetDecLayerIdList[i], PicOutputFlag is set equal to 0.

–	Otherwise, if the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, if LayerInitialisedFlag[ nuh_layer_id ] is equal to 0, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).