| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCT3V-F0044 |
| --- | --- |

| Title: | 3D/MV-HEVC HLS: HEVC compatible slice segment header in 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Jian-Liang Lin2, Shawmin Lei2 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, xianguo.zhang, jl.lin, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to move syntax elements related to 3D-HEVC from slice segment header to slice segment header extension. In this way, slice segment header in 3D-HEVC can be compatible with that in HEVC.

# Introduction

In the current 3D-HEVC [1], some syntax elements related to 3D-HEVC are signalled in the slice segment header (SSH) as depicted in Table 1, where MvHevcCompatibilityFlag is set equal to (!vps_extension2_flag). However, vps_extension2_flag is signaled in the video parameter set in 3D-HEVC but not in HEVC [2]. Thus an HEVC conformance decoder will fail to parse SSH in 3D-HEVC.

Table 1. Syntax elements related to 3D-HEVC in slice segment header

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| else if( layer_id && !MvHevcCompatibilityFlag ) { |  |
| slice_ic_enable_flag | u(1) |
| if( slice_ic_enable_flag ) |  |
| slice_ic_disable_merge_zero_idx_flag | u(1) |
| } |  |
| five_minus_max_num_merge_cand | ue(v) |
| … |  |
| } |  |

# Proposed method

It is proposed to signal all the syntax elements related to 3D-HEVC in slice segment header extension (SSHE) instead of slice segment header as depicted in Table 2 and Table 3. In this proposed HEVC compatible design, SSH in 3D-HEVC can be parsed by an HEVC conformance decoder, thus the SSH coding/parsing module in HEVC can be reused by 3D-HEVC.

Table 2. Proposed SSH

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| else if( layer_id && !MvHevcCompatibilityFlag ) { |  |
| slice_ic_enable_flag | u(1) |
| if( slice_ic_enable_flag ) |  |
| slice_ic_disable_merge_zero_idx_flag | u(1) |
| } |  |
| five_minus_max_num_merge_cand | ue(v) |
| … |  |
| } |  |

Table 3. Proposed SSHE

| slice_segment_header_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !( weighted_pred_flag && slice_type = = P ) && !( weighted_bipred_flag && slice_type = = B ) && |  |
| layer_id && !MvHevcCompatibilityFlag ) |  |
| { |  |
| slice_ic_enable_flag | u(1) |
| if( slice_ic_enable_flag ) |  |
| slice_ic_disable_merge_zero_idx_flag | u(1) |
| } |  |
| … |  |
| } |  |

# Conclusion

In this proposal, an HEVC compatible SSH signaling method is proposed in 3D-HEVC. By signaling all syntax elements related to 3D-HEVC in SSHE instead of SSH, SSH in 3D-HEVC can be parsed by an HEVC conformance decoder, thus the SSH coding/parsing module in HEVC can be reused by 3D-HEVC.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC Draft Text 1,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1001, Aug, 2013.

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent),” Document of Joint Collaborative Team on Video Coding, JCTVC-L1003, Jan, 2013.