| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0045 |
| --- | --- |

| Title: | 3D-HEVC HLS: Constraints on camera parameter signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen, Jian-Liang Lin, Yu-Pao Tsai, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, yiwen.chen, jl.lin, yupao.tsai, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, camera parameters of a texture layer can be replaced with those of a depth layer of the same view, which may cause wrong depth-to-disparity conversion. In this contribution, a pair of missing braces related to camera parameters coded in the video parameter set (VPS) is first added. Then two methods are proposed to solve the ambiguity problem of the camera parameter signaling for layers with the same view order index. The first method is to allow at most one layer per view with its cp_present_flag equal to 1 and to transmit camera parameters in the slice segment header extension only when cp_present_flag of the layer and cp_in_slice_segment_header_flag are both equal to 1. The second method is to force the camera parameters to be the consistent for all layers of the same view.

# Introduction

In the VPS, when cp_present_flag[ layerId ] is equal to 1, camera parameters are coded for the layer with a layer index equal to layerId. However, the camera parameters of a layer can be overwritten by those of another layer with a larger layer index even when the two layers have the same view order index. For example, assuming layer 2 is the texture layer and layer 3 is the depth layer of the same view, syntax-wise the camera parameters of the texture layer can be replaced with those coded in the depth layer, subsequently causing wrong depth-to-disparity conversion. In this contribution, one bug in the VPS text is first fixed, and then two solutions to the mentioned problem are proposed.

# Proposed bug-fix and methods

## Bug-fix for the missing braces in VPS

As shown in Table 1, a pair of missing braces is added.

## Method 1: Restrict cp_present_flag to prevent overwriting

It is proposed to add a constraint on cp_present_flag as highlighted in green in the semantics of cp_present_flag below Table 1. The constraint allows only up to one layer of a view to signal camera parameters in the VPS. Besides, it is proposed to allow camera parameters coded in the slice segment header extension only when both cp_in_slice_segment_header_flag and cp_present_flag[ nuh_layer_id ] are equal to 1. In this way, camera parameters of a later layer cannot overwrite those of an earlier layer of the same view.

Table 1.  Proposed video parameter set syntax

| VPS_header_extension2( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( layerId ! = 0 ) { |  |
| cp_present_flag[ layerId ] | u(1) |
| … |  |
| cp_precision | ue(v) |
| cp_in_slice_segment_header_flag | u(1) |
| if( !cp_in_slice_segment_header_flag ) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if( cp_present_flag[ layerId ] ) { |  |
| viewIdx = ViewOrderIdx[ layerId ] |  |
| for( j = 0; j < viewIdx; j++ ) { |  |
| vps_cp_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_off[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_scale_plus_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_off_plus_off[ viewIdx ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |

cp_present_flag[ layerId ] equal to 1 indicates that vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ], with j equal to VpsViewIdx[ layerId ], may be present for the layer with nuh_layer_id equal to layerId. cp_present_flag[ layerId ] equal to 0 indicates that vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ], with j equal to VpsViewIdx[ layerId ], are not present. When VpsDepthFlag[ layerId ] is equal to 1 and depth_refinement_flag[ layerId ] or view_synthesis_pred_flag[ layerId ] is equal to 1, the value of cp_present_flag[ layerId ] shall be equal to 1 or at least the cp_present_flag for one of the precedent layer with the same view order index shall be equal to 1. When not present, the value of cp_present_flag[ layerId ] is inferred to be equal to 0. When the cp_present_flag for any precedent layer with the same view order index is equal to 1, the value of cp_present_flag[ layerId ] shall be equal to 0.

Table 2.  Proposed slice segment header extension syntax

| slice_segment_header_extension( ) { | Descriptor |
| --- | --- |
| if( cp_in_slice_segment_header_flag && cp_present_flag[ nuh_layer_id ] ) { |  |
| for ( i = 0; i < ViewIdx; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

## Method 2: Force using the same camera parameters

In method 2, we simply add a normative constraint saying that camera parameters with the same view shall be the consistent. The updated semantics are as follows.

cp_precision specifies the precision of vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] in the VPS and cp_scale[ i], cp_off[ i], cp_inv_scale_plus_scale[ i], and cp_inv_off_plus_off[ i ] in the slice segment header. The value of cp_precision shall be in the range of 0 to 5, inclusive.

cp_in_slice_segment_header_flag equal to 1 specifies that the syntax elements of vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] are not present in the video parameter set. A value of camera_parameter_in_slice_header_flag equal to 0 specifies that the vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] syntax elements are present in the slice segment header.

vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] specify conversion parameters for converting a depth value to a disparity value.

When present, the value of the VPS syntax elements cp_precision, cp_in_slice_segment_header_flag, vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] shall be the consistent for all layers of the same view order index.

In slice segment header extension, the value of the syntax elements cp_precision, cp_in_slice_segment_header_flag, vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] shall be consistent for all layers of the same view order index.

# Conclusion

In this contribution, one bug-fix for VPS is first provided. Then two methods are proposed to solve the conflict when camera parameters are inconsistent between two layers of the same view. Method 1 proposes to allow up to only one layer with cp_present_flag equal to 1 for each view and allow camera parameters coded in the slice segment header extension only when cp_present_flag of the layer and cp_in_slice_segment_header_flag are both equal to 1. Method 2 proposes to force the camera parameters to be consistent for all layers of the same view.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 5,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1005, Aug., 2013.