| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0045 |
| --- | --- |

| Title: | 3D-HEVC HLS: Constraints on camera parameter signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Lin Chang, Yi-Wen Chen, Jian-Liang Lin, Yu-Pao Tsai, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yulin.chang, yiwen.chen, jl.lin, yupao.tsai, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current 3D-HEVC, camera parameters are transmitted according to the cp_present_flag for each layer. If coding tools such as view synthesis prediction (VSP) and depth-oriented neighboring block disparity vector (DoNBDV) that require camera parameters are turned on for a certain layer, the cp_present_flag for such layer shall also be turned on. However, if both texture and depth layers with the same view order index have their cp_present_flag turned on, the camera parameter in the later layer will overwrite that in the precedent layer. In this contribution, it is proposed to solve the ambiguity of the camera parameter signaling for layers with the same view order index. Two methods are proposed. The first method is to restrict the cp_present_flag for layers with the same view order index. The second method is to force the camera parameters to be the same for the layers with the same view order index.

# Introduction

In current HTM-8.0, the camera parameter is coded according to the cp_present_flag for each layer with layer index layerId. If cp_present_flag[ layerId ] equals to 1, the camera parameter is coded for that layer. If only depth layers are coded or depth layers are coded before texture layers, the depth oriented coding tools can be normally functioned because there is one constraint that the cp_present_flag shall be 1 if depth oriented coding tool is used. However, in current design, the camera parameter of a layer with smaller layer index can be overwritten by the camera parameter of a layer with larger layer index if the two layers have the same view order index. In this case, the two layers with the same view order index might not have the same camera parameter, and the depth-to-disparity conversion may not correctly be carried out.

# Proposed schemes

## Method 1: Restrict cp_present_flag to prevent overwriting

To solve this issue, it is proposed to add a constraint on cp_present_flag that if any cp_present_flag equals to 1 for other layer with the same view order index, then the cp_present_flag[ layerId ] for the current layer shall be 0 to prevent overwriting of the existing camera parameter. And the camera parameter for the layer with layer index layerId is inherited from the previously coded camera parameter of other layers with the same view order index.

As the VPS extension shown in Table 1, when the cp_present_flag for any precedent layer with the same view order index viewIdx equals to 1, then the value of cp_present_flag[ layerId ] shall be equal to 0. In the slice segment header extension shown in Table 2, a condition check for cp_present_flag[nuh_layer_id] is added to further constraint the coding of camera parameters with the same view order index. If the cp_in_slice_segment_header_flag is turned on in slice header extension, the camera parameter for the current layer with layer id nuh_layer_id is inherited from the previously coded camera parameter of other layers with the same view order index.

Table 1 Syntax Change for VPS

| VPS_header_extension2( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( layerId ! = 0 ) { |  |
| cp_present_flag[ layerId ] | u(1) |
| … |  |
| cp_precision | ue(v) |
| cp_in_slice_segment_header_flag | u(1) |
| if( !cp_in_slice_segment_header_flag ) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if( cp_present_flag[ layerId ] ) { |  |
| viewIdx = ViewOrderIdx[ layerId ] |  |
| for( j = 0; j < viewIdx; j++ ) { |  |
| vps_cp_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_off[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_scale_plus_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_off_plus_off[ viewIdx ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |

cp_present_flag[ layerId ] equal to 1 indicates that vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ], with j equal to VpsViewIdx[ layerId ], may be present for the layer with nuh_layer_id equal to layerId. cp_present_flag[ layerId ] equal to 0 indicates that vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ], with j equal to VpsViewIdx[ layerId ], are not present. When VpsDepthFlag[ layerId ] is equal to 1 and depth_refinement_flag[ layerId ] or view_synthesis_pred_flag[ layerId ] is equal to 1, the value of cp_present_flag[ layerId ] shall be equal to 1. When not present, the value of cp_present_flag[ layerId ] is inferred to be equal to 0. When the cp_present_flag for any precedent layer with the same view order index equals to 1, then the value of cp_present_flag[ layerId ] shall be equal to 0.

- Table 2. Slice segment header extension syntax

| slice_segment_header_extension( ) { | Descriptor |
| --- | --- |
| if( cp_in_slice_segment_header_flag && cp_present_flag[nuh_layer_id] ) { |  |
| for ( i = 0; i < ViewIdx; i++ ) { |  |
| cp_scale[ i ] | se(v) |
| cp_off[ i ] | se(v) |
| cp_inv_scale_plus_scale[ i ] | se(v) |
| cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| } |  |

## Method 2: Force same camera parameter

In method 2, it is proposed to add a constraint on the syntax elements associated with the camera parameters such as cp_precision, cp_in_slice_segment_header_flag, vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ]. To be specific, all the syntax elements associated with the camera parameters of the layers with the same view order index shall be the same. The updated semantics are as following:

cp_precision specifies the precision of vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] in the VPS and cp_scale[ i], cp_off[ i], cp_inv_scale_plus_scale[ i], and cp_inv_off_plus_off[ i ] in the slice segment header. The value of cp_precision shall be in the range of 0 to 5, inclusive.

cp_in_slice_segment_header_flag equal to 1 specifies that the syntax elements of vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] are not present in the video parameter set. A value of camera_parameter_in_slice_header_flag equal to 0 specifies that the vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] syntax elements are present in the slice segment header.

vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] specify conversion parameters for converting a depth value to a disparity value.

When present, the value of the VPS syntax elements cp_precision, cp_in_slice_segment_header_flag, vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] shall be the same in all layers of the same view order index.

In slice segment header extension, the value of the syntax elements cp_precision, cp_in_slice_segment_header_flag, vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ] shall be the same in all layers of the same view order index.

# Conclusion

In this contribution, two methods to solve the conflict when multiple camera parameters exist for same view order index are proposed. Method 1 proposes to restrict the cp_present_flag if a precedent layer with the same order index already signaled camera parameters. Method 2 proposes to force the camera parameters to be the same in all layers of the same view order index.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- G. Tech, K. Wegner, Y. Chen, S. Yea, “3D-HEVC test model 5,” Document of Joint Collaborative Team on 3D Video Coding Extension Development, JCT3V-E1005, Aug., 2013.