| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0045 |
| --- | --- |

| Title: | 3D-HEVC HLS: On camera parameter signalling Working Draft Changes |
| --- | --- |
| Status: | Input Document |

_____________________________

# Proposed WD Changes

## View order index based camera parameter signaling in VPS

This document describes a view order index based camera parameter signaling method and its semantic changes as described in JCT3V-F0045 and JCT3V-F0136. As shown in Table 1, yellow highlighted texts are the parts added.

Table 1.  Proposed video parameter set syntax

| vps_extension2( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| layerId = layer_id_in_nuh[ i ] |  |
| if ( layerId ! = 0 ) { |  |
| cp_present_flag[ layerId ] | u(1) |
| … |  |
| } |  |
| cp_precision | ue(v) |
| cp_in_slice_segment_header_flag | u(1) |
| if( !cp_in_slice_segment_header_flag ) |  |
| for( iviewIdx = 0; iviewIdx <= vps_max_layers_minus1NumViews - 1; iviewIdx++ ) |  |
| layerId = layer_id_in_nuh[ i ] |  |
| cp_present_flag[ viewIdx ] | u(1) |
| if( cp_present_flag[ layerId viewIdx] ) { |  |
| viewIdx = ViewOrderIdx[ layerId ] |  |
| for( j = 0; j < viewIdx; j++ ) { |  |
| vps_cp_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_off[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_scale_plus_scale[ viewIdx ][ j ] | se(v) |
| vps_cp_inv_off_plus_off[ viewIdx ][ j ] | se(v) |
| } |  |
| } |  |
| … | u(1) |
| } |  |

cp_present_flag[ viewIdx ] equal to 1 indicates that vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ], with j equal to VpsViewIdx[ layerId ], may be present for the layer with ViewIdx equal to viewIdx. cp_present_flag[ viewIdx  ] equal to 0 indicates that vps_cp_scale[ viewIdx ][ j ], vps_cp_off[ viewIdx ][ j ], vps_cp_inv_scale_plus_scale[ viewIdx ][ j ], and vps_cp_inv_off_plus_off[ viewIdx ][ j ], with j equal to VpsViewIdx[ layerId ], are not present. When VpsDepthFlag[ layerId ] is equal to 1 and depth_refinement_flag[ layerId ] or view_synthesis_pred_flag[ layerId ] is equal to 1, the value of cp_present_flag[ VpsViewIdx[ layerId ]  ] shall be equal to 1. When not present, the value of cp_present_flag[ VpsViewIdx[ layerId ]   ] is inferred to be equal to 0.

The camera parameters of the texture and depth layers in the same view shall be the same as the camera parameters of the texture layer when both layers exist. If only the depth layer exists in a view, the camera parameter of the depth layer is used for the view where the depth layer belongs.