| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0128 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0047 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On poc reset for long-term reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hahyun Lee Jung Won Kang Jinho Lee Jin Soo Choi | Tel: Email: | 82-42-860-6138 hanilee@etri.re.kr jungwon@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

When poc_reset_flag is set equal to 1, the picOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture is decremented by PicOrderCntVal of current picture and then PicOrderCntVal of current picture is set equal to 0. It is asserted that, in the decoding process for reference picture set, the picture order count values, PocLtCurr and PocLtFoll, are incorrectly derived when poc_reset_flag is set equal to 1. This contribution proposes two alternative solutions to derive the correct PocLtCurr and PocLtFoll values for deriving the RPS when poc_reset_flag is equal to 1.

# Problem Statement

At the 14th JCT-VC meeting, poc_reset_flag has been adopted in MV-HEVC/SHVC draft text in order to ensure that the POC values of all pictures in the same access unit are the same when there are IRAP pictures and non-IRAP pictures in the same assess unit.

According to the current MV-HEVC/SHVC draft text, when poc_reset_flag is equal to 1, the following steps apply in the ordered listed:

- The PicOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture is decremented by PicOrderCntVal

- PicOrderCntVal is set equal to 0

MV-HEVC/SHVC draft text defines five lists of picture order count values for deriving the RPS in F.8.3.2. These five lists are PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll with NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll number of respectively. The five lists and the five variables are derived as follows:

- If the current picture is an IDR picture, PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll are all set to be empty, and NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll are all set equal to 0.

- Otherwise, the following applies:

for( i = 0, j = 0, k = 0; i < NumNegativePics[ CurrRpsIdx ] ; i++ )
	if( UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
		PocStCurrBefore[ j++ ] = PicOrderCntVal + DeltaPocS0[ CurrRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ CurrRpsIdx ][ i ]
NumPocStCurrBefore = j

for( i = 0, j = 0; i < NumPositivePics[ CurrRpsIdx ]; i++ )
	if( UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		PocStCurrAfter[ j++ ] = PicOrderCntVal + DeltaPocS1[ CurrRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ CurrRpsIdx ][ i ]
NumPocStCurrAfter = j
NumPocStFoll = k		
for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {
	pocLt = PocLsbLt[ i ]
	if( delta_poc_msb_present_flag[ i ] )
		pocLt  +=  PicOrderCntVal − 	DeltaPocMsbCycleLt[ i ] * MaxPicOrderCntLsb −
				PicOrderCntVal & ( MaxPicOrderCntLsb − 1 )
	if( UsedByCurrPicLt[ i ] ) {
		PocLtCurr[ j ] = pocLt
		CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]
	} else {
		PocLtFoll[ k ] = pocLt
		FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]
	}
}
NumPocLtCurr = j
NumPocLtFoll = k

Where PicOrderCntVal is the picture order count of the current picture as specified in subclause F.8.3.1

Figure 1 shows example when poc_reset_flag is set equal to 1, the PicOrderCntVal of each reference picture that is in the DPB and belongs to the same layer as the current picture is decremented by PicOrderCntVal of current picture and then the PicOrderCntVal of current picture is set equal to 0. Table 1 shows the long-term reference picture RPS signaled in the current picture. For simplicity, the only one layer is described.

- The POC of pictures when poc_reset_flag is set equal to 0

- The POC of pictures when poc_reset_flag is set equal to 1

Fig.1. Example of POC for pictures according to the poc_reset_flag

Table1.  Long-term reference pictures RPS signaled in the current picture

|  | i=0 | i=1 | i=2 |
| --- | --- | --- | --- |
| poc_lsb_lt[i] | 20 | 10 | 20 |
| used_by_curr_pic_lt_flag[i] | 0 | 1 | 0 |
| delta_poc_msb_present_flag[i] | 1 | 1 | 1 |
| delta_poc_msb_cycle_lt[i] | 1 | 4 | 3 |

According to the current draft text, when poc_reset_flag is equal to 1, PicOrderCntVal of long-term reference picture is derived incorrectly as below.

For examples: 1st long-term reference picture

pocLt = PocLsbLt[1] = 10

if( delta_poc_msb_present_flag[ i ] )
		pocLt  +=  PicOrderCntVal − DeltaPocMsbCycleLt[ i ] * MaxPicOrderCntLsb −
				PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) = 10 + 0 – 5*32 – 0 & (32-1) = -150

PocLtCurr[ 1 ] = pocLt = -150

As the decremented PicOrderCntVal of 1st long-term reference is equal to -161 shown in Figure 1 (b), there is a mismatch between derived PocLtCurr[1] and decremented PicOrderCntVal values.

In addition, the current MV-HEVC/SHVC draft text describes that long-term reference pictures are identified either by their PicOrderCntVal values or their slice_pic_order_cnt_lsb values. When poc_reset_flag is equal to 0, since slice_pic_order_cnt_lsb value and PicOrderCntVal modulo MaxPicOrderCntLsb is the same, there are no problems, However, when the poc_reset_flag is equal to 1, there could be mismatch between slice_pic_order_cnt_lsb and PicOrderCntVal modulo MaxPicOrderCntLsb as shown in Figure 1 (b). If the slice_pic_order_cnt_lsb is used to identify the long-term reference picture, there is no problem but their slice_pic_order_cnt_lsb values have to be stored. However, in general, instead of using slice_pic_order_cnt_lsb of reference picture, their PicOrderCntVal modulo MaxPicOrderCntLsb value is used. The SHM and HTM reference software also use PicOrderCntVal modulo MaxPicOrderCntLsb value to identify long-term reference pictures. According to the reference software, for example, in the 1st long-term reference picture shown in figure 1 (b), since PocLtCurr(=10) derived from RPS signaled in current picture is different from PicOrderCntVal modulo MaxPicOrderCntLsb(=31) of long-term reference picture, the intended long-term reference picture can’t be identified.

Therefore this contribution proposes two alternative solutions to calculate the correct PocLtCurr and PocLtFoll values for deriving the RPS when poc_reset_flag is equal to 1.

# Proposal

## Alternative 1

It is proposed to change the derivation process for reference picture set as shown in red. Also, it is asserted that the slice_pic_order_cnt_lsb, which is a signalled value in the slice segment header of each picture, of the reference pictures in the DPB has to be compared with PocLtCurr[i] or PocLtFoll[i] to identify the long-term reference picture. Therefore, the SHM and HTM reference software has to be fixed.

F. 8.3.2 Decoding process for reference picture set

for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {
	pocLt = PocLsbLt[ i ]
	if( delta_poc_msb_present_flag[ i ] )
		pocLt  +=  PicOrderCntVal − 	DeltaPocMsbCycleLt[ i ] * MaxPicOrderCntLsb −
				PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) slice_pic_order_cnt_lsb
	if( UsedByCurrPicLt[ i ] ) {
		PocLtCurr[ j ] = pocLt
		CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]
	} else {
		PocLtFoll[ k ] = pocLt
		FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]
	}
}
NumPocLtCurr = j
NumPocLtFoll = k

Where PicOrderCntVal is the picture order count of the current picture as specified in subclause F.8.3.1

For examples: 1st long-term reference picture as shown in Figure 1 (b)

pocLt = PocLsbLt[1] = 10

if( delta_poc_msb_present_flag[ i ] )
		pocLt  +=  PicOrderCntVal − DeltaPocMsbCycleLt[ i ] * MaxPicOrderCntLsb −
				slice_pic_order_cnt_lsb = 10 + 0 – 5*32 – 11 = -161

## Alternative 2

It is proposed to change derivation process for reference picture set in red. It is asserted that decremented PicOrderCntVal modulo MaxPicOrderCntLsb of the reference pictures in the DPB has to be compared with PocLtCurr[i] or PocLtFoll[i] to identify the long-term reference picture.

F. 8.3.2 Decoding process for reference picture set

Short-term reference pictures are identified by their PicOrderCntVal values. Long-term reference pictures are identified either by their PicOrderCntVal values or their slice_pic_order_cnt_lsb PicOrderCntVal & (MaxPicOrderCntLsb-1) values.

Five lists of picture order count values are constructed to derive the RPS. These five lists are PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll, with NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll number of elements, respectively. The five lists and the five variables are derived as follows:

- If the current picture is an IDR picture, PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll are all set to be empty, and NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll are all set equal to 0.

- Otherwise, the following applies:

for( i = 0, j = 0, k = 0; i < NumNegativePics[ CurrRpsIdx ] ; i++ )
	if( UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
	   PocStCurrBefore[ j++ ] = PicOrderCntVal + DeltaPocS0[ CurrRpsIdx ][ i ]
	else
       PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ CurrRpsIdx ][ i ]
    NumPocStCurrBefore = j
for( i = 0, j = 0; i < NumPositivePics[ CurrRpsIdx ]; i++ )
	if( UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		PocStCurrAfter[ j++ ] = PicOrderCntVal + DeltaPocS1[ CurrRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ CurrRpsIdx ][ i ]
NumPocStCurrAfter = j
NumPocStFoll = k

if ( poc_reset_flag ) {

for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {

delta_poc_lsb = PocLstLt[i] – slice_pic_order_cnt_lsb

pocLt = delta_poc_lsb & (MaxPicOrderCntLsb-1)

if( delta_poc_msb_present_flag[ i ] )
	if (delta_poc_lsb < 0)

pocLt += -( DeltaPocMsbCycleLt[ i ]+1)*MaxPicOrderCntLsb

else

pocLt += -( DeltaPocMsbCycleLt[ i ])*MaxPicOrderCntLsb

if( UsedByCurrPicLt[ i ] ) {

PocLtCurr[ j ] = pocLt
	  CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]

} else {
	PocLtFoll[ j ] = pocLt
		FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]

}

}

} else {

for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {

pocLt = PocLsbLt[i]

if( delta_poc_msb_present_flag[ i ] )
      pocLt += PicOrderCntVal – DeltaPocMsbCycleLt[i]*MaxPicOrderCntLsb –

PicOrderCntVal&(MaxPicOrderCntLsb-1)

if( UsedByCurrPicLt[ i ] ) {

PocLtCurr[ j ] = pocLt
CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]

} else {
	PocLtFoll[ j ] = pocLt
	FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]

}

}

}	
NumPocLtCurr = j
NumPocLtFoll = k

For examples: 1st long-term reference picture as shown in Figure 1 (b)

delta_poc_lsb = PocLsbLt[1] – slice_pic_order_cnt_lsb = 10 -11 = -1

pocLt = delta_poc_lsb & (MaxPicOrderCntLsb -1 ) = -1 &31 = 31 (=decremented PicOrderCntVal modulo MaxPicOrderCntLsb)

if( delta_poc_msb_present_flag[ i ] ==1)

if (delta_poc_lsb (-1) < 0 )
		pocLt  += - (DeltaPocMsbCycleLt[ 1 ]+1)*MaxPicOrderCntLsb = 31 – (5+1)*32 = -161 (=decremented PicOrederCntVal)

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).