| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0135r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0048r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Carriage of auxiliary pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The carriage methods of auxiliary pictures are studied and more specific syntax and constraints are proposed. First, we propose an indication method of auxiliary pictures with new NAL unit types, and show the pros and cons compared to the method specifying a new scalability dimension (proposed by Nokia in JCTVC-O0041). We suggest adopting one of them as a starting point and more specific syntax and constraints on top of that. The items of this contribution are listed below.

- Carriage method of auxiliary pictures

  - With new NAL unit type (proposed): Option-1

  - With new scalability dimension (Nokia, JCTVC-O0041): Option-2

- Indication of auxiliary layers in VPS

  - Indication of used auxiliary pictures for each layer : for Option-1

  - Grouped signalling of layer dependencies : for Option-2

- Constraints for Options 1&2

- Modification of default_one_target_output_layer_flag : for Option-2

# Introduction

In this proposal, two methods for carriage of auxiliary pictures are discussed. The first method is to indicate auxiliary NAL units with new NAL unit types. A primary picture and the associated auxiliary pictures shall have the same nuh_layer_id. The picture types of auxiliary pictures (IDR, CRA, BLA, TSA, STSA, or TRAIL) are inferred to be equal to the picture type of the associated primary picture. The other method is to allocate a layer for each auxiliary picture, which is basically identical to Nokia’s proposal (JCTVC-O0041). Auxiliary pictures have different nuh_layer_id values than the associated primary picture. On top of JCTVC-O0041, more efficient signalling of layer dependencies and additional constraints are proposed.

The intention of this contribution compares the pros and the cons of two approaches, and proposes specific designs for each method.

# Auxiliary picture indication methods

In this section, two alternative methods to indicate auxiliary pictures are introduced and compared. We suggest adopting one of them and make more details.

## Indication of auxiliary pictures with NAL unit type (Option-1)

In this method, the auxiliary pictures (depth map, alpha channel, chroma enhancement picture) are indicated by new NAL unit types in HEVC-extension.

A new NAL unit type can be assigned to indicate an auxiliary NAL unit typ. AUX_NUT is the Name of nual_type_type for auxiliary pictures. It is proposed to specify auxiliary picture types for alpha planes, depth pictures, and chroma enhancement pictures. The proposed syntax aux_id specifies the type of auxiliary picture, which is signalled in a slice header. Hence, in this approach, a layer shall contain a primary picture and may or may not contain auxiliary pictures.

Alternatively, for easy access of auxiliary picture types by systems, multiple NAL unit types can be assigned for depth map, alpha plane, and enhanced chroma pictures. Even though more NAL unit types are burned, each auxiliary picture type is parsed by only NAL unit headers.

### Specification text for allocating one NAL unit type

A nal_unit_type, namely AUX_NUT, is assigned to indicate auxiliary pictures. The types of auxiliary pictures are signalled in slice headers of VCL NAL units.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| … | … | … | … |
| 39 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 41 | AUX_NUT | Coded slice segment of an auxiliary coded picture slice_segment_layer_rbsp( ) | non-VCL |
| 42..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if(nal_unit_type == AUX_NUT) |  |
| aux_id | u(8) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |

aux_id specifies the type of auxiliary pictures. aux_id equal to 0 specifies that the current slice is the coded slice of a depth map picture. aux_id equal to 1 specifies that the current slice is the coded slice of a alpha map picture. aux_id equal to 2 specifies that the current slice is the coded slice of a chroma enhancement picture. aux_id shall be in the range of 0 to 2, inclusive, for bitstreams conforming to this version of this Specification. Although the value of aux_id shall be in the range of 0 to 4, inclusive, in this version of this Specification, decoders shall allow values of aux_id in the range of 0 to 255, inclusive.

### Specification text for allocating multiple NAL unit types

Multiple NAL unit types are allocated for indicating various auxiliary picture types. For example, DEPTH_NUT for depth map, ALPHA_NUT for alpha plane, and CHROMA_EXT_NUT for enhanced chroma pictures can be assigned. The variable AuxId, specifying the auxiliary type of pictures, can be inferred from nal_unit_type.

Table 7-2 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| … | … | … | … |
| 39 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 41 | DEPTH_NUT | Coded slice segment of an depth map picture slice_segment_layer_rbsp( ) | non-VCL |
| 42 | ALPHA_NUT | Coded slice segment of an alpha map picture slice_segment_layer_rbsp( ) | non-VCL |
| 43 | CHROMA_EXT_NUT | Coded slice segment of an enhanced chroma picture slice_segment_layer_rbsp( ) | non-VCL |
| 44..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

if( nal_unit_type > 40 && nal_unit_type < 44)
AuxId = nal_unit_type – 40

else
AuxId = 0

## Indication of auxiliary pictures with a scalability dimension (Option-2)

Instead of assigning a new NAL unit type, auxiliary pictures can be identified by a new scalability dimension. Then, one layer carries one type of auxiliary pictures. The AuxId as a scalability dimension can be mapped from nuh_layer_id. This method has been proposed by JCTVC-O0041 (Nokia) and its previous versions.

Table Error: Reference source not found-3 – Mapping of ScalabiltyId to scalability dimensions

| scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| … |  |  |
| 3 | Auxiliary | AuxId |
| … |  |  |

## Comparison of two alternative methods

The pros and the cons of two method are compared in this section. Adopting one of them based on this comparison is suggested by needs of auxiliary pictures. Our suggestion is the first option (NAL unit type indication), but both are applicable to carry auxiliary pictures.

- On option-1 : NAL unit type

  - Pros: 1) From a nuh_layer_id without mapping to scalability dimesions, the corresponding auxiliary pictures of a primary picture can be identified.

  - Pros: 2) The layer dependendcy information is more efficiently represented by signalling only dependencies between primary pictures.

  - Pros: 3) Auxiliary pictures with the corresponding primary picture in DPB are synchronously handled.

  - Pros: 4) Many NAL unit types are reserved, while nuh_layer_id (6 bits) is not enough to represent multiple scalability dimensions.

  - Pros: 5) A single layer bitstream with auxiliary pictures is available.

- On option-2 : Scalability dimension (same as decribed in JCTVC-O0041)

  - Pros: 1) Easy bitstream extraction is possible with nuh_layer_id.

  - Pros: 2) The presence of auxiliary pictures are indicated in VPS.

  - Pros: 3) No nal unit type is burned.

  - Pros: 4) Decoders can choose the CPB HRD parameters with or without auxiliary pictures.

# Indication of auxiliary picture types for each layer in VPS (for options-1)

If option-1 or option-2 is adopted, the following syntaxes are proposed. The auxiliary picture types used for each layer are indicated by aux_mask_flag[ i ] and aux_picture_type_present_flag[ i ][ j ]. This indiciation in VPS is beneficial for bitstream manipulation and session negotiation.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_aux_picture_use_flag | u(1) |
| if(vps_aux_picture_use_flag) { |  |
| for( i = 0, NumAuxTypes = 0; i < 16; i++ ) { |  |
| aux_mask_flag[ i ] | u(1) |
| NumAuxTypes += aux_mask_flag[ i ] |  |
| } |  |
| if( NumAuxTypes > 0 ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| for( j = 0; j < NumAuxTypes; j++ ) |  |
| aux_picture_type_present_flag[ i ][ j ] | u(1) |
| } |  |
| … |  |
| } |  |

vps_aux_picture_use_flag equal to 1 specifies that an auxiliary picture is present in a bitstream, and aux_mask_flag[i] is present in VPS. vps_aux_picture_use_flag equal to 0 specifies that an auxiliary picture is not present in a bitstream, and aux_mask_flag is inferred equal to 0.

aux_mask_flag [ i ] equal to 1 indicates that auxiliary pictures with the i-th auxiliay picture type are present in a bitstream. aux_mask_flag [ i ] equal to 0 indicates that auxiliary pictures with the i-th auxialiary picture type are not present in a bitstream.

aux_picture_type_present_flag[i][j] equal to 1 specifies the ith layer have an auxiliary picture with the jth present auxiliary type. aux_picture_type_present_flag[i][j] equal to 0 specifies the ith layer do not have an auxiliary picture with the jth present auxiliary type.

# Grouped signalling of layer dependencies in VPS (for option-2)

If option-2 is adopted, the following syntaxes are proposed. The auxiliary picture types used for the CVS are identified with aux_mask_flag[ i ]. A layer of primary pictures and layers of auxiliary pictures associated the primary pictures are grouped into a group of layer (GOL). Layer dependencies are indicated for each GOL. All layer belonging to a GOL have the same layer dependency. This GOL representation is more efficient than the layer-wise representation. vps_aux_picture_grouped_flag can decide whether a primary picture layer and associated auxiliary picture layers are grouped or they are independently signaled in VPS and managed in DPB.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_aux_picture_use_flag | u(1) |
| if(vps_aux_picture_use_flag) { |  |
| for( i = 0, NumAuxTypes = 0; i < 16; i++ ) { |  |
| aux_mask_flag[ i ] | u(1) |
| NumAuxTypes += aux_mask_flag[ i ] |  |
| vps_aux_picture_grouped_flag | u(1) |
| } |  |
| NumGroupLayers = 1 |  |
| if(vps_aux_picture_grouped_flag) |  |
| NumGroupLayers += NumAuxTypes |  |
| for( i = 1; i <= vps_max_layers_minus1; i+= NumGroupLayers ) { |  |
| for( j = 0; j < i; j+=NumGroupLayers ) { |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i+= NumGroupLayers ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i+= NumGroupLayers ) |  |
| for( j = 0; j < i; j+=NumGroupLayers ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| … |  |
| } |  |

vps_aux_picture_use_flag equal to 1 specifies that an auxiliary picture is present in a bitstream, and aux_mask_flag[i] is present in VPS. vps_aux_picture_use_flag equal to 0 specifiest hat an auxiliary picture is not present in a bitstream, and aux_mask_flag is inferred equal to 0.

aux_mask_flag [ i ] equal to 1 indicates that auxiliary pictures with the i-th auxiliay picture type are present in a bitstream. aux_mask_flag [ i ] equal to 0 indicates that auxiliary pictures with the i-th auxialiary picture type are not present in a bitstream.

vps_aux_picture_grouped_flag equal to 1 specifies that the nuh_layer_id of the i-th auxiliary pictures associated with a primary picture with nuh_layer_id equal to k is equal to (k+i). The auxiliary pictures have the same layer dependencies as the corresponding primary pictures. vps_aux_picture_grouped_flag equal to 0 specifies that the above restriction may or may not apply. If not present, vps_aux_picture_grouped_flag is inferred to be equal to 0.

The variables NumDirectRefLayers[ i ],and RefLayerId[ i ][ j ]SamplePredEnabledFlag[ i ][ j ], MotionPredEnabledFlag[ i ][ j ] and DirectRefLayerIdx[ i ][ j ]are derived as follows:

for( i = 0; i <= vps_max_layers_minus1; i += NumGroupLayers ) {
for( k = 0; k < NumGroupLayers; k++ ) {
  iNuhLId = layer_id_in_nuh[ i ] + k
  NumDirectRefLayers[ iNuhLId ] = 0
  for( j = 0; j < i; j++ )
    if( direct_dependency_flag[ i ][ j ] ){
      RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ] + k
	SamplePredEnabledFlag[ iNuhLId ][ j ] = (( direct_dependency_type[ i ][ j ] + 1 ) & 1 )
	MotionPredEnabledFlag[ iNuhLId ][ j ] = ((( direct_dependency_type[ i ][ j ] + 1 ) & 2 ) >> 1 )
	DirectRefLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ]+k] = NumDirectRefLayers[ iNuhLId ] – 1	
    }
}

# Constraints for options 1&2

If option-1 is adopted or option-2 is done, we assert that the following constraints for the adopted method shall be agreed.

For option-1,

- A layer consists of a primary picture (VCL NAL unit) and its multiple auxiliary pictures (non-VCL NAL units). Thus, an auxiliary picture shall have only one corresponding primary picture. A primary picture can have multiple auxiliary pictures with different auxiliary picture type, e.g. depth map, alpha plane, and additional chroma data.

- A layer shall have at least a primary picture. A layer containing only auxiliary pictures cannot be present.

- NAL units of auxiliary pictures have the same nul_layer_id and TemporalID (temporal_id) as NAL units of the associated primary pictures.

- Au auxiliary picture cannot be used for inter-layer prediction of primary pictures and other auxiliary picture with different AuxIds as a reference picture.

For option-2,

- A layer of auxiliary pictures shall have a corresponding layer of the primary pictures. A primary picture layer can be associated with multiple auxiliary picture layers with different auxiliary picture types.

- A layer of auxiliary pictures cannot be a target output layer, when the corresponding layer of the primary pictures is not a target output layer.

- Within a layer, a value of AuxId shall not be changed in a CVS.

- NAL units of auxiliary picture layers have the same values of dimension_id[ i ][ j ] (except for AuxId) and TemporalID (temporal_id) as NAL units of the associated primary picture layer.

- An auxiliary picture cannot be used for inter-layer prediction of primary pictures and other auxiliary picture with different AuxIds as a reference picture.

# Modifications of default_one_target_output_layer_flag semantics (for Option-2)

If the option-2 is adopted, assuming that a layer with auxiliary pictures cannot be a target output layer when the layer with the primary pictures associated with the auxiliary pictures is not a target output layer, we assert that the semantics of the syntax element default_one_target_output_layer_flag shall be modified as follows:

default_one_target_output_layer_flag equal to 1 specifies that only the highest layer in each of the default output layer sets is a target output layer. default_one_target_output_layer_flag equal to 0 specifies that all layers in each of the default output layer sets are target output layers.

->

default_one_target_output_layer_flag equal to 1 specifies that only the highest layer with AuxId equal to 0 (containing primary pictures, but not auxiliary pictures) in each of the default output layer sets is a target output layer. default_one_target_output_layer_flag equal to 0 specifies that all layers in each of the default output layer sets are target output layers.

Or alternatively,

default_one_target_output_layer_flag equal to 1 specifies that the highest layers with primary pictures and its associated auxiliary pictures in each of the default output layer sets are target output layers. default_one_target_output_layer_flag equal to 0 specifies that all layers in each of the default output layer sets are target output layers.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).