| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0136r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0049r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Signalling decoded picture buffer size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Three alternative syntaxes to indicate the maximum decoded buffer sizes are proposed. : 1) DPB signalling for each operation point and for each representation format. 2) DPB signaling for each operation point and additionally for each layer. 3) Explicitly selecting the level of DPB signaling (hybrid of option 1&2). We assert that those syntaxes are helpful for session negotiation and decoder set-up.

# Introduction

Signalling the maximum sizes of decoded picture buffers (DPBs) in VPS is proposed. Basically, the DPB size (sps_max_dec_pic_buffering_minus1[ i ]) and the related syntax elements (sps_max_num_reorder_pics[ i ] & sps_max_latency_increase_plus1[ i ]) are signaled in SPS for each layer. The maximum capacities of all layers identical to those syntax elements are signaled in VPS. We assert that signaling maximum DPB sizes for each operation point (layer set + sub-layer) in VPS extension is helpful for session negotiation and decoder set-up. Moreover, in order to assist the layer-wise management of DPBs and the exact buffer allocation according to representation formats (width, height, bit-depth, and color format), additional syntaxes indicating maximum DPB sizes for each layer or for each representation format can be employed.

The proposal has three options: 1) DPB signalling for each operation point and for each representation format. 2) DPB signaling for each operation point and additionally for each layer. 3) Explicitly selecting the level of DPB signaling (hybrid of option 1&2). One of three options can be adopted, but our preference is the third option.

Note that signaling the maximum reorder and latency information for each layer or each layer set is not desirable, because the ordering shall be aligned across all layers from the current AU definition.

# Option-1: DPB signalling for each operation point and representation format

In option-1, maximum DBP sizes for each operation points (layer set + temporal sub-layer) are signalled. To assist the exact allocation of DPB according to representation formats, maximum DPB size for each representation format can be optionally signalled.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_number_layer_sets_minus1 | u(10) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) { |  |
| for( j = vps_max_sub_layers_minus1; j >= 0; j-- ) { |  |
| max_dpb_size_for_layer_set_minus1 [ i ][ j ] | u(10) |
| max_dpb_size_for_rep_format_idx_present_flag[ i ][ j ] | u(1) |
| if(max_dpb_size_for_ rep_format_idx_present_flag[ i ][ j ]) |  |
| for( k = 0; k < vps_num_rep_formats_minus1; k++ ) |  |
| max_dpb_size_for_rep_format_idx_minus1[ i ][ j ][ k ] | u(7) |
| } |  |
| } |  |
| … |  |
| } |  |

max_dpb_size_for_ layer_set_minus1[ i ][ j ] plus 1 specifies the maximum required size of the decoded picture buffer of the ith layer set for the CVS in units of picture storage buffers when HighestTid is equal to j. The value of max_dpb_size_for_ layer_set_minus1 [ i ][ j ] shall be in the range of 0 to MaxDpbSize − 1, inclusive. When j is greater than 0, max_dpb_size_for_ layer_set_minus1 [ i ][ j ] shall be greater than or equal to max_dpb_size_for_ layer_set_minus1 [ i ][ j - 1 ].

max_dpb_size_for_rep_format_idx_present_flag[ i ][ j ] equal to 1 indicates that max_dpb_size_for_ rep_format_idx _minus1[ i ][ j ][ k ] is present. max_dpb_size_for_rep_format_idx_present_flag[ i ][ j ] equal to 0 indicates that max_dpb_size_for_ rep_format_idx _minus1[ i ][ j ][ k ] is not present.

max_dpb_size_for_ rep_format_idx_minus1[ i ][ j ][ k ] plus 1 specifies the maximum required size of the decoded picture buffer of the ith layer set associated with the kth representation format for the CVS in units of picture storage buffers when HighestTid is equal to j. The value of max_dpb_size_for_rep_format_idx_minus1[ i ][ j ][ k ] shall be in the range of 0 to MaxDpbSize − 1, inclusive. When j is greater than 0, max_dpb_size_for_rep_format_idx_minus1[ i ][ j ][ k ] shall be greater than or equal to max_dpb_size_for_rep_format_idx_minus1[ i ][ j - 1 ][ k ].

# Option-2: DPB signalling for each operation point and for each layer of the operation point.

In option-2, maximum DBP sizes for each operation points (layer set + temporal sub-layer) are signalled. If the DPB operates in layer-wise, maximum DPB size for each layer of each operation point can be signalled additionally.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_number_layer_sets_minus1 | u(10) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) { |  |
| for( j = vps_max_sub_layers_minus1; j >= 0; j-- ) { |  |
| max_dpb_size_for_ layer_set_minus1 [ i ][ j ] | u(10) |
| max_dpb_size_for_layer_present _flag[ i ][ j ] | u(1) |
| if(max_dpb_size_for_layer_present _flag[ i ][ j ]) |  |
| for( k = 0; k <= vps_max_layer_id; k++ ) |  |
| if ( layer_id_included_flag[ i ][ k ] ) |  |
| max_dpb_size_for_ layer_minus1 [ i ][ j ][ k ] | u(7) |
| } |  |
| } |  |
| … |  |
| } |  |

max_dpb_size_for_ layer_set_minus1[ i ][ j ] plus 1 specifies the maximum required size of the decoded picture buffer of the ith layer set for the CVS in units of picture storage buffers when HighestTid is equal to j. The value of max_dpb_size_for_ layer_set_minus1 [ i ][ j ] shall be in the range of 0 to MaxDpbSize − 1, inclusive. When j is greater than 0, max_dpb_size_for_ layer_set_minus1 [ i ][ j ] shall be greater than or equal to max_dpb_size_for_ layer_set_minus1 [ i ][ j - 1 ].

max_dpb_size_for_layer_present_flag[ i ][ j ] equal to 1 indicates that max_dpb_size_for_ layer_minus1[ i ][ j ][ k ] is present. max_dpb_size_for_layer_present_flag[ i ][ j ] equal to 0 indicates that max_dpb_size_for_layer _minus1[ i ][ j ][ k ] is not present.

max_dpb_size_for_layer_minus1[ i ][ j ][ k ] plus 1 specifies the maximum required size of the decoded picture buffer of the k-th layer associated with the i-th layer set for the CVS in units of picture storage buffers when HighestTid is equal to j. The value of max_dpb_size_for_layer_minus1[ i ][ j ][ k ] shall be in the range of 0 to MaxDpbSize − 1, inclusive. When j is greater than 0, max_dpb_size_for_layer _minus1[ i ][ j ][ k ] shall be greater than or equal to max_dpb_size_for_layer _minus1[ i ][ j - 1 ][ k ].

# Option-3: Selecting the type of DPB signalling.

In option-3, the level of DPB signalling can be selected for DPB design. A new syntax dpb_signal_type indicates the type of DPB signalling. By DPB behaving models, a desired DBP size information can be delivered.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_number_layer_sets_minus1 | u(10) |
| vps_dpb_signal_present_flag | u(1) |
| if( vps_dpb_signal_present_flag ) { |  |
| dpb_signal_type | u(2) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) { |  |
| for( j = vps_max_sub_layers_minus1; j >= 0; j-- ) { |  |
| max_dpb_size_for_ layer_set_minus1 [ i ][ j ] | u(10) |
| if(dpb_signal_type & 0x01) | u(2) |
| for( k = 0; k < vps_num_rep_formats_minus1; k++ ) |  |
| max_dpb_size_for_ rep_format_idx _minus1[ i ][ j ][ k ] | u(7) |
| if(dpb_signal_type & 0x02) { |  |
| for( k = 0; k <= vps_max_layer_id; k++ ) |  |
| if ( layer_id_included_flag[ i ][ k ] ) |  |
| max_dpb_size_for_ layer_minus1 [ i ][ j ][ k ] | u(7) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

vps_dpb_signal_present_flag equal to 1indicates that max DPB size for each layer set and sub-layer is signaled. vps_dpb_signal_present_flag equal to 0 indicates that max DPB size for each layer set and sub-layer is not signaled.

dpb_signal_type specifies whether max_dpb_size_for_ rep_format_idx _minus1[ i ][ j ][ k ] or max_dpb_size_for_ layer_minus1 [ i ][ j ][ k ] is signaled or not.

max_dpb_size_for_ layer_set_minus1[ i ][ j ] plus 1 specifies the maximum required size of the decoded picture buffer of the i-th layer set for the CVS in units of picture storage buffers when HighestTid is equal to j. The value of max_dpb_size_for_ layer_set_minus1 [ i ][ j ] shall be in the range of 0 to MaxDpbSize − 1, inclusive. When j is greater than 0, max_dpb_size_for_ layer_set_minus1 [ i ][ j ] shall be greater than or equal to max_dpb_size_for_ layer_set_minus1 [ i ][ j - 1 ].

max_dpb_size_for_ rep_format_idx_minus1[ i ][ j ][ k ] plus 1 specifies the maximum required size of the decoded picture buffer of the i-th layer set associated with the kth representation format for the CVS in units of picture storage buffers when HighestTid is equal to j. The value of max_dpb_size_for_rep_format_idx_minus1[ i ][ j ][ k ] shall be in the range of 0 to MaxDpbSize − 1, inclusive. When j is greater than 0, max_dpb_size_for_rep_format_idx_minus1[ i ][ j ][ k ] shall be greater than or equal to max_dpb_size_for_rep_format_idx_minus1[ i ][ j - 1 ][ k ].

max_dpb_size_for_layer_minus1[ i ][ j ][ k ] plus 1 specifies the maximum required size of the decoded picture buffer of the k-th layer associated with the i-th layer set for the CVS in units of picture storage buffers when HighestTid is equal to j. The value of max_dpb_size_for_layer_minus1[ i ][ j ][ k ] shall be in the range of 0 to MaxDpbSize − 1, inclusive. When j is greater than 0, max_dpb_size_for_layer _minus1[ i ][ j ][ k ] shall be greater than or equal to max_dpb_size_for_layer _minus1[ i ][ j - 1 ][ k ].

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).