| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0137 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0050 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Extended layer identifier |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Two alternative methods to represent more than 64 layers are proposed. The first one is to signal extra LSB bits of nuh_layer_id in non-VCL NAL units as well as VCL NAL units. The second one is a replacement of nuh_layer_id with ext_layer_id having more than 6bits, when nuh_layer_id is equal to its maximum value. The syntax ext_layer_id is signalled inside NAL unit payload.

# Introduction

During the last two meetings, the methods extending the maximum number of layers more than 64 have been discussed. The basic idea of the previous proposals (N0135/E0115 and N0355/E0092) is to signal the extra bits to be used as LSB bits of nuh_layer_id. In this meeting, we propose two alternative methods to represent more than 64 layers. The first one is the following-up proposal of the previous contributions with more details that specify how to signal extra LSB bits of nuh_layer_id in non-VCL NAL units as well as VCL NAL units. The second one is a replacement of nuh_layer_id with ext_layer_id having more than 6bits, when nuh_layer_id is equal to its maximum value. The syntax ext_layer_id is signalled inside NAL unit payload. Hence, ext_layer_id cannot be simply access by systems for bitstream extraction. One of two alternative solutions can be adopted, but our preference is the first option.

# Option-1: Signaling extra LSB bits of nuh_layer_id

In option-1, extra LSB bits of nuh_layer_id are signalled in slice headers, parameter set (SPS, PPS), and SEI messages. The flag layer_id_extension_flag indicates whether the extension mechanism of nuh_layer_id is used or not. If layer_id_extension_flag is equal to 1, num_layer_id_extension_bits_minus1, ext_max_layers_minus1, and ext_max_layer_id are signaled to represent the increased number of layers. In VCL and non-VCL NAL unit payloads, slice_ext_layer_id, sps_ext_layer_id, pps_ext_layer_id, and sei_ext_layer_id are signaled to indicate the extra LSB bits of nuh_layer_id.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| layer_id_extension_flag | u(1) |
| if( layer_id_extension_flag ) { |  |
| num_layer_id_extension_bits_minus1 | u(3) |
| ext_max_layers_minus1 | u(v) |
| ext_max_layer_id | u(v) |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(v) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(v) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| if( NumViews > 1 ) |  |
| view_id_len_minus1 | u(v) |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i < MaxNumLayers; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < MaxNumLayers-1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i < MaxNumLayers; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0) |  |
| vps_rep_format_idx[ i ] | u(4) |
| max_one_active_ref_layer_flag | u(1) |
| cross_layer_irap_aligned_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i < MaxNumLayers; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| single_layer_for_non_irap_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_vui_alignment_bit_equal_to_one | u(1) |
| vps_vui( ) |  |
| } |  |
| } |  |

layer_id_extension_flag equal to 1 indicates that the extra bits are assigned for layer identifiers to increase the maximum number of layers. vps_layer_id_extension_flag equal to 0 indicates that the layer identifier extension is not used. If not present, vps_layer_id_extension_flag is inferred to be equal to 0.

num_layer_id_extension_bits_minus1 plus1 specifies the number of extra bit to identify layers more than 64.

ext_max_layers_minus1 plus1 specifies the variable MaxNumLayers.

if(layer_id_extension_flag)
	MaxNumLayers = ext_max_layers_minus1 + 1
else
	MaxNumLayers = vps_max_layers_minus1 + 1

ext_max_layer_id specifies the maximum allowed value of the extended layer identifier of all NAL units in the CVS. If not present, ext_max_layer_id is inferred to be equal to vps_max_layer_id.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id > 0) { |  |
| sps_layer_id_extension_flag | u(1) |
| if( sps_layer_id_extension_flag ) |  |
| sps_ext_layer_id | u(8) |
| } |  |
| ... |  |
| } |  |

sps_layer_id_extension_flag equal to 1 indicates that the extra bits are assigned for layer identifiers to increase the maximum number of layers for sequence parameter set. sps_layer_id_extension_flag equal to 0 indicates that the layer identifier extension is not used for sequence parameter set. If not present, sps_layer_id_extension_flag is inferred to be equal to 0.

sps_ext_layer_id specifies the variable LayerId, when sps_layer_id_extension_flag is equal to 1. If not present, sps_ext_layer_id is inferred to be equal to 0.

if ( nuh_unit_type == SPS_NUT ) {
if ( sps_layer_id_extension_flag )
  LayerId = nuh_layer_id << (num_layer_id_extension_bits_minus1 + 1) + sps_ext_layer_id
else
  LayerId = nuh_layer_id

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id > 0) { |  |
| pps_layer_id_extension_flag | u(1) |
| if( pps_layer_id_extension_flag ) |  |
| pps_ext_layer_id | u(8) |
| } |  |
| ... |  |
| } |  |

pps_layer_id_extension_flag equal to 1 indicates that the extra bits are assigned for layer identifiers to increase the maximum number of layers for picture parameter set. pps_layer_id_extension_flag equal to 0 indicates that the layer identifier extension is not used for picture parameter set. If not present, pps_layer_id_extension_flag is inferred to be equal to 0.

pps_ext_layer_id specifies the variable LayerId, when pps_layer_id_extension_flag is equal to 1. If not present, pps_ext_layer_id is inferred to be equal to 0.

if ( nuh_unit_type == PPS_NUT ) {
if ( pps_layer_id_extension_flag )
  LayerId = nuh_layer_id << (num_layer_id_extension_bits_minus1 + 1) + pps_ext_layer_id
else
  LayerId = nuh_layer_id

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| if( nuh_layer_id > 0 && vps_layer_id_extension_flag) |  |
| slice_ext_layer_id | u(v) |
| … |  |
| } |  |

slice_ext_layer_id specifies the variable LayerId, when vps_layer_id_extension_flag is equal to 1. If not present, slice_ext_layer_id is inferred to be equal to 0.

if ( nuh_unit_type < 32 ) {
if ( vps_layer_id_extension_flag )
  LayerId = nuh_layer_id << (num_layer_id_extension_bits_minus1 + 1) + slice_ext_layer_id
else
  LayerId = nuh_layer_id

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( nuh_layer_id > 0) { |  |
| sei_layer_id_extension_flag | u(1) |
| if( sei_layer_id_extension_flag ) |  |
| sei_ext_layer_id | u(8) |
| } |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

sei_layer_id_extension_flag equal to 1 indicates that the extra bits are assigned for layer identifiers to increase the maximum number of layers for SEI message. sei_layer_id_extension_flag equal to 0 indicates that the layer identifier extension is not used for SEI message. If not present, sei_layer_id_extension_flag is inferred to be equal to 0.

sei_ext_layer_id specifies the variable LayerId, when sei_layer_id_extension_flag is equal to 1. If not present, sei_ext_layer_id is inferred to be equal to 0.

if ( nuh_unit_type == PREFIX_SEI_NUT || nuh_unit_type == SUFFIX_SEI_NUT ) {
if (sei_layer_id_extension_flag )
  LayerId = nuh_layer_id << (num_layer_id_extension_bits_minus1 + 1) + sei_ext_layer_id
else
  LayerId = nuh_layer_id

# Option-2: Replacement of nuh_layer_id with ext_layer_id

In option-2, nuh_layer_id (6bits) is replaced with ext_layer_id having more than 6bits, when layer_id_extension_flag in VPS is equal to 1. In VPS, ext_layer_id_len_minis1, ext_max_layers_minus1, and ext_max_layer_id are signaled to represent the increased number of layers. In VCL and non-VCL NAL unit payloads, slice_ext_layer_id, sps_ext_layer_id, pps_ext_layer_id, and sei_ext_layer_id are signaled to indicate the extended layer identiifer. If layer_id_extension_flag is equal to 1 and nuh_layer_id is equal to 63, LayerId is inferred to be equal to those syntax elements (slice_ext_layer_id, sps_ext_layer_id, pps_ext_layer_id, and sei_ext_layer_id).

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| layer_id_extension_flag | u(1) |
| if( layer_id_extension_flag ) { |  |
| ext_layer_id_len_minis1 | u(4) |
| ext_max_layers_minus1 | u(v) |
| ext_max_layer_id | u(v) |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(v) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(v) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| if( NumViews > 1 ) |  |
| view_id_len_minus1 | u(v) |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i < MaxNumLayers; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < MaxNumLayers-1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i < MaxNumLayers; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0) |  |
| vps_rep_format_idx[ i ] | u(4) |
| max_one_active_ref_layer_flag | u(1) |
| cross_layer_irap_aligned_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i < MaxNumLayers; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| single_layer_for_non_irap_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_vui_alignment_bit_equal_to_one | u(1) |
| vps_vui( ) |  |
| } |  |
| } |  |

vps_layer_id_extension_flag equal to 1 indicates that the extra bits are assigned for layer identifiers to increase the maximum number of layers. vps_layer_id_extension_flag equal to 0 indicates that the layer identifier extension is not used. If not present, vps_layer_id_extension_flag is inferred to be equal to 0.

ext_layer_id_len_minis1 plus1 specifies the number of bits to identify the extended layer identifiers.

ext_max_layers_minus1 plus1 specifies the variable MaxNumLayers.

if(layer_id_extension_flag)
	MaxNumLayers = ext_max_layers_minus1 + 1
else
	MaxNumLayers = vps_max_layers_minus1 + 1

ext_max_layer_id specifies the maximum allowed value of the extended layer identifier of all NAL units in the CVS. If not present, ext_max_layer_id is inferred to be equal to vps_max_layer_id.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id == 63 ) { |  |
| sps_layer_id_extension_flag | u(1) |
| if( sps_layer_id_extension_flag ) |  |
| sps_ext_layer_id | u(16) |
| } |  |
| ... |  |
| } |  |

sps_layer_id_extension_flag equal to 1 indicates that the extra bits are assigned for layer identifiers to increase the maximum number of layers for sequence parameter set. sps_layer_id_extension_flag equal to 0 indicates that the layer identifier extension is not used for sequence parameter set. If not present, sps_layer_id_extension_flag is inferred to be equal to 0.

sps_ext_layer_id specifies the variable LayerId, when sps_layer_id_extension_flag is equal to 1. If not present, sps_ext_layer_id is inferred to be equal to 0.

if ( nuh_unit_type == SPS_NUT ) {
if ( sps_layer_id_extension_flag )
  LayerId = sps_ext_layer_id
else
  LayerId = nuh_layer_id

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id == 63 ) { |  |
| pps_layer_id_extension_flag | u(1) |
| if( pps_layer_id_extension_flag ) |  |
| pps_ext_layer_id | u(16) |
| } |  |
| ... |  |
| } |  |

pps_layer_id_extension_flag equal to 1 indicates that the extra bits are assigned for layer identifiers to increase the maximum number of layers for picture parameter set. pps_layer_id_extension_flag equal to 0 indicates that the layer identifier extension is not used for picture parameter set. If not present, pps_layer_id_extension_flag is inferred to be equal to 0.

pps_ext_layer_id specifies the variable LayerId, when pps_layer_id_extension_flag is equal to 1. If not present, pps_ext_layer_id is inferred to be equal to 0.

if ( nuh_unit_type == PPS_NUT ) {
if ( pps_layer_id_extension_flag )
  LayerId = pps_ext_layer_id
else
  LayerId = nuh_layer_id

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| if( nuh_layer_id == 63 && vps_layer_id_extension_flag) |  |
| slice_ext_layer_id | u(v) |
| … |  |
| } |  |

slice_ext_layer_id specifies the variable LayerId, when vps_layer_id_extension_flag is equal to 1. If not present, slice_ext_layer_id is inferred to be equal to 0.

if ( nuh_unit_type < 32 ) {
if ( vps_layer_id_extension_flag )
  LayerId = slice_ext_layer_id
else
  LayerId = nuh_layer_id

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( nuh_layer_id == 63) { |  |
| sei_layer_id_extension_flag | u(1) |
| if( sei_layer_id_extension_flag ) |  |
| sei_ext_layer_id | u(16) |
| } |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

sei_layer_id_extension_flag equal to 1 indicates that the extra bits are assigned for layer identifiers to increase the maximum number of layers for SEI message. sei_layer_id_extension_flag equal to 0 indicates that the layer identifier extension is not used for SEI message. If not present, sei_layer_id_extension_flag is inferred to be equal to 0.

sei_ext_layer_id specifies the variable LayerId, when sei_layer_id_extension_flag is equal to 1. If not present, sei_ext_layer_id is inferred to be equal to 0.

if ( nuh_unit_type == PREFIX_SEI_NUT || nuh_unit_type == SUFFIX_SEI_NUT ) {
if (sei_layer_id_extension_flag )
  LayerId = sei_ext_layer_id
else
  LayerId = nuh_layer_id

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).