| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0138r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0051r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On inter-layer dependency signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Some clean-ups or improvements of syntax related to inter-layer RPS or other layer dependency indication are proposed.

- On inter-layer reference picture set

  - Removal of the inter-layer RPS

  - Or, improvements of the inter-layer RPS ( inter-layer DPB status instead of direct inter-layer references)

- On all_ref_layers_active_flag and max_one_active_ref_layer_flag

  - Signalling those syntax for each layer

  - Optional signalling of max_one_active_ref_layer_flag depending on the value of all_ref_layers_active_flag

- A new flag high_ref_layers_active_flag

  - Indication whether closer dependent layers are used for inter-layer prediction

# Introduction

Layer dependency information in a coded video sequence (CVS) is signalled in VPS with direct_dependency_flag[ i ][ j ]. Within directly dependent layers, the inter-layer references used for the current picture are indicated in slice headers, which are called inter-layer reference picture set (RPS). Also, all_ref_layers_active_flag and max_one_active_ref_layer_flag indicate the active inter-layer references in a CVS. All related syntaxes may have their own roles. However, the roles are somewhat overlapped with each other. In this contribution, we assert that some clean-ups or improvements of existing syntax elements related to inter-layer dependency are needed, as follows:

- On inter-layer reference picture set

  - Removal of the inter-layer RPS

  - Or, improvements of the inter-layer RPS ( inter-layer DPB status instead of direct inter-layer references)

- On all_ref_layers_active_flag and max_one_active_ref_layer_flag

  - Signalling those syntax elemetns for each layer

  - Optional signalling of max_one_active_ref_layer_flag depending on the value of all_ref_layers_active_flag

- A new flag high_ref_layers_active_flag

  - Indication whether closer dependent layers are used for inter-layer prediction

# On inter-layer reference picture set

The inter-layer RPS will be used for picture-level change of inter-layer references and picture-loss detection within an access unit. However, the actual inter-layer references can be chosen with the reference picture list modification process for each picture, if necessary. Moreover, the inter-layer references will not be frequently changed within a CVS. For the picture loss detection, the inter-layer RPS cannot completely detect the picture loss of lower layers, unlike the RPS (inter-picture). This is because the RPS indicates the reference pictures to be used for the current picture and the following pictures, while the inter-layer RPS indicates the reference pictures used for only the picture of the current layer. For example, some decoded pictures belonging to the same AU in a DPB are not indicated by the RPS, when those pictures are used for decoding of upper layers and not used for decoding of the current layer.

Therefore, we propose to remove the syntax elements related to the inter-layer RPS from slice headers due to the lack of usefulness, or align its design with the RPS by signaling the indexes of inter-layer references to be used for decoding of the current layer and the layers with nuh_layer_id greater than nuh_layer_id of the current layer.

When the current picture with nuh_layer_id equal to i is decoded, if the decoded picture of nuh_layer_id less than i, which is indicated by the inter-layer RPS of the current picture and included in TargetDecLayerIdList, is not present in a DPB, that picture turns out to be the lost picture.

This modification of the inter-layer RPS syntax is beneficial for early removal of reference pictures as well as the exact detection of picture loss. If a decoded picture with TemporalId equal to HighestTid is not included in the inter-layer RPS, that picture can be immediately marked as “unused for reference”.

## Specification text of the proposed inter-layer RPS

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

The variables NumDirectRefLayers[ i ], RefLayerId[ i ][ j ], SamplePredEnabledFlag[ i ][ j ], MotionPredEnabledFlag[ i ][ j ], DirectRefLayerIdx[ i ][ j ], IndirectDependencyFlag[ i ][ j ], NumDepLayers[ i ], DepLayerId[ i ][ j ], and DepLayerIdx[ i ][ j ] are derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	NumDirectRefLayers[ iNuhLId ] = 0
	NumDepLayers[ iNuhLId ] = 0
	for( j = 0; j < i; j++ ){
		IndirectDependencyFlag[ i ][ j ] = 0
		for( k = i+1; k <= vps_max_layers_minus1; k++ )
			if( direct_dependency_flag[ k ][ j ] )
				IndirectDependencyFlag[ i ][ j ] = 1
		if( direct_dependency_flag[ i ][ j ] ){
			RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]
			DepLayerId[iNuhLId][ NumDepLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]
			SamplePredEnabledFlag[ iNuhLId ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1 ) & 1 )
			MotionPredEnabledFlag[ iNuhLId ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1 ) & 2 ) >> 1 )
			DirectRefLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ] ] = NumDirectRefLayers[ iNuhLId ] – 1
			DepLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ] ] = NumDepLayers[ iNuhLId ] – 1
		}
		else if(IndirectDependencyFlag[ i ][ j ]) {
			DepLayerId[iNuhLId][ NumDepLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]
			DepLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ] ] = NumDepLayers[ iNuhLId ] – 1
		}
	}
}

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nuh_layer_id > 0 &&! all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_dependent_layers | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0, NumInterlayerRef = 0; i < num_dependent_layer; i++ ) { |  |
| inter_layer_dep_layer_idc [ i ] | u(v) |
| if( inter_layer_pred_enabled_flag && !all_ref_layers_active_flag ) { |  |
| used_by_curr_layer_flag[ i ] | u(1) |
| if( used_by_curr_layer_flag[ i ] ) { |  |
| RefPicLayerId[ NumInterlayerRef ] = DepLayerId [ nuh_layer_id ][ inter_layer_dep_layer_idc[ i ] ] |  |
| NumInterlayerPredRef ++ |  |
| } |  |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture.

num_dependent_layers specifies the number of pictures that may be used in decoding of the current picture and the pictures in the current access unit with nuh_layer_id greater than the current nuh_layer_id for inter-layer prediction. The length of num_dependent_layers syntax element is Ceil( Log2( NumDepLayers[ nuh_layer_id ] ) ) bits. The value of num_dependent_layers syntax shall be in the range of 0 to NumDepLayers[ nuh_layer_id ]− 1, inclusive.

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id = = 0 || NumDirectRefLayers[ nuh_layer_id ]  = =  0 )
	NumActiveRefLayerPics = 0
else if( all_ref_layers_active_flag )
	NumActiveRefLayerPics = NumDirectRefLayers[ nuh_layer_id ]
else if( !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = NumInterlayerPredRef

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

inter_layer_dep_layer_idc[ i ] specifies the variable, RefPicLayerIdInDPB[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture and the pictures with nuh_layer_id greater than nuh_layer_id of the current picture for inter-layer prediction. The length of the syntax element inter_layer_dep_layer_idc[ i ] is Ceil( Log2( nuh_layer_id  ) ) bits. The value of inter_layer_dep_layer_idc[ i ] shall be in the range of 0 to nuh_layer_id − 1, inclusive. When not present, the value of inter_layer_dep_layer_idc[ i ] is inferred to be equal to i.

used_by_curr_layer_flag [ i ] specifies the variable, RefPicLayerId [ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. When not present, the value of used_by_curr_layer_flag [ i ] is inferred to be equal to 0.

The variables RefPicLayerId[ i ] for all values of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, are derived as follows:

for( i = 0; i < num_dependent_layer; i++)
		if(used_by_curr_layer_flag[ i ]) {
			RefPicLayerId[ i ] = DepLayerId [ nuh_layer_id ][ inter_layer_dep_layer_idc[ i ] ]

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, either of the following two conditions shall be true:

–	The value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] is greater than TemporalId.

–	The values of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] and TemporalId are both equal to 0 and the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is an IRAP picture.

- F.8.1.2.1 Marking process for sub-layer non-reference pictures not needed for inter-layer prediction

Input to this process is:

–	a nuh_layer_id value latestDecLayerId

Output of this process is:

–	potentially updated marking as "unused for reference" for some decoded pictures

NOTE – This process marks pictures that are not needed for inter or inter-layer prediction as "unused for reference". When TemporalId is less than HighestTid, the current picture may be used for reference in inter prediction and this process is not invoked.

- The variables numTargetDecLayers, and latestDecIdx are derived as follows:

–	numTargetDecLayers is set equal to the number of entries in TargetDecLayerIdList.

–	latestDecIdx is set equal to the value of i for which TargetDecLayerIdList[ i ] is equal to latestDecLayerId.

- For i in the range of 0 to latestDecIdx, inclusive, the following applies for marking of pictures as "unused for reference":

–	Let currPic be the picture in the current access unit with nuh_layer_id equal to TargetDecLayerIdList[ i ].

–	When currPic is marked as "used for reference" and is a sub-layer non-reference picture, the following applies:

–	The variable currTid is set equal to the value of TemporalId of currPic.

–	The variable remainingInterLayerReferencesFlag is derived as specified in the following:

remainingInterLayerReferencesFlag = 0
		if ( currTid<=  ( max_tid_il_ref_pics_plus1[ LayerIdxInVps[ TargetDecLayerIdList[ i ] ] ] –1 ) )
			for( i =0; i <= latestDecIdx; i++ )
				for( j = 0; j < num_dependent_layers; j++ )
					if( TargetDecLayerIdList[ i ] ==  
                               RefLayerId[ nuh_layer_id ][ inter_layer_dep_layer_idc[ i ] ])

remainingInterLayerReferencesFlag = 1

–	When remainingInterLayerReferenceFlag is equal to 0, currPic is marked as "unused for reference".

# On all_ref_layers_active_flag and max_one_active_ref_layer_flag

In this section, improvements of the syntax elements all_ref_layers_active_flag and max_one_active_ref_layer_flag are proposed.

In the current HEVC-extension spec., all_ref_layers_active_flag equal to 1 indicates that the decoded pictures of all directly dependent layers of each layer are included in the inter-layer reference picture set of the picture of each layer, while max_one_active_ref_layer_flag equal to 1 specifies that at most one picture is used for inter-layer prediction for each picture in the CVS.

The flag max_one_active_ref_layer_flag is applicable for SHVC, while all_ref_layers_active_flag[ i ] seems to be mainly used for MV-HEVC. Both flags may reduce redundant syntax by avoiding fully signalling of the inter-layer RPS syntaxes in slice headers.

However, both flags are set for all layers in a CVS. In some cases, some layers may have one reference layer, while other layers have multiple reference layers. On the other hand, all reference layers are active for some layers, while only one reference is active in other layers. Especially, those cases could be present in a bitstream with multiple scalability dimensions

Therefore, we assert that signaling both syntax elements for each layer is more bit-efficient, especially to cope with bitstreams having multiple scalability dimensions.

Moreover, both flags are correlated. Both flags shall not be equal to 1 at the same time, except for the case that each layer has only one directly dependent layer. However, when all_ref_layers_active_flag is equal to 1 and each layer has only one directly dependent layer, signaling max_one_active_ref_layer_flag does not affect any decoding process. Therefore, if all_ref_layers_active_flag is equal to 1, max_one_active_ref_layer_flag can be inferred to be equal to 0 without signaling.

## Syntax and Semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++) { |  |
| all_ref_layers_active_flag[ i ] | u(1) |
| if( all_ref_layers_active_flag[ i ] ) |  |
| max_one_active_ref_layer_flag [ i ] | u(1) |
| } |  |
| … |  |
| } |  |

all_ref_layers_active_flag[ i ] equal to 1 specifies that for each picture of the ith layer, the reference layer pictures of all direct reference layers of the ith layer are present in the same access unit as the decoded picture and are included in the inter-layer reference picture set of the picture. all_ref_layers_active_flag equal to 0 specifies that the above restriction may or may not apply.

max_one_active_ref_layer_flag [ i ] equal to 1 specifies that at most one picture is used for inter-layer prediction for each picture of the ith layer in the CVS. max_one_active_ref_layer_flag equal to 0 specifies that more than one picture may be used for inter-layer prediction for each picture of the ith layer in the CVS. When not present, max_one_active_ref_layer_flag [ i ] is inferred to be equal to 0.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if(nuh_layer_id > 0 && !all_ref_layers_active_flag[LayerIdxInVps[ nuh_layer_id ] ] && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag[LayerIdxInVps[ nuh_layer_id ] ] ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

# A new flag high_ref_layers_active_flag

In this section, a new flag high_ref_layers_active_flag is proposed. high_ref_layers_active_flag equal to 1 indicates that inter_layer_pred_layer_idc[ i ] is inferred to equal to NumDirectRefLayers[ nuh_layer_id ] – 1 - i when i is in the range of 0 to NumActiveRefLayerPics – 1.

Using this flag is more bit-efficient than explicitly signaling all indexes. Since closer layers tend to have more correlation, far layers will not be used as reference pictures in most cases.

## Syntax and Semantics

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if(nuh_layer_id > 0 && !all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| high_ref_layers_active_flag | u(1) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] && ! high_ref_layers_active_flag ) { |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

high_ref_layers_active_flag equal to 1 specifies that inter_layer_pred_layer_idc[ i ] is inferred to equal to NumDirectRefLayers[ nuh_layer_id ] – 1 – I, when i is in the range of 0 to NumActiveRefLayerPics – 1. high_ref_layers_active_flag equal to 0 specifies that inter_layer_pred_layer_idc[ i ] is explicitly signaled.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).