| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0149 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0056 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: on splicing and layer-wise start-up of the decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The following items are proposed in order to reportedly make splicing of coded video sequences that start with non-aligned IRAP access units easier:

- A base-layer IRAP picture that initiates a layer-wise start-up process (i.e. has NoClrasOutputFlag equal to 1) causes marking of all pictures in the DPB as “unused for reference”.

- A slice_reserved_flag to indicate if a base-layer IDR picture initiates the layer-wise start-up process.

- A base-layer picture that initiates the layer-wise start-up process causes NoOutputOfPriorPicsFlags of all layers to be set equal to no_output_of_prior_pics_flag of the base layer picture.

# Introduction

Since the Vienna meeting MV-HEVC/SHVC has supported access units where some but not all layers contain IDR pictures as well as a so-called layer-wise start-up process (see JCTVC-N0066 for background), in which the decoding of layers is started when they contain IRAP pictures. The layer-wise start-up process is initiated at the start of a bitstream, by a BLA picture in the base layer, or when triggered by external means.

It is suggested in this contribution that it would be convenient to enable concatenation of two coded video sequences (CVSs) such that the latter one contains an IRAP picture in the base layer in its first access unit but not necessarily in all layers. The spliced CVS should trigger the layer-wise decoding start-up process. That is because the first access unit of the spliced CVS does not contain an IRAP picture in all its layers and hence some reference pictures for the non-IRAP pictures in that access unit are not available (in the concatenated bitstream) and cannot therefore be decoded. The splicer is therefore required to modify the first access unit of the spliced CVS such that it triggers the layer-wise start-up process. The splicer has to take care of two things:

- All pictures in the DPB prior to the splice point have to be marked as “unused for reference”.

- Cause NoClrasOutputFlag to be set equal to 1 at the splice point to initiate the layer-wise start-up process. The layer-wise start-up process is initiated when the spliced CVS starts with a BLA picture. However, the slice header of an IDR picture differs from that of a BLA picture, as it lacks the POC and RPS related syntax elements. It is asserted that the operation of a splicer would become easier if it did not need to rewrite the slice headers of base layer IDR pictures to become BLA pictures and instead indicate a base-layer IDR picture to initiate layer-wise start-up otherwise.

Related to an aspect of JCTVC-N0128, it was discussed whether no_output_of_prior_pics_flag and the NoOutputOfPriorPicsFlags variable can be different in different layers. It seemed agreed that the flag effect should be layer-specific. However, it is asserted in this contribution that when a spliced CVS contains an IRAP picture in the base layer but not in all layers, a splicer probably would like to set NoOutputOfPriorPicsFlags for all layers equal to no_output_of_prior_pics_flag of the base layer in the spliced CVS, i.e. the splicer should be able to control if pictures at any layer from the CVS are output before the splice point.

# Overview of the Proposal

The following items are proposed:

- A base-layer IRAP picture that initiates a layer-wise start-up process (i.e. has NoClrasOutputFlag equal to 1) causes marking of all pictures in the DPB as “unused for reference”.

- A slice_reserved_flag to indicate if a base-layer IDR picture initiates the layer-wise start-up process.

- A base-layer picture that initiates the layer-wise start-up process causes NoOutputOfPriorPicsFlags of all layers to be set equal to no_output_of_prior_pics_flag of the base layer picture.

Note that if the DPB process is modified to operate on sub-DPB basis (where each sub-DPB contains pictures sharing the same spatial resolution, for example), the impact of NoOutputOfPriorPicsFlag for a picture with a particular nuh_layer_id should be changed to concern pictures with that particular nuh_layer_id only. However, proposal 3 above applies similarly to a DPB operating on sub-DPB basis.

The detailed proposal is indicated with the change marks in the accompanying specification text document.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).