| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0153 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0058 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: on output layer sets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

This document includes the detailed proposal as change marks relative to the MV-HEVC specification.

| Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 Jul. –2 Aug. 2013 |  |  |  | Document: JCT3V-E1004-v6 |
| --- | --- | --- | --- | --- |
| Title: | MV-HEVC Draft Text 5 |  |  |  |
| Status: | Output Document of JCT-3V |  |  |  |
| Purpose: | Draft of MV-HEVC |  |  |  |
| Author(s) or Contact(s): | Gerhard Tech Fraunhofer HHI Krzysztof Wegner Poznan University of Technology Ying Chen Qualcomm Incorporated Miska Hannuksela Nokia Corporation Jill Boyce Vidyo | Email: Email: Email: Email: Email: | gerhard.tech@hhi.fraunhofer.de kwegner@multimedia.edu.pl cheny@qti.qualcomm.com miska.hannuksela@nokia.com jill@vidyo.com |  |
| Source: | Editors |  |  |  |

_____________________________

Contents

Page

Contents	ii

8	Decoding process	1

8.1	General decoding process	1

8.1.4	Decoding process for a coded picture with nuh_layer_id equal to 0	2

8.2	NAL unit decoding process	3

8.3	Slice decoding process	3

8.3.3	Decoding process for generating unavailable reference pictures	3

8.3.3.1	General decoding process for generating unavailable reference pictures	3

Annex C  Hypothetical reference decoder	4

C.4	Bitstream conformance	4

Annex D  Supplemental enhancement information	5

D.1	SEI payload syntax	5

D.1.1	General SEI message syntax	5

Annex F  Common syntax, semantics and decoding processes for multi-layer video coding extensions	6

F.1	Scope	6

F.2	Normative references	6

F.3	Definitions	6

F.4	Abbreviations	7

F.5	Conventions	7

F.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	7

F.7	Syntax and semantics	7

F.7.1	Method of specifying syntax in tabular form	7

F.7.2	Specification of syntax functions, categories, and descriptors	7

F.7.3	Syntax in tabular form	7

F.7.3.1	NAL unit syntax	7

F.7.3.2	Raw byte sequence payloads and RBSP trailing bits syntax	8

F.7.3.3	Profile, tier and level syntax	18

F.7.3.4	Scaling list data syntax	18

F.7.3.5	Supplemental enhancement information message syntax	18

F.7.3.6	Slice segment header syntax	19

F.7.3.7	Short-term reference picture set syntax	21

F.7.3.8	Slice segment data syntax	21

F.7.4	Semantics	22

F.7.4.1	General	22

F.7.4.2	NAL unit semantics	22

F.7.4.3	Raw byte sequence payloads, trailing bits, and byte alignment semantics	24

F.7.4.4	Profile, tier and level semantics	34

F.7.4.5	Scaling list data semantics	35

F.7.4.6	Supplemental enhancement information message semantics	35

F.7.4.7	Slice segment header semantics	35

F.7.4.8	Short-term reference picture set semantics	36

F.7.4.9	Slice segment data semantics	36

F.8	Decoding process	37

F.8.1	General decoding process	37

F.8.1.1	Decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0	38

F.8.1.2	Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0	38

F.8.1.3	Generation of unavailable reference pictures for pictures first in decoding order within a layer	39

F.8.2	NAL unit decoding process	40

F.8.3	Slice decoding processes	40

F.8.3.1	Decoding process for picture order count	40

F.8.3.2	Decoding process for reference picture set	41

F.8.4	Decoding process for coding units coded in intra prediction mode	45

F.8.5	Decoding process for coding units coded in inter prediction mode	45

F.8.6	Scaling, transformation and array construction process prior to deblocking filter process	45

F.8.7	In-loop filter process	45

F.9	Parsing process	45

F.10	Specification of bitstream subsets	45

F.11	(Void)	45

F.12	Byte stream format	45

F.13	Hypothetical reference decoder	45

F.13.1	General	45

F.13.2	Operation of coded picture buffer (CPB)	45

F.13.3	Operation of the decoded picture buffer (DPB)	46

F.13.3.1	General	46

F.13.3.2	Removal of pictures from the DPB	46

F.13.3.3	Picture output	46

F.13.3.4	Current decoded picture marking and storage	46

F.13.4	Bitstream conformance	46

F.13.5	Decoder conformance	48

F.13.5.1	General	48

F.13.5.2	Operation of the output order DPB	48

F.14	SEI messages	49

F.14.1	SEI message syntax	50

F.14.1.1	Layers present SEI message syntax	50

F.14.1.2	Inter-layer constrained tile sets SEI message syntax	50

F.14.2	SEI message semantics	50

F.14.2.1	Layers not present SEI message semantics	50

F.14.2.2	Inter-layer constrained tile sets SEI message semantics	51

F.15	Video usability information	52

F.15.1	General	52

F.15.2	VUI syntax	52

F.15.3	VUI semantics	52

F.15.3.1	VUI parameters semantics	52

F.15.3.2	HRD parameters semantics	53

F.15.3.3	Sub-layer HRD parameters semantics	53

Annex G  Multiview coding	54

G.1	Scope	54

G.2	Normative references	54

G.3	Definitions	54

G.4	Abbreviations	54

G.5	Conventions	54

G.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	54

G.7	Syntax and semantics	54

G.8	Decoding processes	54

G.8.1	General decoding process	54

G.8.1.1	Decoding process for a coded picture with nuh_layer_id greater than 0	54

G.8.1.2	Decoding process for inter-layer reference picture set	55

G.8.1.3	Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0	55

G.8.2	NAL unit decoding process	55

G.8.3	Slice decoding processes	55

G.8.3.1	Decoding process for picture order count	55

G.8.3.2	Decoding process for reference picture set	55

G.8.3.3	Decoding process for generating unavailable reference pictures	55

G.8.3.4	Decoding process for reference picture lists construction	56

G.8.4	Decoding process for coding units coded in intra prediction mode	56

G.8.5	Decoding process for coding units coded in inter prediction mode	56

G.8.6	Scaling, transformation and array construction process prior to deblocking filter process	57

G.8.7	In-loop filter process	57

G.9	Parsing process	57

G.10	Specification of bitstream subsets	57

G.11	Profiles, tiers, and levels	57

G.11.1	Profiles	57

G.11.1.1	General	57

G.11.1.2	Stereo Main profile	57

G.11.2	Tiers and levels	57

G.11.2.1	General tier and level limits	57

G.11.2.2	Profile-specific level limits for the Main, Main 10, and Stereo Main profiles	58

G.12	Byte stream format	58

G.13	Hypothetical reference decoder	58

G.14	SEI messages	58

G.14.1	SEI message syntax	58

G.14.1.1	3D reference displays information SEI message syntax	58

G.14.2	SEI message semantics	59

G.14.2.1	3D reference displays information SEI message semantics	59

G.15	Video usability information	61

Replace subclauses 8.1, 8.2 and 8.3.3.1 with the following and add subclause 8.1.1.

# Decoding process

## General decoding process

Input to this process is a bitstream. Output of this process is a list of decoded pictures.

The layer identifier list TargetDecLayerIdList, which specifies the list of nuh_layer_id values, in increasing order of nuh_layer_id values, of the NAL units to be decoded, is specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetDecLayerIdList, TargetDecLayerIdList is set by the external means.

–	Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in subclause C.1, TargetDecLayerIdList is set as specified in subclause C.1.

–	Otherwise, TargetDecLayerIdList contains only one nuh_layer_id value that is equal to 0.

The variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in subclause C.1, HighestTid is set as specified in subclause C.1.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

The sub-bitstream extraction process as specified in clause 10 is applied with the bitstream, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to a bitstream referred to as BitstreamToDecode.

The decoding processes specified in the remainder of this subclause apply to each coded picture, referred to as the current picture and denoted by the variable CurrPic, in BitstreamToDecode.

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows:

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, the term "the bitstream" (or part thereof, e.g. a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

When the current picture is an IRAP picture and has nuh_layer_id equal to 0, the following applies:

–	The variable NoClrasOutputFlag is specified as follows:

–	If the current picture is the first picture in the bitstream, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if the current picture is a BLA picture, NoClrasOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set NoClrasOutputFlag, NoClrasOutputFlag is set by the external means.

–	Otherwise, NoClrasOutputFlag is set equal to 0.

–	When NoClrasOutputFlag is equal to 1, the variable LayerInitialisedFlag[ i ] is set equal to 0 for all values of i from 0 to 63, inclusive, and the variable FirstPicInLayerDecodedFlag[ i ] is set equal to 0 for all values of i from 1 to 63, inclusive.

The decoding process is specified such that all decoders will produce numerically identical cropped decoded pictures. Any decoding process that produces identical cropped decoded pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

When the current picture is an IRAP picture, the following applies:

–	If the current picture with a particular value of nuh_layer_id is an IDR picture, a BLA picture, the first picture with that particular value of nuh_layer_id in the bitstream in decoding order, or the first picture with that particular value of nuh_layer_id that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value for the current picture, the variable HandleCraAsBlaFlag is set equal to the value provided by the external means and the variable NoRaslOutputFlag is set equal to HandleCraAsBlaFlag.

–	Otherwise, the variable HandleCraAsBlaFlag is set equal to 0 and the variable NoRaslOutputFlag is set equal to 0.

When the current picture is an IRAP picture and one of the following conditions is true, LayerInitialisedFlag[ nuh_layer_id ] is set equal to 1:

–	nuh_layer_id is equal to 0.

–	LayerInitialisedFlag[ nuh_layer_id ] is equal to 0 and LayerInitialisedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to RefLayerId[ nuh_layer_id ][ j ], where j is in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] – 1, inclusive.

When the current picture has nuh_layer_id equal to 0, the decoding process for a coded picture with nuh_layer_id equal to 0 as specified in subclause 8.1.1 is invoked.

### Decoding process for a coded picture with nuh_layer_id equal to 0

When the current picture is a BLA picture that has nal_unit_type equal to BLA_W_LP or is a CRA picture, the following applies:

–	If some external means not specified in this Specification is available to set the variable UseAltCpbParamsFlag to a value, UseAltCpbParamsFlag is set equal to the value provided by the external means.

–	Otherwise, the value of UseAltCpbParamsFlag is set equal to 0.

Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1, and 2 being assigned to SL, SCb, and SCr, respectively.

NOTE – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause 8.2.

- The processes in subclause 8.3 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause 8.3.1. This needs to be invoked only for the first slice segment of a picture.

–	The decoding process for RPS in subclause 8.3.2 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice segment of a picture.

–	When the current picture is a BLA picture or is a CRA picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.3 is invoked, which needs to be invoked only for the first slice segment of a picture.

–	PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause 8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0) and, when decoding a B slice, reference picture list 1 (RefPicList1).

- The processes in subclauses 8.4, 8.5, 8.6, and 8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the decoded picture is marked as "used for short-term reference".

## NAL unit decoding process

Inputs to this process are VCL NAL units of the current picture and their associated non-VCL NAL units.

Outputs of this process are the parsed RBSP syntax structures encapsulated within the NAL units of the access unit containing the current picture.

The decoding process for each NAL unit extracts the RBSP syntax structure from the NAL unit and then parses the RBSP syntax structure.

## Slice decoding process

### Decoding process for generating unavailable reference pictures

#### General decoding process for generating unavailable reference pictures

This process is invoked once per coded picture when the current picture is a BLA picture or is a CRA picture with NoRaslOutputFlag equal to 1.

NOTE – This process is primarily specified only for the specification of syntax constraints for RASL pictures. The entire specification of the decoding process for RASL pictures associated with an IRAP picture that has NoRaslOutputFlag equal to 1 is included herein only for purposes of specifying constraints on the allowed syntax content of such RASL pictures. During the decoding process, any RASL pictures associated with an IRAP picture that has NoRaslOutputFlag equal to 1 may be ignored, as these pictures are not specified for output and have no effect on the decoding process of any other pictures that are specified for output. However, in HRD operations as specified in Annex C, RASL access units may need to be taken into consideration in derivation of CPB arrival and removal times.

When this process is invoked, the following applies:

- For each RefPicSetStFoll[ i ], with i in the range of 0 to NumPocStFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.3.2, and the following applies:

- The value of PicOrderCntVal for the generated picture is set equal to PocStFoll[ i ].

- The value of PicOutputFlag for the generated picture is set equal to 0.

- The generated picture is marked as "used for short-term reference".

- RefPicSetStFoll[ i ] is set to be the generated reference picture.

- The value of nuh_layer_id for the generated picture is inferred to be equal to nuh_layer_id.

- For each RefPicSetLtFoll[ i ], with i in the range of 0 to NumPocLtFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.3.2, and the following applies:

- The value of PicOrderCntVal for the generated picture is set equal to PocLtFoll[ i ].

- The value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( PocLtFoll[ i ] & ( MaxPicOrderCntLsb – 1 ) ).

- The value of PicOutputFlag for the generated picture is set equal to 0.

- The generated picture is marked as "used for long-term reference".

- RefPicSetLtFoll[ i ] is set to be the generated reference picture.

- The value of nuh_layer_id for the generated picture is inferred to be equal to nuh_layer_id.

Modify clause C.4 as follows:

- Annex C

Hypothetical reference decoder

(This annex forms an integral part of this Recommendation | International Standard)

  - Bitstream conformance

Replace the paragraph

"The first coded picture in a bitstream shall be an IRAP picture, i.e. an IDR picture, a CRA picture or a BLA picture".

with

"The first access unit in a bitstream shall be an IRAP access unit."

Modify subclause D.1.1 as follows:

- Annex D

Supplemental enhancement information

(This annex forms an integral part of this Recommendation | International Standard)

  - SEI payload syntax

    - General SEI message syntax

Add rows enclosed by "...".

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| ... |  |
| else if( payloadType = = XXX ) |  |
| layers_not_present( payloadSize ) |  |
| else if( payloadType = = XXX ) |  |
| inter_layer_constrained_tile_sets( payloadSize ) |  |
| else if( payloadType = = XXX ) |  |
| three_dimensional_reference_displays_info( payloadSize ) |  |
| ... |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| ... |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

- Annex F

Common syntax, semantics and decoding processes for multi-layer video coding extensions

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies the common syntax, semantics and decoding processes for multi-layer video coding extensions.

  - Scope

Common syntax, semantics and decoding processes for multi-layer video coding extensions are specified in this annex with reference made to clauses 2-9 and Annexes A-E and E.

  - Normative references

The specifications in clause 2 apply.

  - Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in clause 3. These definitions are either not present in clause 3 or replace definitions in clause 3.

- access unit: A set of NAL units that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain the VCL NAL units of all coded pictures associated with the same output time and their associated non-VCL NAL units.

NOTE 1 – Pictures in the same access unit are associated with the same picture order count.

- associated IRAP picture: The previous IRAP picture in decoding order within the same layer (if present).

- base layer: A layer in which all VCL NAL units have nuh_layer_id equal to 0.

- coded picture: A coded representation of a picture comprising VCL NAL units with a particular value of nuh_layer_id within an access unit and containing all coding tree units of the picture.

- coded video sequence (CVS): A sequence of access units that consists, in decoding order, of an initial IRAP access unit, followed by zero or more access units that are not initial IRAP access units, including all subsequent access units up to but not including any subsequent access unit that is an initial IRAP access unit.

- collocated sample: A sample TBD.

- direct reference layer: A layer that may be used for inter-layer prediction of another layer.

- indirect reference layer: A layer that is not a direct reference layer of another layer but is a direct reference layer of a layer that is a direct reference layer or indirect reference layer of a direct reference layer of the layer.

- initial intra random access point (IRAP) access unit: An IRAP access unit in which the coded picture with nuh_layer_id equal to 0 has NoRaslOutputFlag equal to 1.

- inter-layer prediction: A prediction in a manner that is dependent on data elements (e.g. sample values or motion vectors) of reference pictures with a different value of nuh_layer_id than that of the current picture.

- intra random access point (IRAP) access unit: An access unit in which the coded picture with nuh_layer_id equal to 0 is an IRAP picture.

- layer initialisation picture (LIP): A picture that is an IRAP picture with NoRaslOutputFlag equal to 1 or that is contained in an initial IRAP access unit.

- leading picture: A picture that is in the same layer as the associated IRAP picture and precedes the associated IRAP picture in output order.

- non-base layer: A layer in which all VCL NAL units have the same nuh_layer_id value greater than 0.

- picture order count: A variable that is associated with each picture and that uniquely identifies the associated picture among all pictures with the same value of nuh_layer_id in the CVS, and, when the associated picture is to be output from the decoded picture buffer, indicates the position of the associated picture in output order relative to the output order positions of the other pictures with the same value of nuh_layer_id in the same CVS that are to be output from the decoded picture buffer.

- reference layer picture: A picture in a direct reference layer which is used for inter-layer prediction of the current picture and is in the same access unit as the current picture.

- reference picture list: A list of reference pictures that is used for inter prediction or inter-layer prediction of a P or B slice.

- target output layer: A layer that is to be output.

- trailing picture: A picture that is in the same layer as the associated IRAP picture and follows the associated IRAP picture in output order.

- output time: A time when a decoded picture is to be output as specified in Annex C, if the timing information is present in the coded video sequence.

- view: A sequence of pictures associated with the same value of ViewOrderIdx.

NOTE 2 – A view typically represents a sequence of pictures captured by one camera.

  - Abbreviations

The specifications in clause 4 apply.

  - Conventions

The specifications in clause 5 apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause 6 apply.

  - Syntax and semantics

This clause specifies syntax and semantics for CVSs that conform to one or more of the profiles specified in this annex.

    - Method of specifying syntax in tabular form

The specifications in subclause 7.1 apply.

    - Specification of syntax functions, categories, and descriptors

The specifications in subclause 7.2 apply.

    - Syntax in tabular form

      - NAL unit syntax

The specifications in subclause 7.3.1 apply.

        - General NAL unit syntax

The specifications in subclause 7.3.1.1 apply.

        - NAL unit header syntax

The specifications in subclause 7.3.1.2 apply.

      - Raw byte sequence payloads and RBSP trailing bits syntax

        - Video parameter set RBSP

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_max_layers_minus1 | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_extension_offset //vps_reserved_0xffff_16bits | u(16) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_sub_layer_ordering_info_present_flag ? 0 : vps_max_sub_layers_minus1 ); i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| vps_max_layer_id | u(6) |
| vps_num_layer_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_layer_id; j++ ) |  |
| layer_id_included_flag[ i ][ j ] | u(1) |
| vps_timing_info_present_flag | u(1) |
| if( vps_timing_info_present_flag ) { |  |
| vps_num_units_in_tick | u(32) |
| vps_time_scale | u(32) |
| vps_poc_proportional_to_timing_flag | u(1) |
| if( vps_poc_proportional_to_timing_flag ) |  |
| vps_num_ticks_poc_diff_one_minus1 | ue(v) |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_layer_set_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_extension_alignment_bit_equal_to_one | u(1) |
| vps_extension( ) |  |
| vps_extension2_flag | u(1) |
| if( vps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

          - Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| if( NumViews > 1 ) |  |
| view_id_len_minus1 | u(4) |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) { |  |
| output_layer_flag[ i ][ j ] | u(1) |
| if( output_layer_flag[ i ][ j ] ) |  |
| min_alt_output_layer_idx[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0 ) |  |
| vps_rep_format_idx[ i ] | u(4) |
| max_one_active_ref_layer_flag | u(1) |
| cross_layer_irap_aligned_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| vps_shvc_reserved_zero_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_vui_alignment_bit_equal_to_one | u(1) |
| vps_vui( ) |  |
| } |  |
| } |  |

          - Representation format syntax

| rep_format( ) { | Descriptor |
| --- | --- |
| chroma_format_vps_idc | u(2) |
| if( chroma_format_vps_idc = = 3 ) |  |
| separate_colour_plane_vps_flag | u(1) |
| pic_width_vps_in_luma_samples | u(16) |
| pic_height_vps_in_luma_samples | u(16) |
| bit_depth_vps_luma_minus8 | u(4) |
| bit_depth_vps_chroma_minus8 | u(4) |
| } |  |

          - VPS VUI syntax

| vps_vui( ){ | Descriptor |
| --- | --- |
| bit_rate_present_vps_flag | u(1) |
| pic_rate_present_vps_flag | u(1) |
| if( bit_rate_present_vps_flag ¦ ¦ pic_rate_present_vps_flag ) |  |
| for( i = 0; i <= vps_number_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_sub_layers_minus1; j++ ) { |  |
| if( bit_rate_present_vps_flag ) |  |
| bit_rate_present_flag[ i ][ j ] | u(1) |
| if( pic_rate_present_vps_flag ) |  |
| pic_rate_present_flag[ i ][ j ] | u(1) |
| if( bit_rate_present_flag[ i ][ j ] ) { |  |
| avg_bit_rate[ i ][ j ] | u(16) |
| max_bit_rate[ i ][ j ] | u(16) |
| } |  |
| if( pic_rate_present_flag[ i ][ j ] ) { |  |
| constant_pic_rate_idc[ i ][ j ] | u(2) |
| avg_pic_rate[ i ][ j ] | u(16) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < NumDirectRefLayers[ layer_id_in_nuh[ i ] ]; j++ ) { |  |
| tile_boundaries_aligned_flag[ i ][ j ] | u(1) |
| ilp_restricted_ref_layers_flag | u(1) |
| if( ilp_restricted_ref_layers_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < NumDirectRefLayers[ layer_id_in_nuh[ i ] ]; j++ ) { |  |
| min_spatial_segment_offset_plus1[ i ][ j ] | ue(v) |
| if( min_spatial_segment_offset_plus1[ i ][ j ] > 0 ) { |  |
| ctu_based_offset_enabled_flag[ i ][ j ] | u(1) |
| if( ctu_based_offset_enabled_flag[ i ][ j ] ) |  |
| min_horizontal_ctu_offset_plus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

        - Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id = = 0 ) { |  |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id > 0 ) |  |
| update_rep_format_flag | u(1) |
| if( update_rep_format_flag ) { |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| } |  |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| if( update_rep_format_flag ) { |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| if( nuh_layer_id > 0 ) |  |
| sps_infer_scaling_list_flag | u(1) |
| if( sps_infer_scaling_list_flag ) |  |
| sps_scaling_list_ref_layer_id | u(6) |
| else { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

          - Sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_shvc_reserved_zero_idc | ue(v) |
| } |  |

        - Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| dependent_slice_segments_enabled_flag | u(1) |
| output_flag_present_flag | u(1) |
| num_extra_slice_header_bits | u(3) |
| sign_data_hiding_enabled_flag | u(1) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| diff_cu_qp_delta_depth | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| transquant_bypass_enabled_flag | u(1) |
| tiles_enabled_flag | u(1) |
| entropy_coding_sync_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| pps_loop_filter_across_slices_enabled_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( nuh_layer_id > 0 ) |  |
| pps_infer_scaling_list_flag | u(1) |
| if( pps_infer_scaling_list_flag ) |  |
| pps_scaling_list_ref_layer_id | u(6) |
| else { |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| lists_modification_present_flag | u(1) |
| log2_parallel_merge_level_minus2 | ue(v) |
| slice_segment_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

        - Supplemental enhancement information RBSP syntax

The specifications in subclause 7.3.2.4 apply.

        - Access unit delimiter RBSP syntax

The specifications in subclause 7.3.2.5 apply.

        - End of sequence RBSP syntax

The specifications in subclause 7.3.2.6 apply.

        - End of bitstream RBSP syntax

The specifications in subclause 7.3.2.7 apply.

        - Filler data RBSP syntax

The specifications in subclause 7.3.2.8 apply.

        - Slice segment layer RBSP syntax

The specifications in subclause 7.3.2.9 apply.

        - RBSP slice segment trailing bits syntax

The specifications in subclause 7.3.2.10 apply.

        - RBSP trailing bits syntax

The specifications in subclause 7.3.2.11 apply.

        - Byte alignment syntax

The specifications in subclause 7.3.2.12 apply.

      - Profile, tier and level syntax

| profile_tier_level( profilePresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profilePresentFlag ) { |  |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| general_profile_compatibility_flag[ j ] | u(1) |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_reserved_zero_44bits | u(44) |
| } |  |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| } |  |
| if( maxNumSubLayersMinus1 > 0 ) |  |
| for( i = maxNumSubLayersMinus1; i < 8; i++ ) |  |
| reserved_zero_2bits[ i ] | u(2) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| if( sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_progressive_source_flag[ i ] | u(1) |
| sub_layer_interlaced_source_flag[ i ] | u(1) |
| sub_layer_non_packed_constraint_flag[ i ] | u(1) |
| sub_layer_frame_only_constraint_flag[ i ] | u(1) |
| sub_layer_reserved_zero_44bits[ i ] | u(44) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

      - Scaling list data syntax

The specifications in subclause 7.3.4 apply.

      - Supplemental enhancement information message syntax

The specifications in subclause 7.3.5 apply.

      - Slice segment header syntax

        - General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| i = 0 |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| poc_reset_flag | u(1) |
| } |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| discardable_flag | u(1) |
| } |  |
| for( i = 1; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( nuh_layer_id > 0 ¦ ¦ ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if( nuh_layer_id > 0 && all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

        - Reference picture list modification syntax

The specifications in subclause 7.3.6.2 apply.

        - Weighted prediction parameters syntax

The specifications in subclause 7.3.6.3 apply.

      - Short-term reference picture set syntax

The specifications in subclause 7.3.7 apply.

      - Slice segment data syntax

        - General slice segment data syntax

The specifications in subclause 7.3.8.1 apply.

        - Coding tree unit syntax

The specifications in subclause 7.3.8.2 apply.

        - Sample adaptive offset syntax

The specifications in subclause 7.3.8.3 apply.

        - Coding quadtree syntax

The specifications in subclause 7.3.8.4 apply.

        - Coding unit syntax

The specifications in subclause 7.3.8.5 apply.

        - Prediction unit syntax

The specifications in subclause 7.3.8.6 apply.

        - PCM sample syntax

The specifications in subclause 7.3.8.7 apply.

        - Transform tree syntax

The specifications in subclause 7.3.8.8 apply.

        - Motion vector difference syntax

The specifications in subclause 7.3.8.9 apply.

        - Transform unit syntax

The specifications in subclause 7.3.8.10 apply.

        - Residual coding syntax

The specifications in subclause 7.3.8.11 apply.

    - Semantics

      - General

      - NAL unit semantics

        - General NAL unit semantics

The specifications in subclause 7.4.2.1 apply.

        - NAL unit header semantics

The specifications in subclause 7.4.2.2 apply with following modifications and additions.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7 1.

When one picture picA of a layer layerA has nal_unit_type equal to TSA_N or TSA_R, each picture in the same access unit as picA in a direct or indirect reference layer of layerA shall have nal_unit_type equal to TSA_N or TSA_R.

When one picture picA of a layer layerA has nal_unit_type equal to STSA_N or STSA_R, each picture in the same access unit as picA in a direct or indirect reference layer of layerA shall have nal_unit_type equal to STSA_N or STSA_R.

nuh_layer_id specifies the identifier of the layer.

When nal_unit_type is equal to AUD_NUT, the value of nuh_layer_id shall be equal to the minimum of the nuh_layer_id values of all VCL NAL units in the access unit.

When nal_unit_type is equal to VPS_NUT, the value of nuh_layer_id shall be equal to 0. Decoder shall ignore NAL units with nal_unit_type equal to VPS_NUT and nuh_layer_id greater than 0.

        - Encapsulation of an SODB within an RBSP (informative)

The specifications in subclause 7.4.2.3 apply.

        - Order of NAL units and association to coded pictures, access units, and coded video sequences

          - General

The specifications in subclause 7.4.2.4.1 apply with the following additions.

A coded picture with nuh_layer_id equal to nuhLayerIdA shall precede, in decoding order, all coded pictures with nuh_layer_id greater than nuhLayerIdA in the same access unit.

          - Order of VPS, SPS and PPS RBSPs and their activation

The specifications in subclause 7.4.2.4.2 apply with the following additions.

The contents of the hrd_parameters( ) syntax structure shall remain unchanged within a sequence of activated SPS RBSPs, in their activation order, from any activated SPS RBSP until the end of the bitstream or up to but excluding an SPS RBSP that is activated within the next access unit in which at least one of the following conditions is true:

- The access unit includes a picture for each nuh_layer_id value in TargetDecLayerIdList and each picture in the access unit is an IDR picture.

- The access unit includes an IRAP picture with nuh_layer_id equal to 0 for which NoClrasOutputFlag is equal to 1.

An activated VPS RBSP shall remain active until the end of the bitstream or until it is deactivated by another VPS RBSP in an access unit in which at least one of the following conditions is true:

- The access unit includes a picture for each nuh_layer_id value in TargetDecLayerIdList and each picture in the access unit is an IDR picture.

- The access unit includes an IRAP picture with nuh_layer_id equal to 0 for which NoClrasOutputFlag is equal to 1.

- Each PPS RBSP is initially considered not active for any layer with nuh_layer_id greater than 0 at the start of the operation of the decoding process. At most one PPS RBSP is considered active for each non-zero nuh_layer_id value at any given moment during the operation of the decoding process, and the activation of any particular PPS RBSP for a particular non-zero nuh_layer_id value results in the deactivation of the previously-active PPS RBSP for that non-zero nuh_layer_id value (if any).

- When a PPS RBSP (with a particular value of pps_pic_parameter_set_id) is not active for a nuh_layer_id value and it is referred to by a coded slice segment NAL unit (using a value of slice_pic_parameter_set_id equal to the pps_pic_parameter_set_id and having that value of nuh_layer_id), it is activated for that nuh_layer_id value. This PPS RBSP is called the active layer PPS RBSP for that nuh_layer_id value until it is deactivated by the activation of another PPS RBSP for the same layer. A PPS RBSP, with that particular value of pps_pic_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId less than or equal to the TemporalId of the PPS NAL unit or provided through external means. The nuh_layer_id value of the NAL unit containing the PPS RBSP that is activated for nuh_layer_id equal to nuhLayerIdA shall be less than or equal to nuhLayerIdA. The same PPS RBSP may be the active layer PPS for more than one nuh_layer_id value.

- Any PPS NAL unit containing the value of pps_pic_parameter_set_id for the active layer PPS RBSP for a coded picture shall have the same content as that of the active layer PPS RBSP for the coded picture, unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

- Each SPS RBSP is initially considered not active for any layer with nuh_layer_id greater than 0 at the start of the operation of the decoding process. At most one SPS RBSP is considered active for each non-zero nuh_layer_id value at any given moment during the operation of the decoding process, and the activation of any particular SPS RBSP for a particular non-zero nuh_layer_id value results in the deactivation of the previously-active SPS RBSP for that non-zero nuh_layer_id value (if any).

- When an SPS RBSP (with a particular value of sps_seq_parameter_set_id) is not already active for a nuh_layer_id value and it is referred to by activation of a PPS RBSP for that nuh_layer_id value (in which pps_seq_parameter_set_id is equal to the sps_seq_parameter_set_id value), it is activated for that nuh_layer_id value. This SPS RBSP is called the active layer SPS RBSP for that nuh_layer_id value until it is deactivated by the activation of another SPS RBSP for the same layer. An SPS RBSP, with that particular value of sps_seq_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means. An activated SPS RBSP for a particular nuh_layer_id value shall remain active for a sequence of pictures in decoding order with that nuh_layer_id value starting from a LIP picture having that nuh_layer_id value, inclusive, until either the next LIP picture with that nuh_layer_id value, exclusive, or the end of the CVS, whichever is earlier. The nuh_layer_id value the NAL unit containing the SPS RBSP that is activated for nuh_layer_id equal to nuhLayerIdA shall be less than or equal to nuhLayerIdA. The same SPS RBSP may be the active layer SPS for more than one nuh_layer_id value.

- Any SPS NAL unit containing the value of sps_seq_parameter_set_id for the active layer SPS RBSP shall have the same content as that of the active layer SPS RBSP unless it follows the last coded picture for which the active layer SPS is required to be active and precedes the first NAL unit activating a SPS of the same value of seq_parameter_set_id.

During operation of the decoding process for VCL NAL units with a non-zero nuh_layer_id value, the values of parameters of the active layer SPS for that non-zero nuh_layer_id value, and the active layer PPS RBSP for that non-zero nuh_layer_id value are considered in effect.

          - Order of access units and their association to CVS

The specifications in subclause 7.4.2.4.3 apply.

          - Order of NAL units and coded pictures and association to access units

The specifications in subclause 7.4.2.4.4 apply.

          - Order of VCL NAL units and association to coded pictures

The specifications in subclause 7.4.2.4.5 apply.

      - Raw byte sequence payloads, trailing bits, and byte alignment semantics

        - Video parameter set RBSP semantics

The specifications in subclause 7.4.3.1 apply with following modifications and additions.

–	layerSetLayerIdList is replaced by LayerSetLayerIdList.

–	numLayersInIdList is replaced by NumLayersInIdList.

vps_extension_offset specifies the byte offset, starting from the beginning of the VPS NAL unit, of the next set of fixed-length coded information starting from avc_base_layer_flag, when present, in the VPS NAL unit. When present, emulation prevention bytes that appear in the VPS NAL unit are counted for purposes of byte offset identification.

NOTE – VPS information for non-base layer or view starts from a byte-aligned position of the VPS NAL unit, with fixed-length coded information that is essential for session negotiation and/or capability exchange. The byte offset specified by vps_extension_offset would then help to locate and access those essential information in the VPS NAL unit without the need of entropy decoding, which may not be equipped with some network entities that may desire to access only the information in the VPS that is essential for session negotiation and/or capability exchange.

vps_extension_flag equal to 0 specifies that no vps_extension( ) syntax structure is present in the VPS RBSP syntax structure. vps_extension_flag equal to 1 specifies that the vps_extension( ) syntax structure is present in the VPS RBSP syntax structure. When vps_max_layers_minus1 is greater than 0, vps_extension_flag shall be equal to 1.

vps_extension_alignment_bit_equal_to_one shall be equal to 1.

vps_extension2_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the VPS RBSP syntax structure. vps_extension2_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for vps_extension2_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension2_flag in a VPS NAL unit.

          - Video parameter set extension semantics

avc_base_layer_flag equal to 1 specifies that the base layer conforms to Rec. ITU-T H.264 | ISO/IEC 14496-10. avc_base_layer_flag equal to 0 specifies that the base layer conforms to this Specification.

When avc_base_layer_flag equal to 1, in the Rec. ITU-T H.264 | ISO/IEC 14496-10 conforming base layer, after applying the Rec. ITU-T H.264 | ISO/IEC 14496-10 decoding process for reference picture lists construction the output reference picture lists refPicList0 and refPicList1 (when applicable) does not contain any pictures for which the TemporalId is greater than TemporalId of the coded picture.  All sub-bitstreams of the Rec. ITU-T H.264 | ISO/IEC 14496-10 conforming base layer, that can be derived using the sub-bitstream extraction process as specified in Rec. ITUT H.264 | ISO/IEC 14496-10 subclause G.8.8.1 with any value for temporal_id as the input shall result in a set of CVSs, with each CVS conforming to one or more of the profiles specified in Rec. ITUT H.264 | ISO/IEC 14496-10 Annexes A, G and H.

vps_vui_offset specifies the byte offset, starting from the beginning of the VPS NAL unit, of the set of fixed-length coded information starting from bit_rate_present_vps_flag, when present, in the VPS NAL unit. When present, emulation prevention bytes that appear in the VPS NAL unit are counted for purposes of byte offset identification.
splitting_flag equal to 1 indicates that the dimension_id[ i ][ j ] syntax elements are not present and that the binary representation of the nuh_layer_id value in the NAL unit header are split into NumScalabilityTypes segments with lengths, in bits, according to the values of dimension_id_len_minus1[ j ] and that the values of dimension_id[ LayerIdxInVps[ nuh_layer_id ] ][ j ] are inferred from the NumScalabilityTypes segments. splitting_flag equal to 0 indicates that the syntax elements dimension_id[ i ][ j ] are present.

NOTE 1 – When splitting_flag is equal to 1, scalable identifiers can be derived from the nuh_layer_id syntax element in the NAL unit header by a bit masked copy. The respective bit mask for the i-th scalable dimension is defined by the value of the dimension_id_len_minus1[ i ] syntax element and dimBitOffset[ i ] as specified in the semantics of dimension_id_len_minus1[ j ].

scalability_mask_flag[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table F-1 are present. scalability_mask_flag[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table F-1 – Mapping of ScalabiltyId to scalability dimensions

| scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | Multiview | View Order Index |
| 2-15 | Reserved |  |

NOTE 2 – It is anticipated that in future 3D extensions of this Specification, scalability mask index 0 will be used to indicate depth maps. It is anticipated that in future scalability extensions of this Specification, scalability mask index 2 will be used to indicate spatial/SNR scalability.

dimension_id_len_minus1[ j ] plus 1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

When splitting_flag is equal to 1, the following applies:

–	The variable dimBitOffset[ 0 ] is set equal to 0 and for j in the range of 1 to  NumScalabilityTypes − 1 , inclusive, dimBitOffset[ j ] is derived as follows:

dimBitOffset[ j ]=dimIdx=0j−1dimension_id_len_minus1[ dimIdx ]+1	(F-1)

–	The value of dimension_id_len_minus1[ NumScalabilityTypes − 1 ] is inferred to be equal to 5 − dimBitOffset[ NumScalabilityTypes − 1 ].

–	The value of dimBitOffset[ NumScalabilityTypes ] is set equal to 6.

It is a requirement of bitstream conformance that when NumScalabilityTypes is greater than 0, dimBitOffset[ NumScalabilityTypes − 1 ] shall be less than 6.

vps_nuh_layer_id_present_flag equal to 1 specifies that layer_id_in_nuh[ i ] for i from 0 to vps_max_layers_minus1, inclusive, are present. vps_nuh_layer_id_present_flag equal to 0 specifies that layer_id_in_nuh[ i ] for i from 0 to vps_max_layers_minus1, inclusive, are not present.

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. For i in the range of 0 to vps_max_layers_minus1, inclusive, when layer_id_in_nuh[ i ] is not present, the value is inferred to be equal to i.

When i is greater than 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ].

For i from 0 to vps_max_layers_minus1, inclusive, the variable LayerIdxInVps[ layer_id_in_nuh[ i ] ] is set equal to i.

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th layer. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits.

If splitting_flag is equal to 1, for i from 0 to vps_max_layers_minus1, inclusive, and j from 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ i ][ j ] is inferred to be equal to ( ( layer_id_in_nuh[ i ] & ( (1 << dimBitOffset[ j + 1 ] ) − 1) ) >> dimBitOffset[ j ] ).

Otherwise, for j from 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ 0 ][ j ] is inferred to be equal to 0.

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer, the variable ViewOrderIdx[ layer_id_in_nuh[ i ] ] specifying the view order index of the i-th layer, and the variable ViewScalExtLayerFlag specifying whether the i-th layer is a view scalability extension layer are derived as follows:

NumViews = 1

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {

lId = layer_id_in_nuh[ i ]

for( smIdx= 0, j = 0; smIdx < 16; smIdx++ )

if( scalability_mask_flag[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

ViewOrderIdx[ lId ] = ScalabilityId[ i ][ 1 ]

if( i > 0  &&  ( ViewOrderIdx[ lId ]  !=  ScalabilityId[ i – 1][ 1 ] ) )

NumViews++

ViewScalExtLayerFlag[ lId ] = ( ViewOrderIdx[ lId ] > 0 )

}

view_id_len_minus1 plus 1 specifies the length, in bits, of the view_id_val[ i ] syntax element.

view_id_val[ i ] specifies the view identifier of the i-th view specified by the VPS. The length of the view_id_val[ i ] syntax element is view_id_len_minus1 + 1 bits. When not present, the value of view_id_val[ i ] is inferred to be equal to 0.

For each layer with nuh_layer_id equal to nuhLayerId, the value ViewId[ nuhLayerId ] is set equal to view_id_val[ ViewOrderIdx[ nuhLayerId ] ].

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

The variables NumDirectRefLayers[ i ] and RefLayerId[ i ][ j ] are derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	NumDirectRefLayers[ iNuhLId ] = 0
	for( j = 0; j < i; j++ )
		if( direct_dependency_flag[ i ][ j ] )
			RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]
}

The variables NumRecursiveRefLayers[ i ] and RecursiveRefLayerFlag[ i ][ j ] are derived as follows:

–	NumRecursiveRefLayers[ i ] and RecursiveRefLayerFlag[ i ][ j ] are first initialized to 0 for all values of i and j in the range of 0 to 63, inclusive.

–	The function setRefLayerFlags( currLayerId ) is specified as follows. The function is used to modify RecursiveRefLayerFlag[ i ][ j ].

for( j = 0; j < NumDirectRefLayers[ currLayerId ]; j++ )
	refLayerId = RefLayerId[ currLayerId ][ j ]
	setRefLayerFlags( refLayerId )
	for( k = 0; k <= 63; k++ )
		RecursiveRefLayerFlag[ currLayerId ][ k ] = 
			RecursiveRefLayerFlag[ currLayerId ][ k ] | RecursiveRefLayerFlag[ refLayerId ][ k ]

–	The function setRefLayerFlags( currLayerId ) is called as follows:

for( i = 0; i <= vps_max_layers_minus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	setRefLayerFlags( iNuhLId )
	for( j = 0; j < i; j++ )
		NumRecursiveRefLayers[ iNuhLId ] += RecursiveRefLayerFlag[ iNuhLId ][ j ]

max_tid_ref_present_flag equal to 1 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ] is present. max_tid_ref_present_flag equal to 0 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ] is not present.

max_tid_il_ref_pics_plus1[ i ] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction. max_tid_il_ref_pics_plus1[ i ] greater than 0 specifies that within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ] – 1 are not used as reference for inter-layer prediction. When not present, max_tid_il_ref_pics_plus1[ i ] is inferred to be equal to 7.

all_ref_layers_active_flag equal to 1 specifies that for each picture referring to the VPS, the reference layer pictures of all direct reference layers of the layer containing the picture are present in the same access unit as the picture and are included in the inter-layer reference picture set of the picture. all_ref_layers_active_flag equal to 0 specifies that the above restriction may or may not apply.

vps_number_layer_sets_minus1 plus 1 specifies the number of layer sets that are specified by the VPS. The value of vps_number_layer_sets_minus1 shall be in the range of 0 to 1023, inclusive, and shall be equal to vps_num_layer_sets_minus1.

vps_num_profile_tier_level_minus1 plus 1 specifies the number of profile_tier_level( ) syntax structures in the VPS.

vps_profile_present_flag[ i ] equal to 1 specifies that the profile and tier information for layer set i is present in the i-th profile_tier_level( ) syntax structure. vps_profile_present_flag[ i ] equal to 0 specifies that profile and tier information is not present in the i-th profile_tier_level( ) syntax structure and is inferred.

profile_ref_minus1[ i ] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the (profile _ref_minus1[ i ] + 1)-th layer set. The value of profile_ref_minus1[ i ] + 1 shall be less than i.

more_output_layer_sets_than_default_flag equal to 1 specifies that the number of output layer sets specified by the VPS is greater than vps_number_layer_sets_minus1 + 1. more_output_layer_sets_than_default_flag equal to 0 specifies that the number of output layer sets specified by the VPS is equal to vps_number_layer_sets_minus1 + 1.

num_add_output_layer_sets_minus1 plus 1 specifies the number of output layer sets in addition to the default output layer sets specified by the VPS. The default output layer sets refer to the first vps_number_layer_sets_minus1 + 1 output layer sets specified by the VPS. For the default output layer sets, either only the highest layer is a target output layer or all layers are target output layers.

default_one_target_output_layer_flag equal to 1 specifies that only the highest layer in each of the default output layer sets is a target output layer. default_one_target_output_layer_flag equal to 0 specifies that all layers in each of the default output layer sets are target output layers.

output_layer_set_idx_minus1[ i ] plus 1 specifies the index of the layer set for the i-th output layer set. The value of output_layer_set_idx_minus1[ i ] shall be in the range of 0 to vps_num_layer_sets_minus1 − 1, inclusive. The length of the output_layer_set_idx_minus1[ i ] syntax element is Ceil( Log2( vps_num_layer_sets_minus1 ) ) bits.

The layer set for the i-th output layer set with i in the range of 0 to vps_num_layer_sets_minus1, inclusive, is inferred to be the i-th layer set.

output_layer_flag[ i ][ j ] equal to 1 specifies that the j-th layer in the i-th output layer set is a target output layer. output_layer_flag[ i ][ j ] equal to 0 specifies that the j-th layer in the i-th output layer set is not a target output layer.

min_alt_output_layer_idx[ i ][ j ] is used to derive variable MinAltOutputLayerId[ i ][ j ]. The value of min_alt_output_layer_idx[ i ][ j ] shall be in the range of 0 to NumRecursiveRefLayers[ LayerSetLayerIdList[ lsIdx ][ j ] – 1, inclusive.

Variables OutputLayerFlag[ i ][ j ] and MinAltOutputLayerId[ i ][ j ] where i is the index of the output layer set and j is a nuh_layer_id value are derived as follows:

for( i = 0; i < numOutputLayerSets; i++) {
	for( j = 0; j <= 63; j++ ) {
		OutputLayerFlag[ i ][ j ] = 0
		MinAltOutputLayerId[ i ][ j ] = 0
	}
	if( i <= vps_number_layer_sets_minus1 )
		if( default_one_target_output_layer_flag ) {
			layerId = LayerSetLayerIdList[ i ][ NumLayersInIdList[ i ] – 1 ]
			OutputLayerFlag[ i ][ layerId ] = 1
			MinAltOutputLayerId[ i ][ layerId ] = layerId
		} else
			for( j = 0; j < NumLayersInIdList[ i ]; j++ ) {
				layerId = LayerSetLayerIdList[ i ][ j ]
				OutputLayerFlag[ i ][ layerId ] = 1
				MinAltOutputLayerId[ i ][ layerId ] = layerId
			}
	else {
		lsIdx = output_layer_set_idx_minus1[ i ] + 1
		for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) {
			layerId = LayerSetLayerIdList[ i ][ j ]
			OutputLayerFlag[ i ][ layerId ] = output_layer_flag[ i ][ j ]
			MinAltOutputLayerId[ i ][ layerId ] = LayerSetLayerIdList[ i ][ min_alt_output_layer_idx[ i ][ j ] ]
		}
	}
}

profile_level_tier_idx[ i ] specifies the index, into the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to i-th output layer set. The length of the profile_level_tier_idx[ i ] syntax element is Ceil( Log2( vps_num_profile_tier_level_minus1 + 1 ) ) bits. The value of profile_level_tier_idx[ 0 ] is inferred to be equal to 0. The value of profile_level_tier_idx[ i ] shall be in the range of 0 to vps_num_profile_tier_level_minus1, inclusive.

rep_format_idx_present_flag equal to 1 specifies that the syntax elements vps_num_rep_formats_minus1 and vps_rep_format_idx[ i ] are present. rep_format_idx_present_flag equal to 0 specifies that the syntax elements vps_num_rep_formats_minus1 and vps_rep_format_idx[ i ] are not present.

vps_num_rep_formats_minus1 plus 1 specifies the number of the following rep_format( ) syntax structures in the VPS. When not present, the value of vps_num_rep_formats_minus1 is inferred to be equal to vps_max_layers_minus1.

vps_rep_format_idx[ i ] specifies the index, into the list of rep_format( ) syntax structures in the VPS, of the rep_format( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. When not present, the value of vps_rep_format_idx[ i ] is inferred to be equal to 0. The value of vps_rep_format_idx[ i ] shall be in the range of 0 to vps_num_rep_formats_minus1, inclusive

max_one_active_ref_layer_flag equal to 1 specifies that at most one picture is used for inter-layer prediction for each picture in the CVS. max_one_active_ref_layer_flag equal to 0 specifies that more than one picture may be used for inter-layer prediction for each picture in the CVS.

cross_layer_irap_aligned_flag equal to 1 specifies that IRAP pictures in the CVS are cross-layer aligned, i.e. when a picture pictureA of a layer layerA in an access unit is an IRAP picture, each picture pictureB in the same access unit that belongs to a direct reference layer of layerA or that belongs to a layer for which layerA is a direct reference layer of that layer is an IRAP picture and the VCL NAL units of pictureB have the same value of nal_unit_type as that of pictureA. cross_layer_irap_aligned_flag equal to 0 specifies that the above restriction may or may not apply.

direct_dep_type_len_minus2 plus 2 specifies the number of bits of the direct_dependency_type[ i ][ j ] syntax element. In bitstreams conforming to this version of this Specification the value of direct_dep_type_len_minus2 shall be equal 0. Although the value of direct_dep_type_len_minus2 shall be equal to 0 in this version of this Specification, decoders shall allow other values of direct_dep_type_len_minus2 in the range of 0 to 30, inclusive, to appear in the syntax.

direct_dependency_type[ i ][ j ] indicates the type of dependency between the layer with nuh_layer_id equal layer_id_in_nuh[ i ] and the layer with nuh_layer_id equal to layer_id_in_nuh[ j ]. direct_dependency_type[ i ][ j ] equal to 0 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for both inter-layer sample prediction and inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer sample prediction but not for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 2 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer motion prediction but not for inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. Although the value of direct_dependency_type[ i ][ j ] shall be in the range of 0 to 2, inclusive, in this version of this Specification, decoders shall allow values of direct_dependency_type[ i ][ j ] in the range of 3 to 232 − 2, inclusive, to appear in the syntax.

vps_shvc_reserved_zero_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for vps_shvc_reserved_zero_flag are reserved for future use by ITU-T | ISO/IEC. Although the value of vps_shvc_reserved_zero_flag is required to be equal to 0 in this version of this Specification, decoders shall allow other values of vps_shvc_reserved_zero_flag to appear in the syntax.

NOTE 3 – It is anticipated that in future scalable extensions of this Specification, this field will be used to indicate either that all the VCL NAL units of an access unit have the same nuh_layer_id value or that two nuh_layer_id values are used by the VCL NAL units of an access unit and the picture with the greater nuh_layer_id value is an IRAP picture.

vps_vui_present_flag equal to 1 specifies that the vps_vui( ) syntax structure is present in the VPS. vps_vui_present_flag equal to 0 specifies that the vps_vui( ) syntax structure is not present in the VPS.

vps_vui_alignment_bit_equal_to_one shall be equal to 1.

          - Representation format semantics

chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8 are used for inference of the values of the SPS syntax elements chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, for each SPS that refers to the VPS.

For each of these syntax elements, all constraints, if any, that apply to the value of the corresponding SPS syntax element also apply.

          - VPS VUI semantics

bit_rate_present_vps_flag equal to 1 specifies that the syntax element bit_rate_present_flag[ i ][ j ] is present. bit_rate_present_vps_flag equal to 0 specifies that the syntax element bit_rate_present_flag[ i ][ j ] is not present.

pic_rate_present_vps_flag equal to 1 specifies that the syntax element pic_rate_present_flag[ i ][ j ] is present. pic_rate_present_vps_flag equal to 0 specifies that the syntax element pic_rate_present_flag[ i ][ j ] is not present.

bit_rate_present_flag[ i ][ j ] equal to 1 specifies that the bit rate information for the j-th subset of the i-th layer set is present. bit_rate_present_flag[ i ] equal to 0 specifies that the bit rate information for the j-th subset of the i-th layer set is not present. The j-th subset of a layer set is the output of the sub-bitstream extraction process when it is invoked with the layer set, j, and the layer identifier list associated with the layer set as inputs. When not present, the value of bit_rate_present_flag[ i ][ j ] is inferred to be equal to 0.

pic_rate_present_flag[ i ][ j ] equal to 1 specifies that picture rate information for the j-th subset of the i-th layer set is present. pic_rate_present_flag[ i ][ j ] equal to 0 specifies that picture rate information for the j-th subset of the i-th layer set is not present. When not present, the value of pic_rate_present_flag[ i ][ j ] is inferred to be equal to 0.

avg_bit_rate[ i ][ j ] indicates the average bit rate of the j-th subset of the i-th layer set, in bits per second. The value is given by BitRateBPS( avg_bit_rate[ i ][ j ] ) with the function BitRateBPS( ) being specified as follows:

BitRateBPS( x ) = ( x & ( 214 − 1 ) ) * 10( 2 + ( x  >>  14 ) )	(F-2)

The average bit rate is derived according to the access unit removal time specified in clause D.13. In the following, bTotal is the number of bits in all NAL units of the j-th subset of the i-th layer set, t1 is the removal time (in seconds) of the first access unit to which the VPS applies, and t2 is the removal time (in seconds) of the last access unit (in decoding order) to which the VPS applies. With x specifying the value of avg_bit_rate[ i ][ j ], the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

( x & ( 214 − 1 ) )  = =  Round( bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) ) )	(F-3)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

( x & ( 214 − 1 ) )  = =  0		(F-4)

max_bit_rate_layer[ i ][ j ] indicates an upper bound for the bit rate of the j-th subset of the i-th layer set in any one-second time window of access unit removal time as specified in clause D.13. The upper bound for the bit rate in bits per second is given by BitRateBPS( max_bit_rate_layer[ i ][ j ] ). The bit rate values are derived according to the access unit removal time specified in clause D.13. In the following, t1 is any point in time (in seconds), t2 is set equal to t1 + 1 ÷ 100, and bTotal is the number of bits in all NAL units of access units with a removal time greater than or equal to t1 and less than t2. With x specifying the value of max_bit_rate_layer[ i ][ j ], the following condition shall be obeyed for all values of t1:

( x & ( 214 − 1 ) )  >=  bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x  >>  14 ) ) )	(F-5)

constant_pic_rate_idc[ i ][ j ] indicates whether the picture rate of the j-th subset of the i-th layer set is constant. In the following, a temporal segment tSeg is any set of two or more consecutive access units, in decoding order, of the j-th subset of the i-th layer set, auTotal( tSeg ) is the number of access units in the temporal segment tSeg, t1( tSeg ) is the removal time (in seconds) of the first access unit (in decoding order) of the temporal segment tSeg, t2( tSeg ) is the removal time (in seconds) of the last access unit (in decoding order) of the temporal segment tSeg, and avgPicRate( tSeg ) is the average picture rate in the temporal segment tSeg, and is specified as follows:

avgPicRate( tSeg )  = =  Round( auTotal( tSeg ) * 256 ÷ ( t2( tSeg ) − t1( tSeg ) ) )	(F-6)

If the j-th subset of the i-th layer set only contains one or two access units or the value of avgPicRate( tSeg ) is constant over all the temporal segments, the picture rate is constant; otherwise, the picture rate is not constant.

constant_pic_rate_idc[ i ][ j ] equal to 0 indicates that the picture rate of the j-th subset of the i-th layer set is not constant. constant_pic_rate_idc[ i ][ j ] equal to 1 indicates that the picture rate of the j-th subset of the i-th layer set is constant. constant_pic_rate_idc[ i ][ j ] equal to 2 indicates that the picture rate of the j-th subset of the i-th layer set may or may not be constant. The value of constant_pic_rate_idc[ i ][ j ] shall be in the range of 0 to 2, inclusive.

avg_pic_rate[ i ] indicates the average picture rate, in units of picture per 256 seconds, of the j-th subset of the layer set. With auTotal being the number of access units in the j-th subset of the i-th layer set, t1 being the removal time (in seconds) of the first access unit to which the VPS applies, and t2 being the removal time (in seconds) of the last access unit (in decoding order) to which the VPS applies, the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

avg_pic_rate[ i ]  = =  Round( auTotal * 256 ÷ ( t2 − t1 ) )	(F-7)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

avg_pic_rate[ i ]  = =  0		(F-8)

tile_boundaries_aligned_flag[ i ][ j ] equal to 1 indicates that, when any two samples of one picture of the i-th layer specified by the VPS belong to one tile, the two collocated samples, when both present in the picture of the j-th direct reference layer of the i-th layer, belong to one tile, and when any two samples of one picture of the i-th layer belong to different tiles, the two collocated samples, when both present in the picture of the j-th direct reference layer of the i-th layer belong to different tiles. tile_boundaries_aligned_flag equal to 0 indicates that such a restriction may or may not apply.

ilp_restricted_ref_layers_flag equal to 1 indicates that additional restrictions on inter-layer prediction as specified below apply for each direct reference layer of each layer specified by the VPS. ilp_restricted_ref_layers_flag equal to 0 indicates that additional restrictions on inter-layer prediction may or may not apply.

The variables refCtbLog2SizeY[ i ][ j ], refPicWidthInCtbsY[ i ][ j ], and refPicHeightInCtbsY[ i ][ j ] are set equal to CtbLog2SizeY, PicWidthInCtbsY, and PicHeightInCtbsY, respectively, of the j-th direct reference layer of the i-th layer.

min_spatial_segment_offset_plus1[ i ][ j ] indicates the spatial region, in each picture of the j-th direct reference layer of the i-th layer, that is not used for inter-layer prediction for decoding of any picture of the i-th layer, by itself or together with min_horizontal_ctu_offset_plus1[ i ][ j ], as specified below. The value of min_spatial_segment_offset_plus1[ i ][ j ] shall be in the range of 0 to refPicWidthInCtbsY[ i ][ j ] * refPicHeightInCtbsY[ i ][ j ], inclusive. When not present, the value of min_spatial_segment_offset_plus1[ i ][ j ] is inferred to be equal to 0.

ctu_based_offset_enabled_flag[ i ][ j ] equal to 1 specifies that the spatial region, in units of CTUs, in each picture of the j-th direct reference layer of the i-th layer, that is not used for inter-layer prediction for decoding of any picture of the i-th layer is indicated by min_spatial_segment_offset_plus1[ i ][ j ] and min_horizontal_ctu_offset_plus1[ i ][ j ] together. ctu_based_offset_enabled_flag[ i ][ j ] equal to 0 specifies that the spatial region, in units of slice segments, tiles, or CTU rows, in each picture of the j-th direct reference layer of the i-th layer, that is not used for inter-layer prediction for decoding of any picture of the i-th layer is indicated by min_spatial_segment_offset_plus1[ i ] only. When not present, the value of ctu_based_offset_enabled_flag[ i ] is inferred to be equal to 0.

min_horizontal_ctu_offset_plus1[ i ][ j ], when ctu_based_offset_enabled_flag[ i ][ j ] is equal to 1, indicates the spatial region, in each picture of the j-th direct reference layer of the i-th layer, that is not used for inter-layer prediction for decoding of any picture of the i-th layer, together with min_spatial_segment_offset_plus1[ i ][ j ], as specified below. The value of min_horizontal_ctu_offset_plus1[ i ][ j ] shall be in the range of 0 to refPicWidthInCtbsY[ i ][ j ], inclusive.

When ctu_based_offset_enabled_flag[ i ][ j ] is equal to 1, the variable minHorizontalCtbOffset[ i ][ j ] is derived as follows:

minHorizontalCtbOffset[ i ][ j ] = ( min_horizontal_ctu_offset_plus1[ i ][ j ] > 0 ) ?	(F-9)
							( min_horizontal_ctu_offset_plus1[ i ][ j ] – 1 ) : ( refPicWidthInCtbsY[ i ][ j ] - 1 )

The variables curCtbLog2SizeY[ i ], curPicWidthInCtbsY[ i ], and curPicHeightInCtbsY[ i ] are set equal to CtbLog2SizeY, PicWidthInCtbsY, and PicHeightInCtbsY, respectively, of the i-th layer.

The variable colCtbAddr[ i ][ j ] that denotes the raster scan address of the collocated CTU, in a picture in the j-th direct reference layer of the i-th layer, of the CTU with raster scan address equal to ctbAddr in a picture of the i-th layer is derived as follows:

xAddrOfCtb[ i ][ j ] = ( ctbAddr % curPicWidthInCtbsY )  <<  curCtbLog2SizeY	(F-10)

yAddrOfCtb[ i ][ j ] = ( ctbAddr / curPicWidthInCtbsY )  <<  curCtbLog2SizeY	(F-11)

xColCtb[ i ][ j ] = xAddrOfCtb[ i ][ j ]  >>  refCtbLog2SizeY[ i ][ j ]	(F-12)

yColCtb[ i ][ j ] = yAddrOfCtb[ i ][ j ]  >>  refCtbLog2SizeY[ i ][ j ]	(F-13)

colCtbAddr[ i ][ j ] = xColCtb[ i ][ j ] + ( yColCtb[ i ][ j ] * refPicWidthInCtbsY[ i ][ j ] )	(F-14)

When min_spatial_segment_offset_plus1[ i ][ j ] is greater than 0, it is a requirement of bitstream conformance that the following shall apply:

- If ctu_based_offset_enabled_flag[ i ][ j ] is equal to 0, exactly one of the following applies:

- In each PPS referred to by a picture in the j-th direct reference layer of the i-th layer, tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 0, and the following applies:

- Let slice segment A be any slice segment of a picture of the i-th layer and ctbAddr be the raster scan address of the last CTU in slice segment A. Let slice segment B be the slice segment that belongs to the same access unit as slice segment A, belongs to the j-th direct reference layer of the i-th layer, and contains the CTU with raster scan address colCtbAddr[ i ][ j ]. Let slice segment C be the slice segment that is in the same picture as slice segment B and follows slice segment B in decoding order, and between slice segment B and that slice segment there are min_spatial_segment_offset_plus1[ i ] - 1 slice segments in decoding order. When slice segment C is present, the syntax elements of slice segment A are constrained such that no sample or syntax elements values in slice segment C or any slice segment of the same picture following C in decoding order are used for inter-layer prediction in the decoding process of any samples within slice segment A.

- In each PPS referred to by a picture in the j-th direct reference layer of the i-th layer, tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 0, and the following applies:

- Let tile A be any tile in any picture picA of the i-th layer and ctbAddr be the raster scan address of the last CTU in tile A. Let tile B be the tile that is in the picture picB belonging to the same access unit as picA and belonging to the j-th direct reference layer of the i-th layer and that contains the CTU with raster scan address colCtbAddr[ i ][ j ]. Let tile C be the tile that is also in picB and follows tile B in decoding order, and between tile B and that tile there are min_spatial_segment_offset_plus1[ i ] - 1 tiles in decoding order. When slice segment C is present, the syntax elements of tile A are constrained such that no sample or syntax elements values in tile C or any tile of the same picture following C in decoding order are used for inter-layer prediction in the decoding process of any samples within tile A.

- In each PPS referred to by a picture in the j-th direct reference layer of the i-th layer, tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 1, and the following applies:

- Let CTU row A be any CTU row in any picture picA of the i-th layer and ctbAddr be the raster scan address of the last CTU in CTU row A. Let CTU row B be the CTU row that is in the picture picB belonging to the same access unit as picA and belonging to the j-th direct reference layer of the i-th layer and that contains the CTU with raster scan address colCtbAddr[ i ][ j ]. Let CTU row C be the CTU row that is also in picB and follows CTU row B in decoding order, and between CTU row B and that CTU row there are min_spatial_segment_offset_plus1[ i ] - 1 CTU rows in decoding order. When CTU row C is present, the syntax elements of CTU row A are constrained such that no sample or syntax elements values in CTU row C or row of the same picture following C are used for inter-layer prediction in the decoding process of any samples within CTU row A.

- Otherwise (ctu_based_offset_enabled_flag[ i ][ j ] is equal to 1), the following applies:

- The variable refCtbAddr[ i ][ j ] is derived as follows:

xOffset[ i ][ j ] = ( ( xColCtb[ i ][ j ] + minHorizontalCtbOffset[ i ][ j ] ) > ( refPicWidthInCtbsY[ i ][ j ] - 1 ) ) ? 		( refPicWidthInCtbsY[ i ][ j ] - 1 –xColCtb[ i ][ j ] ) : ( minHorizontalCtbOffset[ i ][ j ] )
		(F-15)

yOffset[ i ][ j ] = ( min_spatial_segment_offset_plus1[ i ][ j ] – 1 ) * refPicWidthInCtbsY[ i ][ j ]	(F-16)

refCtbAddr[ i ][ j ] = colCtbAddr[ i ][ j ] + xOffset[ i ][ j ] + yOffset[ i ][ j ]	(F-17)

- Let CTU A be any CTU in any picture picA of the i-th layer, and ctbAddr be the raster scan address ctbAddr of CTU A. Let CTU B be a CTU that is in the picture belonging to the same access unit as picA and belonging to the j-th direct reference layer of the i-th layer and that has raster scan address greater than refCtbAddr[ i ][ j ]. When CTU B is present, the syntax elements of CTU A are constrained such that no sample or syntax elements values in CTU B are used for inter-layer prediction in the decoding process of any samples within CTU A.

        - Sequence parameter set RBSP semantics

The specifications in subclause 7.4.3.2 apply, with following additions and modifications.

NOTE 1 – All SPSs, regardless of the values of their nuh_layer_id, share the same value space for sps_seq_parameter_set_id. In other words, an SPS with nuh_layer_id equal to X and sps_seq_parameter_set_id equal to A would update the previously received SPS with nuh_layer_id not equal to X and sps_seq_parameter_set_id equal to A.

sps_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in each CVS referring to the SPS. The value of sps_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive. When not present sps_max_sub_layers_minus1 is inferred to be equal to vps_max_sub_layers_minus1.

update_rep_format_flag equal to 1 specifies that the syntax elements chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 are explicitly signalled in the SPS and all the layers with nuh_layer_id greater than zero that refer to this SPS use these values instead of those signalled in the VPS when the nuh_layer_id of the SPS is greater than 0. update_rep_format_flag equal to 0 specifies that the syntax elements chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 are not signalled in the SPS and all the layers that refer to this SPS use the values signaled in the VPS. When not present, the value of update_rep_format_flag is inferred to be equal to 1.

When a current picture with nuh_layer_id layerIdCurr greater than 0 refers to an SPS, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 are inferred or constrained as follows:

–	If the nuh_layer_id of the active layer SPS is equal to 0, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 are inferred to be equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS where j is equal to LayerIdxInVps[ layerIdCurr ] and the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 of the active layer SPS are ignored.

NOTE 2 – The values are inferred from the VPS when an active non-base layer references an SPS which is also used by the base layer, in which case the SPS has nuh_layer_id equal to 0. For an active base layer, the values in the active SPS apply.

–	Otherwise (the nuh_layer_id of the active layer SPS is greater than zero), the following applies:

–	If update_rep_format_flag is equal to 0, the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 are inferred to be equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, and bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

–	Otherwise (update_rep_format_flag is equal to 1), it is a requirement of bitstream conformance that the value of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, or bit_depth_chroma_minus8 shall be less than or equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, or bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in subclause 6.2. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive. The value of chroma_format_idc shall be less than or equal to chroma_format_vps_idc.

separate_colour_plane_flag equal to 1 specifies that the three colour components of the 4:4:4 chroma format are coded separately. separate_colour_plane_flag equal to 0 specifies that the colour components are not coded separately. When separate_colour_plane_flag is not present, it is inferred to be equal to 0. When separate_colour_plane_flag is equal to 1, the coded picture consists of three separate components, each of which consists of coded samples of one colour plane (Y, Cb, or Cr) and uses the monochrome coding syntax. In this case, each colour plane is associated with a specific colour_plane_id value. The value of separate_colour_plane_flag shall be less than or equal to separate_colour_plane_vps_flag

NOTE 3 – There is no dependency in decoding processes between the colour planes having different colour_plane_id values. For example, the decoding process of a monochrome picture with one value of colour_plane_id does not use any data from monochrome pictures having different values of colour_plane_id for inter prediction.

Depending on the value of separate_colour_plane_flag, the value of the variable ChromaArrayType is assigned as follows:

–	If separate_colour_plane_flag is equal to 0, ChromaArrayType is set equal to chroma_format_idc.

–	Otherwise (separate_colour_plane_flag is equal to 1), ChromaArrayType is set equal to 0.

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. The value of pic_width_in_luma_samples shall be less than or equal to pic_width_vps_in_luma_samples.

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. The value of pic_height_in_luma_samples shall be less than or equal to pic_height_vps_in_luma_samples.

bit_depth_luma_minus8 specifies the bit depth of the samples of the luma array BitDepthY and the value of the luma quantization parameter range offset QpBdOffsetY as follows:

BitDepthY	= 8 + bit_depth_luma_minus8		(F-18)

QpBdOffsetY	= 6 * bit_depth_luma_minus8		(F-19)

bit_depth_luma_minus8 shall be in the range of 0 to 6, inclusive. bit_depth_luma_minus8 shall be less than or equal to bit_depth_vps_luma_minus8.

bit_depth_chroma_minus8 specifies the bit depth of the samples of the chroma arrays BitDepthC and the value of the chroma quantization parameter range offset QpBdOffsetC as follows:

BitDepthC	= 8 + bit_depth_chroma_minus8		(F-20)

QpBdOffsetC	= 6 * bit_depth_chroma_minus8		(F-21)

bit_depth_chroma_minus8 shall be in the range of 0 to 6, inclusive. bit_depth_chroma_minus8 shall be less than or equal to bit_depth_vps_chroma_minus8.

sps_infer_scaling_list_flag equal to 1 specifies that the syntax elements of the scaling list data syntax structure of the SPS are inferred to be equal to those of the SPS that is active for the layer with nuh_layer_id equal to sps_scaling_list_ref_layer_id. sps_infer_scaling_list_flag equal to 0 specifies that the syntax elements of the scaling list data syntax structure are not inferred. When not present, the value of sps_infer_scaling_list_flag is inferred to be 0.

sps_scaling_list_ref_layer_id specifies the value of the nuh_layer_id of the layer for which the active SPS is associated with the same scaling list data as the current SPS.

The value of sps_scaling_list_ref_layer_id shall be in the range of 0 to 62, inclusive.

When avc_base_layer_flag is equal to 1, it is a requirement of bitstream conformance that the value of sps_scaling_list_ref_layer_id shall be greater than 0.

It is a requirement of bitstream conformance that, when an SPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB and sps_infer_scaling_list_flag in the SPS is equal to 1, sps_infer_scaling_list_flag shall be equal to 0 for the SPS that is active for the layer with nuh_layer_id equal to sps_scaling_list_ref_layer_id.

It is a requirement of bitstream conformance that, when an SPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB, the layer with nuh_layer_id equal to sps_scaling_list_ref_layer_id shall be a direct or indirect reference layer of the layer with nuh_layer_id equal to nuhLayerIdB.

sps_scaling_list_data_present_flag equal to 1 specifies that the scaling list data syntax structure is present in the SPS. sps_scaling_list_data_present_flag equal to 0 specifies that the scaling list data syntax structure is not present in the SPS. When not present, the value of sps_scaling_list_data_present_flag is inferred to be equal to 0.

sps_temporal_id_nesting_flag, when sps_max_sub_layers_minus1 is greater than 0, specifies whether inter prediction is additionally restricted for CVSs referring to the SPS. When vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag shall be equal to 1. When sps_max_sub_layers_minus1 is equal to 0, sps_temporal_id_nesting_flag shall be equal to 1. When not present sps_temporal_id_nesting_flag is inferred to be equal to vps_temporal_id_nesting_flag.

NOTE 4 – The syntax element sps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e. switching from decoding up to any TemporalId tIdN to decoding up to any TemporalId tIdM that is greater than tIdN, is always possible in the CVS.

sps_extension_flag equal to 0 specifies that no sps_extension( ) syntax structure is present in the SPS RBSP syntax structure. sps_extension_flag equal to 1 specifies that the sps_extension( ) syntax structure is present in the SPS RBSP syntax structure.

sps_extension2_flag equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. sps_extension2_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for sps_extension2_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all sps_extension_data_flag syntax elements that follow the value 1 for sps_extension2_flag in an SPS NAL unit.

          - Sequence parameter set extension semantics

inter_view_mv_vert_constraint_flag equal to 1 specifies that vertical component of motion vectors used for inter-layer prediction are constrained in the CVS. When inter_view_mv_vert_constraint_flag is equal to 1, the vertical component of the motion vectors used for inter-layer prediction shall be equal to or less than 56 in units of luma samples. When inter_view_mv_vert_constraint_flag is equal to 0, no constraint for of the vertical component of the motion vectors used for inter-layer prediction is signalled by this flag. When not present, the inter_view_mv_vert_constraint_flag is inferred to be equal to 0.

sps_shvc_reserved_zero_idc shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for sps_shvc_reserved_zero_idc are reserved for future use by ITU-T | ISO/IEC. Although the value of sps_shvc_reserved_zero_idc is required to be equal to 0 in this version of this Specification, decoders shall allow other values of sps_shvc_reserved_zero_idc to appear in the syntax.

NOTE – It is anticipated that in future scalable extensions of this Specification, this field will be used to indicate that the scaled reference layer offset parameters are present in the SPS.

        - Picture parameter set RBSP semantics

The specifications in subclause 7.4.3.3 apply, with the following modifications:

NOTE – All PPSs, regardless of the values of their nuh_layer_id, share the same value space for pps_pic_parameter_set_id. In other words, a PPS with nuh_layer_id equal to X and pps_pic_parameter_set_id equal to A would update the previously received PPS with nuh_layer_id not equal to X and pps_pic_parameter_set_id equal to A.

num_extra_slice_header_bits specifies the number of extra slice header bits that are present in the slice header RBSP for coded pictures referring to the PPS. num_extra_slice_header_bits shall be in the range of 0 to 2, inclusive, in bitstreams conforming to this version of this Specification. Other values for num_extra_slice_header_bits are reserved for future use by ITU-T | ISO/IEC. However, decoders shall allow num_extra_slice_header_bits to have any value.

When cross_layer_irap_aligned_flag equal to 0, it is a requirement of bitstream conformance that num_extra_slice_header_bits shall be greater than or equal to 1.

pps_infer_scaling_list_flag equal to 1 specifies that the syntax elements of the scaling list data syntax structure of the PPS are inferred to be equal to those of the PPS that is active for the layer with nuh_layer_id equal to pps_scaling_list_ref_layer_id. pps_infer_scaling_list_flag equal to 0 specifies that the syntax elements of the scaling list data syntax structure of the PPS are not inferred. When not present, the value of pps_infer_scaling_list_flag is inferred to be 0.

pps_scaling_list_ref_layer_id specifies the value of the nuh_layer_id of the layer for which the active PPS is associated with the same scaling list data as the current PPS.

The value of pps_scaling_list_ref_layer_id shall be in the range of 0 to 62, inclusive.

When avc_base_layer_flag is equal to 1, it is a requirement of bitstream conformance that pps_scaling_list_ref_layer_id shall be greater than 0.

It is a requirement of bitstream conformance that, when a PPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB and pps_infer_scaling_list_flag in the PPS is equal to 1, pps_infer_scaling_list_flag shall be equal to 0 for the PPS that is active for the layer with nuh_layer_id equal to pps_scaling_list_ref_layer_id.

It is a requirement of bitstream conformance that, when a PPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB, the layer with nuh_layer_id equal to pps_scaling_list_ref_layer_id shall be a direct or indirect reference layer of the layer with nuh_layer_id equal to nuhLayerIdB.

pps_scaling_list_data_present_flag equal to 1 specifies that parameters are present in the PPS to modify the scaling lists specified by the active SPS. pps_scaling_list_data_present_flag equal to 0 specifies that the scaling list data used for the pictures referring to the PPS are inferred to be equal to those specified by the active SPS. When scaling_list_enabled_flag is equal to 0, the value of pps_scaling_list_data_present_flag shall be equal to 0. When scaling_list_enabled_flag is equal to 1, sps_scaling_list_data_present_flag is equal to 0, and pps_scaling_list_data_present_flag is equal to 0, the default scaling list data are used to derive the array ScalingFactor as described in the scaling list data semantics 7.4.5.

        - Supplemental enhancement information RBSP semantics

The specifications in subclause 7.4.3.4 apply.

        - Access unit delimiter RBSP semantics

The specifications in subclause 7.4.3.5 apply.

        - End of sequence RBSP semantics

The specifications in subclause 7.4.3.6 apply.

        - End of bitstream RBSP semantics

The specifications in subclause 7.4.3.7 apply.

        - Filler data RBSP semantics

The specifications in subclause 7.4.3.8 apply.

        - Slice segment layer RBSP semantics

The specifications in subclause 7.4.3.9 apply.

        - RBSP slice segment trailing bits semantics

The specifications in subclause 7.4.3.10 apply.

        - RBSP trailing bits semantics

The specifications in subclause 7.4.3.11 apply.

        - Byte alignment semantics

The specifications in subclause 7.4.3.12 apply.

      - Profile, tier and level semantics

The profile_tier_level( ) syntax structure provides profile, tier and level information used for a layer set. When the profile_tier_level( ) syntax structure is included in a vps_extension( ) syntax structure, the applicable layer set to which the profile_tier_level( ) syntax structure applies is specified by the corresponding lsIdx variable in the vps_extension( ) syntax structure. When the profile_tier_level( ) syntax structure is included in a VPS, but not in a vps_extension( ) syntax structure, the applicable layer set to which the profile_tier_level( ) syntax structure applies is the layer set specified by the index 0. When the profile_tier_level( ) syntax structure is included in an SPS, the layer set to which the profile_tier_level( ) syntax structure applies is the layer set specified by the index 0.

For interpretation of the following semantics, CVS refers to the CVS subset associated with the layer set to which the profile_tier_level( ) syntax structure applies.

When the syntax elements general_profile_space, general_tier_flag, general_profile_idc, general_profile_compatibility_flag[ j ], general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag, general_frame_only_constraint_flag, general_reserved_zero_44bits are not present for the applicable layer set, they are inferred to be equal to the corresponding values of the layer set specified by the index ( profile_layer_set_ref_minus1[ lsIdx ] +1 ).

When the syntax elements sub_layer_profile_space[ i ], sub_layer_tier_flag[ i ], sub_layer_profile_idc[ i ], sub_layer_profile_compatibility_flag[ i ][ j ], sub_layer_progressive_source_flag[ i ], sub_layer_interlaced_source_flag[ i ], sub_layer_non_packed_constraint_flag[ i ], sub_layer_frame_only_constraint_flag[ i ], sub_layer_reserved_zero_44bits[ i ] are not present for the applicable layer set, and they are present in or inferred for the layer set specified by the index ( profile_layer_set_ref_minus1[ lsIdx ] +1 ) they are inferred to be equal to the corresponding values of the layer set specified by the index ( profile_layer_set_ref_minus1[ lsIdx ] +1 ).

The specifications in subclause 7.4.4 apply, with following modifications.

general_tier_flag specifies the tier context for the interpretation of general_level_idc as specified in Annex A or subclause E.11.

general_profile_idc, when general_profile_space is equal to 0, indicates a profile to which the CVS conforms as specified in Annex A or in subclause E.11. Bitstreams shall not contain values of general_profile_idc other than those specified in Annex A or subclause E.11. Other values of general_profile_idc are reserved for future use by ITU-T | ISO/IEC.

general_profile_compatibility_flag[ j ] equal to 1, when general_profile_space is equal to 0, indicates that the CVS conforms to the profile indicated by general_profile_idc equal to i as specified in Annex A or in subclause E.11. When general_profile_space is equal to 0, general_profile_compatibility_flag[ general_profile_idc ] shall be equal to 1. The value of general_profile_compatibility_flag[ j ] shall be equal to 0 for any value of j that is not specified as an allowed value of general_profile_idc in Annex A or in subclause E.11.

general_level_idc indicates a level to which the CVS conforms as specified in Annex A or subclause E.11. Bitstreams shall not contain values of general_level_idc other than those specified in Annex A or subclause E.11. Other values of general_level_idc are reserved for future use by ITU-T | ISO/IEC.

sub_layer_profile_present_flag[ i ] equal to 1, specifies that profile information is present in the profile_tier_level( ) syntax structure for the representation of the sub-layer with TemporalId equal to i. sub_layer_profile_present_flag[ i ] equal to 0 specifies that profile information is not present in the profile_tier_level( ) syntax structure for the representations of the sub-layer with TemporalId equal to i. When profilePresentFlag is equal to 0, sub_layer_profile_present_flag[ i ] shall be equal to 0.

      - Scaling list data semantics

The specifications in subclause 7.4.5 apply.

      - Supplemental enhancement information message semantics

The specifications in subclause 7.4.6 apply.

      - Slice segment header semantics

        - General slice segment header semantics

The specifications in subclause 7.4.7.1 apply with the following modifications.

–	"When nal_unit_type has a value in the range of 16 to 23, inclusive (IRAP picture), slice_type shall be equal to 2." is replaced by "When nal_unit_type has a value in the range of 16 to 23 and nuh_layer_id is equal to 0, inclusive (IRAP picture), slice_type shall be equal to 2."

poc_reset_flag equal to 1 specifies that the derived picture order count for the current picture is equal to 0. poc_reset_flag equal to 0 specifies that the derived picture order count for the current picture may or may not be equal to 0. It is a requirement of bitstream conformance that when cross_layer_irap_aligned_flag is equal to 1, the value of poc_reset_flag shall be equal to 0. When not present, the value of poc_reset_flag is inferred to be equal to 0.

discardable_flag equal to 1 specifies that the coded picture is not used as a reference picture for inter prediction and is not used as an inter-layer reference picture in the decoding process of subsequent pictures in decoding order. discardable_flag equal to 0 specifies that the coded picture may be used as a reference picture for inter prediction and may be used as an inter-layer reference picture in the decoding process of subsequent pictures in decoding order. When not present, the value of discardable_flag is inferred to be equal to 0.

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture.

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictures that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0 )
	NumActiveRefLayerPics = 0
else if( all_ref_layers_active_flag )
	NumActiveRefLayerPics = NumDirectRefLayers[ nuh_layer_id ]
else if( !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element inter_layer_pred_layer_idc[ i ] is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive. When not present, the value of inter_layer_pred_layer_idc[ i ] is inferred to be equal to i.

When i is greater than 0, inter_layer_pred_layer_idc[ i ] shall be greater than inter_layer_pred_layer_idc[ i − 1 ].

The variables RefPicLayerId[ i ] for all values of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, are derived as follows:

for( i = 0, j = 0; i < NumActiveRefLayerPics; i++)
	RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

All slices of a picture shall have the same value of inter_layer_pred_layer_idc[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, either of the following two conditions shall be true:

–	The value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] is greater than TemporalId.

–	The values of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] and TemporalId are both equal to 0 and the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is an IRAP picture.

        - Reference picture list modification semantics

The specifications in subclause 7.4.7.2 apply with following modifications.

–	Equation (7-43) specifying the derivation of NumPicTotalCurr is replaced by:

NumPicTotalCurr = 0
for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++)
	if(UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
		NumPicTotalCurr++
for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++)		(F-22)
	if(UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		NumPicTotalCurr++
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
	if( UsedByCurrPicLt[ i ] )
		NumPicTotalCurr++
NumPicTotalCurr  +=  NumActiveRefLayerPics

        - Weighted prediction parameters semantics

The specifications in subclause 7.4.7.3 apply.

      - Short-term reference picture set semantics

The specifications in subclause 7.4.8 apply.

      - Slice segment data semantics

        - General slice segment data semantics

The specifications in subclause 7.4.9.1 apply.

        - Coding tree unit semantics

The specifications in subclause 7.4.9.2 apply.

        - Sample adaptive offset semantics

The specifications in subclause 7.4.9.3 apply.

        - Coding quadtree semantics

The specifications in subclause 7.4.9.4 apply.

        - Coding unit semantics

The specifications in subclause 7.4.9.5 apply.

        - Prediction unit semantics

The specifications in subclause 7.4.9.6 apply.

        - PCM sample semantics

The specifications in subclause 7.4.9.7 apply.

        - Transform tree semantics

The specifications in subclause 7.4.9.8 apply.

        - Motion vector difference semantics

The specifications in subclause 7.4.9.9 apply.

        - Transform unit semantics

The specifications in subclause 7.4.9.10 apply.

        - Residual coding semantics

The specifications in subclause 7.4.9.11 apply.

  - Decoding process

    - General decoding process

The specifications in subclause 8.1 apply with following additions.

The variable TargetOutputLayerSetIdx is specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetOutputLayerSetIdx, TargetOutputLayerSetIdx is set by the external means.

–	Otherwise, if the decoding process is invoked in a bitstream conformance test as specified in subclause C.1, TargetOutputLayerSetIdx is  set as specified in subclause C.1. [Ed. (MH): The cross-reference here assumes that clause F.13 will be integrated to Annex C.]

––	Otherwise, TargetOutputLayerSetIdx is set equal to 0.

When the current picture has nuh_layer_id greater than 0, the following applies.

–	Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the following decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the following decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb, and SCr, respectively.

NOTE – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

–	The decoding process operates as follows for the current picture CurrPic.

–	For the decoding of the slice segment header of the first slice, in decoding order, of the current picture, the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause  is invoked.

–	When ViewScalExtLayerFlag[ nuh_layer_id ] is equal to 1, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause E.8.1 is invoked.

–	After all slices of the current picture have been decoded, the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause D.8.1.2 is invoked.

When the decoding of the last picture of an access unit has been completed, PicOutputFlag for each picture in the access unit is updated as follows:

for( i = 0; i <= 63; i++ )
	outputLayerIdNeeded[ i ] = 0
for( i = 0; i <= 63; i++ )
	if( OutputLayerFlag[ TargetOutputLayerSetIdx ][ i ] )
		if( picture with nuh_layer_id equal to i is present in the access unit  &&  
			PicOutputFlag of the picture with with nuh_layer_id equal to i in the access unit is equal to 1 ) 
			outputLayerIdNeeded[ i ] = 1
		else {
			foundFlag = 0
			for( j = i – 1; j >= MinAltOutputLayerId[ TargetOutputLayerSetIdx ][ i ]  &&  !foundFlag; j--) 
				if( RecursiveRefLayerFlag[ i ][ j ] &&
					picture with nuh_layer_id equal to j is present in the access unit &&
					PicOutputFlag of the picture with with nuh_layer_id equal to i in access unit is equal to 1  ) {
					outputLayerIdNeeded[ j ] = 1
					foundFlag = 1
				}
		}
for( i = 0; i <= 63; i++ )
	if( picture with nuh_layer_id equal to i is present in the access unit &&
		PicOutputFlag of the picture with nuh_layer_id equal to i in the access unit is equal to 1 &&
		outputLayerIdNeeded[ i ]  is equal to 0 )
		PicOutputFlag of of the picture with nuh_layer_id equal to i in the access unit is set equal to 0

      - Decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0

Each picture referred to in this subclause is a complete coded picture.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause 4.

- The processes in subclause D.8.3 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause D.8.3.1. This needs to be invoked only for the first slice segment of a picture. It is a requirement of bitstream conformance that PicOrderCntVal shall remain unchanged within an access unit.

–	The decoding process for RPS in subclause D.8.3.2 is invoked, wherein only reference pictures with a nuh_layer_id equal to that of CurrPic may be marked as "unused for reference" or "used for long-term reference" and any picture with a different value of nuh_layer_id is not marked. This needs to be invoked only for the first slice segment of a picture.

–	When FirstPicInLayerDecodedFlag[ nuh_layer_id ] is equal to 0, the decoding process for generating unavailable reference pictures specified in subclause D.8.1.3 is invoked, which needs to be invoked only for the first slice segment of a picture.

      - Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0

PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, if LayerInitialisedFlag[ nuh_layer_id ] is equal to 0, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

The following applies:

–	If discardable_flag is equal to 1, the decoded picture is marked as "unused for reference".

–	Otherwise, the decoded picture is marked as "used for short-term reference".

When TemporalId is equal to HighestTid, the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in subclause D.8.1.2.1 is invoked with latestDecLayerId equal to nuh_layer_id as input.

FirstPicInLayerDecodedFlag[ nuh_layer_id ] is set equal to 1.

        - Marking process for sub-layer non-reference pictures not needed for inter-layer prediction

Input to this process is:

–	a nuh_layer_id value latestDecLayerId

Output of this process is:

–	potentially updated marking as "unused for reference" for some decoded pictures

NOTE – This process marks pictures that are not needed for inter or inter-layer prediction as "unused for reference". When TemporalId is less than HighestTid, the current picture may be used for reference in inter prediction and this process is not invoked.

- The variables numTargetDecLayers, and latestDecIdx are derived as follows:

–	numTargetDecLayers is set equal to the number of entries in TargetDecLayerIdList.

–	latestDecIdx is set equal to the value of i for which TargetDecLayerIdList[ i ] is equal to latestDecLayerId.

- For i in the range of 0 to latestDecIdx, inclusive, the following applies for marking of pictures as "unused for reference":

–	Let currPic be the picture in the current access unit with nuh_layer_id equal to TargetDecLayerIdList[ i ].

–	When currPic is marked as "used for reference" and is a sub-layer non-reference picture, the following applies:

–	The variable currTid is set equal to the value of TemporalId of currPic.

–	The variable remainingInterLayerReferencesFlag is derived as specified in the following:

remainingInterLayerReferencesFlag = 0
		if ( currTid  <=  ( max_tid_il_ref_pics_plus1[ LayerIdxInVps[ TargetDecLayerIdList[ i ] ] ] –1 ) )
			for( j = latestDecIdx + 1; j < numTargetDecLayers; j++ )
				for( k = 0; k < NumDirectRefLayers[ TargetDecLayerIdList[ j ] ]; k++ )
					if( TargetDecLayerIdList[ i ]  = =  RefLayerId[ TargetDecLayerIdList[ j ] ][ k ] )
						remainingInterLayerReferencesFlag = 1

–	When remainingInterLayerReferenceFlag is equal to 0, currPic is marked as "unused for reference".

      - Generation of unavailable reference pictures for pictures first in decoding order within a layer

- This process is invoked for a picture with nuh_layer_id equal to layerId, when FirstPicInLayerDecodedFlag[ layerId ] is equal to 0.

NOTE – A cross-layer random access skipped (CL-RAS) picture is a picture with nuh_layer_id equal to layerId such that LayerInitialisedFlag[ layerId ] is equal to 0 when the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 is invoked. The entire specification of the decoding process for CL-RAS pictures is included only for purposes of specifying constraints on the allowed syntax content of such CL-RAS pictures. During the decoding process, any CL-RAS pictures may be ignored, as these pictures are not specified for output and have no effect on the decoding process of any other pictures that are specified for output. However, in HRD operations as specified in Annex , CL-RAS pictures may need to be taken into consideration in derivation of CPB arrival and removal times.

When this process is invoked, the following applies:

–	For each RefPicSetStCurrBefore[ i ], with i in the range of 0 to NumPocStCurrBefore – 1, inclusive, that is equal to “no-reference picture”, a picture is generated as specified in subclause 8.3.3.2, and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocStCurrBefore[ i ].

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for short-term reference".

–	RefPicSetStCurrBefore[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

–	For each RefPicSetStCurrAfter[ i ], with i in the range of 0 to NumPocStCurrAfter – 1, inclusive, that is equal to “no-reference picture”, a picture is generated as specified in subclause 8.3.3.2, and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocStCurrAfter[ i ].

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for short-term reference".

–	RefPicSetStCurrAfter[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

–	For each RefPicSetStFoll[ i ], with i in the range of 0 to NumPocStFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.3.2, and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocStFoll[ i ].

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for short-term reference".

–	RefPicSetStFoll[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

–	For each RefPicSetLtCurr[ i ], with i in the range of 0 to NumPocLtCurr – 1, inclusive, that is equal to “no-reference picture”, a picture is generated as specified in subclause 8.3.3.2, and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocLtCurr[ i ].

–	The value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( PocLtCurr[ i ] & ( MaxPicOrderCntLsb – 1 ) ).

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for long-term reference".

–	RefPicSetLtCurr[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

–	For each RefPicSetLtFoll[ i ], with i in the range of 0 to NumPocLtFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.3.2, and the following applies:

–	The value of PicOrderCntVal for the generated picture is set equal to PocLtFoll[ i ].

–	The value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( PocLtFoll[ i ] & ( MaxPicOrderCntLsb – 1 ) ).

–	The value of PicOutputFlag for the generated picture is set equal to 0.

–	The generated picture is marked as "used for long-term reference".

–	RefPicSetLtFoll[ i ] is set to be the generated reference picture.

–	The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

    - NAL unit decoding process

The specifications in subclause 8.2 apply.

    - Slice decoding processes

      - Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in merge mode and motion vector prediction, and for decoder conformance checking (see subclause C.5).

Each coded picture is associated with a picture order count variable, denoted as PicOrderCntVal.

- When the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows:

- Let prevTid0Pic be the previous picture in decoding order that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture, and let prevPicOrderCnt be equal to PicOrderCntVal of prevTid0Pic.

- The variable prevPicOrderCntLsb is set equal to prevPicOrderCnt & ( MaxPicOrderCntLsb − 1 ).

- The variable prevPicOrderCntMsb is set equal to prevPicOrderCnt − prevPicOrderCntLsb.

- The variable PicOrderCntMsb of the current picture is derived as follows:

- If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, PicOrderCntMsb is set equal to 0.

- Otherwise, PicOrderCntMsb is derived as follows:

if( ( slice_pic_order_cnt_lsb < prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − slice_pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(F-23)
else if( (slice_pic_order_cnt_lsb > prevPicOrderCntLsb )  &&
		( (slice_pic_order_cnt_lsb − prevPicOrderCntLsb ) > ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as follows:

PicOrderCntVal = PicOrderCntMsb + slice_pic_order_cnt_lsb	(F-24)

NOTE 1 – All IDR pictures will have PicOrderCntVal equal to 0 since slice_pic_order_cnt_lsb is inferred to be 0 for IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

When poc_reset_flag is equal to 1, the following steps apply in the order listed:

- The PicOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture is decremented by PicOrderCntVal.

- PicOrderCntVal is set equal to 0.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive. In one CVS, the PicOrderCntVal values for any two coded pictures in the same layer shall not be the same.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(F-25)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(F-26)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that are not in the range of −215 to 215 − 1, inclusive.

NOTE 2 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

      - Decoding process for reference picture set

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction for the slice as specified in subclause 8.3.3. This process may result in one or more reference pictures in the DPB being marked as "unused for reference" or "used for long-term reference". This subclause marks only the pictures with the same value of nuh_layer_id and does not mark any picture with a nuh_layer_id different from the current picture.

NOTE 1 – The RPS is an absolute description of the reference pictures used in the decoding process of the current and future coded pictures. The RPS signalling is explicit in the sense that all reference pictures included in the RPS are listed explicitly.

A decoded picture in the DPB can be marked as "unused for reference", "used for short-term reference", or "used for long-term reference", but only one among these three at any given moment during the operation of the decoding process. Assigning one of these markings to a picture implicitly removes another of these markings when applicable. When a picture is referred to as being marked as "used for reference", this collectively refers to the picture being marked as "used for short-term reference" or "used for long-term reference" (but not both).

The variable currPicLayerId is set to be the nuh_layer_id of the current picture.

When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, all reference pictures with nuh_layer_id equal to currPicLayerId currently in the DPB (if any) are marked as "unused for reference".

Short-term reference pictures are identified by their PicOrderCntVal values. Long-term reference pictures are identified either by their PicOrderCntVal values or their slice_pic_order_cnt_lsb values.

Five lists of picture order count values are constructed to derive the RPS. These five lists are PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll, with NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll number of elements, respectively. The five lists and the five variables are derived as follows:

- If the current picture is an IDR picture, PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll are all set to be empty, and NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll are all set equal to 0.

- Otherwise, the following applies:

for( i = 0, j = 0, k = 0; i < NumNegativePics[ CurrRpsIdx ] ; i++ )
	if( UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
		PocStCurrBefore[ j++ ] = PicOrderCntVal + DeltaPocS0[ CurrRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ CurrRpsIdx ][ i ]
NumPocStCurrBefore = j

for( i = 0, j = 0; i < NumPositivePics[ CurrRpsIdx ]; i++ )
	if( UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		PocStCurrAfter[ j++ ] = PicOrderCntVal + DeltaPocS1[ CurrRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ CurrRpsIdx ][ i ]
NumPocStCurrAfter = j
NumPocStFoll = k		(F-27)
for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {
	pocLt = PocLsbLt[ i ]
	if( delta_poc_msb_present_flag[ i ] )
		pocLt  +=  PicOrderCntVal − 	DeltaPocMsbCycleLt[ i ] * MaxPicOrderCntLsb −
				PicOrderCntVal & ( MaxPicOrderCntLsb − 1 )
	if( UsedByCurrPicLt[ i ] ) {
		PocLtCurr[ j ] = pocLt
		CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]
	} else {
		PocLtFoll[ k ] = pocLt
		FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]
	}
}
NumPocLtCurr = j
NumPocLtFoll = k

where PicOrderCntVal is the picture order count of the current picture as specified in subclause D.8.3.1.

NOTE 2 – A value of CurrRpsIdx in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive, indicates that a candidate short-term RPS from the active SPS is being used, where CurrRpsIdx is the index of the candidate short-term RPS into the list of candidate short-term RPSs signalled in the active SPS. CurrRpsIdx equal to num_short_term_ref_pic_sets indicates that the short-term RPS of the current picture is directly signalled in the slice header.

For each i in the range of 0 to NumPocLtCurr − 1, inclusive, when CurrDeltaPocMsbPresentFlag[ i ] is equal to 1, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStCurrBefore[ j ].

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStCurrAfter[ j ].

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, where j is not equal to i, for which PocLtCurr[ i ] is equal to PocLtCurr[ j ].

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when FollDeltaPocMsbPresentFlag[ i ] is equal to 1, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStCurrBefore[ j ].

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStCurrAfter[ j ].

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtFoll − 1, inclusive, where j is not equal to i, for which PocLtFoll[ i ] is equal to PocLtFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, for which PocLtFoll[ i ] is equal to PocLtCurr[ j ].

For each i in the range of 0 to NumPocLtCurr − 1, inclusive, when CurrDeltaPocMsbPresentFlag[ i ] is equal to 0, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStCurrBefore[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStCurrAfter[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, where j is not equal to i, for which PocLtCurr[ i ] is equal to ( PocLtCurr[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when FollDeltaPocMsbPresentFlag[ i ] is equal to 0, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStCurrBefore[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStCurrAfter[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocLtFoll − 1, inclusive, where j is not equal to i, for which PocLtFoll[ i ] is equal to ( PocLtFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocLtCurr[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

The variable NumPicTotalCurr is derived as specified in subclause 7.4.7.2. It is a requirement of bitstream conformance that the following applies to the value of NumPicTotalCurr:

- If nuh_layer_id is equal to 0 and the current picture is a BLA picture or a CRA picture, the value of NumPicTotalCurr shall be equal to 0.

- Otherwise, when the current picture contains a P or B slice, the value of NumPicTotalCurr shall not be equal to 0.

The RPS of the current picture consists of five RPS lists; RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetStFoll, RefPicSetLtCurr and RefPicSetLtFoll. RefPicSetStCurrBefore, RefPicSetStCurrAfter, and RefPicSetStFoll are collectively referred to as the short-term RPS. RefPicSetLtCurr and RefPicSetLtFoll are collectively referred to as the long-term RPS.

NOTE 3 – RefPicSetStCurrBefore, RefPicSetStCurrAfter, and RefPicSetLtCurr contain all reference pictures that may be used for inter prediction of the current picture and one or more pictures that follow the current picture in decoding order. RefPicSetStFoll and RefPicSetLtFoll consist of all reference pictures that are not used for inter prediction of the current picture but may be used in inter prediction for one or more pictures that follow the current picture in decoding order.

The derivation process for the RPS and picture marking are performed according to the following ordered steps:

- The following applies:

for( i = 0; i < NumPocLtCurr; i++ )
	if( !CurrDeltaPocMsbPresentFlag[ i ] )
		if( there is a reference picture picX in the DPB with slice_pic_order_cnt_lsb equal to PocLtCurr[ i ] and nuh_layer_id equal to currPicLayerId)
			RefPicSetLtCurr[ i ] = picX
		else
			RefPicSetLtCurr[ i ] = "no reference picture"
	else
		if( there is a reference picture picX in the DPB with PicOrderCntVal equal to PocLtCurr[ i ] and nuh_layer_id equal to currPicLayerId)
			RefPicSetLtCurr[ i ] = picX
		else
			RefPicSetLtCurr[ i ] = "no reference picture"	(F-28)
for( i = 0; i < NumPocLtFoll; i++ )
	if( !FollDeltaPocMsbPresentFlag[ i ] )
		if( there is a reference picture picX in the DPB with slice_pic_order_cnt_lsb equal to PocLtFoll[ i ] and nuh_layer_id equal to currPicLayerId)
			RefPicSetLtFoll[ i ] = picX
		else
			RefPicSetLtFoll[ i ] = "no reference picture"
	else
		if( there is a reference picture picX in the DPB with PicOrderCntVal equal to PocLtFoll[ i ] and nuh_layer_id equal to currPicLayerId)
			RefPicSetLtFoll[ i ] = picX
		else
			RefPicSetLtFoll[ i ] = "no reference picture"

- All reference pictures that are included in RefPicSetLtCurr and RefPicSetLtFoll and with nuh_layer_id equal to currPicLayerId are marked as "used for long-term reference".

- The following applies:

for( i = 0; i < NumPocStCurrBefore; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrBefore[ i ] and nuh_layer_id equal to currPicLayerId)
		RefPicSetStCurrBefore[ i ] = picX
	else
		RefPicSetStCurrBefore[ i ] = "no reference picture"

for( i = 0; i < NumPocStCurrAfter; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrAfter[ i ] and nuh_layer_id equal to currPicLayerId)
		RefPicSetStCurrAfter[ i ] = picX
	else
		RefPicSetStCurrAfter[ i ] = "no reference picture"	(F-29)

for( i = 0; i < NumPocStFoll; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStFoll[ i ] and nuh_layer_id equal to currPicLayerId)
		RefPicSetStFoll[ i ] = picX
	else
		RefPicSetStFoll[ i ] = "no reference picture"

- All reference pictures in the DPB that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetStFoll and with nuh_layer_id equal to currPicLayerId are marked as "unused for reference".

NOTE 4 – There may be one or more entries in the RPS lists that are equal to "no reference picture" because the corresponding pictures are not present in the DPB. Entries in RefPicSetStFoll or RefPicSetLtFoll that are equal to "no reference picture" should be ignored. An unintentional picture loss should be inferred for each entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr that is equal to "no reference picture".

NOTE 5 – A picture cannot be included in more than one of the five RPS lists.

It is a requirement of bitstream conformance that the RPS is restricted as follows:

- There shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr for which one or more of the following are true:

  - The entry is equal to "no reference picture".

  - The entry is a sub-layer non-reference picture and has TemporalId equal to that of the current picture.

  - The entry is a picture that has TemporalId greater than that of the current picture.

- There shall be no entry in RefPicSetLtCurr or RefPicSetLtFoll for which the difference between the picture order count value of the current picture and the picture order count value of the entry is greater than or equal to 224.

- When the current picture is a TSA picture, there shall be no picture included in the RPS with TemporalId greater than or equal to the TemporalId of the current picture.

- When the current picture is an STSA picture, there shall be no picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr that has TemporalId equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, an STSA picture that has TemporalId equal to that of the current picture, there shall be no picture that has TemporalId equal to that of the current picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr that precedes the STSA picture in decoding order.

- When the current picture is a CRA picture, there shall be no picture included in the RPS that precedes, in decoding order, any preceding IRAP picture in decoding order (when present).

- When the current picture is a trailing picture, there shall be no picture in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr that was generated by the decoding process for generating unavailable reference pictures as specified in subclause 8.3.3.

- When the current picture is a trailing picture, there shall be no picture in the RPS that precedes the associated IRAP picture in output order or decoding order.

- When the current picture is a RADL picture, there shall be no picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr that is any of the following:

  - A RASL picture

  - A picture that was generated by the decoding process for generating unavailable reference pictures as specified in subclause 8.3.3

  - A picture that precedes the associated IRAP picture in decoding order

- When the sps_temporal_id_nesting_flag is equal to 1, the following applies:

  - Let tIdA be the value of TemporalId of the current picture picA.

Any picture picB with TemporalId equal to tIdB that is less than or equal to tIdA shall not be included in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr of picA when there exists a picture picC that has TemporalId less than tIdB, follows picB in decoding order, and precedes picA in decoding order.

    - Decoding process for coding units coded in intra prediction mode

The specifications in subclause 8.4 apply.

    - Decoding process for coding units coded in inter prediction mode

The specifications in subclause 8.5 apply.

    - Scaling, transformation and array construction process prior to deblocking filter process

The specifications in subclause 8.6 apply.

    - In-loop filter process

The specifications in subclause 8.7 apply.

  - Parsing process

The specifications in clause 9 apply.

  - Specification of bitstream subsets

The specifications in clause 10 apply.

  - (Void)

  - Byte stream format

The specifications in Annex B apply.

  - Hypothetical reference decoder

    - General

The specifications in subclause C.1 apply with the following addition:

An output layer set index TargetOutputLayerSetIdx is selected. The selected output layer set shall refer to the layer set consisting of layers of OpLayerIdList.

    - Operation of coded picture buffer (CPB)

The specifications in subclause C.2 apply with the following modifications.

–	Replace "a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_RADL or BLA_N_LP" with "a BLA access unit for which each coded picture has nal_unit_type equal to BLA_W_RADL or BLA_N_LP".

–	Replace "a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_LP" with "a BLA access unit for which each coded picture has nal_unit_type equal to BLA_W_LP".

–	Replace "picture n" with "access unit n".

–	Replace "AuNominalRemovalTime[ prevNonDiscardablePic ] is the nominal removal time of the preceding picture in decoding order with TemporalId equal to 0 that is not a RASL, RADL or sub-layer non-reference picture", with "AuNominalRemovalTime[ prevNonDiscardablePic ] is the nominal removal time of the preceding access unit in decoding order, each picture of which is with TemporalId equal to 0 that is not a RASL, RADL or sub-layer non-reference picture".

    - Operation of the decoded picture buffer (DPB)

      - General

The specifications in this subclause apply independently to each set of DPB parameters selected as specified in subclause C.1.

The decoded picture buffer contains picture storage buffers. Each of the picture storage buffers may contain a decoded picture that is marked as "used for reference" or is held for future output. The processes specified in subclauses D.13.3.2, D.13.3.3 and D.13.3.4 are sequentially applied as specified below.

Decoded pictures with the same DPB output time and with PicOutputFlag equal to 1 are output in ascending order of nuh_layer_id values of these decoded pictures.

Let picture n be the coded picture or decoded picture of the access unit n for a particular value of nuh_layer_id, wherein n is a non-negative integer number.

      - Removal of pictures from the DPB

The specifications in subclause C.3.2 apply separately for each set of decoded pictures with a particular value of nuh_layer_id with the following modifications.

–	Replace "The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of access unit n (containing the current picture) and proceeds as follows:" with "The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of the picture n and proceeds as follows:".

      - Picture output

The specifications in subclause C.3.3 apply with the following modifications.

–	Replace "The output of the current picture is specified as follows, " with "For each picture of the current access unit, the output of the picture is specified as follows for each picture of the access unit ".

      - Current decoded picture marking and storage

The process specified in this subclause happens instantaneously at the CPB removal time of the last decoding unit of picture n, CpbRemovalTime[ n ].

The current decoded picture is stored in the DPB in an empty picture storage buffer, the DPB fullness is incremented by one, and the current picture is marked as "used for short-term reference".

    - Bitstream conformance

A bitstream of coded data conforming to this Specification shall fulfil all requirements specified in this subclause.

The bitstream shall be constructed according to the syntax, semantics, and constraints specified in this Specification outside of this annex.

The first access unit in a bitstream shall be an IRAP access unit.

The bitstream is tested by the HRD for conformance as specified in subclause C.1.

Let the nuh_layer_id of the current picture be currPicLayerId.

For each current picture, let the variables maxPicOrderCnt and minPicOrderCnt be set equal to the maximum and the minimum, respectively, of the PicOrderCntVal values of the following pictures with nuh_layer_id equal to currPicLayerId:

–	The current picture.

–	The previous picture in decoding order that has TemporalId equal to 0 and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture.

–	The short-term reference pictures in the RPS of the current picture.

–	All pictures n that have PicOutputFlag equal to 1, AuCpbRemovalTime[ n ] less than AuCpbRemovalTime[ currPic ], and DpbOutputTime[ n ] greater than or equal to AuCpbRemovalTime[ currPic ], where currPic is the current picture.

All of the following conditions shall be fulfilled for each of the bitstream conformance tests:

- For each access unit n, with n greater than 0, associated with a buffering period SEI message, let the variable deltaTime90k[ n ] be specified as follows:

deltaTime90k[ n ] = 90000 * ( AuNominalRemovalTime[ n ] − AuFinalArrivalTime[ n − 1 ] )	(F-30)

The value of InitCpbRemovalDelay[ SchedSelIdx ] is constrained as follows:

–	If cbr_flag[ SchedSelIdx ] is equal to 0, the following condition shall be true:

InitCpbRemovalDelay[ SchedSelIdx ]  <=  Ceil( deltaTime90k[ n ] )	(F-31)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1), the following condition shall be true:

Floor( deltaTime90k[ n ] ) <= 	InitCpbRemovalDelay[ SchedSelIdx ] <= Ceil( deltaTime90k[ n ] )	(F-32)

- NOTE 1 – The exact number of bits in the CPB at the removal time of each picture may depend on which buffering period SEI message is selected to initialize the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialize the HRD, as the HRD may be initialized at any one of the buffering period SEI messages.

- A CPB overflow is specified as the condition in which the total number of bits in the CPB is greater than the CPB size. The CPB shall never overflow.

- A CPB underflow is specified as the condition in which the nominal CPB removal time of decoding unit m DuNominalRemovalTime( m ) is less than the final CPB arrival time of decoding unit m DuFinalArrivalTime( m ) for at least one value of m. When low_delay_hrd_flag[ HighestTid ] is equal to 0, the CPB shall never underflow.

- When SubPicHrdFlag is equal to 1, low_delay_hrd_flag[ HighestTid ] is equal to 1, and the nominal removal time of a decoding unit m of access unit n is less than the final CPB arrival time of decoding unit m (i.e. DuNominalRemovalTime[ m ] < DuFinalArrivalTime[ m ]), the nominal removal time of access unit n shall be less than the final CPB arrival time of access unit n (i.e. AuNominalRemovalTime[ n ] < AuFinalArrivalTime[ n ]).

- The nominal removal times of access units from the CPB (starting from the second access unit in decoding order) shall satisfy the constraints on AuNominalRemovalTime[ n ] and AuCpbRemovalTime[ n ] expressed in subclauses A.4.1 through A.4.2.

- For each current picture, after invocation of the process for removal of pictures from the DPB as specified in subclause C.3.2, the number of decoded pictures in the DPB, including all pictures n that are marked as "used for reference", or that have PicOutputFlag equal to 1 and AuCpbRemovalTime[ n ] less than AuCpbRemovalTime[ currPic ], where currPic is the current picture, shall be less than or equal to sps_max_dec_pic_buffering_minus1[ HighestTid ].

- All reference pictures shall be present in the DPB when needed for prediction. Each picture that has PicOutputFlag equal to 1 shall be present in the DPB at its DPB output time unless it is removed from the DPB before its output time by one of the processes specified in subclause C.3.

- For each current picture, the value of maxPicOrderCnt − minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2.

- The value of DpbOutputInterval[ n ] as given by Equation C-17, which is the difference between the output time of an access unit and that of the first access unit following it in output order and having PicOutputFlag equal to 1, shall satisfy the constraint expressed in subclause A.4.1 for the profile, tier and level specified in the bitstream using the decoding process specified in clauses 2 through 10.

- For each current picture, when sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 1, let tmpCpbRemovalDelaySum be derived as follows:

tmpCpbRemovalDelaySum = 0
for( i = 0; i < num_decoding_units_minus1; i++ )	(F-33)
	tmpCpbRemovalDelaySum  +=  du_cpb_removal_delay_increment_minus1[ i ] + 1

The value of ClockSubTick * tmpCpbRemovalDelaySum shall be equal to the difference between the nominal CPB removal time of the current access unit and the nominal CPB removal time of the first decoding unit in the current access unit in decoding order.

    - Decoder conformance

      - General

The specifications in subclause C.5.1 apply.

      - Operation of the output order DPB

        - General

The decoded picture buffer contains picture storage buffers. The number of picture storage buffers for nuh_layer_id equal to 0 is derived from the active SPS. The number of picture storage buffers for each non-zero nuh_layer_id value is derived from the active layer SPS for that non-zero nuh_layer_id value. Each of the picture storage buffers contains a decoded picture that is marked as "used for reference" or is held for future output. The process for output and removal of pictures from the DPB as specified in subclause D.13.5.2.2 is invoked, followed by the invocation of the process for picture decoding, marking, additional bumping, and storage as specified in subclause D.13.5.2.3. The "bumping" process is specified in subclause D.13.5.2.4 and is invoked as specified in subclauses D.13.5.2.2 and D.13.5.2.3.

Let picture n be the coded picture or decoded picture of the access unit n for a particular value of nuh_layer_id, wherein n is a non-negative integer number.

        - Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the current picture is removed from the CPB and proceeds as follows:

The decoding process for RPS as specified in subclause D.8.3.2 is invoked.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers  containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause D.13.5.2.4, and the DPB fullness is set equal to 0.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1 or with nuh_layer_id not equal to 0), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. The variable currLayerId is set equal to nuh_layer_id of the current decoded picture and when one or more of the following conditions are true, the "bumping" process specified in subclause D.13.5.2.4 is invoked repeatedly while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied, until none of the following conditions are true:

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId (when currLayerId is not equal to 0).

–	sps_max_latency_increase_plus1[ HighestTid ] of the active SPS (when currLayerId is equal to 0) or the active layer SPS for the value of currLayerId is not equal to 0 and there is at least one picture with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] is greater than or equal to SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB is greater than or equal to sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

        - Picture decoding, marking, additional bumping, and storage

The processes specified in this subclause happen instantaneously when the last decoding unit of access unit n containing the current picture is removed from the CPB.

The variable currLayerId is set equal to nuh_layer_id of the current decoded picture.

For each picture in the DPB that is marked as "needed for output" and that has a nuh_layer_id value equal to currLayerId, the associated variable PicLatencyCount[ currLayerId ] is set equal to PicLatencyCount[ currLayerId ] + 1.

The current picture is considered as decoded after the last decoding unit of the picture is decoded. The current decoded picture is stored in an empty picture storage buffer in the DPB, and the following applies:

–	If the current decoded picture has PicOutputFlag equal to 1, it is marked as "needed for output" and its associated variable PicLatencyCount[ currLayerId ] is set equal to 0.

–	Otherwise (the current decoded picture has PicOutputFlag equal to 0), it is marked as "not needed for output".

The current decoded picture is marked as "used for short-term reference".

When one or more of the following conditions are true, the "bumping" process specified in subclause D.13.5.2.4 is invoked repeatedly until none of the following conditions are true:

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId, if not equal to 0.

–	sps_max_latency_increase_plus1[ HighestTid ] is not equal to 0 and there is at least one picture with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] that is greater than or equal to SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId (when currLayerId is not equal to 0).

        - "Bumping" process

The "bumping" process consists of the following ordered steps:

- The pictures that are first for output are selected as the ones having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- These pictures are cropped, using the conformance cropping window specified in the active SPS for the picture with nuh_layer_id equal to 0 or in the active layer SPS for a non-zero nuh_layer_id value equal to that of the picture, the cropped pictures are output in ascending order of nuh_layer_id, and the pictures are marked as "not needed for output".

- Each picture storage buffer that contains a picture marked as "unused for reference" and that was one of the pictures cropped and output is emptied.

  - SEI messages

The specifications in Annex D together with the extensions and modifications specified in this subclause apply.

    - SEI message syntax

      - Layers present SEI message syntax

| layers_not_present( payloadSize ) { | Descriptor |
| --- | --- |
| lp_sei_active_vps_id | u(4) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) |  |
| layer_not_present_flag[ i ] | u(1) |
| } |  |

      - Inter-layer constrained tile sets SEI message syntax

| inter_layer_constrained_tile_sets( payloadSize ) { | Descriptor |
| --- | --- |
| il_all_tiles_exact_sample_value_match_flag | u(1) |
| il_one_tile_per_tile_set_flag | u(1) |
| if( !one_tile_per_tile_set_flag ) { |  |
| il_num_sets_in_message_minus1 | ue(v) |
| if( il_num_sets_in_message_minus1 ) |  |
| skipped_tile_set_present_flag | u(1) |
| numSignificantSets = il_num_sets_in_message_minus1 – skipped_tile_set_present_flag + 1 |  |
| for( i = 0; i <= numSignificantSets; i++ ) { |  |
| ilcts_id[ i ] | ue(v) |
| il_num_tile_rects_in_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_tile_rects_in_set_minus1[ i ]; j++ ) { |  |
| il_top_left_tile_index[ i ][ j ] | ue(v) |
| il_bottom_right_tile_index[ i ][ j ] | ue(v) |
| } |  |
| ilc_idc[ i ] | u(2) |
| if ( !il_all_tiles_exact_sample_value_match_flag ) |  |
| il_exact_sample_value_match_flag[ i ] | u(1) |
| } |  |
| } else |  |
| all_tiles_ilc_idc | u(2) |
| } |  |

    - SEI message semantics

Table F-2 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| Layers not present | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains a layers present change SEI message or the end of the CVS, whichever is earlier in decoding order |
| Inter-layer constrained tile sets | - The CVS containing the SEI message |

      - Layers not present SEI message semantics

The layers not present SEI message provides a mechanism for signalling that VCL NAL units of particular layers indicated by the VPS are not present in a particular set of access units.

The target access units are defined as the set of access units starting from the access unit containing the layers not present SEI message up to but not including the next access unit, in decoding order, that contains a layers not present change SEI message or the end of the CVS, whichever is earlier in decoding order.

When present, the layers not present SEI message applies to the target access units.

A layers not present SEI message shall not be included in a scalable nesting SEI message.

A layers not present SEI message shall not be included in an SEI NAL unit with TemporalId greater than 0.

lp_sei_active_vps_id identifies the active VPS of the CVS containing the layers not present SEI message. The value of lp_sei_active_vps_id shall be equal to the value of vps_video_parameter_set_id of the active VPS for the VCL NAL units of the access unit containing the SEI message.

layer_not_present_flag[ i ] equal to 1 indicates that there are no VCL NAL units with nuh_layer_id equal to layer_id_in_nuh[ i ] present in the target access units. layer_not_present_flag[ i ] equal to 0 indicates that there may or may not be VCL NAL units with nuh_layer_id equal to layer_id_in_nuh[ i ] present in the target access units.

When layer_not_present_flag[ i ] is equal to 0 and i is greater than 0, layer_not_present_flag[ LayerIdxInVps[ RefLayerId[ layer_id_in_nuh[ i ] ][ j ] ] ] shall be equal to 0 for all values of j in the range of 0 to NumDirectRefLayers[ layer_id_in_nuh[ i ] ] − 1, inclusive.

      - Inter-layer constrained tile sets SEI message semantics

The scope of the inter-layer constrained tile sets SEI message is the complete CVS. When an inter-layer tile sets SEI message is present in any access unit of a CVS, it shall be present for the first access unit of the CVS in decoding order and may also be present for other access units of the CVS.

The inter-layer constrained tile sets SEI message shall not be present for a layer when tiles_enabled_flag is equal to 0 for any PPS that is active in the layer.

The inter-layer constrained tile sets SEI message shall not be present for a layer unless every PPS that is active for the layer has tile_boundaries_aligned_flag equal to 1 or fulfills the conditions that would be indicated by tile_boundaries_aligned_flag being equal to 1.

The presence of the inter-layer tile sets SEI message indicates that the inter-layer inter prediction process is constrained such that no sample value outside each identified tile set, and no sample value at a fractional sample position that is derived using one or more sample values outside the identified tile set, is used for inter prediction of any sample within the identified tile set.

NOTE 1 – When loop filtering and resampling filter is applied across tile boundaries, inter-layer prediction of any samples within an inter-layer constrained tile set that refers to samples within 8 samples from an inter-layer constrained tile set boundary that is not also a picture boundary may result in propagation of mismatch error. An encoder can avoid such potential error propagation by avoiding the use of motion vectors that cause such references.

When more than one inter-layer constrained tile sets SEI message is present within the access units of a CVS, they shall contain identical content.

The number of inter-layer constrained tile sets SEI messages in each access unit shall not exceed 5.

il_all_tiles_exact_sample_value_match_flag equal to equal to 1 indicates that, within the CVS, when the coding tree blocks that are outside of any identified tile are not decoded and the boundaries of the identified tile is treated as picture boundaries for purposes of the decoding process, the value of each sample in the identified tile would be exactly the same as the value of the sample that would be obtained when all the coding tree blocks of all pictures in the CVS are decoded.  il_all_tiles_exact_sample_value_match_flag equal to 0 indicates that, within the CVS, when the coding tree blocks that are outside of any identified tile are not decoded and the boundaries of the identified tile is treated as picture boundaries for purposes of the decoding process, the value of each sample in the identified tile may or may not be exactly the same as the value of the same sample when all the coding tree blocks of all pictures in the CVS are decoded.

il_one_tile_per_tile_set_flag equal to 1 indicates that each inter-layer constrained tile set contains one tile, and il_num_sets_in_message_minus1 is not present. If il_one_tile_per_tile_set_flag is equal to zero, tile sets are signalled explicitly.

il_num_sets_in_message_minus1 plus 1 specifies the number of inter-layer tile sets identified in the SEI message. The value of il_num_sets_in_message_minus1 shall be in the range of 0 to 255, inclusive.

skipped_tile_set_present_flag equal to 1 indicates that, within the CVS, the tile set consists of those remaining tiles that are not included in any earlier tile sets in the same message and all the prediction blocks that are inside the identified tile set having nuh_layer_id equal to ictsNuhLayerId are inter-layer predicted from inter-layer reference pictures with nuh_layer_id equal to RefLayerId[ ictsNuhLayerId ][ NumDirectRefLayers[ ictsNuhLayerId ] – 1 ] and no residual_coding syntax structure is present in any transform unit of the identified tile set, where ictsNuhLayerId is the value of nuh_layer_id of this message. skipped_tile_set_present_flag equal to 0 does not indicate a bitstream constraint within the CVS.

ilcts_id[ i ] contains an identifying number that may be used to identify the purpose of the i-th identified tile set (for example, to identify an area to be extracted from the coded video sequence for a particular purpose). The value of ilcts_id[ i ] shall be in the range of 0 to 232 − 2, inclusive.

Values of ilcts_id[ i ] from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of ilcts_id[ i ] from 256 to 511 and from 231 to 232 − 2 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of ilcts_id[ i ] in the range of 256 to 511 or in the range of 231 to 232 − 2 shall ignore (remove from the bitstream and discard) it.

il_num_tile_rects_in_set_minus1[ i ] plus 1 specifies the number of rectangular regions of tiles in the i-th identified inter-layer constrained tile set. The value of il_num_tile_rects_in_set_minus1[ i ] shall be in the range of 0 to (num_tile_columns_minus1 + 1) * (num_tile_rows_minus1 + 1) − 1, inclusive.

il_top_left_tile_index[ i ][ j ] and il_bottom_right_tile_index[ i ][ j ] identify the tile position of the top-left tile and the tile position of the bottom-right tile in a rectangular region of the i-th identified inter-layer constrained tile set, respectively, in tile raster scan order.

il_exact_sample_value_match_flag[ i ] equal to 1 indicates that, within the CVS, when the coding tree blocks that do not belong to the inter-layer constrained tile set are not decoded and the boundaries of the inter-layer constrained tile set are treated as picture boundaries for purposes of the decoding process, the value of each sample in the inter-layer constrained tile set would be exactly the same as the value of the sample that would be obtained when all the coding tree blocks of all pictures in the coded video sequence are decoded. il_exact_sample_value_match_flag[ i ] equal to 0 indicates that, within the CVS, when the coding tree blocks that are outside of the i-th identified inter-layer constrained tile set are not decoded and the boundaries of the inter-layer constrained tile set are treated as picture boundaries for purposes of the decoding process, the value of each sample in the identified tile set may or may not be exactly the same as the value of the same sample when all the coding tree blocks of the picture are decoded.

NOTE 2 – It should be feasible to use il_exact_sample_value_match_flag equal to 1 when using certain combinations of loop_filter_across_tiles_enabled_flag, pps_loop_filter_across_slices_enabled_flag, pps_deblocking_filter_disabled_flag, slice_loop_filter_across_slices_enabled_flag, slice_deblocking_filter_disabled_flag, sample_adaptive_offset_enabled_flag, slice_sao_luma_flag, and slice_sao_chroma_flag.

ilc_idc[ i ] equal to 1 indicates that, within the CVS, no samples outside of the i-th identified tile set and no samples at a fractional sample position that is derived using one or more samples outside of the i-th identified tile set are used for inter-layer prediction of any sample within the i-th identified tile set with nuh_layer_id equal to ictsNuhLayerId, where ictsNuhLayerId is the value of nuh_layer_id of this message. ilc_idc[ i ][ j ] equal to 2 indicates that, within the CVS, no prediction block in the i-th identified tile set with nuh_layer_id equal to ictsNuhLayerId is predicted from an inter-layer reference picture. ilc_idc[ i ] equal to 0 indicates that, within the CVS, the inter-layer prediction process may or may not be constrained for the prediction block in the i-th identified tile set having nuh_layer_id equal to ictsNuhLayerId. The value of ilc_idc[ i ] equal to 3 is reserved.

all_tiles_ilc_idc equal to 1 indicates that, within the CVS, no sample value outside of each identified tile and no sample value at a fractional sample position that is derived using one or more samples outside of the identified tile is used for inter-layer prediction of any sample within the identified tile with nuh_layer_id equal to ictsNuhLayerId, where ictsNuhLayerId is the value of nuh_layer_id of this message. all_tiles_ilc_idc equal to 2 indicates that, within the CVS, no prediction block in each identified tile with nuh_layer_id equal to ictsNuhLayerId is predicted from an inter-layer reference picture. all_tiles_ilc_idc equal to 0 indicates that, within the CVS, the inter-layer prediction process may or may not be constrained for the tile having nuh_layer_id equal to ictsNuhLayerId. The value of all_tiles_ilc_idc equal to 3 is reserved.

  - Video usability information

    - General

The specifications in clause E.1 apply.

    - VUI syntax

The specifications in clause E.2 apply.

    - VUI semantics

      - VUI parameters semantics

The specifications in clause E.3.1 apply with the following modifications and additions.

vui_timing_info_present_flag equal to 1 specifies that vui_num_units_in_tick, vui_time_scale, vui_poc_proportional_to_timing_flag, and vui_hrd_parameters_present_flag are present in the vui_parameters( ) syntax structure. vui_timing_info_present_flag equal to 0 specifies that vui_num_units_in_tick, vui_time_scale, vui_poc_proportional_to_timing_flag, and vui_hrd_parameters_present_flag are not present in the vui_parameters( ) syntax structure. It is a requirement of bitstream conformance that, when nuh_layer_id is greater than 0, vui_timing_info_present_flag shall be equal to 0.

      - HRD parameters semantics

The specifications in clause E.3.2 apply.

      - Sub-layer HRD parameters semantics

The specifications in clause E.3.3 apply.

- Annex G

Multiview coding

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies multiview coding that use the syntax, semantics, and decoding processes specified in clauses 2-9 and Annex A-F.

  - Scope

Decoding processes and bitstreams conforming to this annex are completely specified in this annex with reference made to clauses 2-9 and Annexes A-F.

  - Normative references

The specifications in clause 2 apply.

  - Definitions

The specification in clause D.3 apply.

  - Abbreviations

The specification in clause 4 apply.

  - Conventions

The specification in clause 5 apply.

  - Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specification in clause 6 apply.

  - Syntax and semantics

The specification in clause D.7 apply.

  - Decoding processes

    - General decoding process

The specification in subclause D.8.1 applies.

      - Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause E.8.2.

- The processes in subclause E.8.1.2 and E.8.3.4 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Prior to decoding the first slice of the current picture, subclause E.8.1.2 is invoked.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause E.8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

- The processes in subclauses D.8.5, E.8.5, E.8.6, and E.8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause E.8.1.3 is invoked.

      - Decoding process for inter-layer reference picture set

Outputs of this process are updated lists of inter-layer pictures RefPicSetInterLayer0 and RefPicSetInterLayer1 and the variables NumActiveRefLayerPics0 and NumActiveRefLayerPics1.

The lists RefPicSetInterLayer0 and RefPicSetInterLayer1 are first emptied, NumActiveRefLayerPics0 and NumActiveRefLayerPics1 are set equal to 0 and the following applies:

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
										nuh_layer_id equal to RefPicLayerId[ i ] ) {
		if( ( ViewId[ nuh_layer_id ]  <=  ViewId[ 0 ]   &&
									ViewId[ nuh_layer_id ]  <=  ViewId[ RefPicLayerId[ i ] ] )  | |
			( ViewId[ nuh_layer_id ]  >=  ViewId[ 0 ]  &&
									ViewId[ nuh_layer_id ]  >=  ViewId[ RefPicLayerId[ i ] ] ) ) {
			RefPicSetInterLayer0[ NumActiveRefLayerPics0 ] = picX
			RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] is marked as "used for long-term reference"
		} else {
			RefPicSetInterLayer1[ NumActiveRefLayerPics1 ] = picX
			RefPicSetInterLayer1[ NumActiveRefLayerPics1++ ] is marked as "used for long-term reference"
		}
	} else
		RefPicSetInterLayer0[ NumActiveRefLayerPics0++ ] = "no reference picture"
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer0 or RefPicSetInterLayer1.

If the current picture is a RADL picture, there shall be no entry in RefPicSetInterLayer0 or RefPicSetInterLayer1 that is a RASL picture.

NOTE – An access unit may contain both RASL and RADL pictures.

      - Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies:

for( i = 0; i < NumActiveRefLayerPics0; i++ )
	RefPicSetInterLayer0[ i ] is marked as "used for short-term reference"

for( i = 0; i <  NumActiveRefLayerPics1; i++ )
	RefPicSetInterLayer1[ i ] is marked as "used for short-term reference"

    - NAL unit decoding process

The specification in subclause 8.3 apply.

    - Slice decoding processes

      - Decoding process for picture order count

The specifications in subclause D.8.3.1 apply.

      - Decoding process for reference picture set

The specifications in subclause D.8.3.2apply.

      - Decoding process for generating unavailable reference pictures

The specifications in subclause 8.3.3 apply.

      - Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause 8.5.3.3.2. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture lists RefPicList0 and, for B slices, RefPicList1 are derived as follows:

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPicTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumActiveRefLayerPics0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer0[ i ]
	for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )	(G-1)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i < NumActiveRefLayerPics1; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer1[ i ]
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++)			(G-2)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPicTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i< NumActiveRefLayerPics1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer1[ i ]
	for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )	(G-3)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i< NumActiveRefLayerPics0; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer0[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(G-4)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

    - Decoding process for coding units coded in intra prediction mode

The specifications in subclause 8.4 apply.

    - Decoding process for coding units coded in inter prediction mode

The specifications in subclause D.8.5.apply.

    - Scaling, transformation and array construction process prior to deblocking filter process

The specifications in subclause 8.6 apply.

    - In-loop filter process

The specifications in subclause 8.7 apply.

  - Parsing process

The specifications in clause 9 apply.

  - Specification of bitstream subsets

The specifications in clause 10 apply.

  - Profiles, tiers, and levels

    - Profiles

      - General

      - Stereo Main profile

Bitstreams conforming to the Stereo Main profile shall obey the following constraints:

- The sub-bitstream resulting from the sub-bitstream extraction process with any value of tIdTarget and a value of 0 in layerIdListTarget as inputs shall conform to the Main profile.

- The bitstream shall contain one layer with nuh_layer_id equal to i for which ViewScalExtLayerFlag[ i ] is equal to 1.

- When ViewScalExtLayerFlag[ i ] is equal to 1, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_extension( ) syntax structure of the active layer SPS RBSP of any coded pictures with nuh_layer_id equal to i.

- When ViewScalExtLayerFlag[ i ] is equal to 1, ScalabilityId[ LayerIdxInVps [ i ]][ smIdx ] shall be equal to 0 for any smIdx value from 0 to 15, inclusive, that is not equal to 1, for any coded picture with nuh_layer_id equal to i.

- SPSs shall have sps_shvc_reserved_zero_idc equal to 0 only.

- When ViewScalExtLayerFlag[ i ] is equal to 1, the values of pic_width_in_luma_samples and pic_height_in_luma_samples in the active SPS for the layer with nuh_layer_id equal to i shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in the active SPSs of all direct reference layers of that layer.

- The bitstream shall contain a sub-bitstream consisting of two layers having nuh_layer_id equal to 0 and nuhLayerIdA for which ScalabilityId[ LayerIdxInVps [ nuhLayerIdA ]][ smIdx ] shall be equal to 0 for any smIdx from 0 to 15, inclusive that is not equal to 1.

- VPSs shall have avc_base_layer_flag equal to 0 only.

    - Tiers and levels

      - General tier and level limits

The specifications in A.4.1 apply with the following modifications.

Replace item d) by the following:

- The value of sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 shall be less than or equal to MaxDpbSize, which is derived as follows:

if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf, 16 )
else if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, 16 )	(G-5)
else if( PicSizeInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxDpbSize = maxDpbPicBuf

where MaxLumaPs is specified in Table A-1 and maxDpbPicBuf is equal to 6.

      - Profile-specific level limits for the Main, Main 10, and Stereo Main profiles

The specifications in A.4.1 apply with the following modifications.

Replace item b) by the following:

- The difference between consecutive output times of pictures of the same layer from the DPB, as specified in subclause C.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( PicSizeInSamplesY  MaxLumaSr, fR ) for the value of PicSizeInSamplesY of picture n, where MaxLumaSr is the value specified in Table A-2 for picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

  - Byte stream format

The specifications in subclause D.12 apply.

  - Hypothetical reference decoder

The specifications in subclause D.13 and its subclauses apply.

  - SEI messages

The specifications in Annex D and subclause D.14 together with the extensions and modifications specified in this subclause apply.

    - SEI message syntax

      - 3D reference displays information SEI message syntax

| three_dimensional_reference_displays_info( payloadSize ) { | Descriptor |
| --- | --- |
| prec_ref_display_width | ue(v) |
| ref_viewing_distance_flag | u(1) |
| if( ref_viewing_distance_flag ) |  |
| prec_ref_viewing_dist | ue(v) |
| num_ref_displays_minus1 | ue(v) |
| for( i = 0; i <= num_ref_displays_minus1; i++ ) { |  |
| left_view_id[ i ] | ue(v) |
| right_view_id[ i ] | ue(v) |
| exponent_ref_display_width[ i ] | u(6) |
| mantissa_ref_display_width[ i ] | u(v) |
| if( ref_viewing_distance_flag ) { |  |
| exponent_ref_viewing_distance[ i ] | u(6) |
| mantissa_ref_viewing_distance[ i ] | u(v) |
| } |  |
| additional_shift_present_flag[ i ] | u(1) |
| if( additional_shift_present_flag[ i ] ) |  |
| num_sample_shift_plus512[ i ] | u(10) |
| } |  |
| three_dimensional_reference_displays_extension_flag | u(1) |
| } |  |

    - SEI message semantics

Table G-3 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| 3D reference displays information | - The access unit containing the SEI message and up to but not including the next access unit, in both decoding and displaying order, that contains the SEI message |

      - 3D reference displays information SEI message semantics

A 3D reference displays information SEI message contains information about the reference display width(s) and reference viewing distance(s) as well as information about the corresponding reference stereo-pair(s) i.e. the pair(s) of views to be displayed for the viewer's left and right eyes on the reference display at the reference viewing distance. This information enables a view renderer to generate a proper stereo-pair for the target screen width and the viewing distance. The reference display width and viewing distance values are signalled in units of centimetres. The reference pair of view specified in this SEI message can be used to extract or infer parameters related to the distance between the camera centers in the reference stereo-pair, which can be used for generation of views for the target display. For multi-view displays, the reference stereo-pair corresponds to a pair of views that can be simultaneously observed by the viewer's left and right eyes.

When present, this SEI message shall be associated an IRAP access unit or with a non-IRAP access unit, when all access units that follow this access unit in the decoding order, also follow it in the display order. The 3D reference display information SEI message should be applied for the access unit, it is associated with and the access units which follow this access unit in both the output and decoding order until the next IRAP access unit or the next access unit containing a 3D reference displays information SEI message.

NOTE 1  – The 3D reference displays information SEI message specifies display parameters for which the 3D sequence was optimized and the corresponding reference parameters. Each reference display (i.e. a reference display width and possibly a corresponding viewing distance) is associated with one reference pair of views by signalling their ViewId. The difference between the values of ViewId is referred to as the baseline distance (i.e. the distance between the centers of the cameras used to obtain the video sequence).

The following equations can be used for determining the baseline distance and horizontal shift for the receiver's display when the ratio between the receiver's viewing distance and the reference viewing distance is the same as the ratio between the receiver screen width and the reference screen width:

baseline[ i ]  = refBaseline[ i ] * ( refDisplayWidth[ i ] ÷ displayWidth )

shift[ i ] = refShift[ i ] * ( refDisplayWidth[ i ] ÷ displayWidth )

where refBaseline[ i ] is equal to right_view_id[ i ] − left_view_id[ i ] signalled in this SEI message. Other parameters related to the view generation may be obtained determined by using a similar equation.

parameter[ i ] = refParameter[ i ] * ( refDisplayWidth[ i ] ÷ displayWidth )

where refParameter[ i ] is a parameter related to view generation that corresponds to the reference pair of views signalled by left_view_id[ i ] and right_view_id[ i ]. In the above equations, the width of the visible part of the display used for showing the video sequence should be understood under "display width". The same equations can also be used for determining the pair of views and horizontal shift and/or other view synthesis parameters when the viewing distance is not scaled proportionally to the screen width compared to the reference display parameters. In this case, the effect of applying  above equations would be to keep the perceived depth in the same proportion to the viewing distance as in the reference setup.

When the view synthesis related parameters that correspond to the reference stereo-pair change from one access unit to another, they should be scaled with the same scaling factor as the parameters in the access unit that the SEI message is associated with. Therefore, the above equation should also be applied to obtain the parameters for a following access unit, where the refParameter is the parameter related to the reference stereo-pair associated the following access unit.

The horizontal shift for the receiver's display should also be modified by scaling it with the same factor as that used to scale the baseline distance (or other view synthesis parameters).

prec_ref_display_width specifies the exponent of the maximum allowable truncation error for refDisplayWidth[ i ] as given by 2−prec_ref_display_width. The value of prec_ref_display_width shall be in the range of 0 to 31, inclusive.

ref_viewing_distance_flag equal to 1 indicates the presence of reference viewing distance. ref_viewing_distance_flag equal to 0 indicates that the reference viewing distance is not present in the bitstream.

prec_ref_viewing_dist specifies the exponent of the maximum allowable truncation error for refViewingDist[ i ] as given by 2−prec_ref_viewing_dist. The value of prec_ref_viewing_dist shall be in the range of 0 to 31, inclusive.

num_ref_displays_minus1 plus 1 specifies the number of reference displays that are signalled in the bitstream. The value of num_ref_displays_minus1 shall be in the range of 0 to 31, inclusive.

left_view_id[ i ] indicates the ViewId of the left view of a stereo-pair corresponding to the i-th reference display.

right_view_id[ i ] indicates the ViewId of the right view of a stereo-pair corresponding to the i-th reference display.

exponent_ref_display_width[ i ] specifies the exponent part of the reference display width of the i-th reference display. The value of exponent_ref_display_width[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified reference display width.

mantissa_ref_display_width[ i ] specifies the mantissa part of the reference display width of the i-th reference display. The variable refDispWidthBits specifying the number of bits of the mantissa_ref_display_width[ i ] syntax element is derived as follows:

- If exponent_ref_display_width[ i ] is equal to 0, refDispWidthBits is set equal to Max( 0, prec_ref_display_width − 30 ).

- Otherwise ( 0 < exponent_ref_display_width[ i ] < 63 ), refDispWidthBits is set equal to Max( 0, exponent_ref_display_width[ i ] + prec_ref_display_width − 31 ).

exponent_ref_viewing_distance[ i ] specifies the exponent part of the reference viewing distance of the i-th reference display. The value of exponent_ref_viewing_distance[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified reference display width.

mantissa_ref_viewing_distance[ i ] specifies the mantissa part of the reference viewing distance of the i-th reference display. The variable refViewDistBits specifying the number of bits of the mantissa_ref_viewing_distance[ i ] syntax element is derived as follows:

- If exponent_ref_viewing_distance[ i ] is equal to 0, the refViewDistBits is set equal to Max( 0, prec_ref_viewing_distance − 30 ).

- Otherwise ( 0 < exponent_ref_viewing_distance[ i ] < 63 ), refViewDistBits is set equal to Max( 0, exponent_ref_viewing_distance[ i ] + prec_ref_viewing_distance − 31 ).

The variables in the x row of Table G-4 are derived as follows from the respective variables or values in the e, n, and v rows of Table G-4 as follows:

- If e is not equal to 0, the following applies:

x = 2(e−31) * ( 1 + n ÷2v )	(G-6)

- Otherwise (e is equal to 0), the following applies:

x = 2−( 30+v ) * n	(G-7)

NOTE 2 – The above specification is similar to that found in IEC 60559 60559:1989, Binary floating-point arithmetic for microprocessor systems.

Table G-4 – Association between camera parameter variables and syntax elements

| x | refDisplayWidth[ i ] | refViewingDistance[ i ] |
| --- | --- | --- |
| e | exponent_ref_display_width[ i ] | exponent_ref_viewing_distance[ i ] |
| n | mantissa_ref_display_width[ i ] | mantissa_ref_viewing_distance[ i ] |
| v | refDispWidthBits | refViewDistBits |

additional_shift_present_flag[ i ] equal to 1 indicates that the information about additional horizontal shift of the left and right views for the i-th reference display is present in the bitstream. additional_shift_present_flag[ i ] equal to 0 indicates that the information about additional horizontal shift of the left and right views for the i-th reference display is not present in the bitstream.

num_sample_shift_plus512[ i ] indicates the recommended additional horizontal shift for a stereo-pair corresponding to the i-th reference baseline and the i-th reference display.

- If ( num_sample_shift_plus512[ i ] − 512 ) is less than 0, it is recommended that the left view of the stereo-pair corresponding to the i-th reference baseline and the i-th reference display is shifted in the left direction by ( 512 − num_sample_shift_plus512[ i ] ) samples with respect to the right view of the stereo-pair.

- Otherwise, if num_sample_shift_plus512[ i ] is equal to 512, it is recommended that shifting is not applied.

- Otherwise, ( ( num_sample_shift_plus512[ i ] − 512 ) is greater than 0 ), it is recommended that the left view in the stereo-pair corresponding to the i-th reference baseline and the i-th reference display should be shifted in the right direction by ( 512 − num_sample_shift_plus512[ i ] ) samples with respect to the right view of the stereo-pair.

The value of num_sample_shift_plus512[ i ] shall be in the range of 0 to 1023, inclusive.

three_dimensional_reference_displays_extension_flag equal to 0 indicates that no additional data follows within the reference displays SEI message. The value of three_dimensional_reference_displays_extension_flag shall be equal to 0  in bitstreams conforming to this version of this Specification. The value of 1 for three_dimensional_reference_displays_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follows the value of 1 for three_dimensional_reference_displays_extension_flag in a reference displays SEI message.

NOTE 3  – Shifting the left view in the left (or right) direction by x samples with respect to the right view can be performed by the following two-step processing:

- Shift the left view by x/2 samples in the left (or right) direction, and shift the right view by x/2 samples in the right (or left) direction.

- Fill the left and right image margins of x/2 samples in width in both the left and right views in background colour.

The following pseudo code explains the recommended shifting processing in the case of shifting the left view in the left direction by x samples with respect to the right view.

for( i = x/2; i < width − x/2; i++ )
		for( j=0;  j < height; j++ ) {
			leftView[ j ][ i ] = leftView[ j ][ i + x/2 ]
			rightView[ j ][ width − 1 − i ] = rightView[ j ][ width − 1 − i − x/2 ]
		}
for( i = 0; i < x/2; i++)
		for( j = 0; j < height; j++ ) {
			leftView[ j ][ width − 1 − i ] = leftView[ j ][ i ] = backgroundColour
			rightView[ j ][ width − 1 − i ] = rightView[ j ][ i ] = backgroundColour
	}

The following pseudo code explains the recommended shifting processing in the case of shifting the left view in the right direction by x samples with respect to the right view.

for( i = x/2; i < width − x/2; i++ )
		for( j = 0; j < height; j++ ){
			leftView[ j ][ width − 1 − i ] = leftView[ j ][ width − 1 − i − x/2 ]
			rightView[ j ][ i ] = rightView[ j ][ i + x/2 ]
	}
for( i=0; i < x/2; i++ )
		for( j = 0; j < height; j++ ) {
			leftView[ j ][ width − 1 − i ] = leftView[ j ][ i ] = backgroundColour
			rightView[ j ][ width − 1 − i ] = rightView[ j ][ i ] = backgroundColour
	}

The variable backgroundColour may take different values in different systems, for example black or gray.

  - Video usability information

The specifications in clause D.15 apply.