| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0166 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0060 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On non-HEVC base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Discussion |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution discusses the need of having a bitstream format that encapsulates a non-HEVC-coded base layer and MV-HEVC/SHVC enhancement layers and asserts that no such need is envisioned. The contribution proposes to use external means to provide decoded base-layer pictures for the enhancement layer decoding process. Furthermore, the contribution proposes to provide certain DPB-related properties of non-HEVC-coded base-layer pictures either through external means or using a specific NAL unit within the HEVC bitstream.

As the changes are asserted to be substantial and may require verification by both expert review and software implementation, the contribution has been submitted for discussion.

# Introduction

## On the need of bitstream format for hybrid codec scalability

It is asserted that in the following common transport mechanisms and file formats there are mechanisms to transmit or store the base layer separately from the enhancement layer(s):

- ISO Base Media File Format (ISOBMFF): a non-HEVC-coded base layer can be stored as a track (e.g. of sample entry type ‘avc1’), while the enhancement layer(s) can be stored as another track which is linked to the base-layer track using so-called track references.

- Real-time Transport Protocol (RTP): either RTP session multiplexing or synchronization source (SSRC) multiplexing can be used to logically separate different layers. See e.g. IETF Internet Draft draft-westerlund-avtcore-multiplex-architecture-03 for further details.

- MPEG-2 transport stream (TS): Each layer can have a different PID value. See e.g. ISO/IEC 13818-1:2007 Amd. 3 for details on SVC carriage over MPEG-2 TS.

It is further asserted that storing a non-HEVC-coded base layer separately from HEVC-extension-coded enhancement layer(s) in the above-mentioned systems makes the signaling of properties of layers easier (as systems usually assume one coding format being used per a logical channel) and does not cause substantial disadvantages. It is therefore concluded in this contribution that a bitstream format encapsulating a non-HEVC-coded base layer and HEVC-extension-coded enhancement layers is not necessary for the above-mentioned transport mechanisms and file formats.

## Decoded non-HEVC-coded base-layer pictures in the MV-HEVC/SHVC DPB

It is asserted that in the case of hybrid codec scalability (the base layer being non-HEVC-coded):

- The decoded pictures of the non-HEVC-coded base layer have to be provided as reference for inter-layer prediction to the HEVC-extension (SHVC or MV-HEVC) decoder immediately after their decoding without considering the DPB delays specified for the base layer (e.g. in the AVC bitstream). Otherwise, the delay of the decoding system could be unreasonably high.

- Each decoded non-HEVC-coded base-layer picture has to be considered in the MV-HEVC/SHVC DPB similarly to a decoded HEVC-coded base-layer picture, as the output of decoded pictures is synchronized across layers (i.e. all pictures of the same access unit are output at the same time). It seems unreasonable to attempt to constrain the output times of non-HEVC-coded base-layer pictures (according to the base-layer specification, such as AVC) to be identical to the output times of the respective enhancement layer pictures. Hence, conceptually two separate copies of each decoded base-layer picture appear in the non-HEVC base-layer decoder/HRD and in the HEVC-extension decoder/HRD.

It is asserted that in order for the DPB of SHVC/MV-HEVC to operate correctly in the case of hybrid codec scalability, the base layer pictures shall be at least conceptually included in the DPB operation of the scalable bitstream and be assigned the following properties:

- NoOutputOfPriorPicsFlag

- PicOutputFlag

- PicOrderCntVal

- Reference picture set

It is asserted that these properties are needed for the base layer because the DPB for the base layer cannot operate independently of the other layers due to the following reasons:

- The base layer has output characteristics that may be related to the other layers, e.g.:

  - In some operation points, the base layer may be among the output layers, in some the base layer might not be among output layers.

  - The output of a base layer picture in a certain access unit should be synchronized with the output of the pictures of other layers in the same access.

  - The base layer pictures may be assigned or it may be desirable to assign the base layer information that is specific to the output operation, such as no_output_of_prior_pics_flag and pic_output_flag.

- The marking of base layer pictures as "unused for reference" shall not precede using the base layer picture as an inter-layer reference picture. (In H.264/AVC, a picture can be marked as "unused for reference" immediately after its decoding.)

- The marking of base layer pictures as "used for reference" and "unused for reference" affects the emptying of picture storage buffers and hence the total memory use of the DPB.

The contribution therefore proposes to assign non-HEVC-coded base layer pictures the above-mentioned properties. It is proposed that the assignment can be made through external means or through indicating the properties in specific NAL units in the HEVC bitstream.

# Overview of the Proposal

The contribution includes the following proposals:

- Removing avc_base_layer_flag from the VPS extension and including instead:

  - non_hevc_base_layer_flag specifying whether the base layer is HEVC-coded

  - base_layer_prop_present_in_bitstream_flag specifying whether the DPB related properties of non-HEVC-coded base layer are present in the bitstream (and encapsulated in specific NAL units as outlined below) or whether these properties are provided by external means

- A new NAL unit types, NON_HEVC_VCL, containing the DPB related properties (a part of the slice segment header as outlined below) of a non-HEVC-coded base-layer picture.

- The slice segment header is split into two parts, the generic part applicable to HEVC-coded and non-HEVC-coded slices and the HEVC-specific part. The generic part includes:

  - Slice address: first_slice_in_pic_flag and slice_segment_address

  - Output related syntax elements: no_output_of_prior_pics_flag, pic_output_flag

  - Generic syntax elements: slice_pic_parameter_set_id, dependent_slice_segment_flag, extra_slice_header_bits, slice_type, colour_plane_id

  - Picture order count related syntax elements

  - Reference picture set related syntax elements

- When non_hevc_base_layer_flag is equal to 1, the decoded pictures are provided by external means and the decoded picture is included in the DPB.

- If base_layer_prop_present_in_bitstream is equal to 0, syntax element and variable values (nal_unit_type, TemporalId, NoOuputOfPriorPicsFlag, PicOutputFlag, reference picture set) associated with the non-HEVC-coded base layer picture are provided by external means. Otherwise, the NON_HEVC_VCL NAL unit is decoded to obtain these syntax elements and variables.

The detailed proposal is provided with the accompanying document including the change-marked specification text.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).